3GPP TSG-RAN WG2 Meeting #131	R2-2505633

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.9.4

Source:	Nokia, Nokia Shanghai Bell, Google, Huawei

Title:	On support of inter-cell PWS reception for NB-IoT NTN

WID/SID:	IoT_NTN_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The R19 WID for IoT NTN has been updated in RAN plenary#105 meeting to cover the objective for support of PWS (Public Warning System). In detail, the support of PWS messages for NB-IoT is expected to reuse the LTE mechanisms.

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In RAN2-130 meetings, it was agreed to further discuss the support of continued reception of PWS segmentation of a message from different cells in intra-gNB scenario.  Issue RRC-3 was identified for this in the post-meeting email discussion.

RAN2-130 Agreements:

1.	A UE should be able to camp on an NB-IoT cell (at least for NTN) for the purpose of receiving PWS broadcast, without the cell being a part of UEs PLMNs. Come back in the next meeting to discuss the exact solution to make this possible (i.e. not necessarily introducing an acceptable cell category for NB-IoT)

2.	Regarding support continued reception of PWS segmentation of a message from different cells, can come back in the next meeting with an actual TP. In any case we will not target other scenarios than the intra-gNB case

In this contribution, the sourcing companies elaborate more on the support of PWS segmentation in the inter-cell intra-eNB scenario with an actual TP.

# 2	Discussion

## 2.1	Inter-cell PWS segmentation reception (open issue RRC-3)

According to current TS36.331 specification, the PWS message may be segmented to accomodate a subset of the message information within the SIB (which has size limitations). However, for the UE receiving the PWS message, the segment feature is only supported for the intra-cell case. i.e., if the RRC Idle UE enters a new cell, the UE shall discard any previously buffered warning message segment(s).

#### 36.331: 5.2.2.4 System information acquisition by the UE … 1> if the UE is ETWS capable: 2> upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment: 3> discard any previously buffered warningMessageSegment; 3> clear, if any, the current values of messageIdentifier and serialNumber for SystemInformationBlockType11;

Observation 1: In legacy, the PWS message segmentation feature is only supported for the intra-cell scenario.

Different from TN, in NTN, the cell (satellite) is moving hence the UE may experience cell change frequently. For Earth-Moving cell (EMC), a UE may experience a cell change every ~6 seconds as indicated in TR38.821 Table 7.3.2.1.4-1. Therefore, the NW may not manage to broadcast all segments of a PWS message in the current NTN serving cell to a UE due to the large PWS message size (and corresponding huge number of segments) and limited cell availability time (~6 seconds). This may cause the UE cannot receive all the segments for the PWS message if the serving cell is changed before the UE receives all the segments.

Furthermore, whether UE can receive all the segments of a PWS message also depends on when the NW starts to broadcast the segments and the UE’s location. The satellite/eNB may start to broadcast PWS message after it receives the PWS message from CN. However, as illustrated in Figure 1, when the satellite/eNB starts to broadcast PWS segments at current moment, the UE located in the cell coverage edge cannot receive all the segments from current serving cell since its cell coverage will be moved out very soon. Similarly, in an Earth Fixed Cell scenario, the UE may also start the PWS message segment acquisition in one cell, but then experience a cell change (according to t-service).

Figure 1: The UE may not receive all segments from current (earth moving) serving cell

If the legacy mechanism of discarding buffered PWS segments upon cell change is followed, there might be the risk of non-succesful delivery of PWS message due to frequent cell change in NTN scenario.

Observation 2: In NTN, all the segments of PWS message may not be delivered to UE successfully within a single cell.

In our understanding, due to the frequent cell change in NTN and the short serving time duration of a NTN cell, it is not a corner case where UE cannot receive all the segments of a PWS message from a single cell. Therefore, we propose to support the continued reception of PWS segmentation of a message from different cells.

As discussed in RAN2-130 meeting, to limit network complexity and avoid the impact to other WGs, it is beneficial to limit the continued reception to different cells provided by the same eNB. By doing this the network will avoid coordination and information sharing between eNBs thereby avoiding any impact to other WGs.

Proposal 1: RAN2 to confirm support for continued reception of PWS message segments from different cells provided by the same eNB (inter-cell, intra-eNB case).

The PWS information (SIB11 and SIB12) already includes a message identifier, serial number and segment number. By checking the message identifier and serial number in PWS SIBs, the UE would know whether the segments are from the same PWS message hence it can be assembled or not. In other words, the same pair means the same PWS message and the segments can be assembled even if the segments are received from different cells.

SystemInformationBlockType11 information element -- ASN1START SystemInformationBlockType11 ::= SEQUENCE { messageIdentifier BIT STRING (SIZE (16)), serialNumber BIT STRING (SIZE (16)), warningMessageSegmentType ENUMERATED {notLastSegment, lastSegment}, warningMessageSegmentNumber INTEGER (0..63), warningMessageSegment OCTET STRING, dataCodingScheme OCTET STRING (SIZE (1)) OPTIONAL, -- Cond Segment1 ..., lateNonCriticalExtension OCTET STRING OPTIONAL } -- ASN1STOP

To support the inter-cell segments reception, the eNB should broadcast a new flag to inform UE that the new function is enabled by network. Hence the UE should not discard the segments as in legacy operation when it changes the serving cell. The indication/flag can be provided in the SIB31-NB.

Proposal 2: Introduce a new indication in SIB for inter-cell PWS segmentation reception.

To support the intra-eNB aspect, the UE shall be able to determine whether two cells are provided by the same eNB.

Observation 3: The UE must be able to determine the eNB identity of the current cell to facilitate inter-cell intra-eNB PWS segmentation reception.

Today, the SIB1-NB includes the 28-bit CellIdentity information element, which can unambiguously identify a cell within a PLMN.

SystemInformationBlockType1-NB message -- ASN1START SystemInformationBlockType1-NB ::= SEQUENCE { hyperSFN-MSB-r13 BIT STRING (SIZE (8)), cellAccessRelatedInfo-r13 SEQUENCE { plmn-IdentityList-r13 PLMN-IdentityList-NB-r13, trackingAreaCode-r13 TrackingAreaCode, cellIdentity-r13 CellIdentity, cellBarred-r13 ENUMERATED {barred, notBarred}, intraFreqReselection-r13 ENUMERATED {allowed, notAllowed} }, #### – CellIdentity The IE CellIdentity is used to unambiguously identify a cell within a PLMN. CellIdentity information element -- ASN1START CellIdentity ::= BIT STRING (SIZE (28)) -- ASN1STOP

The CellIdentity information is defined in TS 36.413 as follows:

#### 9.2.1.37 Global eNB ID This information element is used to globally identify an eNB (see TS 36.401 [2]). IE/Group Name; Presence; Range; IE type and reference; Semantics description. PLMN Identity; M; 9.2.3.8. CHOICE eNB ID; M. >Macro eNB ID. >>Macro eNB ID; M; BIT STRING (SIZE(20)); Equal to the 20 leftmost bits of the Cell Identity IE contained in the E-UTRAN CGI IE (see subclause 9.2.1.38) of each cell served by the eNB.. >Home eNB ID. >>Home eNB ID; M; BIT STRING (SIZE(28)); Equal to the Cell Identity IE contained in the E-UTRAN CGI IE (see subclause 9.2.1.38) of the cell served by the eNB.. >Short Macro eNB ID. >> Short Macro eNB ID; M; BIT STRING (SIZE(18)); Equal to the 18 leftmost bits of the Cell Identity IE (see subclause 9.2.1.38) of each cell served by the eNB.. >Long Macro eNB ID. >> Long Macro eNB ID; M; BIT STRING (SIZE(21)); Equal to the 21 leftmost bits of the Cell Identity IE (see subclause 9.2.1.38) of each cell served by the eNB..

Thus, the CellIdentity may consist of either a “legacy” Macro eNB ID (20 leftmost bits), a short Macro eNB ID (18 leftmost bits) or a long Macro eNB ID (21 leftmost bits) for NTN eNB, with the remaining bits used for the local cell ID within that eNB. Currently, the UE is not aware whether the “legacy”, short or the long version is used for eNB ID and therefore the UE cannot unambiguously determine whether two cells originate from the same eNB. The reason is that the “legacy”, long and short eNB ID indications are not conveyed to the UE, but only the CellIdentity IE in SIB1-NB.

We therefore propose RAN2 discusses whether an indication of eNB ID length can be introduced in system information to indicate whether the macro eNB ID, the short or the long macro eNB ID is used. The information can be provided in SIB1-NB, SIB31-NB or the relevant PWS SIB with our preference for the SIB31-NB because it can then apply to any of the PWS SIBs and without enlarging the SIB1-NB. It is worth noting that this feature is already supported in NR (TS 38.331, section 5.2.2.4.10) where the gNB-ID-Length can be used to indicate which bits in the cellIdentity provide the gNB ID.

Observation 4: The TS 38.331 specification includes gNB-ID-Length that defines which bits in CellIdentity provide the gNB ID.

Proposal 3: Specify eNB-ID-Length in SIB31-NB to indicate the number of bits used for providing the eNB ID in the CellIdentity.

Assuming eNB-ID-Length is provided in SIB31-NB, the TP is provided in the following Text Proposal section.

Proposal 4: RAN2 to discuss the TP to support continued reception of PWS message segments in inter-cell intra-eNB scenario.

# 3	Conclusion

This document has made the following observations:

Observation 1: In legacy, the PWS message segmentation feature is only supported for the intra-cell scenario.

Observation 2: In NTN, all the segments of PWS message may not be delivered to UE successfully within a single cell.

Observation 3: The UE must be able to determine the eNB identity of the current cell to facilitate inter-cell intra-eNB PWS segmentation reception.

Observation 4: The TS 38.331 specification includes gNB-ID-Length that defines which bits in CellIdentity provide the gNB ID.

And proposed the following:

Proposal 1: RAN2 to confirm support for continued reception of PWS message segments from different cells provided by the same eNB (inter-cell, intra-eNB case).

Proposal 2: Introduce a new indication in SIB for inter-cell PWS segmentation reception.

Proposal 3: Specify eNB-ID-Length in SIB31-NB to indicate the number of bits used for providing the eNB ID in the CellIdentity.

Proposal 4: RAN2 to discuss the TP to support continued reception of PWS message segments in inter-cell intra-eNB scenario.

# Text Proposal

#### 5.2.2.4	System information acquisition by the UE

The UE shall:

1>	apply the specified BCCH configuration defined in 9.1.1.1 or BR-BCCH configuration defined in 9.1.1.8;

1>	if the procedure is triggered by a system information change notification:

2>	if the UE uses an idle DRX cycle longer than the modification period:

3>	start acquiring the required system information, as defined in 5.2.2.3, from the next eDRX acquisition period boundary;

2>	else

3>	start acquiring the required system information, as defined in 5.2.2.3, from the beginning of the modification period following the one in which the change notification was received;

NOTE 1:	The UE continues using the previously received system information until the new system information has been acquired.

1>	if the UE is in RRC_IDLE and enters a cell for which the UE does not have stored a valid version of the system information required in RRC_IDLE, as defined in 5.2.2.3:

2>	acquire, using the system information acquisition procedure as defined in 5.2.3, the system information required in RRC_IDLE, as defined in 5.2.2.3;

1>	following successful handover completion to a PCell for which the UE does not have stored a valid version of the system information required in RRC_CONNECTED, as defined in 5.2.2.3:

2>	acquire, using the system information acquisition procedure as defined in 5.2.3, the system information required in RRC_CONNECTED, as defined in 5.2.2.3;

2>	upon acquiring the concerned system information:

3>	discard the corresponding radio resource configuration information included in the radioResourceConfigCommon previously received in a dedicated message, if any;

1>	following a request from CDMA2000 upper layers:

2>	acquire SystemInformationBlockType8, as defined in 5.2.3;

1>	neither initiate the RRC connection establishment/resume procedure nor initiate transmission of the RRCConnectionReestablishmentRequest message until the UE has a valid version of the MasterInformationBlock (MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB in NB-IoT) and SystemInformationBlockType1 (SystemInformationBlockType1-NB in NB-IoT) messages as well as SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT), and for NB-IoT, SystemInformationBlockType22-NB;

1>	not initiate the RRC connection establishment/resume procedure subject to EAB until the UE has a valid version of SystemInformationBlockType14, if broadcast;

1>	if the UE is ETWS capable:

2>	upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment:

3>	if the UE is not connected to NTN:

4>	discard any previously buffered warningMessageSegment;

4>	clear, if any, the current values of messageIdentifier and serialNumber for SystemInformationBlockType11;

2>	when the UE acquires SystemInformationBlockType1 following ETWS indication, upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment:

3>	if schedulingInfoList indicates that SystemInformationBlockType10 is present:

4>	if the UE is in CE:

5>	start acquiring SystemInformationBlockType10;

4>	else

5>	start acquiring SystemInformationBlockType10 immediately;

3>	if schedulingInfoList indicates that SystemInformationBlockType11 is present:

4>	start acquiring SystemInformationBlockType11 immediately;

NOTE 2:	UEs shall start acquiring SystemInformationBlockType10 and SystemInformationBlockType11 as described above even when systemInfoValueTag in SystemInformationBlockType1 has not changed.

1>	if the UE is CMAS capable:

2>	upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment:

3>	if the UE is not connected to NTN:

4>	discard any previously buffered warningMessageSegment;

4>	clear, if any, stored values of messageIdentifier and serialNumber for SystemInformationBlockType12 associated with the discarded warningMessageSegment;

2>	when the UE acquires SystemInformationBlockType1 following CMAS indication, upon entering a cell during RRC_IDLE, following successful handover and upon connection re-establishment:

3>	if schedulingInfoList indicates that SystemInformationBlockType12 is present:

4>	acquire SystemInformationBlockType12;

NOTE 3:	UEs shall start acquiring SystemInformationBlockType12 as described above even when systemInfoValueTag in SystemInformationBlockType1 has not changed.

1>	if the UE is interested to receive MBMS services:

2>	if the UE is capable of MBMS reception as specified in 5.8:

3>	if schedulingInfoList indicates that SystemInformationBlockType13 is present and the UE does not have stored a valid version of this system information block:

4>	acquire SystemInformationBlockType13;

3>	else if SystemInformationBlockType13 is present in SystemInformationBlockType1-MBMS and the UE does not have stored a valid version of this system information block:

4>	acquire SystemInformationBlockType13 from SystemInformationBlockType1-MBMS;

2>	if the UE is capable of SC-PTM reception as specified in 5.8a:

3>	if schedulingInfoList indicates that SystemInformationBlockType20 (SystemInformationBlockType20-NB in NB-IoT) is present and the UE does not have stored a valid version of this system information block:

4>	acquire SystemInformationBlockType20 (SystemInformationBlockType20-NB in NB-IoT);

2>	if the UE is capable of MBMS Service Continuity:

3>	if schedulingInfoList indicates that SystemInformationBlockType15 (SystemInformationBlockType15-NB in NB-IoT) is present and the UE does not have stored a valid version of this system information block:

4>	acquire SystemInformationBlockType15 (SystemInformationBlockType15-NB in NB-IoT);

1>	if the UE is EAB capable:

2>	when the UE does not have stored a valid version of SystemInformationBlockType14 upon entering RRC_IDLE, or when the UE acquires SystemInformationBlockType1 following EAB parameters change notification, or upon entering a cell during RRC_IDLE, or before establishing an RRC connection if using eDRX with DRX cycle longer than the modification period:

3>	if schedulingInfoList indicates that SystemInformationBlockType14 is present:

4>	start acquiring SystemInformationBlockType14 immediately;

3>	else:

4>	discard SystemInformationBlockType14, if previously received;

NOTE 4:	EAB capable UEs start acquiring SystemInformationBlockType14 as described above even when systemInfoValueTag in SystemInformationBlockType1 has not changed.

NOTE 5:	EAB capable UEs maintain an up to date SystemInformationBlockType14 in RRC_IDLE.

1>	if the UE is capable of sidelink communication and is configured by upper layers to receive or transmit sidelink communication:

2>	if the cell used for sidelink communication meets the S-criteria as defined in TS 36.304 [4]; and

2>	if schedulingInfoList indicates that SystemInformationBlockType18 is present and the UE does not have stored a valid version of this system information block:

3>	acquire SystemInformationBlockType18;

1>	if the UE is capable of sidelink discovery and is configured by upper layers to receive or transmit sidelink discovery announcements on the primary frequency:

2>	if schedulingInfoList of the serving cell/ PCell indicates that SystemInformationBlockType19 is present and the UE does not have stored a valid version of this system information block:

3>	acquire SystemInformationBlockType19;

1>	if the UE is capable of sidelink discovery and, for each of the one or more frequencies included in discInterFreqList, if included in SystemInformationBlockType19 and for which the UE is configured by upper layers to receive sidelink discovery announcements on:

2>	if SystemInformationBlockType19 of the serving cell/ PCell does not provide the corresponding reception resources; and

2>	if schedulingInfoList of the cell on the concerned frequency indicates that SystemInformationBlockType19 is present and the UE does not have stored a valid version of this system information block:

3>	acquire SystemInformationBlockType19;

1>	if the UE is capable of sidelink discovery and, for each of the one or more frequencies included in discInterFreqList, if included in SystemInformationBlockType19 and for which the UE is configured by upper layers to transmit sidelink discovery announcements on:

2>	if SystemInformationBlockType19 of the serving cell/ PCell includes discTxResourcesInterFreq which is set to acquireSI-FromCarrier; and

2>	if schedulingInfoList of the cell on the concerned frequency indicates that SystemInformationBlockType19 is present and the UE does not have stored a valid version of this system information block:

3>	acquire SystemInformationBlockType19;

1>	if the UE is a NB-IoT UE connected to EPC and if ab-Enabled included in MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB is set to TRUE:

2>	not initiate the RRC connection establishment/resume procedure for all access causes except mobile terminating calls until the UE has acquired the SystemInformationBlockType14-NB;

1>	if the UE is capable of V2X sidelink communication and is configured by upper layers to receive or transmit V2X sidelink communication on a frequency:

2>	if schedulingInfoList on the serving cell/PCell indicates that SystemInformationBlockType21 is present and the UE does not have stored valid version of this system information block:

3>	acquire SystemInformationBlockType21 from serving cell/PCell;

2>	if schedulingInfoList on the serving cell/PCell indicates that SystemInformationBlockType26 is present and the UE does not have stored valid version of this system information block;

3>	acquire SystemInformationBlockType26 from serving cell/PCell;

1>	if the UE is capable of V2X sidelink communication and is configured by upper layers to receive V2X sidelink communication on a frequency, which is not primary frequency:

2>	if neither SystemInformationBlockType21 nor SystemInformationBlockType26 of the serving cell/ PCell provide reception resource pool for V2X sidelink communication for the concerned frequency; and

2>	if the cell used for V2X sidelink communication on the concerned frequency meets the S-criteria as defined in TS 36.304 [4]:

3>	if schedulingInfoList on the concerned frequency indicates that SystemInformationBlockType21 is present and the UE does not have stored a valid version of this system information block:

4>	acquire SystemInformationBlockType21 from the concerned frequency;

3>	if schedulingInfoList on the concerned frequency indicates that SystemInformationBlockType26 is present and the UE does not have stored a valid version of this system information block:

4>	acquire SystemInformationBlockType26 from the concerned frequency;

1>	if the UE is capable of V2X sidelink communication and is configured by upper layers to transmit V2X sidelink communication on a frequency, which is not primary frequency and is not included in v2x-InterFreqInfoList in SystemInformationBlockType21 nor SystemInformationBlockType26 of the serving cell/PCell:

2>	if the cell used for V2X sidelink communication on the concerned frequency meets the S-criteria as defined in TS 36.304 [4]:

3>	if schedulingInfoList on the concerned frequency indicates that SystemInformationBlockType21 is present and the UE does not have stored a valid version of this system information block:

4>	acquire SystemInformationBlockType21 from the concerned frequency;

3>	if schedulingInfoList on the concerned frequency indicates that SystemInformationBlockType26 is present and the UE does not have stored a valid version of this system information block:

4>	acquire SystemInformationBlockType26 from the concerned frequency;

1>	if the NB-IoT UE supports NPRACH resources using preamble format 2:

2>	if schedulingInfoList indicates that SystemInformationBlockType23-NB is present and the UE does not have stored a valid version of this system information block:

3>	acquire SystemInformationBlockType23-NB;

1>	following a request from positioning upper layers:

2>	acquire SystemInformationBlockPos, as defined in 5.2.3;

1>	if the UE is capable of NR sidelink communication and is configured by upper layers to receive or transmit NR sidelink communication on a frequency:

2>	if schedulingInfoList on the serving cell/PCell indicates that SystemInformationBlockType28 is present and the UE does not have stored valid version of this system information block:

3>	acquire SystemInformationBlockType28 from serving cell/PCell;

1>	if the UE connected to 5GC is a BL UE or a UE in CE:

2>	when the UE does not have stored a valid version of SystemInformationBlockType25 upon entering RRC_IDLE, or when the UE acquires SystemInformationBlockType1-BR following UAC parameters change notification, or upon entering a cell during RRC_IDLE, or before establishing, resuming or re-establishing an RRC connection if using an eDRX cycle longer than the modification period:

3>	if schedulingInfoList indicates that SystemInformationBlockType25 is present:

4>	start acquiring SystemInformationBlockType25 immediately before establishing, resuming or re-establishing an RRC connection;

3>	else:

4>	discard SystemInformationBlockType25, if previously received;

NOTE 5a:	When connected to 5GC, BL UEs or a UEs in CE start acquiring SystemInformationBlockType25 as described above even when systemInfoValueTag in SystemInformationBlockType1-BR has not changed.

NOTE 5b:	When connected to 5GC, BL UEs or a UEs in CE maintain an up to date SystemInformationBlockType25 in RRC_IDLE.

1>	if the UE is a NB-IoT UE connected to 5GC and if ab-Enabled5GC included in MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB is set to TRUE:

2>	not initiate the RRC connection establishment/ resume/ re-establishment procedure for all access causes until the UE has acquired the SystemInformationBlockType14-NB;

1>	if the UE is NTN capable:

2>	if schedulingInfoList indicates that SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) is present:

3>	immediately before establishing, resuming or re-establishing an RRC connection; or

3>	immediately before EDT or transmission using PUR; or

3>	if in RRC_CONNECTED and T317 is not running:

4>	acquire SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT);

2>	if the UE supports discontinuous coverage; and

2>	if schedulingInfoList indicates that SystemInformationBlockType32 (SystemInformationBlockType32-NB in NB-IoT) is present and the UE does not have a valid version of this system information block:

3>	acquire SystemInformationBlockType32 (SystemInformationBlockType32-NB in NB-IoT);

The UE may apply the received SIBs or posSIBs immediately, i.e. the UE does not need to delay using a SIB or posSIB until all SI messages have been received. The UE may delay applying the received SIBs until completing lower layer procedures associated with a received or a UE originated RRC message, e.g. an ongoing random access procedure.

NOTE 6:	While attempting to acquire a particular SIB/posSIB, if the UE detects from schedulingInfoList/ posSchedulingInfoList that it is no longer present, the UE should stop trying to acquire the particular SIB/ posSIB.

#### 5.2.2.18	Actions upon reception of SystemInformationBlockType11

Upon receiving SystemInformationBlockType11(-NB), the UE shall:

1>	upon entering a cell during RRC_IDLE, or upon connection re-establishment:

2>	if the intercellPWSReception indicated in SIB31 is set to true:

3>	derive the eNB identity from eNB-ID-Length indicated in SIB31 and cellIdentity indicated in SIB1:

4>	determine whether the derived eNB identity of the serving cell is different from the stored eNB identity (if any);

4>	store the derived eNB identity of current serving cell.

2>	if the intercellPWSReception indicated in SIB31 is set to false; or

2>	if the UE determines that the derived eNB identity of the serving cell is different from the stored eNB identity:

3>	discard any previously buffered warningMessageSegment;

3>	clear, if any, the current values of messageIdentifier and serialNumber for SystemInformationBlockType11;

1>	if there is no current value for messageIdentifier and serialNumber for SystemInformationBlockType11; or

1>	if either the received value of messageIdentifier or of serialNumber or of both are different from the current values of messageIdentifier and serialNumber for SystemInformationBlockType11:

2>	use the received values of messageIdentifier and serialNumber for SystemInformationBlockType11 as the current values of messageIdentifier and serialNumber for SystemInformationBlockType11;

2>	discard any previously buffered warningMessageSegment;

2>	if all segments of a warning message have been received:

3>	assemble the warning message from the received warningMessageSegment;

3>	forward the received warning message, messageIdentifier, serialNumber and dataCodingScheme to upper layers;

3>	stop reception of SystemInformationBlockType11;

3>	discard the current values of messageIdentifier and serialNumber for SystemInformationBlockType11;

2>	else:

3>	store the received warningMessageSegment;

3>	continue reception of SystemInformationBlockType11;

1>	else if all segments of a warning message have been received:

2>	assemble the warning message from the received warningMessageSegment;

2>	forward the received complete warning message, messageIdentifier, serialNumber and dataCodingScheme to upper layers;

2>	stop reception of SystemInformationBlockType11;

2>	discard the current values of messageIdentifier and serialNumber for SystemInformationBlockType11;

1>	else:

2>	store the received warningMessageSegment;

2>	continue reception of SystemInformationBlockType11;

The UE should discard any stored warningMessageSegment and the current value of messageIdentifier and serialNumber for SystemInformationBlockType11 if the complete warning message has not been assembled within a period of 3 hours.

#### 5.2.2.19	Actions upon reception of SystemInformationBlockType12

Upon receiving SystemInformationBlockType12(-NB), the UE shall:

1>	upon entering a cell during RRC_IDLE, or upon connection re-establishment:

2>	if the intercellPWSReception indicated in SIB31 is set to true:

3>	derive the eNB identity from eNB-ID-Length indicated in SIB31 and cellIdentity indicated in SIB1:

4>	determine whether the derived eNB identity of the serving cell is different from the stored eNB identity (if any);

4>	store the derived eNB identity of current serving cell.

2>	if the intercellPWSReception indicated in SIB31 is set to false; or

2>	if the UE determines that the derived eNB identity of the serving cell is different from the stored eNB identity:

3>	discard any previously buffered warningMessageSegment;

3>	clear, if any, the current values of messageIdentifier and serialNumber for SystemInformationBlockType12;

1>	if the SystemInformationBlockType12 contains a complete warning message and the complete geographical area coordinates (if any):

2>	forward the received warning message, messageIdentifier, serialNumber, dataCodingScheme and the geographical area coordinates (if any) to upper layers;

2>	continue reception of SystemInformationBlockType12;

1>	else:

2>	if the received values of messageIdentifier and serialNumber are the same (each value is the same) as a pair for which a warning message and the geographical area coordinates (if any) are currently being assembled:

3>	store the received warningMessageSegment;

3>	store the received warningAreaCoordinatesSegment (if any);

3>	if all segments of a warning message and geographical area coordinates (if any) have been received:

4>	assemble the warning message from the received warningMessageSegment;

4>	assemble the geographical area coordinates from the received warningAreaCoordinatesSegment (if any);

4>	forward the received warning message, messageIdentifier, serialNumber, dataCodingScheme and geographical area coordinates (if any) to upper layers;

4>	stop assembling a warning message and warning area coordinates (if any) for this messageIdentifier and serialNumber and delete all stored information held for it;

3>	continue reception of SystemInformationBlockType12;

2>	else if the received values of messageIdentifier and/or serialNumber are not the same as any of the pairs for which a warning message is currently being assembled:

3>	start assembling a warning message for this messageIdentifier and serialNumber pair;

3>	start assembling the geographical area coordinates (if any) for this messageIdentifier and serialNumber pair;

3>	store the received warningMessageSegment;

3>	store the received warningAreaCoordinatesSegment (if any);

3>	continue reception of SystemInformationBlockType12;

The UE should discard warningMessageSegment and warningAreaCoordinatesSegment (if any) and the associated values of messageIdentifier and serialNumber for SystemInformationBlockType12 if the complete warning message and the warning area coordinates (if any) have not been assembled within a period of 3 hours.

NOTE:	The number of warning messages that a UE can re-assemble simultaneously is a function of UE implementation.

#### –	SystemInformationBlockType31-NB

The IE SystemInformationBlockType31-NB contains satellite assistance information. SystemInformationBlockType31-NB is only signalled in a NTN cell.

SystemInformationBlockType31-NB information element

-- ASN1START

SystemInformationBlockType31-NB-r17 ::= SEQUENCE {

servingSatelliteInfo-r17		ServingSatelliteInfo-r17,

lateNonCriticalExtension		OCTET STRING					OPTIONAL,

...,

[[	servingSatelliteInfo-v1820	ServingSatelliteInfo-v1820		OPTIONAL	-- Need OR

]],

[[

intercellPWSReception-r19	BOOLEAN									OPTIONAL,

eNB-ID-Length-r19			ENUMERATED {Macro, Short, Long}	OPTIONAL,	--Need OR

]]

}

-- ASN1STOP

SystemInformationBlockType31-NB field descriptions

eNB-ID-Length Indicates the length of eNB ID out of the 28-bit long cellIdentity indicated in SIB1. Macro indicates the 20 leftmost bits equal the eNB ID. Short indicates the 18 leftmost bits equal the eNB ID, while long indicates the 21 leftmost bits equal the eNB ID. The cellIdentity information is defined in TS 36.413 [39].

interCellPWSReception Indicates whether the UE shall retain or discard buffered warningMessageSegment when entering a new cell in RRC_IDLE or during RRC connection re-establishment. If the value is set to true, the UE may retain the previously buffered warningMessageSegment and continue its assembly upon reception of additional segments in new cell. If set to false, the UE shall discard any previously buffered warningMessageSegment upon entering a new cell or re-establishing the RRC connection. For this release, support for inter-cell PWS message segmentation reception is restricted to the intra-eNB case in NTN scenario.

# Reference

[1] R2-2503663	Remaining issues on support of PWS for NB-IoT NTN, Nokia, Nokia Shanghai Bell

[2] R2-2504655	Enhancements to support PWS in NB-IoT NTN, Ericsson

[3] R2-2503503	Remaining issues on PWS support for NB-IoT, Huawei

[4] R2-2503799	Open issues on the support for PWS in NB-IoT, Google

# Annex

The following agreements have been reached from RAN2-127bis to RAN2-130 meetings.

RAN2-127bis Agreements:

1.	RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS

2.	The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes.

3.	For NB-IoT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/ skip some unnecessary fields):

- SystemInformationBlockType10-NB for primary ETWS notification;

- SystemInformationBlockType11-NB for secondary ETWS notification;

- SystemInformationBlockType12-NB for CMAS notification.

4.	Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB):

- etws-Indication;

- cmas-Indication.

5.	Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (can come back to clarify further what immediately means)

6.	Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases)

RAN2-128 Agreements:

1.	We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements.

2.	We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed

3.	Add the following PWS indication in Paging-NB:

- etws-Indication;

- cmas-Indication.

RAN2-129 Agreements:

1.	Introduce geographic information in ETWS messages for geo-fencing in IoT NTN cells

2.	Introduce the Warning Area Coordinates IE for ETWS to describe an emergency area in system information.

3.	Introduce the Warning Area Coordinates IE to existing ETWS SIBs, i.e., SIB10-NB/11-NB, without introducing the need for segmentation of SIB10-NB

4.	We don’t introduce a new SIB for carrying geographical information for emergency areas

5.	Send a LS to CT1 and CT4 indicating that RAN2 currently supports a maximum of 64 segments for SIB11-NB and SIB12-NB with a maximum TBS of 680bits and asking to provide feedback if they think we need to work on changes to allow longer messages (that would require more segments). Also indicate some possible drawbacks

RAN2-129bis Agreements:

1.	In case of PWS notification the network may release a RRC_CONNECTED UE to idle, in case the UE reports the capability to receive PWS in idle (no other spec impact other than the introduction of a UE capability for supporting PWS reception in idle)

RAN2-130 Agreements:

1.	A UE should be able to camp on an NB-IoT cell (at least for NTN) for the purpose of receiving PWS broadcast, without the cell being a part of UEs PLMNs. Come back in the next meeting to discuss the exact solution to make this possible (i.e. not necessarily introducing an acceptable cell category for NB-IoT)

2.	Regarding support continued reception of PWS segmentation of a message from different cells, can come back in the next meeting with an actual TP. In any case we will not target other scenarios than the intra-gNB case