3GPP TSG-RAN WG2 Meeting #131	R2-2505641

Bengaluru, India, 25 - 29 August 2025

Agenda Item:	8.6.4

Source:	ITRI

Title:	Discussion on early TA for conditional LTM

Document for: Discussion

- Background

In previous RAN2 meetings, many agreements were made on early TA acquisition and maintenance for the conditional LTM. The early TA value makes the RACH-less cell switch possible. However, it may become invalid when the conditional LTM cell switch is triggered.

RAN2#128 Agreements [1]:

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

RAN2#129 Agreements [2]:

- CLTM TAT is started/re-started when UE receives TA value for the candidate cell sent by the MAC CE from current serving cell.

- If the CLTM TAT associated with one candidate cell is running, UE considers the TA for the candidate cell is valid; if CLTM TAT associated with one candidate cell expires, UE considers the TA for the candidate cell is invalid.

- If UE has valid TA value at the time CLTM execution, UE performs RACH-less CLTM, otherwise UE performs RACH base CLTM.

- The CLTM TAT is per candidate configuration.

RAN2#129bis Agreements [3]:

- Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running.

- C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

RAN2#130 Agreements [4]:

- Upon CLTM execution, if TA is valid (i.e. TAT timer is running) at the first available CG occasion, and there is CG resource configured, RACH-less CLTM will be performed.

- During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, fallback to RACH-based CLTM.

In this contribution, we provide our views and proposals to support early TA re-acquisition, facilitating TA value update.

2	Discussion

Early TA acquisition is a key that makes the RACH-less cell switch possible. In other words, it is important for latency reduction during a cell switch.

For the network-triggered LTM, the UE can receive the TA value of the target cell via the cell switch commend MAC CE and perform the cell switch immediately.

By contrast, the conditional LTM is UE-triggered. That is, even if an early TA value has been acquired, e.g. via PDCCH ordered early RACH, it may become invalid when the conditional LTM is triggered. That is the reason why we introduced a TA validity timer for each candidate cell in the conditional LTM.

The expiry of a TA validity timer means the corresponding early TA value becomes invalid. Without the valid early TA, the RACH procedure needs to be performed during the cell switch.

Therefore, early TA re-acquisition should be supported in the conditional LTM to facilitate RACH-less cell switch as much as possible.

Proposal 1: Early TA re-acquisition is supported in the conditional LTM.

The TA value is essential for ensuring uplink synchronization between the UE and the network.

A valid TA enables the UE to adjust its transmission timing so that the uplink signal arrives at the network precisely as expected.

However, if the TA value expires, it indicates that the uplink synchronization has been lost. This may suggest that the network is no longer aware of the UE’s current location. In such cases, a preamble and preamble transmission time may be required for the UE to inform the network of its updated position and re-acquire a TA value.

Therefore, the network should allocate the resources such as the preamble and preamble transmission time to allow the UE to perform early TA re-acquisition.

Proposal 2: The network should allocate the resources such as the preamble and preamble transmission time to enable early TA re-acquisition.

# 3	Conclusion

This contribution makes the following proposals:

Proposal 1: Early TA re-acquisition is supported in the conditional LTM.

Proposal 2: The network should allocate the resources such as the preamble and preamble transmission time to enable early TA re-acquisition.

# 4	Reference

- Report of 3GPP TSG RAN WG2 meeting #128, Hefei, China, Oct 14th – 18th, 2024.

- Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece, 17 - 21 February, 2025.

- Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan, China, 7th– 11th April 2025.

- Report of 3GPP TSG RAN WG2 meeting #130, St. Julian’s, Malta, May 19th – 23rd, 2025.