# 3GPP TSG-RAN WG2 Meeting #131		                  								R2-2505649

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	8.2.2

Source:	Robert Bosch GmbH

Title:	Discussions on ambient IoT paging

Document for:	Discussion and Decision

# Ambient IoT Paging

When an A-IoT device receives a paging message from a reader, it starts with the random access procedure. For any reason such as collision of msg1 or not selection of the device by the reader, the device will not succeed with the random access procedure. Besides, the device may not start the random access procedure due to some internal reasons such as energy harvesting or buffer status. In such cases, the reader may repeat the paging in the next paging occasion. If the random access procedure for a device is not successful and the reader does not receive msg1 from a device, the paging may be repeated in several upcoming paging occasions which can be waste of energy and resources. The reason of not successful random access can be lack of enough energy or the need to perform energy harvesting in a device as well as high congestion of msg1 transmissions by several devices.

Observation 1: Several re-transmission of a paging message can be useless since the device is not able to start random access procedure due to internal reasons or it fails to access the medium due to high congestions.

A reader can stop further paging of a device, if it is informed about not possibility of successful random access by a device at least in the next access occasions. Therefore, it is reasonable that a device informs the reader about the reason of failure and the reader stops the upcoming pagings. However, if the reason for failure is congestion, the device needs dedicated resources to inform the reader other than it will again fail to send such an information. Besides, if the reason of failure is the lack of energy, the information from the device to the reader should not consume a lot of energy.

Proposal 1: A reader may mandate a device to send n bits in a D2R message to indicate the reason of its failure for e.g., energy harvesting or congestion using dedicated resources.

Proposal 2: The paging message may contain resource information for the D2R message which carries the reason of device failure in the random access procedure.

It is important for a reader to know the reason of failure since it may consider different behaviors in case of different reasons of failures. For example, if congestion is the reason of failure for a device, the reader can adapt the paging message and configure the device to use CFRA using indicated resources in the paging message.

Proposal 3: A reader may stop paging of a device or adapt the content of paging for e.g., adapting random access type if it knows the reason of failure for a device.

# Conclusions

Based on the discussion in this contribution, we have the following observation and proposals:

Observation 1: Several re-transmission of a paging message can be useless since the device is not able to start random access procedure due to internal reasons or it fails to access the medium due to high congestions.

Proposal 1: A reader may mandate a device to send n bits in a D2R message to indicate the reason of its failure for e.g., energy harvesting or congestion using dedicated resources.

Proposal 2: The paging message may contain resource information for the D2R message which carries the reason of device failure in the random access procedure.

Proposal 3: A reader may stop paging of a device or adapt the content of paging for e.g., adapting random access type if it knows the reason of failure for a device.