3GPP TSG-RAN WG2 Meeting #131		R2-2505650

Bangalore, India, Aug 25th – 29th 2025

Agenda Item: 		8.1.3

Source:		Sony

Title:	Discussion on open issues on NW side data collection

Document for:		Discussion & Decision

- Introduction

In this contribution we provide our views on the remaining open issues on NW side data collection.

- Discussions

In the email discussion [POST130][026][AI PHY] 38.331 CR (Ericsson), the following identified open issues remain unresolved, in this paper we discuss and provide our proposals for each issue.

Open issue RRC-21: Time related content of logged data Issue description: It has not yet been clarified what information needs to be included with the logged data, to indicate a time gap between the logged data entries (i.e. a gap that is longer than the logging data periodicity). This issue refers to the RAN2#129bis agreement: - For temporal domain, the network is made aware whether there is a gap between two consecutive samples. FFS amount of gap and whether this is implicit or explicit RAN2#130 agreement: Data is collected on per data logging configuration basis and UE indicates data logging configuration ID. An indication of the “gap” is needed. “Gap” is time interval larger than the configured logging periodicity. FFS if timestamp and relative time stamp for each group is needed per “group”.. The issue is captured as an editor’s note in the running CR, in clause 5.7.10.3 and 6.2.2. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

In the spec of logged MDT, we have the following design of time information [1]:

For each MDT measurement the UE includes a relative time stamp. The base unit for time information in the Logged MDT reports is the second. In the log associated to periodical logging configuration, the time stamp indicates the point in time when periodic logging timer expires. The time stamp is counted in seconds from the moment the logged measurement configuration is received at the UE, relative to the absolute time stamp received within the configuration. The absolute time stamp is the current network time at the point when Logged MDT is configured to the UE. The UE echoes back this absolute reference time. The time format for Logged MDT report is: YY-MM-DD HH:MM:SS.

Based on the MDT framework, we think that an absolute timestamp is need. There are two options for this absolute timestamp, one is to include it in the data collection configuration from NW to UE, and UE will report the time gap between logged data entries using relative timestamp to the absolute time stamp received within the configuration. The other option is to use an absolute timestamp for the first logged data entry, and then use a relative timestamp for all subsequent entries within that continuous logging period. The absolute time stamp is common for all groups, and the relative time stamp could be multiple of logging periodicity. Instead of relative time, the UE can also report the number of missed logging periodicities. This approach provides a precise starting point for each logging entry while minimizing the data overhead for the entries that follow.

Proposal 1 (RRC-21): RAN2 to discuss and select one of the two to indicate the gap:

- NW indicate an absolute time stamp in the data collection configuration and this is common for all groups, then UE report the time gap between two logged entries as relative time stamp to the configured absolute time stamp, or as the number of missed periodicities.

- The UE report an absolute timestamp for the first group of logged data, and then use a relative timestamp for all subsequent groups, or report the number of missed periodicities between two groups.

###### Open issue RRC-25: Dynamic activation/deactivation of data collection configurations for logging Issue description: RAN2#127 made the following agreement regarding NW-side data collection: 1 As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support.. The second FFS on dynamic activation/deactivation has not yet been addressed, as also pointed out in the stage-2 running CR draft. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

In the legacy MDT framework, logged measurement will be activated upon UE receiving the logged MDT configuration. In the case of data collection for AI model training, in RAN2#130, we agreed that data is collected on per data logging configuration basis. When network needs data for training, it will provide a data collection configuration for the UE to start collecting data and UE should immediately start logging data based on the configuration, similarly as the MDT framework. We think network will provide multiple configurations only if a particular UE should be involved in data collection and as per the provided configurations. In other words, network can always provide different data collection configurations for different UEs. We don’t see the motivation for dynamically activate/deactivate logging of a specific data collection configuration.

Proposal 2 (RRC-25): Dynamic activation/deactivation of data collection configuration for logging is not supported.

Open issue RRC-29: Whether data availability indication should be sent when the UE has data below the threshold and low power state is sent, and what cause should be included then Whether data availability indication should be sent when the UE has data and low power state is sent and what cause should be included then. The UE may have some data when indicating low power state. Even though this data volume may be lower than full buffer/threshold, it is still useful to let the network know about that so that the data can be fetched. Proposed resolution: It is suggested that companies provide contributions to resolve the issue.

The motivation for sending low power state indication is for UE to save energy, and this means UE will not participate in data logging and NW should not fetch data in the low power state for UE to save energy. The available data should be fetched when UE return to normal power state and send data availability indication.  If we let the network fetch data in UE low power state, we are going against the purpose of buffer threshold and low power state.

Proposal 3 (RRC-29)： Data availability indication is not sent when indicating low power state.

Open issue RRC-31: The release of logged AIML data in UE’s buffer Issue description: RAN2 needs to discuss and capture in the spec when the logged data for AIML in UE buffer will be released. In addition to the retain/release of logged data during HO covered in Open issue RRC-7, the following can be also considered: - Power off or deregistration - 48 hrs after the release of logged measurement configuration - Explicit indication from the serving gNB. Proposed resolution: It is suggested that companies provide contributions to resolve this issue. (maybe also for RRC-35)

Open issue RRC-35: Release/retain un-retrieved data upon inter-RAT handover Issue description: RAN2 agreed to introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO. It needs to be discussed whether this is applicable for inter-RAT handover also. Since it is agreed that UE releases the logged L1 measurements for AI/ML while moving to RRC_IDLE/RRC_INACTIVE and RLF, if this agreement is applied for inter-RAT HO, there will be changes in LTE for releasing AI/ML measurements in LTE RLF, moving to LTE idle etc. Moreover, it is not possible to retrieve the data by eNB. A solution is to always discard logged L1 measurements during inter-RAT handover without considering the indication. Proposed resolution: This open issue was resolved by the RAN#130 agreement below, since the UE discards the logged data if it does not receive the 1-bit indication in HO command. 1 (RRC-18) 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration with synch is introduced. In case of HO, the source sends the 1-bit indication to target cell in HO preparation message. This 1-bit indication is included in HO command by target cell (if the target cell wants to keep the data). We should have single UE behaviour, when it receives the indication it keeps it, otherwise it removes it. Notify RAN3.

As to when the logged data in UE buffer will be released, we think NW could have some control since the data is collected for NW side model training. In RAN2#129bis, it is agreed that UE releases the logged L1 measurements for AI/ML while moving to RRC_IDLE/RRC_INACTIVE/RLF.

RAN2#129bis agreement:

Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data

In RLF case, if the UE fails to re-establish the connection within the T311 timer (e.g., the target cell rejects the request or no suitable cell is found), the UE moves to the RRC_IDLE state. It will then perform normal cell re-selection to find a new suitable cell and re-establish a new RRC connection. In this case (i.e. UE moves to RRC_IDLE after RLF), the logged data before RLF happens could be beneficial for model training to improve model performance in the corresponding condition. Therefore, we propose the NW should have the option to indicate UE to retain logged data for a specific time period (e.g. 48 hours) after going to RRC_IDLE when RLF happens

Proposal 4 (RRC-31 &RRC-35)： When needed, NW can send an indication to UE to keep the data during RRC Idle when RLF happens. A specific time period for keeping the data can either be configurable or fixed in the specification.

Open issue RRC-34: Whether enhancements for NW-side data collection are per use case or common for all AI-related use cases Issue description: It can be discussed whether the enhancements for NW-side data collection are per use case or common for all AI-related use cases, e.g., AS layer memory, logged data availability, low-power state indication. Proposed resolution: It is suggested that companies provide contributions to resolve the issue.

For NW side data collection, the model is on network side and UE task is only to provide data. From this point of view, UE does not need to distinguish between different use cases.  We don’t see any difference of UE behaviour (e.g. data buffer, availability indication, low power state indication) for different use cases. A common framework reduces the number of RRC messages, IEs, and procedures that need to be defined and implemented. A common approach ensures consistent behavior across all AI-related features. Therefore, we propose to a single, unified framework for data collection, which avoids redundant signaling and complex specifications

Proposal 5 (RRC-34):  The enhancements for NW-side data collection should be common for all AI-related use cases.

###### Open issue RRC-36: How data is forwarded to OAM or source gNB after HO Issue description: RAN2 agreed that target gNB can fetch data collected in the source gNB after HO. We need to decide how this data is forwarded to OAM or source gNB, e.g. via inter-node RRC message or in some other way. RAN3 involvement may be needed. Proposed resolution: It is suggested that companies provide contributions to resolve the issue

From RAN2 perspective, how the data is forwarded from the target gNB to either the source gNB or OAM is out of RAN2 scope because RAN2 does not define inter node messages. The relevant WGs (e.g., RAN3/SA5) should resolve this issue. The LS to RAN3 (R2-2504950), which notifies RAN3 the related RAN2 agreements on handling of logged data collected for NW-side AI/ML model training at handover, would be enough.

Proposal 6 (RRC-36): The relevant WGs (e.g., RAN3/SA5) should resolve this issue (How data is forwarded to OAM or source gNB after HO).

- Summary

In this contribution, we have the following proposal for RAN2 to agree:

Proposal 1 (RRC-21): Two options can be adopted to indicate the gap:

- NW indicate an absolute time stamp in the data collection configuration and this is common for all groups, then UE report the time gap between two logged entries as relative time stamp to the configured absolute time stamp, or as the number of missed periodicities.

- The UE can report an absolute timestamp for the first group of logged data and then use a relative timestamp for all subsequent groups, or report the number of missed periodicities between two groups.

Proposal 2 (RRC-25): Dynamic activation/deactivation of data collection configuration for logging is not supported.

Proposal 3 (RRC-29)： Data availability indication is not sent  when indicating low power state.

Proposal 4 (RRC-31 & RRC-35)： When needed, NW can send an indication to UE to keep the data during RRC Idle when RLF happens. A specific time period for keeping the data can either be configurable or fixed in the specification.

Proposal 5 (RRC-34):  The enhancements for NW-side data collection should be common for all AI-related use cases.

Proposal 6 (RRC-36): The relevant WGs (e.g., RAN3/SA5) should resolve this issue (How data is forwarded to OAM or source gNB after HO).

- References

- TS 37.321 V18.3.0, Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2 (Release 18).