3GPP TSG-RAN WG2 Meeting #131							       	 R2-2505673

Bengaluru, India, 25 - 29 August, 2025

Title: 	Discussion on UE-side data collection

Source: 	Huawei, HiSilicon

Agenda item:	8.3.2.3

Document for:	Discussion/Decision

# Introduction

In RAN2 #130 meeting, the following agreements were reached on UE-side data collection:

Agreements for UE side data collection For UE sided data collection – use AI/ML PHY request/configuration framework as baseline. FFS enhancements/or differences

In this document, we will follow up on the agreement to use AI/ML PHY request/configuration framework as a baseline and will further discuss UE side data collection for RRM measurement prediction and event prediction excluding those aspects that are under study in AI/ML PHY.

# Discussion

## General procedure for UE side data collection

Following RAN2 meeting agreements on AI/ML PHY request/configuration, we can reuse the procedure of UE side data collection agreed for beam management case, summarized by the agreements copied below.

RAN2 #129 agreement: - Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g., start/stop indication). RAN2 #129bis agreement: - The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following: • An indication on start/stop of data collection • Preferred configuration from a list of candidate configurations provided by NW. - Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

As can be seen in the highlighted agreement above, the UE can send a request including a preference to start/stop the data collection and can also indicate its preferred configuration from a set of configurations provided by the network. We propose to confirm the same should apply to AIML for mobility.

Proposal 1: The procedure agreed for UE data collection request for AIML based beam management can be reused for AIML based mobility, including:

      - The UE can request measurement configuration for data collection of AI/ML for mobility cases. The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW.

      - Use UAI message for UE request of data collection measurement configuration. It is up to UE implementation when to send the request.

In RAN2#129bis meeting, following agreements were reached regarding the supported use case for the UE sided model.

- L3 Beam-level prediction specification impact will be studied - Things to continue study for specification and RAN2 impact: UE sided model; Network sided model. Measurement event prediction; Yes; No (up to NW implementation). Temporal domain case A; Yes, all sub cases; Yes. Temporal domain case B; Yes, all sub cases; Yes. Frequency domain; Yes, all sub cases; Yes. Spatial domain; Pending; Pending. L3 beam level measurement prediction; Yes; Yes.

Depending on UE capability or OTT server requirement, UE may require data collection for different scenarios. Hence, the contents of the data collection configuration and the contents of the data collection request will depend on the use case that is targeted by the UE. We analyze this case by case in the following sections.

## Content for data collection configuration

### RRM measurement prediction

For RRM measurement prediction, intra-frequency temporal domain case A and case B, inter-frequency domain and spatial domain is supported using UE side model. In this section, we discuss the contents of configuration by the network.

Temporal domain Case A is described in TR 38.744 as follows:

In intra-frequency temporal domain case A, continuous measurement results in PW are predicted by continuous historical measurement result(s) in OW. Then OW and PW slide forward with either sampling period(s) (with sliding L1/L3 filtering option) or measurement period(s) (with non-sliding L1/L3 filtering option), where measurement result(s) are actually measured before sliding. One example is illustrated in Figure 5.2.1.1-1: Figure 5.2.1.1-1: Example of intra-frequency temporal domain case A

Temporal domain Case B is described in TR 38.744 as follows:

In intra-frequency temporal domain case B, measurement results in PW are predicted by historical measurement result(s) in OW. Then OW and PW slide forward with either sampling period(s) (with sliding L1/L3 filtering option) or measurement period(s) (with non-sliding L1/L3 filtering option) and measurement result(s) in previous PW is/are skipped during window sliding. Example 1 and example 2 are illustrated in Figure 5.2.1.1-2 and Figure 5.2.1.1-3 respectively, between which example 2 is recommended as baseline for evaluation. Figure 5.2.1.1-2: Skipping pattern example 1 of intra-frequency temporal domain case B Figure 5.2.1.1-3: Skipping pattern example 2 of intra-frequency temporal domain case B

Based on this description, we can observe that:

- In temporal domain case A, UE continuously measures signal to get data for model input, so the legacy RRM measurement configuration can be reused for data collection.

- In temporal domain case B, we can observe that some samples are skipped for measurement and are instead intended to be predicted during inference. As discussed previously, the NW may want to switch off some RS(es) in case the UEs are able to perform prediction on them. However, if the UEs train for different skipping patterns, this may not be possible. Hence, it is preferential for the network to indicate the intended skipping pattern to the UE, similarly as done with set A/B configuration for beam management use case.

Observation 1: It is preferential for the network to indicate the intended skipping pattern to the UE, similarly as done with set A/B configuration for beam management use case.

In AI/ML PHY for beam management case, we agreed that the contents of candidate configurations provided by NW are as following:

For beam management, candidate data collection configuration includes at least: - CSI-ResourceConfigId of Set A - CSI-ResourceConfigId of Set B - One/two associated IDs (up to whether Set B is equal/subset of Set A or not) according to RAN1 agreements

Similarly, for spatial domain prediction in AI for mobility case, configurations like set A and set B can also be considered in candidate data collection configuration.

In summary, we can conclude as following proposal:

Proposal 2: One or more configurations can be provided by the NW which can include:

- L3 and/or L1 measurement configuration

- For intra-frequency temporal domain case B: time instances/skipping pattern related information for measurements

- For spatial domain: set A and set B beam configurations (similar as in BM case)

- Associated ID (optional)

### RRM measurement event prediction

According to the summary of performance results for measurement event prediction in TR 38.744, we can observe that:

F1 score is higher for shorter TTT values

Based on this conclusion, TTT will impact the model inference accuracy. Furthermore, the network may employ only certain TTT values for the event configured in a particular cell. Hence, in order to ensure the trained models match network implementation and to prevent misalignment between the UE and the NW, it is beneficial for the NW to indicate its preferred TTT ranges or other key parameters (e.g., filtering coefficients, hysteresis) to UE.

Proposal 3: NW can indicate its preferred TTT ranges to UE or other key measurement event configuration parameters (e.g., filtering coefficients, hysteresis). The exact parameters can be discussed in the WI phase.

## Content of data collection request

### RRM measurement prediction

As clarified in the previous section, the contents of the candidate configurations will depend on the scenario and exact use case the UE intends to train for. Therefore, in order for the network to prepare proper candidate data collection configurations, it needs to be aware of the use case in which the UE is interested. For example, in case the UE would like to collect data for inter-frequency prediction, it needs to indicate which two frequencies it is interested in. The network may need to configure proper gap configuration in this situation. On the other hand, if the UE indicates that it will collect the data for temporal domain prediction case B, the network may need to provide the UE with a skipping pattern.

Proposal 4: The content of the UE request for data collection should include the indication to indicate which use case the UE intends to collect data for, e.g., temporal domain case A/B, frequency domain prediction, spatial domain prediction.

### RRM measurement event prediction

Both direct and indirect method for measurement event prediction use actual measurement results of serving cell and/or neighboring cells as model input for training. So, the UE request contents can be reused for RRM measurement prediction case. However, to allow the UE to train the model for specific settings the network uses in a particular cell, the UE should also indicate the event types that it intends to train for. This way the NW can provide typical TTT or other configuration parameters that it uses for these events.

Proposal 5: The UE should be able to indicate the measurement event type in data collection request.

# Conclusion

In this paper, our proposals are summarized as follows:

Proposal 1: The procedure agreed for UE data collection request for AIML based beam management can be reused for AIML based mobility, including:

- The UE can request measurement configuration for data collection of AI/ML for mobility cases. The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW.

- Use UAI message for UE request of data collection measurement configuration. It is up to UE implementation when to send the request.

Observation 1: It is preferential for the network to indicate the intended skipping pattern to the UE, similarly as done with set A/B configuration for beam management use case.

Proposal 2: One or more configurations can be provided by the NW which can include:

- L3 and/or L1 measurement configuration

- For intra-frequency temporal domain case B: time instances/skipping pattern related information for measurements

- For spatial domain: set A and set B beam configurations (similar as in BM case)

- Associated ID (optional)

Proposal 3: NW can indicate its preferred TTT ranges to UE or other key measurement event configuration parameters (e.g., filtering coefficients, hysteresis). The exact parameters can be discussed in the WI phase.

Proposal 4: The content of the UE request for data collection should include the indication to indicate which use case the UE intends to collect data for, e.g., temporal domain case A/B, frequency domain prediction, spatial domain prediction.

Proposal 5: The UE should be able to indicate the measurement event type in data collection request.

# Reference

[1] 3GPP TR 38.744 v1.1.0

[2] 3GPP TSG-RAN WG2 Meeting #130 chair note of AI/ML for mobility

[3] [POST130][026][AI PHY] 38.331 CR (Ericsson)