3GPP TSG-RAN WG2 Meeting #131					   			 R2-2505674

Bangalore, India, 25 - 29 August, 2025

Title: 	Discussion on NW-sided model support

Source: 	Huawei, HiSilicon

Agenda item:	8.3.3

Document for:	Discussion/Decision

# Introduction

In this paper, we discuss the NW-sided model corresponding remaining issues in AI mobility, mainly including the following three aspects:

- The analysis for specification impact from NW-sided spatial domain prediction

- The potential enhancements of measurement framework and the corresponding specification impacts

- The necessity of assistance information for the NW-sided model

# Discussion

## Spatial Domain Prediction

In the RAN2 #130 meeting, there are the following agreements about spatial domain RRM prediction:

Agreements 1 The scenario of intra-frequency spatial domain prediction (in cell dimension) is not considered by the UE-side model in 5G. RAN2 assumes that a network-sided model requires no specification impacts. Intra-frequency spatial domain prediction (in cell dimension) involves measuring one or more cells as input to the model to derive L3 filtered cell-level measurements for the same time instance of another cell(s). 2 The specification impact of intra-frequency spatial domain prediction in beam dimension will be studied by RAN2 for both UE-side and network-side model.

Then according to the RAN plenary meeting #108, the AI mobility WI has been approved [1], while for the spatial domain RRM prediction, there is a note as follows:

Note 1: For UE sided model, spatial domain is not supported. Can check in RAN#109 whether for network sided model spatial domain is supported or not, based on the final outcome of the SI.

We understand that the UE-sided model spatial domain prediction shall not be discussed in Rel-19 and Rel-20 anymore. Then for the intra-frequency NW-sided spatial domain prediction, we think it still needs further consideration.

Observation 1: The UE-sided spatial domain prediction will no more be discussed in Rel-19 and Rel-20, while NW-sided requires further analysis and conclusion.

For the cell dimension, RAN2 has already agreed that there is no specification impact, as in the RAN2#130 meeting agreements copied above. For the spatial domain prediction in beam dimension, the impacts are the same as for other scenarios (e.g. temporal/frequency domain prediction) and are mainly based on existing RRM framework, as per the following agreements from RAN2#130 meeting:

- For network-side RRM measurement prediction, the legacy RRM measurement configuration and RRM measurement reporting framework can be used. For L1-filtered beam-level RSRP reporting can be configured by setting co-efficient to zero. FFS if there are specification impacts to support subcase 1 and 3 (if supported) and interference for cell level prediction. - To support cell/beam level prediction, one enhancements is to report RRM measurement results per cell at multiple time instances in one measurement report for NW-sided model.

Inference using NW-sided model for spatial domain prediction in beam dimension does not present any additional specification impacts on top of what is already needed/agreed to support other scenarios.

Observation 2: Inference using NW-sided model for spatial domain prediction in beam dimension does not require additional specification impacts on top of what is already needed/agreed to support other scenarios (i.e. temporal/frequency domain).

NW-side data collection for spatial domain in beam dimension

Another aspect which needs to be analyzed is specification impact due to NW-sided data collection support for spatial domain in beam dimension.

As defined in the latest TR 38.744 [2], the intra-frequency spatial domain is designed to reduce the measurement costs, e.g. using measured results of the beams in set “A”, to predict the results of the beams in another set “B”. Therefore, to obtain the training data, the UE needs to be configured with sets of beams to measure. For layer 1 beam measurement, this kind of measurement has been already fully discussed in RAN1 air-interface topic, and can be easily reused for RRM prediction sub case-1 and 3.

Observation 3: NW-side data collection mechanism specified in Rel-19 for beam management use case can be reused to support data collection for NW-side spatial domain prediction using RRM sub-case 1 and 3.

However, layer 3 beam measurements are not currently collected for the sake of beam management use case and will not be supported by the NW-side data collection specified in Rel-19 for AIML for air interface. This would be one additional aspect that needs to be added, but it should be noted that in case beam-level prediction for other scenarios is to be supported (e.g. for temporal domain prediction), the same data needs to be collected, so there is no additional impact to support spatial domain in beam dimension on top of this. What is more, this has been already agreed by RAN2 in RAN2#130 meeting:

For the collection of data for the training of a network-sided model for RRM measurement prediction: - UE can be configured to log, at a certain logging periodicity, one or more of the following: - L3 cell level measurements, - L3 beam level measurements, - L1-filtered beam level measurements (if sub-case 1 and 3 is supported) - Cell ID (FFS CGI of serving cell. If CGI is unavailable, or for neighboring cells the UE logs PCI-ARFCN as a fallback) - Time info (if as agreed by AI/ML and/or if needed)

Observation 4: To support sub-use case 2 for spatial domain prediction in beam dimension at the NW-side, data collection mechanism needs to be extended to collect L3 beam level results which is the same requirement as for beam level prediction for other scenarios (e.g. temporal domain prediction) using NW-sided model and which was already agreed by RAN2.

Based on this analysis we observe and propose the following:

Observation 5: The data collection for spatial domain RRM prediction in beam dimension is already supported using RRM sub-case 1/3, and will only bring limited impacts to support RRM sub-case 2 which are required to support beam level prediction for other scenarios anyway.

Proposal 1: RAN2 to confirm that intra-frequency spatial domain RRM prediction in beam dimension using NW-sided model is feasible, has minor specifications impact and there are no obstacles to include it in the normative phase of the work.

## Potential Impacts on measurement framework

In the last RAN2#130 meeting, the following agreements have been approved for the measurement framework:

Agreements 1 For network-side RRM measurement prediction, the legacy RRM measurement configuration and RRM measurement reporting framework can be used. For L1-filtered beam-level RSRP reporting can be configured by setting co-efficient to zero. FFS if there are specification impacts to support subcase 1 and 3 (if supported) and interference for cell level prediction. 2 To support cell/beam level prediction, one enhancements is to report RRM measurement results per cell at multiple time instances in one measurement report for NW-sided model.

Sub-case 1 and sub-case 3 support

For bullet 1 above, before discussion, we quote the calculation process of Layer 3 filtering from TS 38.331 as follows:

#### 5.5.3.2 Layer 3 filtering The UE shall: 1> for each cell measurement quantity, each beam measurement quantity, each sidelink measurement quantity as needed in clause 5.8.10, for each CLI measurement quantity that the UE performs measurements according to 5.5.3.1, for each L2 U2N Relay UE measurement quantity according to 5.5.3.4, for evaluating the selected or detected NR sidelink U2N Relay UEs according to 5.8.15.3, for evaluating the SyncRef UE according to 5.8.5 and 5.8.6, for evaluating the NR sidelink U2U Relay/Remote UE threshold conditions according to 5.8.16.2 and 5.8.17.2, for evaluating the conditions for selection and reselection of NR sidelink U2U Relay UE according to 5.8.17.3, and for evaluating the detected NR sidelink U2U Relay UEs according to 5.8.17.4: 2> filter the measured result, before using for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation or for evaluating the SyncRef UE, by the following formula: Fn = (1 – a)*Fn-1 + a*Mn where Mn is the latest received measurement result from the physical layer; Fn is the updated filtered measurement result, that is used for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation or for evaluating the SyncRef UE; Fn-1 is the old filtered measurement result, where F0 is set to M1 when the first measurement result from the physical layer is received; and for MeasObjectNR, a = 1/2(ki/4), where ki is the filterCoefficient for the corresponding measurement quantity of the i:th QuantityConfigNR in quantityConfigNR-List, and i is indicated by quantityConfigIndex in MeasObjectNR; for other measurements, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by the quantityConfig; for UTRA-FDD, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by quantityConfigUTRA-FDD in the QuantityConfig; 2> adapt the filter such that the time characteristics of the filter are preserved at different input rates, observing that the filterCoefficient k assumes a sample rate equal to X ms; The value of X is equivalent to one intra-frequency L1 measurement period as defined in TS 38.133 [14] assuming non-DRX operation, and depends on frequency range. NOTE 1: If k is set to 0, no layer 3 filtering is applicable. NOTE 2: The filtering is performed in the same domain as used for evaluation of reporting criteria, for measurement reporting, for U2N/U2U Relay (re)selection evaluation or for evaluating the SyncRef UE, i.e., logarithmic filtering for logarithmic measurements. NOTE 3: The filter input rate is implementation dependent, to fulfil the performance requirements set in TS 38.133 [14]. For further details about the physical layer measurements, see TS 38.133 [14]. NOTE 4: For CLI-RSSI measurement, it is up to UE implementation whether to reset filtering upon BWP switch. NOTE 5: For SSB measurements when multiple altitude range-based ssb-ToMeasure are configured, it is up to UE implementation whether to reset filtering upon entering a different altitude range. NOTE 6: Upon satellite switch with resynchronization, it is up to UE implementation to reset filtering for the serving cell.

If ki, i.e. the co-efficient, is set to 0, the filtered result Fn is equal to Mn. As Note 1 explains, this means no layer 3 filtering is applicable. Meanwhile, Mn is defined as the latest received layer 1 result. Based on this, it can be concluded that sub-case 1 and 3 are feasible without additional specifications impacts.

Proposal 2: RAN2 confirms sub-case 1 and 3 support for NW-sided model is feasible without additional specifications impacts.

Measurement reporting period

It should be noted that currently the minimum reporting period value in specifications is 120ms, which is twice the sampling period which was used in the simulation assumptions, as in [2]. This may lead to the performance of the prediction to be lower than in the simulations. To address this, RAN2 should consider introducing a shorter reporting time interval values in the measurement report configuration.

Proposal 3: Shorter reporting periods should be introduced for measurement report configuration to improve an accuracy of the prediction.

It should be noted that in combination with the previous agreement to have a possibility for the UE to report multiple measurement results in one message, this should not impose additional signalling overhead.

UE-awareness of NW-sided model inference

Another issue for the measurement framework is about the UE awareness of measurements being performed for the sake of NW-sided model prediction, as noted in the TR 38.744 [2]:

##### 6.1.2.2.2 Monitoring and management For performance monitoring of network-sided model, the legacy RRM measurement configuration and reporting framework in RRC layer can be reused i.e. no spec impact is identified. And UE will not be informed about any network-sided functionality management decision. Editor Note1: FFS on UE awareness and preference when legacy measurement result is reported for network-sided model

Firstly, it is not entirely clear what the purpose of such awareness would be. Even though the measurements are used to feed the prediction algorithm at the network side, the results of the prediction or performance monitoring is still to make the best mobility decisions for a particular UE. Therefore, it is unacceptable for these measurements to have lower performance requirements or be ignored by the UE in some situations.

Observation 6: The measurement results reported by the UE for the sake of NW-sided model based RRM measurement/event prediction need to meet the same performance requirements as any other RRM measurements. Otherwise, the mobility performance of the UE would be deteriorated.

Furthermore, how the network configures RRM measurements and how it performs mobility decisions is up to NW implementation. We see no reason for the UE to indicate its preference on how the network should perform RRM tasks, even if AIML based mobility is utilized at the NW side. In fact, the UE has no reason to know whether the mobility is based on traditional methods or AIML algorithms.The same measurement results can be used by the network for many purposes, so such indication would not be even feasible in most cases. Based on these considerations, we can conclude there is no need for the UE to know whether the measurement report is used as an input for NW-sided based measurement/event prediction, and the UE should just measure and report as configured.

Proposal 4: There is no need for the UE awareness or preference indication on whether RRM measurement result is/can be used by the NW as input to NW-sided model based AIML algorithms.

## Assistance information for the NW-side model

Based on the simulation discussion from RAN2#129, the generalization performance for UE speed and frequency was evaluated and the following observation were agreed.

Agreements on generalization over UE speed 1 For intra-frequency temporal domain prediction in FR1/FR2, generalization performs well across all UE speeds 2 Using a mixed dataset (GC#2) slightly improves the accuracy of the AI/ML model compared to GC#1 cases, while offering comparable accuracy as the baseline case (for the same data set size). 3 For GC#1, as the difference between the UE speed that an AI/ML model trained at and the UE speed that the inference is being made decreases, the AI/ML achieves a closer performance to the baseline Agreements on generalization over frequency 1. Generalization using GC#2 always outperform that of GC#1 2. Training and generalization using the knowledge about the input & output frequency (or even an indication) outperforms the case where the model cannot recognize which frequency at the input or output. 3. Considering the generalization parameter of different predicted frequencies in inter-freq prediction, the GC#1 case without any preprocessing based on the information of predicted frequency suffers from significant performance loss. 4. For GC#2 case, the prediction accuracy is acceptable and close to the baseline.

Then in RAN2 #129b, the generalization performance for Uma/UMi is further evaluated and agreed as the follows.

Agreements on generalization over UMa/UMi 1 For Case A and B temporal domain prediction in FR1 and FR2, RAN2 to capture the observation that it is generalizable for the studied cases over cell configurations with different deployment scenarios. 2 For Case A and B temporal domain prediction in FR1 and FR2, RAN2 to capture the observation that GC#2 slightly improves the accuracy of the AI/ML model compared to GC#1, while offering comparable accuracy as baseline. 3 For intra-frequency temporal domain prediction in FR1/FR2, the model trained in UMi scenario shows better prediction accuracy when tested in UMa scenario than the model trained in UMa scenario when tested in UMi scenario. 4 No more generalization work is expected.

From the above agreements, we can observe that the prediction generalizes well among different UE speeds and UMa/UMi scenarios. For frequency domain prediction, the GC#2 can also achieve close accuracy comparing to the baseline. Therefore, we can observe that to achieve a required level of prediction accuracy, no information other than L1/L3 RSRP measurement results are needed.

Proposal 5: No assistance information from the UE is needed to support NW-side model inference other than L1/L3 RSRP beam/cell level measurement results.

# Conclusion

In this contribution, we have the following observations and proposals:

Spatial Domain Prediction

Observation 1: The UE-sided spatial domain prediction will no more be discussed in Rel-19 and Rel-20, while NW-sided requires further analysis and conclusion.

Observation 2: Inference using NW-sided model for spatial domain prediction in beam dimension does not require additional specification impacts on top of what is already needed/agreed to support other scenarios (i.e. temporal/frequency domain).

Observation 3: NW-side data collection mechanism specified in Rel-19 for beam management use case can be reused to support data collection for NW-side spatial domain prediction using RRM sub-case 1 and 3.

Observation 4: To support sub-use case 2 for spatial domain prediction in beam dimension at the NW-side, data collection mechanism needs to be extended to collect L3 beam level results which is the same requirement as for beam level prediction for other scenarios (e.g. temporal domain prediction) using NW-sided model and which was already agreed by RAN2.

Observation 5: The data collection for spatial domain RRM prediction in beam dimension is already supported using RRM sub-case 1/3, and will only bring limited impacts to support RRM sub-case 2 which are required to support beam level prediction for other scenarios anyway.

Proposal 1: RAN2 to confirm that intra-frequency spatial domain RRM prediction in beam dimension using NW-sided model is feasible, has minor specifications impact and there are no obstacles to include it in the normative phase of the work.

Measurement framework impacts

Proposal 2: RAN2 confirms sub-case 1 and 3 support for NW-sided model is feasible without additional specifications impacts.

Proposal 3: Shorter reporting periods should be introduced for measurement report configuration to improve an accuracy of the prediction.

Observation 6: The measurement results reported by the UE for the sake of NW-sided model based RRM measurement/event prediction need to meet the same performance requirements as any other RRM measurements. Otherwise, the mobility performance of the UE would be deteriorated.

Proposal 4: There is no need for the UE awareness or preference indication on whether RRM measurement result is/can be used by the NW as input to NW-sided model based AIML algorithms.

Assistance information for the NW-side model

Proposal 5: No assistance information from the UE is needed to support NW-side model inference other than L1/L3 RSRP beam/cell level measurement results.

# References

- [1] RP-251864		New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR.

- [2] R2-2501822	3GPP TR 38.744 V0.0.7 (2025-4)	Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR.