3GPP TSG-RAN WG2 Meeting #131              			                                R2-2505696

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.6.4

Source: 	Lenovo

Title: 	Remaining issues for CLTM

Document for:	Discussion and decision

Introduction

In the previous RAN2 meeting, we have made a good progress for Intra-CU conditional MCG LTM. This document aims to address the remaining points for the Intra-CU MCG C-LTM work.

Discussion

- Early TA handling

It was agreed that LTM Cell Switch Command MAC CE can be used to trigger cell switch for candidate cells configured for CLTM. It is possible that UE receives LTM Candidate TA Command MAC CE including early TA for CLTM candidate cells e.g., CLTM candidate cell#1. When UE receives LTM cell switch command for CLTM candidate cell#1 without early TA value, RAN2 needs to discuss whether RACH based LTM or RACH-less based LTM is performed by UE. If RACH based LTM is performed for C-LTM execution triggered by LTM cell switch command, UE needs to release the early TA for the target cell if configured. It will impact current specification.

Proposal 1: (MAC-14) If UE has received early TA MAC CE for CLTM candidate cell, UE shall perform RACHless cell switch using TA from early TA MAC CE upon the reception of LTM cell switch command without a TA value included.

We consider the case that one CLTM candidate cell is configured with MIMO 2TA. UE receives LTM candidate TA MAC CE for TAG#1 of CLTM candidate cell. UE performs cell switch towards another TAG e.g., TAG#0 due to the reception of LTM cell switch command including early TA for TAG#0. How to handle the early TA for TAG#1 of the CLTM target cell after LTM cell switch? It was agreed that upon C-LTM execution towards the target cell, UE maintains the TA value and keep the early TA timer running for other candidate cells. Therefore, we can follow the similar principle.

Proposal 2: Upon CLTM execution towards the first TAG of the target cell, UE will maintain the TA value and keep the early TA timer running for the second TAG of the same target cell in case two TAGs are configured.

- CLTM with PDCCH order TA and UE based TA

The following was agreed for the case that only PDCCH order-based TA is considered for CLTM execution.

- Upon CLTM execution, if TA is valid (i.e. TAT timer is running) at the first available CG occasion, and there is CG resource configured, RACH-less CLTM will be performed.

- During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, fallback to RACH-based CLTM.

The section aims to address the case that both PDCCH order-based TA and UE based TA are available when CLTM is triggered. Regarding one CLTM candidate cell, PDCCH order-based TA will be transmitted by LTM Candidate Timing Advance Command MAC CE. In addition, UE based TA is also supported by CLTM. In legacy Rel-18 LTM, the PDCCH order-based TA has high priority if both TA from MAC CE and UE based TA are available. Therefore, we suggest following the similar principle. UE performs RACHless CLTM using TA from MAC CE if this TA is available in the first available CG resource i.e., the TAT is running in the first available CG resource. Otherwise, UE performs RACHless CLTM using UE based TA.

Proposal 3: Upon CLTM execution with TA from MAC CE and UE based TA, UE performs RACHless CLTM using TA from MAC CE if this TA is available in the first available CG resource i.e., the TAT is running in the first available CG resource. Otherwise, UE performs RACHless CLTM using UE based TA.

During CLTM using TA from MAC CE is ongoing, after the first transmission, if the TAT timer expires while RACH-less LTM using TA from MAC CE is ongoing, UE can fall back to RACHless CLTM using UE based TA in case two TAGs are not configured for the CLTM candidate cell.

Regarding the CLTM with 2 TAGs, we need to discuss two sub-cases as follows.

- Sub-case#1: TA from MAC CE and UE based TA are available for the same TAG.  In this sub-case, we can follow the case without two TAGs configured.

- Sub-case#2: TA from MAC CE and UE based TA from the different TAG are available. During CLTM using TA from MAC CE is ongoing, after the first transmission, if the TAT timer expires while RACH-less LTM using TA from MAC CE is ongoing, UE still can fall back to RACHless CLTM using UE based TA. The question is whether to restart T304 since UE accesses to the different TRP. To make the procedure simple, we suggest keeping T304 running.

Proposal 4: During CLTM is ongoing using TA from MAC CE, after the first transmission, if the TAT timer expires while RACH-less LTM using TA from MAC CE is ongoing, UE can fall back to RACHless CLTM using UE based TA.

- Candidate cell/beam selection

Regarding CLTM candidate cells associated with L1 or L3 execution condition, UE will perform CLTM execution if at least one of these CLTM candidate cell meets the execution condition. It is possible that some of these candidate cells have valid early TA but not the others. If the execution conditions from more than one CLTM candidate cells are met, UE can select one of them for cell switch. It is better to select one CLTM candidate cell which has valid early TA. There could be different ways to select a target cell e.g., based on best measurement quality, or select a candidate as target for which the remaining timer alignment timer has the highest value etc. But for simplicity if multiple candidate cells meeting the execution condition have valid early TA, it can be selected as target cell.

Proposal 5: If multiple CLTM candidate cells meet the L1 (or L3) execution condition, the target cell for CLTM execution can be selected among the candidate cells for which the UE has a valid TA. If the valid TA is not available for any candidate cell, the UE can select any candidate cell.

For L1 based CLTM candidate cell without MIMO 2TA, the C-LTM execution is triggered when at least one beam fulfils the CLTM event condition. When multiple candidate beams satisfy the CLTM condition, it is up to UE implementation to select a beam and perform C-LTM. Now, we consider the case that CLTM candidate cell is configured with MIMO 2TA. UE receives LTM TAC MAC CE for TAG#1 of CLTM candidate cell. If multiple beams of LTM candidate cell meet the condition, the beam which has valid early TA should be selected in priority.

Proposal 6: In the case of CLTM candidate cell with MIMO 2TA, UE prioritizes the beam which has valid early TA and satisfy the execution condition.

Conclusion

This document attempted to develop a basic understanding of “conditional LTM” and proposed some first scenarios for RAN2 consideration. In summary, we have following observations and proposals:

Early TA delivery

Proposal 1: (MAC-14) If UE has received early TA MAC CE for CLTM candidate cell, UE shall perform RACHless cell switch using TA from early TA MAC CE upon the reception of LTM cell switch command without a TA value included.

Proposal 2: Upon CLTM execution towards the first TAG of the target cell, UE will maintain the TA value and keep the early TA timer running for the second TAG of the same target cell in case two TAGs are configured.

CLTM with PDCCH order TA and UE based TA

Proposal 3: Upon CLTM execution with TA from MAC CE and UE based TA, UE performs RACHless CLTM using TA from MAC CE if this TA is available in the first available CG resource i.e., the TAT is running in the first available CG resource. Otherwise, UE performs RACHless CLTM using UE based TA.

Proposal 4: During CLTM is ongoing using TA from MAC CE, after the first transmission, if the TAT timer expires while RACH-less LTM using TA from MAC CE is ongoing, UE can fall back to RACHless CLTM using UE based TA.

Candidate cell/beam selection

Proposal 5: If multiple CLTM candidate cells meet the L1 (or L3) execution condition, the target cell for CLTM execution can be selected among the candidate cells for which the UE has a valid TA. If the valid TA is not available for any candidate cell, the UE can select any candidate cell.

Proposal 6: In the case of CLTM candidate cell with MIMO 2TA, UE prioritizes the beam which has valid early TA and satisfy the execution condition.

References

- Chair notes for RAN2#130