3GPP TSG-RAN WG2 #131                                          R2-2505705

Bengaluru, IN, Aug. 25th – 29th, 2025

Agenda Item:	8.7.5 RLC AM Enhancements

Source:	NEC, Fujitsu

Title:	Clarifications on RLC AM

Document for:	Discussion, Decision

# 1.Introduction

In this document, we discuss following open issues:

- Open issue RLC-14, we propose to discard the outdated PDUs even if its transmission has been attempted, for specification clarity

- Open issue PDCP-2, whether PDCP SN gap report should be triggered for the SDUs whose retransmission stops

- Open issue RLC-11, polling triggered but no SDU in buffer to transmit

- Further clarification on enhanced polling trigger

# 2 Discussion

## 2.1 Stop retransmission relevant

Open issue RLC-14 and PDCP-2: Stop retransmission, SDU discard, RLC/PDCP SN gap, PDCP SN gap report

RLC-14 If stopReTxDiscardedSDU is configured, when indicated from upper layer to discard a particular RLC SDU, whether the transmitting side of an AM RLC entity shall discard the corresponding RLC SDU or RLC SDU segment(s) if the RLC SDU or the RLC SDU segment(s) thereof has been submitted to lower layers.

PDCP-2 Whether the PDCP triggers a PDCP SN gap report when at least one byte for the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers.

In legacy, for these PDCP SDUs, which or part of which has been submitted to lower layer, it cannot be discarded, and retransmission of it will continue until it is delivered to receiving RLC/PDCP or reaches the maximum transmission times. This is clear from section 5.4 of RLC spec as follows:

## 5.4 SDU discard procedures When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

The reason of having legacy behaviour is to avoid RLC SN gap, because RLC SN gap will cause Rx window stalling. Consequently, PDCP SN gap shall not be reported for these PDCP SDU, and PDCP receiver side should wait for receiving these PDCP SDUs even though they are outdated.

Observation 1: In Rel-18, for the out-dated PDCP SDU(s), which have been submitted to lower layer, we have following treatment:

- Not to discard them

- Continue their (re)transmission

- They will not create RLC/PDCP SN gap

- They will not trigger PDCP SN gap report

However, with RLC AM enhancement in Rel19, things change. Current specification has very inconsistent treatment for these PDCP SDU, here is our observation:

Observation 2: In Rel-19 with RLC AM enhancement, for the out-dated PDCP SDU(s), which have been submitted to lower layer, we have following treatment in running CR:

- not to discard them,

- but stop their (re)transmission

- they will create RLC/PDCP SN gap

- but they will not trigger PDCP SN gap report

In our opinion, because of RLC AM enhancement, it is neither necessary nor possible to avoid RLC SN gap as well as PDCP SN gap. Fortunately, enhanced RLC Rx side will not wait for these PDUs forever and an SN gap will not cause window stalling anymore. To move PDCP Rx window forward without unnecessary re-ordering delay, it would be better to trigger PDCP SN gap report for these PDCP PDUs whose (re)transmission have stopped. Alternatively, a shorter PDCP layer t-reordering timer should be configured by considering the value of t-RxDiscard timer in RLC layer. For robustness and consistency, both mechanisms could be considered.

Moreover, if retransmission of an RLC SDU stops, it is also unnecessary to buffer it. Keep them in buffer will only occupy more UE buffer, cause confusion and extra specification effort as below:

## 5.3.3.4 Expiry of t-PollRetransmit Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements and excluding RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped); or - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling): - consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or - consider any RLC SDU which has not been positively acknowledged for retransmission. - include a poll in an AMD PDU as described in clause 5.3.3.2.

When the combined approach to avoid unnecessary retransmission was discussed, it was agreed that the discard at Tx side relies on existing mechanism, which may have less impact on specification. However, with current Tx side approach that the transmission or retransmission is stopped when a discard indication is received from upper layer, the specification change is actually larger. For example, specification changes considering the RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped exist in several places in 5.3.3.2 and 5.3.3.4 (as above) in the RLC running CR. If the RLC SDU is discarded at Tx side, all these changes can be avoided. Therefore, we may revisit the issue on whether to discard RLC SDU when stopReTxDiscardedSDU is configured and the discard indication is received from upper layer.

Observation 3: If Tx side discards the RLC SDU for which the (re)transmission is stopped, specification changes related to RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped in 5.3.3.2 and 5.3.3.4 in current RLC running CR can be avoided.

In summary, we think following treatment is better and consistent:

Observation4: In Rel-19 with RLC AM enhancement, for the out-dated PDCP SDU(s), which have been submitted to lower layer, we believe more consistent treatment should be:

- Discard them,

- stop their (re)transmission

- they will create RLC/PDCP SN gap

- they will trigger PDCP SN gap report

Our suggested TP to RLC specification is as below:

## 5.4 SDU discard procedures When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall: If stopRxTxDiscardedSDU is not configured: - discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU. Else - discard the indicated RLC SDU (consequently, this RLC SDU and any segment thereof are not pending for initial transmission or retransmission)

Our suggested TP to PDCP specification is as below:

5.16.1 Transmit operation For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - the discarded PDCP SDU(s) either have not been submitted by any RLC entity to lower layers or their (re)transmission has been stopped before successful delivery is confirmed.

Proposal 1: [RLC-14] Tx side discards the RLC SDUs whose (re)transmission stops

Proposal 2: [PDCP-2] PDCP SN gap report should be triggered for these PDCP SDUs whose (re)transmission stops.

## 2.2 Polling enhancement relevant open issues

RLC-11:  Polling in case that (re)transmission of all buffered SDUs are stopped

| RLC-11 | FFS how to handle the case “ if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with” - As raised by Nokia and Apple below RAN2 agreement in #130 meeting: Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting) Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work | Issue Type: Not essential but important How to address it: based on companies’ contribution |
| --- | --- | --- |

Firstly, in above case, there is no SDUs in the buffer waiting for retransmission or for acknowledgment, so there is no need to poll at all.

Secondly, the case of empty buffer has existed even in legacy and polling bit will not be sent out in this case. As follows described in highlighted words, polling will be included only when there is transmission opportunity and there is buffered SDU submitted for transmission or retransmission. In case there is no buffered SDU for transmission or retransmission, even if polling/polling retransmission is triggered, TX will not be able to send out polling bit.

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall: - increment PDU_WITHOUT_POLL by one. - increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU; - if PDU_WITHOUT_POLL >= pollPDU; or - if BYTE_WITHOUT_POLL >= pollByte: - include a poll in the AMD PDU as described below. Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or - if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling); - include a poll in the AMD PDU as described below.

The case mentioned in FFS in our understanding is more about TX window stalling, if we need to solve it, we should allow the Tx window moves forward considering the SDU whose transmission stops. However, it seems the majority view that there is no need to solve window stalling issue.

Proposal 3: [RLC-11] In case “there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with”, RAN2 confirm no need to poll or retransmit the poll for any acknowledgement (no spec change).

Other open issue: details of the enhanced polling condition

The condition of remaining time-based polling is specified in PDCP running CR, as follows:

For the purpose of remaining-time-based RLC polling, if the remaining time till discardTimer expiry becomes less than the remainingTimeBasedPollingThreshold for the PDCP SDU for which the corresponding PDCP Data PDU has already been submitted to lower layers but for which successful delivery has not been confirmed by lower layers, the transmitting PDCP entity shall: - indicate to lower layers that the condition for remaining-time-based RLC polling is met for the corresponding PDCP Data PDU.

In our understanding, not only for these RLC SDU or RLC SDU segments which has been ACKed, but also for these RLC SDU or RLC SDU segments which is NACKed and pending for retransmission, further polling is not needed. If we agree NACKed ones in addition to ACKed ones are excluded from triggering polling, we at least need to define the NACKed condition checking in RLC specification, because only RLC layer knows the NACKed status.

An example TP to RLC specification is provided as below, and can work together with the existing text in PDCP running CR:

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements and excluding RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped as specified in clause 5.2.3.1.1) after the transmission of the AMD PDU; or - if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling); or - if an indication is received from upper layer (e.g., PDCP) that the condition for remaining-time-based RLC polling has been met for an RLC SDU which is waiting for acknowledgment in the TX buffer: - include a poll in the AMD PDU as described below.

Proposal 4: For these RLC SDUs or RLC SDU segments which has been NACked (in addition to ACked) should be excluded from triggering polling.

# 3 Conclusion

In this contribution, we provided the following observations and proposals regarding the details of RLC AM enhancement:

Observation 1: In Rel-18, for the out-dated PDCP SDU(s), which have been submitted to lower layer, we have following treatment:

- Not to discard them

- Continue their (re)transmission

- They will not create RLC/PDCP SN gap

- They will not trigger PDCP SN gap report

Observation 2: In Rel-19 with RLC AM enhancement, for the out-dated PDCP SDU(s), which have been submitted to lower layer, we have following treatment in running CR:

- not to discard them,

- but stop their (re)transmission

- they will create RLC/PDCP SN gap

- but they will not trigger PDCP SN gap report

Observation 3: If Tx side discards the RLC SDU for which the (re)transmission is stopped, specification changes related to RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped in 5.3.3.2 and 5.3.3.4 in current RLC running CR can be avoided.

Observation4: In Rel-19 with RLC AM enhancement, for the out-dated PDCP SDU(s), which have been submitted to lower layer, we believe more consistent treatment should be:

- Discard them,

- stop their (re)transmission

- they will create RLC/PDCP SN gap

- they will trigger PDCP SN gap report

Proposal 1: [RLC-14] Tx side discards the RLC SDUs whose (re)transmission stops

Proposal 2: [PDCP-2] PDCP SN gap report should be triggered for these PDCP SDUs whose (re)transmission stops.

Proposal 3: [RLC-11] In case “there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with”, RAN2 confirm no need to poll or retransmit the poll for any acknowledgement (no spec change).

Proposal 4: For these RLC SDUs or RLC SDU segments which has been NACked (in addition to ACked) should be excluded from triggering polling.