3GPP TSG-RAN WG2 Meeting #R2-131		        R2-2505729      Bangalore, India, Aug 25-29,2025

Agenda Item:	8.6.4

Source: 	Rakuten

Title:  	Remaining issues of Conditional LTM

WID/SID:           NR_Mob_Ph4-Core- Release 19

Document for:	Discussion, Decision

# Introduction

In RAN #105, the Rel-19 WID on NR mobility enhancements Phase 4 was updated [1], where the objectives relevant to conditional LTM (CLTM) are described below:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

The primary motivation for CLTM is to introduce robustness to the LTM procedure where the UE may not be able to receive the LTM cell-switch command from the serving gNB-DU, which may lead to radio link failure. The conditional LTM is expected to bring the benefits of robustness while retaining the interruption time reduction benefits of LTM.

In this contribution, we discuss remaining issues of CLTM.

# Discussion

## CLTM cell switch notification to the gNB-CU

In Rel 18 LTM, the gNB-DU notifies the gNB-CU after the LTM cell switch command is delivered to the UE. This ensures that the gNB-CU is always aware of the UE’s current serving cell.

In CLTM, the gNB-DU doesn’t deliver a cell switch command, instead the CLTM cell switch is based on CLTM execution condition. Hence, we there is a need to address this issue using an alternative mechanism.

The UE may inform the source gNB-DU of the selected CLTM target cell right when the CLTM execution condition is satisfied, but this may not be reliable as there could be a too-late HO and the message may not successfully reach the source gNB-DU.

Hence, we propose that the UE informs the target gNB-DU of the target cell configuration index sent over RRC, after a successful CLTM cell switch. The target gNB-DU can send the same to the gNB-CU, using which the gNB-CU can identify the UE’s current serving cell.

UE informs the target gNB-DU of the target cell configuration index sent over RRC, after a successful CLTM cell switch.

## CLTM for a MIMO 2TA UE

In RAN2#129bis made the below agreements for MIMO 2 TA.

RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE. For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

When a UE is configured with mTRP and with C-LTM, we have the following.

- There are at least two TRPs configured for the UE, which will have 2TAs corresponding to each TRP.

- The TRPs could belong to different cells, which could belong to same or different gNB-DU, but the same gNB.

- The UE can be configured with one or more C-LTM candidate cells coupled with a CLTM execution condition.

- The C-LTM candidate cells and the execution condition are TRP specific i.e candidate cells and execution condition of one TRP aren’t applicable to the other.

In CLTM, since there is no gNB control over the cell switch execution, there is a possibility where only one of the TRPs satisfy the CLTM execution condition, while the other does not. There are the following feasible alternatives when only one of the TRPs satisfy the CLTM execution condition.

- The UE performs RACH-less C-LTM cell switch for only that TRP. The other TRPs are retained with their serving cell, until either of the following occurs

  - TRP execution condition(s) is satisfied, and C-LTM cell switch can be executed.

  - UE detects RLF for the TRP and the TRP is autonomously discarded by the UE. No RRC re-establishment is performed after RLF detection as there is another TRP active. TRP recovery can be attempted, if configured and feasible.

- One of the TRPs is designated as the master TRP by the gNB and the UE performs C-LTM cell switch only if the execution condition(s) of the master TRP is satisfied.

- C-LTM cell switch is performed only when both TRPs have the execution condition(s) are satisfied.

- C-LTM candidates are never prepared with mTRP (only prepared with a single TRP). If C-LTM execution condition is satisfied for one TRP at the source, the UE performs C-LTM cell switch. The network may configure mTRP again after a successful C-LTM cell switch.

RAN2 discusses and agrees a way forward CLTM handling of a mTRP UE.

# Conclusion

We have the following proposals:

Proposal 1	UE informs the target gNB-DU of the target cell configuration index sent over RRC, after a successful CLTM cell switch.

Proposal 2	RAN2 discusses and agrees a way forward CLTM handling of a mTRP UE.

# References

RP-242356,   “Revised Work Item: NR mobility enhancements Phase 4”

RAN2 Chairman Notes R2_127bis_ChairNotes

RAN2 Chairman Notes R2_128_ChairNotes