3GPP TSG-RAN WG2 Meeting #131                                 R2-2505769

Bangalore, India, Aug.25th- 29th, 2025

Agenda item:		8.2.3

Source:		Samsung

Title:			Discussion on the remaining issues on A-IoT RA procedure

Document for:	Discussion & Decision

- 1 Introduction

In RAN2#130, the AIoT RACH procedures were extensively discussed with the following agreements:

Agreements

- For Msg1 resource selection procedure capture as guidance the countdown behaviour in the MAC specification (use TP in R2-2503952).  Capture a NOTE that other implementation are allowed.   X, Y will be signalled by paging message

- The start of the first set of MSG1 resources is indicated by Paging message directly instead of the new R2D trigger messages.  R2D trigger message is not sent in CFRA procedure.   Come back if RAN1/4 sees any issues.  Send LS to RAN1/RAN4

- FFS  R2D byte alignment dependent on TBS size discussion

Agreements on RA

1	Exclude the option of  MSG2 transmission and any retransmission of MSG2 happens within a predefined time window (based on timer)

2	A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further downselected between option B and C below.  A device receiving MSG2 within this boundary transmits MSG3. The device does not process MSG2 (re)transmission received after the boundary.

Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message

Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS)

Option A (the boundary being the subsequent paging only) is excluded.

For option C, further discuss in terms of complexity at the device vs reader flexibility.

3	Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible.  FFS Discuss further whether to include it.

Agreements on NACK reception:

After MSG3 transmission, upon receiving NACK with its AS ID before subsequent paging or command addressed to this device from the reader, device determines it will perform re-access.   FFS how to specify.

Agreements on RN16/AS ID maintainance:

Confirm a device is not expected to maintain both AS ID and RN16.   After msg2 reception, RN16 becomes AS ID, if new AS ID was not assigned by reader.

This implies that the reader cannot change AS ID and RN16 pair across message 2 retransmission.  How to capture device behavior is FFS

In this contribution, we will share our views on the remaining issues.

- 2 Discussion

Issue 1: CBRA failure detection

For CBRA failure detection, the current running CR captures the condition of reception of A-IoT paging message after initialization of CBRA. The remaining issue is how to determine the CBRA failure after Msg.1 transmission when the device receives the Access trigger message. Two options can be considered:

- Option 1: the reception of next Access Trigger message

This option assumes that the Access Trigger message is transmitted after the Msg.2 transmission for the Msg.1 triggered by the last access trigger message is completed, as shown in the following figure:

- Option 2: the reception of k-th Access Trigger message

This option assumes that multiple access trigger messages can be sent before the reader sends the Msg.2, as shown in the following figure. The intention for such assumption is to allow the reader have the implementation flexibility for Msg.2 scheduling. To realize such flexibility, the reader needs to configure the K value to the devices via, e.g., A-IoT paging message. Moreover, this option requires the device to count the number of Access Trigger messages, which introduces additional complexity.

Considering Rel-19 is the first version of A-IoT, the support of fundamental features is important. Option 2 adds more signalling overhead, and device complexity. Thus, we prefer to Option 1.

Proposal 1: RAN2 is kindly asked to agree that the CBRA failure can be detected when the device receives the next Access Trigger message after Msg.1 transmission.

Issue 2: Frequency index in Msg.2 (to reduce the collision of Msg.1)

In last meeting, RAN2 agreed that it is feasible to include the Frequency index used for Msg. 1 transmission in Msg. 2. The intention is to reduce the collision for Msg. 1 transmission, e.g., different devices transmit the same random ID to the reader by using different frequency domain resource. To realize this, Msg. 2 should include the frequency index associated with each Echoed Random ID. The collision issue is related to the length of random ID, which is 16, and the collision probability is low enough. Thus, there is no need to introduce frequency index in Msg. 2.

Proposal 2: RAN2 is kindly asked to agree that the frequency index in Msg. 2 is not needed for Msg.1 collision resolution.

- 3 Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1: RAN2 is kindly asked to agree that the CBRA failure can be detected when the device receives the next Access Trigger message after Msg.1 transmission.

Proposal 2: RAN2 is kindly asked to agree that the frequency index in Msg. 2 is not needed for Msg.1 collision resolution.