3GPP TSG-RAN2 Meeting #131	R2-2505818

Bangalore, India, Aug 25th – 29th, 2025

Agenda item:	8.2.3

Source:	Qualcomm Incorporated

Title:	Views on Random Access Aspects of Ambient IoT

WID/SID:	Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# Introduction

The Rel-19 Ambient IoT WI was approved in [1] and the RAN2 objectives were described as follows:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included. - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

RAN2 #129 made the following agreements regarding the A-IoT random access.

Agreements

- For Rel-19, only 3-step CBRA is supported for A-IoT

- We will specify both CBRA and CFRA.

- Re-use the subsequent paging message to trigger re-access.  There is no need to differentiate msg1 resource for initial access vs re-access.

Agreements

- NACK based mechanism is supported for D2R messages to determine re-access for at least msg3.  FFS details including whether we need a timer or explicit message and when reader sends feedback

- RAN2 assumes that device randomly selects among FDMA occasions as the baseline.

RAN2 #129bis made the following agreements regarding the A-IoT random access.

- Agreements on msg1

      - In case of CBRA, only 16 bits random ID is included in Msg1.  FFS can be revisited if message type will be needed for other D2R messages purposes

      - RN16 is not included in the first D2R message in the CFRA procedure.  AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader.   FFS if we can assume or need to support multiple device scenario.

Agreements on new R2D message

- A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling.   The R2D message indicates the start of a set of MSG1 resources that were configured in paging message.

- Assumption: The R2D message does not include slot number/count down number.

Agreements on msg 2

- A-IoT Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices.

- Same Msg2 format is used for initial transmission and retransmission of Msg2.

Agreements on NACK

- For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3.   May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion).

- For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access.    No need for a timer.   FFS whether subsequent R2D message is trigger message or paging

- For CFRA, NACK feedback and re-access is not supported.  FFS how to achieve

- FFS on end of procedure

RAN2 #130 made the following agreements regarding the A-IoT random access.

Agreements

- For Msg1 resource selection procedure capture as guidance the countdown behaviour in the MAC specification (use TP in R2-2503952).  Capture a NOTE that other implementation are allowed.   X, Y will be signalled by paging message

- The start of the first set of MSG1 resources is indicated by Paging message directly instead of the new R2D trigger messages.  R2D trigger message is not sent in CFRA procedure.   Come back if RAN1/4 sees any issues.  Send LS to RAN1/RAN4

- FFS  R2D byte alignment dependent on TBS size discussion

- Agreements on RA - 1 Exclude the option of MSG2 transmission and any retransmission of MSG2 happens within a predefined time window (based on timer) 2 A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further downselected between option B and C below. A device receiving MSG2 within this boundary transmits MSG3. The device does not process MSG2 (re)transmission received after the boundary. - Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message - Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS) - Option A (the boundary being the subsequent paging only) is excluded. For option C, further discuss in terms of complexity at the device vs reader flexibility. 3 Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible. FFS Discuss further whether to include it. Agreements on NACK reception: - After MSG3 transmission, upon receiving NACK with its AS ID before subsequent paging or command addressed to this device from the reader, device determines it will perform re-access. FFS how to specify. Agreements on RN16/AS ID maintainance: - Confirm a device is not expected to maintain both AS ID and RN16. After msg2 reception, RN16 becomes AS ID, if new AS ID was not assigned by reader. This implies that the reader cannot change AS ID and RN16 pair across message 2 retransmission. How to capture device behavior is FFS

In this paper, we will discuss the random-access aspects of Ambient IoT (A-IoT).

# Discussion

## A-IoT Msg2 monitoring boundary

When to receive A-IoT Msg2 was extensively discussed in the last RAN2 meeting. The conclusion was that a boundary will be defined that the A-IoT device expect to receive the A-IoT Msg2 response before the boundary and considers the contention resolution failure if the corresponding A-IoT Msg2 is not received before the boundary.

There are at least two options of the boundary for further discussion.

- Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message

- Option C – the boundary is the reception of either the Kth R2D trigger message or the subsequent paging message (K is FFS)

Option B has less collisions of random IDs than option C, since random ID for A-IoT Msg1 are transmitted over the Access Occasions triggered by each A-IoT paging message in the configured boundary. Option C also requires additional frequency index indication along with the random ID in A-IoT Msg2.

Since A-IoT Msg2 may include the successful A-IoT devices across multiple Access Trigger messages in a boundary, the size of A-IoT Msg2 in option C may be expected to be larger than that in option B.

The latency of the successful information sent to A-IoT device is also relative larger in option C which is not friendly for the A-IoT device in terms of energy consumption. In summary, option B is much simpler from A-IoT device implementation.

Thus, the A-IoT device expects to receive the A-IoT Msg2 either before the next Access Trigger message or the subsequent paging message.

Proposal 1: A-IoT device expects to receive the A-IoT Msg2 before either the next Access Trigger message or the subsequent paging message, whichever happens earlier (i.e. Option B).

## Frequency index in Msg2

RAN2#130 agreed

3	Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible.  FFS Discuss further whether to include it.

We understand including the frequency index can reduce the collisions as long as there is no misdetection of frequency index by the Reader (or mismatch between the understanding by device vs reader). So, we have no strong view on whether to include this or not in Rel-19, either way RAN2 needs to conclude and move on.

One option is to not include frequency index in Rel-19 (since that needs further specification work without a clear need in addition to the collision avoidance provided by already agreed RN16) but revisit this in Rel-20 considering the new device types, use cases and scenarios.

Proposal 2: RAN2 discuss and decide on whether to include frequency index (in addition to the RN16) in A-IoT Msg2 in Rel-19 or leave it for Rel-20.

## ‘Bit Duration’ and ‘Frequency Resource Indication’ field

RAN1 has made the following agreements about the frequency domain resource indication.

Frequency domain resource

Agreement For D2R transmission, - The minimum Tb is 1.39μs. - Support at least the following values of Tb, Tchip, and R from the table agreed in RAN1#120bis. Tchip (μs). Tb (μs). 133.33; 66.67; 33.33; 16.67; 11.11; 8.33; 5.56; 4.17; 2.78; 2.08; 1.39; 1.04; 0.69; 0.52. 266.67; R=1; R=2; R=4; R=8; R=12; R=16; R=24; R=32; R=48; R=64; R=96; R=128; R=256. 133.33; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32; R=48; R=64; R=96; R=128. 66.67; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32; R=48; R=64. 33.33; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32. 22.22; R=1; R=2; R=4; R=8; R=16. 16.67; R=1; R=2; R=4; R=6; R=8; R=12; R=16. 11.11; R=1; R=2; R=4; R=8. 8.33; R=1; R=2; R=4; R=6; R=8. 5.56; R=1; R=2; R=4. 4.17; R=1; R=2; R=4. 2.78; R=1; R=2. 2.08; R=1; R=2. 1.39; R=1. 1.04; R=1. Note: This does not imply the device to pre-store the table. Agreement At least for CBRA, for FDMA of multiple Msg1 transmissions in response to a R2D transmission triggering random access, support only the same data rate for the FDMed Msg1 transmissions. Agreement For indication of frequency domain resources for Msg 1 transmissions when Y≥1, the reader indicates - a single bit duration Tb which is same for all frequency domain resources - a set of R values, where the possible R values correspond to the agreed table of values of Tb, Tchip, and R - note: the set of R values could be signalled using a bitmap The detailed signalling design is left to RAN2. Agreement Support the following option(s) for frequency domain resource for Msg3 transmission: - Option 2: The frequency domain resource for Msg3 can be determined based on explicit indication in the PRDCH for Msg2 transmission for one or multiples devices. - FFS: support option 1 as a default if PRDCH does not provide the indication or based on a rule - Option 1: the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device Agreement For frequency domain resource for Msg 3 transmission determined based on explicit indication in the PRDCH for Msg 2 transmission for one or multiple devices, the reader indicates: - a single bit duration, which is same for all frequency domain resources, - a set of R values, where the possible R values correspond to the agreed table of values of Tb, Tchip, and R - note: the set of R values could be signalled using a bitmap The mapping relationship between device and its Msg 3 frequency domain resource is left to RAN2. - Note: Device could determine its R value for Msg 3 transmission based on its order of random ID in Msg 2 The detailed signalling design is left to RAN2.

In MAC running CR, the ‘Bit Duration’ field is 3 bits to indicate one value among total eight, and the ‘Frequency Resource Indication’ field is an 8 bits bitmap. The total size of bits including the ‘Bit Duration’ field and ‘Frequency Resource Indication’ field are always 11 bits.

We believe the bit size of ‘Bit Duration’ field and ‘Frequency Resource Indication’ field can be optimized and the signaling overhead can be reduced. Specifically, the bit size of ‘Bit Duration’ field and ‘Frequency Resource Indication’ can be variable and the total size of these two fields is 9 bits.

Observation 1: In current MAC running CR, the Bit Duration and Frequency Resource Indication together unnecessarily takes 11 bits. There is an easy way to represent these fields with just 9 bits in total (and just 6 bits for messages triggering a single device if further optimization is desired).

The ‘Bit duration’ field can start from a ‘0’ or ‘1’ and toggle to other value to indicate the end bit of the ‘Bit duration’ field. The size of the ‘Bit duration’ field is equal to the index of the value of ‘Bit duration’. An example is shown below.

| ×τ, where τ=2×106/15000 | 2 | 1 | 1/2 | 1/4 | 1/8 | 1/16 | 1/32 | 1/96 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit Duration field value | 1 | 01 | 001 | 0001 | 00001 | 000001 | 0000001 | 00000001 |
| Bit Duration (# of bits) | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |

For ‘Frequency Resource Indication’ field, the bitmap is still used, and the size of bitmap is related to the ‘Bit Duration’ field. If ‘Bit Duration’ indicates the (about) 266.67 μs, 8 values of small frequency shit factor have to be supported using the 8 bits bitmap of ‘Frequency Resource Indication’ field. If ‘Bit Duration’ indicates the (about)133.33 μs, 7 values of small frequency shit factor have to be supported with the 7 bits bitmap of ‘Frequency Resource Indication’ field. In the end, 1 bit ‘Frequency Resource Indication’ field is for the value of (about)1.39 μs of the ‘Bit Duration’.

| ×τ, where τ=2×106/15000 | 2 | 1 | 1/2 | 1/4 | 1/8 | 1/16 | 1/32 | 1/96 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency Resource Indication field (bits) | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |

In summary, the total size of the ‘Bit Duration’ field and the ‘Frequency Resource Indication’ field are fixed 9 bits (while each field can be variable size). The scheme can be applied to the ‘D2R Scheduling Info field’ in at least the R2D message triggering multi-device cases (like CBRA).

Proposal 3: For the R2D message triggering multi-device case, the ‘Bit Duration’ and ‘Frequency Resource Indication’ in the D2R scheduling info field is jointly indicated using a total fixed length of 9 bits for the two fields.

Further, for CFA paging message or the R2D Upper Layer Data Transfer message (i.e., the ‘D2R Scheduling Info field’ in R2D message triggering single device cases), since only one value of Frequency Resource Indication is needed for the particular A-IoT device, the ‘Frequency Resource Indication’ field can be further optimized to use 3 bits to indicate one value instead of using a bitmap.

Proposal 4: For the R2D message triggering single device case, the ‘Bit Duration’ and ‘Frequency Resource Indication’ using 3 bits of each field directly indicates the value to the A-IoT device, i.e., bitmap is not used.

## Number of Access Occasions

In last RAN2 meeting, the total number of Access Occasions was agreed using the exponential way with 4 bits.

Issue (1-4) For number of access occasions introduce exponential way, 4 bits, value range FFS

Consequently, the MAC running CR has captured that the maximum number of Access Occasions could be up to 2^15 (i.e., the value when codepoint set to ‘1111’), but needs to be exactly a power of 2.

For ‘Frequency Resource Indication’ field, there could be 1 to 8 candidate locations to indicate up to 8 frequency domain resource(s), and there could be up to 2 time domain resources indicated by the ‘Time Resource Indication’ field.

The potential issue is that the indicated total number of Access Occasions (which is power of 2) cannot always match the true total number of Access Occasions which is calculated by the number of time domain resources multiplied by the number of frequency domain resources multiplied by the number of Access Trigger message(s), especially when the number of frequency resources are configured with 3, 5, 6 and 7 which is not the power of 2. For example, if number of time domain resources is 2, frequency domain resources is 3 and indicated number of Access Occasions is 2^5, it would be unclear how many Access Trigger message(s) (including the Access Occasions triggered by paging message) are used. If 5 Access Trigger message(s) are used, total number of true Access Occasions would be 2 x 3 x 5 = 30. On the other hand, if 6 total Access Triggers (paging + 5 triggers) are used, then the last set of Access Occasions cannot be more than 2. Otherwise, the total Access Occasions will be 2 x 3 x 6 = 36. In short, the true number of Access Occasions cannot be correctly indicated by any codepoint for number of Access Occasions.

Observation 2: There would be mismatch between the true Access Occasions and indicated Access Occasions as some possible Access Occasions are not power of 2.

If RAN2 would like to keep the agreement still using the exponential way to express the total number of Access Occasions, some further clarifications are needed.

One option would be to allow indicating the number of the set of Access Occasions triggered by the last Access Trigger message be smaller than the number of the Access Occasions calculated from the number of frequency domain resources multiplied by the number of time domain resources. Considering the above example, in this option, the last Access Occasion set would only include 2 Access Occasions in it. However, further discussion and clarification would be needed on whether the 2 Occasions are spread across 2 time occasions in a certain frequency, or across 2 different frequencies on a certain time occasion, etc.

Figure 1 Residual AOs in the last AO set (total true AOs is equal to indicated but further clarification needed for last AO set)

Another option is that the last set of Access Occasions is considered to be invalid unless it exactly matches with number of time domain resources multiplied by number of frequency domain resources. Only the same number of the set of Access Occasions (calculated from the number of time domain resources multiplied by the number of frequency domain resources) is triggered by each Access Trigger message. Taking the same example above, in this solution option, specification would be clarified that total 5 Access Occasion sets are used, totaling to true 30 Access Occasions even though the indicated codepoint represents 32.

Figure 2 Floor of the indicated AO Sets (total true AOs are less than indicated)

The third option is that Reader provides the total amount of the Access Occasions at least equal to the indicated value but if needed larger than the indicated number of Access Occasions from A-IoT paging message to match with the full set of Access Occasions where each set consists of number of Access Occasions which is equal to number of time domain resources multiplied by the number of frequency domain resources, so that A-IoT device can randomly select across all the possible Access Occasion sets based on the indicated number of Access Occasions. Continuing with the above example, in this option, the Reader actually provides 36 Access Occasions (= 2 (time) x 3 (freq) x 6 (sets)) although it indicates 32 in the number of Access Occasions field. The last 4 Access Occasions have to be wasted.

Figure 3 Ceiling of the indicated AO Sets (total true AOs are more than indicated)

Finally, another way around is for RAN2 to consider revisiting the agreements made in the last meeting, e.g., using the linear value for the number of Access Occasions or indicating the number of Access Trigger message(s) instead of the total number of Access Occasions. In such case, the number of Access Occasions can be easily calculated by taking product of number of time resources and frequency resources in each set and further multiply by ‘number of Access Trigger messages + 1’.

Observation 3: There are various ways to solve the mismatch issues, however all of them need some further specification work.

- Opt a: The last AO set includes lower number of AOs compared to other sets - Total true AOs is equal to indicated but further clarification needed for the AOs in the last AO set.

- Opt b: The last AO set which is not a full set is dropped - Total true AOs is less than indicated, this needs spec update to disallow device to select the AOs in the last set.

- Opt c: The last AO set is also made a full set of AOs - Total true AOs is more than indicated, this needs spec update to allow device to select from the additional AOs in the last set, otherwise those are wasted.

- Opt d: Revert earlier RAN2 agreement and indicate Number of Access Trigger Messages/sets instead of ‘Number of Access Occasions’ – Needs spec update.

RAN2 should discuss the mismatch between the true number of Access Occasions and the indicated number of Access Occasions by A-IoT paging message and decide on a solution. As discussed above, for each of the options, further specification work is needed in any case.

Proposal 5: RAN2 to discuss the mismatch between the true number of Access Occasions and the indicated number of Access Occasions by A-IoT paging message and decide on a solution.

## End of procedure indication

It is possible that the NACK feedback indication and/or other R2D message like A-IoT Msg2 are missed by the A-IoT device(s) when the R2D link is poor. Then, it can cause ambiguity between Reader and A-IoT device, e.g., Reader requires A-IoT device to re-access by sending NACK feedback indication. However, A-IoT device assumes random access is successful due to failed reception of NACK indication. Thus, an explicit ACK indication should be introduced to indicate the end of the whole procedure for the particular A-IoT device. Only if A-IoT device receives the ACK indication, the random access is determined as successful. Otherwise (if the ACK is missing), A-IoT device should perform re-access in the next A-IoT paging message.

Proposal 6: An explicit ACK is introduced to indicate the end of the whole procedure for the particular A-IoT device.

# Conclusion

Based on the discussion and observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: A-IoT device expects to receive the A-IoT Msg2 before either the next Access Trigger message or the subsequent paging message, whichever happens earlier (i.e. Option B).

Proposal 2: RAN2 discuss and decide on whether to include frequency index (in addition to the RN16) in A-IoT Msg2 in Rel-19 or leave it for Rel-20.

Observation 1: In current MAC running CR, the Bit Duration and Frequency Resource Indication together unnecessarily takes 11 bits. There is an easy way to represent these fields with just 9 bits in total (and just 6 bits for messages triggering a single device if further optimization is desired).

Proposal 3: For the R2D message triggering multi-device case, the ‘Bit Duration’ and ‘Frequency Resource Indication’ in the D2R scheduling info field is jointly indicated using a total fixed length of 9 bits for the two fields.

Proposal 4: For the R2D message triggering single device case, the ‘Bit Duration’ and ‘Frequency Resource Indication’ using 3 bits of each field directly indicates the value to the A-IoT device, i.e., bitmap is not used.

Observation 2: There would be mismatch between the true Access Occasions and indicated Access Occasions as some possible Access Occasions are not power of 2.

Observation 3: There are various ways to solve the mismatch issues, however all of them need some further specification work.

- Opt a: The last AO set includes lower number of AOs compared to other sets - Total true AOs is equal to indicated but further clarification needed for the AOs in the last AO set.

- Opt b: The last AO set which is not a full set is dropped - Total true AOs is less than indicated, this needs spec update to disallow device to select the AOs in the last set.

- Opt c: The last AO set is also made a full set of AOs - Total true AOs is more than indicated, this needs spec update to allow device to select from the additional AOs in the last set, otherwise those are wasted.

- Opt d: Revert earlier RAN2 agreement and indicate Number of Access Trigger Messages/sets instead of ‘Number of Access Occasions’ – Needs spec update.

Proposal 5: RAN2 to discuss the mismatch between the true number of Access Occasions and the indicated number of Access Occasions by A-IoT paging message and decide on a solution.

Proposal 6: An explicit ACK is introduced to indicate the end of the whole procedure for the particular A-IoT device.

# References

- RP-250796, Solutions for Ambient IoT (Internet of Things) in NR.