3GPP TSG-RAN WG2 #131	R2-2505838

Bengaluru, India, 25th – 29th August, 2025

Agenda Item:	8.1.2.2

Source:	Ericsson

Title:	LCM for UE-side models for beam management

Document for:	Discussion

# 1	Introduction

In this contribution we share our views on remaining open issues that were the outcome of the email discussion [POST130][026][AI PHY] 38.331 Running CR (Ericsson).

# 2	Discussion

## 2.1 Discussion on RRC open issues

### Open issue RRC-3: UE data collection request

In this section we discuss aspects related to this open issue, beyond those addressed in the email discussion [POST130][037][AI PHY] UE candidate data collection (Xiaomi/Ericsson). Based on RAN2 agreements and the email discussion, the NW will send a set of candidate UE data collection configurations (in otherConfig), out of which the UE can request preferred configurations. Upon reception of this request, the NW may decide to configure the UE with an actual UE data collection configuration in CSI-ReportConfig.

If the UE is configured with such a UE data collection configuration in CSI-ReportConfig, it needs to be ensured that the UE does not send another request for the corresponding candidate configuration. Ensuring this was not yet discussed in RAN2 and it is important for avoiding redundant requests from the UE.

(RRC-3) The UE does not request to start data collection for candidate configurations for which a corresponding UE data collection configuration is already configured via CSI-ReportConfig.

### Open issue RRC-15: The time duration for an AI functionality to become available for inference when the UE reports applicability

From RAN2 perspective, there is already a processing delay requirement for RRCReconfigurationComplete, until when an RRC configuration shall be processed and applied at the UE (in Section 12, TS 38.331). If a longer time is needed for a model to be loaded in the memory, this should not delay the RRCReconfigurationComplete further or prolong the time until the NW can assume that a UE processed an RRCReconfiguration beyond the existing processing delay requirement. Based on existing RAN2 agreements, the UE has the possibility to first reply that the status of an inference configuration is inapplicable and after it loads the model it can update the applicability, if loading the model takes a longer time than the maximum processing time allowed for RRCReconfigurationComplete.

(RRC-15) RAN2 does not add another processing delay requirement or indication for model loading, in addition to the processing delay requirement for RRCReconfigurationComplete.

### Open issue RRC-44: Whether RRCReconfigurationComplete contains the applicability/inapplicability of all inference configurations

In our understanding, according to RAN2 agreements made so far, the RRCReconfigurationComplete contains the applicable/inapplicable status for all inference configurations and only the UAI contains updates in the applicability status, as implemented in the RRC CR. It was proposed by some companies that this behaviour can be improved by allowing only applicability updates to be sent by the UE in RRCReconfigurationComplete for inference configurations that were not received in the immediate previous RRCReconfiguration (i.e. inference configurations for which the applicability was already reported). While this solution also works, in our view it is an optimization.

Furthermore, if the applicability of all inference configurations is reported in RRCReconfigurationComplete (i.e. every RRCReconfigurationComplete is treated as initial applicability reporting), this is more robust to errors. Otherwise, the RRCReconfigurationComplete will contain a mix of first applicability reports (for inference configurations in the immediate previous RRCReconfiguration) and applicability updates for other configurations (for which the applicability was reported at least once before), making it more difficult to track in the implementation what the applicability status is. Furthermore, in case of errors (e.g. missed applicability reports), the NW and UE cannot re-sync on the applicability status, unless the NW re-configures the same inference configuration. Please note that the UAI contains only updates in the applicability status, according to RAN2 agreements, so the UAI cannot be used for re-syncing either.

(RRC-44) RRCReconfigurationComplete contains the applicability/inapplicability of all inference related configurations, i.e. this message does not include a mix of initial applicability for all new configurations and only updated applicability for older configurations.

### Open issue RRC-45: How/where to capture activation of periodic inference CSI-ReportConfig in specifications

In the LS [R4-2508085], RAN4 asks RAN2 whether

- the applicable periodic CSI report configurations are already activated before reporting the initial applicability status via RRCReconfigurationComplete; or

- the UE starts to activate the applicable periodic CSI report configurations upon reporting the initial applicability via RRCReconfigurationComplete.

In our understanding, the underlying problem that RAN4 is trying to address is that the NW needs to know whether the UE transmits periodic CSI reports or not, in order to decode the PUSCH transmissions from the UE correctly. Specifically, when a periodic CSI transmission coincides with a scheduled PUSCH transmission, the UE multiplexes the UCI (CSI report) into a subset of the resource elements that the gNB granted for the PUSCH transmission. The actual PUSCH transport block is rate matched around the resource elements that are used for the UCI. If the NW assumes that the UE did multiplex UCI into PUSCH but the UE did not, the PUSCH decoding (and the decoding of the UCI) will fail, and vice versa.

Such ambiguity may also arise when the gNB sends an RRCReconfiguration for a legacy (non-AI/ML) periodic CSI report. Since the RRCReconfiguration is an asynchronous procedure, the NW does not know exactly from which slot onwards the UE will start multiplexing the periodic CSI reports into (scheduled) PUSCH slots. However, due to the way that the RRC processing requirements were defined, the ambiguity period is kept rather short, cf. Section 12 in TS 38.331:

“The UE performance requirements for RRC procedures are specified in the following tables. The performance requirement is expressed as the time in [ms] from the end of reception of the network -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> network response message with no access delay other than the TTI-alignment (e.g. excluding delays caused by scheduling, the random access procedure or physical layer synchronisation).“

This essentially means that the UE shall perform all the actions defined in Section 5.3.5.3 upon reception of an RRCReconfiguration, including applying the new configuration and the preparation of the corresponding RRCReconfigurationComplete message, within the processing time specified in Section 12. Hence, the ambiguity lasts only for the duration of the specified processing delay. If the NW sends an uplink grant for PUSCH after that time, it knows that the UE will multiplex the newly configured periodic CSI report into the PUSCH resources and is hence able to decode both correctly. The same holds for all other parts of the configuration, i.e. the NW may assume that the UE has applied the entire RRCReconfiguration and acts accordingly when it schedules the UE for UL or DL after that processing time.

The second option from RAN4 implies that the UE does not start transmitting AI/ML based periodic CSI reports after the specified RRC processing delay ends, even if the UE determines the periodic CSI report configurations to be applicable. The UE behaviour would thus differ from the legacy UE behaviour upon reception of RRCReconfiguration that we summarized above. The drawback is that it creates a second period of ambiguity after the UE sent the RRCReconfigurationComplete containing the applicability report, since it is not defined anymore when the periodic CSI configuration is assumed to be activated (only the start of the activation process would be defined). Thus, only the first option from RAN4 is compliant with RRC procedures.

(RRC-45) RAN2 to reply to RAN4 LS [R4-2508085] that interpretation 1 is aligned with RRC procedures in the RRC specification.

Interpretation (1) can be captured in Section 5.3.5.3 of the RRC specifications in terms of when an applicable periodic CSI report configuration is assumed to be activated, either when setting the content of RRCReconfigurationComplete or when sending the RRCReconfigurationComplete to the lower layers. Either way works in our view, but we have a slight preference for the procedural text on setting the content of RRCReconfigurationComplete, in order to keep all applicability reporting related procedures together and avoid repeating some if clauses when sending the RRCReconfigurationComplete to the lower layers.

(RRC-45) Interpretation 1 from RAN4 for activation of applicable periodic CSI report configurations is specified in Section 5.3.5.3 of TS 38.331, within the procedural text on setting the content of RRCReconfigurationComplete.

However, capturing interpretation 1 for activation in RRC specifications is not sufficient to restrict the decoding ambiguity at NW within the RRC processing delay requirement. Even if the activation process happens at the UE before the end of the processing delay limit (i.e. by the time “when the UE shall be ready for the reception of uplink grant for the UE -> network response message”), the NW finds out about this only later, after it successfully receives the RRCReconfigurationComplete containing the applicability report. This is because the UE decides autonomously whether to activate an AIML periodic CSI report configuration or not, based on whether it determines the configuration to be applicable or inapplicable. Therefore, after the RRC processing delay expires and until reception of the applicability report at the NW, there is still a decoding ambiguity at the NW, which is not compliant with the meaning and purpose of the RRC processing delay requirement.

(RRC-45) Introducing autonomous activation by the UE of periodic CSI report configurations causes:

- inconsistency with the RRC processing delay requirements; and

- ambiguity at the NW regarding when exactly the UE starts sending L1 CSI reports, which causes subsequent decoding errors.

To avoid such additional ambiguity and new UE and NW behaviour compared to the legacy one, we propose that the UE starts transmitting AI/ML-based periodic CSI reports immediately upon completing the processing of the RRCReconfiguration, i.e. according to the same rules and principles as for legacy periodic CSI reports, regardless of whether the configuration is determined to be applicable or inapplicable. To still be able to distinguish between these two cases while being compliant with legacy RRC procedures, RAN1 can specify that the UE shall populate the L1 CSI report with a specified code point indicating “invalid” L1 CSI reports, if/while a CSI report configuration is determined to be inapplicable and is thus inactive.

(RRC-45) RAN2 to ask RAN1 to resolve the decoding ambiguity at the NW regarding whether/when the UE starts sending AIML-based periodic CSI reports. Compliance with the RRC processing delay requirements should be ensured.

### Open issue RRC-46: Whether the associated ID is cell specific or multi-cell specific

Regarding this open issue, RAN1#118 agreed the following:

Agreement Confirm the following Working assumption. Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

Some companies propose to define a multi-cell specific associated ID, where this associated ID could also be extended to be a global associated ID defined per PLMN. In our view, a global/multi-cell associated ID would require a solution for network vendors to coordinate with each other to split the associated ID range among each other, which is very complex. Alternatively, a central entity in the PLMN would have to split the associated ID range among network vendors and this is outside RAN2 scope.

Furthermore, a global/multi-cell associated ID does not ensure that the same associated IDs will indeed be used often across different cells. Since different cells may have different beam configurations/Tx properties, it may as well happen that many different associated IDs need to be used anyway. Thus, this global/multi-cell solution may increase the size of the associated IDs unnecessarily compared to a cell specific associated ID, without providing a clear benefit in practice. Moreover, even if the global/multi-cell associated IDs are often used across cells, the propagation conditions in each cell with the same associated ID may be different, so the UE model(s) would have to be anyway trained on multiple datasets corresponding to measurements from different cells. Thus, a global/multi-cell associated ID does not guarantee that the UE can use fewer models or train them on fewer datasets compared to cell specific associated IDs, so the main purpose of the global/multi-cell associated ID solution may not necessarily be achieved.

The solution of supporting both cell-specific and global/multi-cell specific associated IDs was also proposed in discussions. It seems unnecessary to introduce two solutions and this also complicates the signaling, e.g. a bit may need to be introduced to differentiate between these two cases, a related UE capability discussion may be needed, without a clear benefit. Finally, in our view this solution is not critical for completing the WI, given that the associated ID is anyway optional according to RAN1 and a cell-specific associated ID is already supported based on existing agreements.

(RRC-46) RAN2 to confirm that the associated ID is consistent within a cell, i.e. the same values can be reused in other cells, where they can correspond to different beam configurations.

Another related aspect is how long an associated ID value is assumed valid. Namely, how long should the UE (or UE side server) assume/remember that the associated ID corresponds to the same NW beam properties. This is an issue common to both cell specific and global/multi-cell specific associated IDs. Although the common understanding is that the associated IDs are somewhat static and are associated to the same NW beam properties (within the area where they are consistent, e.g. per cell, per PLMN, etc.), it cannot be assumed that this static association holds indefinitely. If it happens that the NW needs to reset the associated IDs, there may be inconsistency with what the UE (or UE side server) thinks they mean, based on the data stored from a previous training phase.

(RRC-46) RAN2 to discuss how long an associated ID is assumed to be valid (i.e. has the same meaning at the NW side and UE side).

### Open issue RRC-48: Applicability reporting for inference related parameter sets after the corresponding full inference configuration is provided

This open issue refers in our view to an optimization that can be solved by NW implementation. If the NW first provides a set of inference related parameters and then one full inference configuration that corresponding to the set of inference related parameters, it is beneficial also for the NW and not only for the UE if the NW de-configures the set of inference related parameters, to avoid receiving redundant applicability reports for the two corresponding configurations. Thus, the UE should report the applicability of all configurations that were received from the network (full inference configurations and sets of inference related parameters) until the NW de-configures them and it is left to NW implementation when to de-configure the sets of inference related parameters that are associated with full inference configurations.

(RRC-48) The UE reports the applicability of both full inference configurations and associated sets of inference related parameters, if both are configured.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	(RRC-45) Introducing autonomous activation by the UE of periodic CSI report configurations causes:

	inconsistency with the RRC processing delay requirements; and

	ambiguity at the NW regarding when exactly the UE starts sending L1 CSI reports, which causes subsequent decoding errors.

Based on the discussion in the previous sections we propose the following:

Proposal 1	(RRC-3) The UE does not request to start data collection for candidate configurations for which a corresponding UE data collection configuration is already configured via CSI-ReportConfig.

Proposal 2	(RRC-15) RAN2 does not add another processing delay requirement or indication for model loading, in addition to the processing delay requirement for RRCReconfigurationComplete.

Proposal 3	(RRC-44) RRCReconfigurationComplete contains the applicability/inapplicability of all inference related configurations, i.e. this message does not include a mix of initial applicability for all new configurations and only updated applicability for older configurations.

Proposal 4	(RRC-45) RAN2 to reply to RAN4 LS [R4-2508085] that interpretation 1 is aligned with RRC procedures in the RRC specification.

Proposal 5	(RRC-45) Interpretation 1 from RAN4 for activation of applicable periodic CSI report configurations is specified in Section 5.3.5.3 of TS 38.331, within the procedural text on setting the content of RRCReconfigurationComplete.

Proposal 6	(RRC-45) RAN2 to ask RAN1 to resolve the decoding ambiguity at the NW regarding whether/when the UE starts sending AIML-based periodic CSI reports. Compliance with the RRC processing delay requirements should be ensured.

Proposal 7	(RRC-46) RAN2 to confirm that the associated ID is consistent within a cell, i.e. the same values can be reused in other cells, where they can correspond to different beam configurations.

Proposal 8	(RRC-46) RAN2 to discuss how long an associated ID is assumed to be valid (i.e. has the same meaning at the NW side and UE side).

Proposal 9	(RRC-48) The UE reports the applicability of both full inference configurations and associated sets of inference related parameters, if both are configured.