3GPP TSG-RAN WG2 #131	R2-2505868

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	8.6.3

Source: 	Ericsson

Title:  	Handling of SP CSI-RS resources of target cell after LTM cell switch

Document for:	Discussion and Decision

# Introduction

The SP CSI-RS resources was discussed in RAN2#130 and RAN2#129bis where agreements have been made for candidate cells other than the target. This contribution discusses how the SP CSI-RS resources in the target cell should be handled after an LTM cell switch.

# Discussion

UE typically measures two types of CSI-RS reference signal (NZP and CSI-IM) for RRM and early CSI acquisition purpose. However, only the NZP CSI-RS can be used for both RRM and early CSI acquisition while CSI-IM can only be used for early CSI acquisition. In RAN1, there are two types of semi-persistent CSI-RS resource for Rel-19 that affects RAN2, namely either for normal RRM measurements like CSI-RSRP or for the case of early RSRP reporting during an LTM Cell Switch.

NZP CSI-RS can be used for both RRM and early CSI acquisition while CSI-IM only can be used for early CSI acquisition.

During RAN2#130 the issue on how to handle the semi-persistent CSI-RS resources for the candidate cells was discussed, and it was agreed that the UE deactivates the CSI-RS resources for all candidates except the target cell (which becomes the new serving cell), for which we still have an FFS.

RAN2#130: UE deactivates SP CSI-RS resource of candidate cells (other than the target cell) after cell switch. FFS on the target cell.

Important to highlight is that the agreement above only states “after cell switch” which in this case of the target cell is not very precise and needs to be further described. When the cell switch command is sent in the source cell, the cell switch will happen, but the procedure is not complete until either the random access procedure in the target cell is completed (for RACH-based) or the first DL transmission addressing the C-RNTI is received (for RACH-less). This means that there is some time after the cell switch command in the source cell until the cell switch is completed for the UE in the target cell. If the UE can perform the measurements for early CSI acquisition in this timeframe it should do that and send the report in the new serving cell after the LTM cell switch is completed. However, if there has been no opportunity to measure and report during this time, then there is no point for the UE to keep the SP CSI-RS activated once the LTM cell switch is completed as in this case the normal CSI measurement configuration will be applied by the UE. Therefore, we see no real benefit in keeping the SP CSI-RS activated after the LTM cell switch for the target cell, as the legacy CSI measurements can be used in this case.

There is no point for the UE to keep the SP CSI- RS activated once the LTM cell switch is completed as in this case the normal CSI measurement configuration will be applied by the UE.

For this reason, we propose that the UE deactivates the SP CSI-RS resources for the target cell at the time of cell switch completion.

UE deactivates SP CSI-RS resource of the target cell after cell switch.

# Conclusion

In the previous sections we made the following observations:

Observation 1	NZP CSI-RS can be used for both RRM and early CSI acquisition while CSI-IM only can be used for early CSI acquisition.

Observation 2	There is no point for the UE to keep the SP CSI- RS activated once the LTM cell switch is completed as in this case the normal CSI measurement configuration will be applied by the UE.

Based on the discussion in the previous sections we propose the following:

Proposal 1	UE deactivates SP CSI-RS resource of the target cell after cell switch.