3GPP TSG-RAN-WG2 Meeting #131	                                   R2-2505880

Bengaluru, India, August 25th – 29th, 2025

Agenda Item: 8.1.2.2

Source: Sharp

Title: On Remaining Open Issues: LCM for UE-Sided AI/ML Beam Management

Document for: Discussion and Decision

# Introduction

This tdoc addresses selected open issues as identified in the email discussion [POST130][026] [AI PHY] 38.331 CR (Ericsson).

In this tdoc we provide input, propose clarifications, and support agreement on the CR and the associated open issues list as identified in the referenced discussion.

# Open Issue RRC-15: The time duration for an AI functionality to become available for inference when the UE reports applicability

In Rel-19, the UE may receive AI-based inference configurations from the network, which can include full inference configurations or subsets of inference-related parameter sets. When the UE reports applicability of these configurations, the network needs to know when the AI functionality will be usable for producing inference reports. Without knowing the expected time for AI model activation, the network cannot efficiently schedule resources (e.g., for periodic CSI reporting) that depend on AI-based inference results, leading to potential inefficiencies and suboptimal resource utilization.

Proposal 1 (RRC 15): The UE shall include time information for model or functionality inferencing when reporting applicability of a full inference configuration or an inference related parameter set.

Proposal 2 (RRC 15): The network may configure the UE to report time information for model or functionality inferencing.

# Open issue RRC-44: Whether RRCReconfigurationComplete contains the applicability/inapplicability of all inference configurations

Based on agreements in RAN2#129 [1], the UE sends the initial applicability report for a full inference configuration in RRCReconfigurationComplete upon reception. Subsequent updates to applicability are sent via UAI.

RAN2#129 agreement [1]:

- If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

During RAN2#130 [2], concerns were raised about the efficiency of reporting applicability for both full inference configurations (Option A) and inference related parameter sets (Option B). It was noted that when the network provides a full inference configuration linked to an applicable inference related parameter set, reporting both IDs repeatedly can be inefficient. A potential solution discussed was to allow the UE to report either the applicability of the inference related parameter set or the full inference configuration based on the parameter set’s applicability, once the full inference configuration is provided by the network.

Despite these discussions, it remains unclear whether RRCReconfigurationComplete must include applicability/inapplicability information for all inference configurations stored at the UE or only for those for which the UE has previously reported applicability/inapplicability.

There are two possible interpretations regarding the content of applicability reporting in RRCReconfigurationComplete (as indicated by the rapporteur):

- Full reporting:
The message contains applicability/inapplicability status for all inference configurations stored at the UE, with only changes afterwards reported via UAI.

- Partial (delta) reporting:
The message includes applicability/inapplicability status only for:

  - The inference configurations included in the immediately preceding RRCReconfiguration message, and

  - Previously configured inference configurations for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report. Subsequent updates are then reported via UAI.

The absence of explicit clarification may lead to inconsistent UE behaviour and unnecessary signalling overhead.

We propose to adopt the following reporting interpretation:

Proposal 3 (RRC 44): RRCReconfigurationComplete shall include applicability/inapplicability status for:

- All inference configurations included in the immediately preceding RRCReconfiguration message, and

- Any previously configured inference configurations for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report.

Proposal 4 (RRC 44): All further changes in applicability status shall be reported by the UE using UAI messages.

Proposal 5 (RRC 1 & 44): The UE shall report an explicit cause value for inapplicability whenever applicability/inapplicability status is reported, both in the initial RRCReconfigurationComplete message and in subsequent UAI messages.

This approach ensures the network gets the initial status for new inference configurations without unnecessary repeated reports. It reduces signalling size by only reporting changes, keeps UE behaviour clear and consistent, and helps the network better understand why some configurations are not applicable through explicit cause reporting.

The cause for inapplicability may not need to be reported every time, therefore, the network can either request the inapplicability cause when needed or configure the UE to report it only when necessary.

Proposal 6 (RRC 1 & 44): The network may request the cause for inapplicability from the UE when needed (for e.g., in RRCReconfiguration message).

# Open issue RRC-45: How/where to capture activation of periodic inference CSI-ReportConfig in specifications

RAN2 agreed that the UE should activate periodic inference configurations in CSI-ReportConfig only if it reports that these configurations are applicable. According to RAN2#129 agreements [1], if Option A is configured, the UE autonomously activates applicable functionalities upon reporting applicability in RRCReconfigurationComplete, without waiting for a subsequent RRCReconfiguration message.

RAN2#129 agreement [1]:

If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

However, there is ambiguity regarding how and where this activation should be specified in the specifications — whether it should be captured solely within RRC specifications or also involve RAN1. Different companies have proposed alternative methods for capturing activation behaviour in the spec, including whether activation should be linked to when RRCReconfigurationComplete is prepared or when it is sent to lower layers.

Additionally, an incoming liaison statement [3] raises the question of whether the UE activates applicable functionalities before reporting applicability or only upon reporting it in RRCReconfigurationComplete. This timing impacts the gNB’s ability to correctly decode uplink control information (UCI). If the UE multiplexes UCI with inference predictions before confirming applicability, the gNB may experience decoding failures due to unexpected data, affecting other information in the same transmission.

Proposal 7 (RRC 45): The UE activates applicable periodic inference configurations when sending the RRCReconfigurationComplete message to lower layers, rather than at message preparation.

This provides a precise activation point at message transmission while ensuring predictable UE behaviour and aligns activation with confirmed applicability to the network.

Proposal 8 (RRC 45): The UE shall explicitly indicate to lower layers to activate periodic inference configurations only if reported as applicable in RRCReconfigurationComplete.

Alternatively, the UE may submit periodic inference configurations to lower layers only when applicable, where submission implies activation and omission implies no activation.
This UE behaviour may prevent activating non-applicable configurations, reducing unnecessary processing and potential confusion at lower layers.

Proposal 9 (RRC 45): The UE shall activate applicable periodic inference configurations only after reporting applicability in RRCReconfigurationComplete.

This is because activating configurations only after reporting applicability avoids ambiguity for the gNB, which depends on applicability reports to correctly process uplink inference predictions.

Proposal 10 (RRC 45): RAN 2 to discuss and agree that the UE shall not multiplex UCI containing inference predictions on PUCCH or PUSCH before activating the periodic inference configuration post-RRCReconfigurationComplete.

Alternatively, the UE may always send an L1 report in UCI for periodic CSI-ReportConfig even if not yet active/applicable.

It is worth noting that RAN1 input may be needed to handle multiplexing constraints at the PHY/MAC level

This will help in avoiding decoding failures and ensures consistent handling of uplink control information at the gNB.

# Open issue RRC-46: Whether the associated ID is cell specific or multi-cell specific

At RAN1#118 [4], it was agreed as a working assumption that:

Agreement Confirm the following Working assumption. Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

It was noted (FFS) whether/how this UE assumption could be applicable across multiple cells, including feasibility study for multi-cell or PLMN-wide scope.
Some companies have expressed that cell-specific IDs are not sufficient for certain multi-cell operation scenarios, such as carrier aggregation, dual connectivity, and inter-cell mobility within the same gNB or CU/DU.

If the associated ID is cell-specific only, the UE may need to re-acquire and reconfigure conditions unnecessarily when moving between cells that share identical configurations.

This may increase signalling overhead and delay feature activation (e.g., AI/ML model reuse, measurement applicability).

If the associated ID meaning is extended across cells without standardisation, there is a risk of inconsistent interpretation if the same ID is assigned different parameters/config. in different cells. Furthermore, it adds complexity: the network must ensure consistency of meaning across all cells sharing that ID.

Proposal 11 (RRC 46): RAN 2 to agree that associated IDs shall be considered cell-specific (by default) unless explicitly indicated otherwise.

Proposal 12 (RRC 46): The network may signal the scope of an associated ID via RRC signalling for example: if its Multi cell (applies across listed cells) or PLMN wide (applies across all cells in the PLMN).

When a multi-cell or PLMN-wide scope is signalled for an associated ID, the gNB shall ensure that the semantics/configuration of the associated ID are identical across all cells within the indicated scope. This maintains backward compatibility with the existing cell-specific assumption defined in Rel-19, allowing UEs that operate under cell-specific associated ID behaviour to continue functioning without modification. At the same time, it reduces RRC signalling overhead and latency in multi-cell scenarios, by enabling the UE to reuse configured network-side conditions without reconfiguration when moving between cells. Explicitly signalling the scope of the associated ID and enforcing consistency across the relevant cells provides flexibility for network-wide coordination and avoids ambiguity, ensuring the UE interprets the associated ID consistently and correctly across all applicable cells.

# Open issue RRC-48: Applicability reporting for inference related parameter sets after the corresponding full inference configuration is provided

In Rel-19, the UE can receive full inference configurations from the network, which may include multiple inference-related parameter sets. According to current agreements, the UE reports applicability for both the full configuration and each related parameter set. This approach may be inefficient. Once a full inference configuration is provided, reporting both the configuration ID and all corresponding parameters set config. IDs upon changes in applicability results in redundant signalling, increased uplink overhead, and additional UE processing.

Proposal 13 (RRC 48): Once the full inference configuration has been provided by the network, the UE may report applicability using either:

- The full inference configuration ID, or

- The inference-related parameter set ID corresponding to the full configuration.

Proposal 14 (RRC 48): The network may indicate via RRC whether the UE should report applicability at the full configuration level or parameter set level, allowing flexibility while maintaining backward compatibility.

Proposal 15 (RRC 48): Reporting both IDs may be optional based on network configuration.

# Conclusions

Based on the discussion above, we have the following observations and proposals:

Proposal 1 (RRC 15): The UE shall include time information for model or functionality inferencing when reporting applicability of a full inference configuration or an inference related parameter set.

Proposal 2 (RRC 15): The network may configure the UE to report time information for model or functionality inferencing.

Proposal 3 (RRC 44): RRCReconfigurationComplete shall include applicability/inapplicability status for:

- All inference configurations included in the immediately preceding RRCReconfiguration message, and

- Any previously configured inference configurations for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report.

Proposal 4 (RRC 44): All further changes in applicability status shall be reported by the UE using UAI messages.

Proposal 5 (RRC 1 & 44): The UE shall report an explicit cause value for inapplicability whenever applicability/inapplicability status is reported, both in the initial RRCReconfigurationComplete message and in subsequent UAI messages.

Proposal 6 (RRC 1 & 44): The network may request the cause for inapplicability from the UE when needed (for e.g., in RRCReconfiguration message).

Proposal 7 (RRC 45): The UE activates applicable periodic inference configurations when sending the RRCReconfigurationComplete message to lower layers, rather than at message preparation.

Proposal 8 (RRC 45): The UE shall explicitly indicate to lower layers to activate periodic inference configurations only if reported as applicable in RRCReconfigurationComplete.

Proposal 9 (RRC 45): The UE shall activate applicable periodic inference configurations only after reporting applicability in RRCReconfigurationComplete.

Proposal 10 (RRC 45): RAN 2 to discuss and agree that the UE shall not multiplex UCI containing inference predictions on PUCCH or PUSCH before activating the periodic inference configuration post-RRCReconfigurationComplete.

Proposal 11 (RRC 46): RAN 2 to agree that associated IDs shall be considered cell-specific (by default) unless explicitly indicated otherwise.

Proposal 12 (RRC 46): The network may signal the scope of an associated ID via RRC signalling for example: if its Multi cell (applies across listed cells) or PLMN wide (applies across all cells in the PLMN).

Proposal 13 (RRC 48): Once the full inference configuration has been provided by the network, the UE may report applicability using either:

- The full inference configuration ID, or

- The inference-related parameter set ID corresponding to the full configuration.

Proposal 14 (RRC 48): The network may indicate via RRC whether the UE should report applicability at the full configuration level or parameter set level, allowing flexibility while maintaining backward compatibility.

Proposal 15 (RRC 48): Reporting both IDs may be optional based on network configuration.

# References

[1] Chair notes, RAN2 129 3GPP TSG-RAN WG2 Meeting #129, Athens, Greece, 2025

[2] Chair notes, RAN2 130 3GPP TSG-RAN WG2 Meeting #130, Malta, EU, 2025

[3] R4-2508085 LS on AI/ML functionality activation

[4] Chair notes, RAN1 #118, Maastricht, NL, August 19th – 23rd, 2024