3GPP TSG-RAN WG2 Meeting #131	R2-2505923

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.2.3

Source:	Nokia

Title:	Open issues in AIoT random access

WID/SID:	Ambient_IoT_solutions - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This Tdoc handles the open issues of AIoT, specifically the frequency indication for the R2D trigger message along with considerations of msg2 retransmissions.

# 2	Discussion

## 2.1	On frequency index inclusion in MSG2

RAN1 agreed in RAN1#121 the values of the Tb and Tchip used for the small frequency shifts, to be as in the table in Figure 1, leading to up to 7 different frequencies of transmissions in case of Tchip=1.04. Furthermore, it is noted that RAN1 does not assume i.e. that a specific table is used to store the table.

Observation 1: AIoT devices may support up to 7 small frequency shifts.

Observation 2: The device is not assumed to pre-store the table to decide the frequency shifts.

Figure 1 Values of Tb and Tchip for small frequency shifts

RAN2 discussed in RAN2#130 whether/how to include the frequency index along with RN16 in MSG2. An advantage to allowing reader to signal dedicated frequency shifts in msg2 and other messages is, that in case number of devices selecting the msg1 resources is lower than the number of available Tb, even for non-collision cases, the reader have the freedom to select Tb in such a way that the devices are spread far in frequency. As an example, having 4 devices selecting resources with Tb duration 2.08, the reader can assign R=64, 16, 4, 1 irrespectively of the device(s) selected Tb in msg1.

Observation 3: From RAN1 agreements, the reader may signal the small frequency shift(s) using little signalling overhead.

- Agreements on RA - 1 Exclude the option of MSG2 transmission and any retransmission of MSG2 happens within a predefined time window (based on timer) 2 A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further downselected between option B and C below. A device receiving MSG2 within this boundary transmits MSG3. The device does not process MSG2 (re)transmission received after the boundary. - Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message - Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS) - Option A (the boundary being the subsequent paging only) is excluded. For option C, further discuss in terms of complexity at the device vs reader flexibility. 3 Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible. FFS Discuss further whether to include it.

Our understanding is that in RAN1 allowing the reader to avoid devices interfering with each other has been argued as one of the reason for the way of signalling the small frequency shifts i.e. giving the reader flexibility handle inter-cell interference coordination. However, the freedom to, does not mean the requirement of, so whether/how the reader determines the used R value can be up to reader implementation. In such cases where the reader strives for simplicity, it can simply set R equal to the device’s selected value.

Observation 4: Allowing the reader to allocate specific D2R frequency resources, enables the reader to ensure best performance as it may allow the reader to;

- Limit interference between devices for simple frequency shift signalling

- Handle collisions of MSG1’s in case a reader is able to receive two different random IDs on the same frequency and then provide each device with a unique frequency shift

- Be forward compatible i.e. using FDMA to allow R19 devices communicate with R20

Proposal 1: RAN2 to include frequency index along with RN16 in msg2, but avoid specifying how the reader should determine the frequency selection i.e. R value.

In terms of signalling available chip durations in the D2R trigger message, it would be simple for the reader to include this as a fixed bitmap to signal the set of R values i.e. 8 bit for all supported R values: { 1, 2, 4, 8, 16, 32, 64, 128 } thus avoiding the lookup operation in a table for the device.

Observation 5: Signalling of small frequency shift in D2R trigger can be a byte aligned bitmap.

Proposal 2: D2R trigger message signals the R value for small frequency shifts as a fixed bitmap with the reader having the mapping table.

## 2.2	Boundary of CBRA failure

Since RAN2 in the early stages agreed that there would be only NACK based reply to msg2 and that the device would re-access based on said NACK, a boundary is needed to be aware of the point in time where the device can assume an ACK. The discussion was downselected to the below options.

A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further downselected between option B and C below. A device receiving MSG2 within this boundary transmits MSG3. The device does not process MSG2 (re)transmission received after the boundary. - Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message - Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS) - Option A (the boundary being the subsequent paging only) is excluded. For option C, further discuss in terms of complexity at the device vs reader flexibility.

It is noted that the option B and C are similar, except that RAN2 should define the number “k” for option C. We understand that the reason for including “k” was the RAN1 agreement that an R2D trigger message can be signalled with “k” time resources in the future. In our view, there are some observations on the current agreements and options;

- The subsequent paging is assumed to be quite far into the future, as it will mainly be present after the a number of msg1’s , whereof the number is equal to the Q-value.

- The “next R2D trigger message” may be wrongly worded, as an R2D message is only to determine future resources, and in practice resources could be assigned to finish a single device i.e. multiple R2D trigger messages could occur with no relation to the paging.

- The “kth” trigger message was due to RAN1 agreeing that an R2D trigger message for msg1 can point till multiple time instances.

From the above considerations, we don’t see a reason to consider the kth message, as there will be no R2D trigger message in between the time resources triggering other messages. We also see that the subsequent paging is to far in time that the device should be expecting any replies i.e. we would expect the reader to finish at least the initial access procedure for AIoT devices fighting for contention before new msg1’s are triggered. However, as we mentioned earlier we thing that the next R2D trigger message may be wrongly worded, as a device should finish the initial access before a new paging. Thus, we propose that the agreement is to have the boundary before the next msg1 trigger message.

Observation 6: A R2D trigger message will provide k resources in advance thus in case multiple R2D resources are allocated, these will not have independent trigger

Proposal 3: A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before the boundary is the reception next msg1 R2D trigger message.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: AIoT devices may support up to 7 different frequency shifts.

Observation 2: The device is not assumed to pre-store the table to decide the frequency shifts.

Observation 3: From RAN1 agreements, the reader may signal the small frequency shift(s) using little signalling overhead.

Observation 4: Allowing the reader to allocate specific D2R frequency resources, enables the reader to ensure best performance as it may allow the reader to;

- Limit interference between devices for simple frequency shift signalling

- Handle collisions of MSG1’s in case a reader is able to receive two different random IDs on the same frequency and then provide each device with a unique frequency shift

- Be forward compatible i.e. using FDMA to allow R19 devices communicate with R20

Proposal 1: RAN2 to include frequency index along with RN16 in msg2, but avoid specifying how the reader should determine the frequency selection i.e. R value.

Observation 5: Signalling of small frequency shift in D2R trigger can be a byte aligned bitmap.

Proposal 2: D2R trigger message signals the R value for small frequency shifts as a fixed bitmap with the reader having the mapping table.

Observation 6: A R2D trigger message will provide k resources in advance thus in case multiple R2D resources are allocated, these will not have independent trigger

Proposal 3: A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before the boundary is the reception next msg1 R2D trigger message.