3GPP TSG-RAN WG2 Meeting #131	R2-2505933

# Bengaluru, IN, 25-29 Augst, 2025

Source:			Skylo Technologies, Lockheed Martin, EchoStar, Sateliot

Title:			[TEI19] Introduction of Satellite Switch with Resynchronization support for IoT NTN

Agenda item:	8.19.1

Document for:	Discussion and Decision

# Introduction

The "satellite switch with re-synchronization" procedure, as specified in TS 38.300 and TS 38.331 for NR-NTN, enables a UE to switch from a serving satellite to a new target satellite.

A similar capability is essential for NB-IoT NTN operation within NGSO constellation. This is necessary for (quasi-)earth-fixed cell deployments and to support a wide range of consumer devices and service offerings like emergency messaging, eSOS, and voice. The serving satellite (source satellite) can proactively indicate an upcoming target satellite to enable the NB-IoT device to perform a seamless satellite switch with re-synchronization. This is quite useful in non-discontinuous NTN coverage situations operating quasi-Earth fixed cells, and particularly suited to IoT NTN scenarios in order to enable resource-efficient quick switchovers as connected mode mobility (e.g., L3-based handover) is not natively supported by NB-IoT [4].

This contribution proposes the introduction of the "Satellite switch with re-synchronization" feature and its corresponding RRC procedure into the E-UTRA system description in TS 36.300, the RRC protocol specification TS 36.331, and MAC layer protocol specification TS 36.321.

The primary target for this proposal is Rel-19. A mirror CR to Rel-18 is also necessary to ensure consistency. The introduction of satellite switch with re-synchronization will enable NB-IoT to fully leverage the benefits of NGSO constellation, supporting a wider range of latency-sensitive applications and enhancing the overall NTN ecosystem.

# Discussion

Specified in TS 38.331, TS 38.321 and TS 38.300, the satellite switch with re-synchronization procedure is currently support for NR-NTN. A UE capable of hard satellite switch with resynchronization in RRC_CONNECTED initiates the procedure when SatSwitchWithReSync and t-Service are included in SIB19. The procedure is executed by the UE in the RRC_CONNECTED state based on network-provided information in SIB19, which contains parameters for a target satellite required (e.g., t-ServiceStart) to perform satellite switch with resynchronization. One of the essential requirements of this satellite switch with resynchronization is that a currently serving cell (i.e., operated by a source satellite) and a target cell (i.e., operated by a target satellite) use the same PCI for their respective cells targeting the same geographical area. Accordingly, with a help of some essential parameters that the source satellite provides in its SIB19, namely ssb-timeoffset and t-ServiceStart pertaining to the target (cell) satellite, UEs know when to start the synchronization process. For (quasi-)earth-fixed deployments in NB-IoT NTN, the rapid movement of satellites across the sky can benefit from a similar procedure in order to facilitate seamless mobility between different satellites. The support of a "satellite switch with re-synchronization" procedure is preferred. This procedure enables a UE in the RRC_CONNECTED state to transition from a serving satellite to a new target satellite seamlessly without having to perform a blind search and synchronization with a new satellite on the horizon. In addition, such a switchover can avoid L3-based handover procedure that is not well supported by NB-IoT.

For NB-IoT, a new or modified SIB would be used to broadcast the satSwitchwithReSync parameters, which contain orbital parameters and configurations for upcoming target satellites. A UE in the RRC_CONNECTED state reads and processes this SIB. Based on its internal clock, position, and the provided satellite orbital parameters, the UE's RRC layer can pre-determine the precise time when it needs to switch to a new satellite.

At the pre-determined time, the UE initiates the switch. It applies the resynchronization parameters from the SIB to adjust its timing and frequency, enabling it to quickly re-synchronize with the new satellite. Since the PCI is the same, the physical layer transition is simplified.

Observation 1: The “Satellite Switch with Re-synchronization: procedure can and should be supported for NB-IoT NTN.

# Proposal

Based on the discussion above, we propose the following example CRs to the E-UTRA specifications to support the “satellite switch with resynchronization” procedure for NB-IoT NTN.

- TS 36.321: Add that Random Access procedure can be initiated by NPDCCH order in clauses 5.1.1 and 5.2a

- TS 36.300: Add a new sub-clause 4.12.1 “Satellite Switch with Resynchronization” to provide a high-level description of the "Satellite switch with re-synchronization" feature.

- TS 36.331: Define necessary parameters and procedures to enable this feature

  - We propose to modify SIB31-NB specified in clause 6.7.3.1, to carry the satSwitchWithReSync parameters.

  - In clause 6.3.1, the changes also specify fallback behavior when epochTime and ul-SyncValidityDuration are absent in the servingSatelliteInfo. The UE shall use the corresponding values from the serving cell as a fallback.

  - The corresponding satSwitchWithResync procedure for UEs in the RRC_CONNECTED state will be described in a new sub-clause in clause 5.6.x

Proposal 1: It is proposed to discuss and agree the CRs [1-3] for TS 36.331, TS 36.300, and TS 36.321 to support this feature in Rel-19.

Since NTN features for E-UTRA were initially introduced in Release 17 and further enhanced in Release 18 for NR, it is critical that this new mobility feature is available in E-UTRA in both Release 18 and Release 19. Proposing the change for Release 19 as the primary target and then creating a mirror CR for Release 18 ensures that the feature is available in the latest releases and maintains a consistent evolution path for E-UTRA.

Proposal 2: It is proposed to agree the mirror CRs for TS 36.331 and TS 36.300 in Rel-18 for the “satellite switch with resynchronization” feature.

# Reference

[1] R2-2505939, 3GPP TS 36.331 CR5149: “SatSwitchwithReSync support for IoT NTN”.

[2] R2-2505934, 3GPP TS 36.300 CR1429: “SatSwitchwithReSync support for IoT NTN”.

[3] R2-2505938, 3GPP TS 36.321 CR1594: “SatSwitchwithReSync support for IoT NTN”.

[4] 3GPP TS 23.401: “GPRS enhancements for E-UTRAN access”.