# 3GPP TSG-WG2 Meeting #131                                            R2-2505941

Bangalore, India, Aug 25th – 29th, 2025

Source: 			CMCC

Title:  		    Discussion on the remaining issues of data transmission for A-IoT

Agenda item:	8.2.4

Document for: 	Discussion

Introduction

With the past meetings, the basic A-IoT data transmission procedure and functions have been discussed and captured in the running CRs.

Agreements

- R2D message scheduling non-first segment (re)transmission does not include upper layer command.

- For the first segment and unsegmented packet (re)transmission, the “offset” indicator in R2D is not present.

- This implies that the R2D message will either have command or offset (but not both).  FFS whether we define two message types or one message type with optional fields.

- The device is expected to send a MAC response to the reader in the D2R occasion. The MAC response contains the NAS message if available at the D2R occasion.  If there is no NAS message available to transmit at the D2R occasion then the response contains MAC with 0 SDU and padding as needed.

- Send LS to CT1 to inform the agreement 1 to CT1 and explain that we have an issue with delayed NAS write success response.   RAN2 would prefer that this is handled by CT1 (and give the example of sending NAS response upon successful reception of write command).  Ask if this can be handled by CT1

- A mandatory length field directly indicates the length of D2R data MAC SDU to support varying lengths of D2R data. The size of length field is 7-bit in bytes.

- The offset indication for transmission/retransmission of the segments after the first segment of a D2R message is 7-bit length in bytes.  Segmented SDUs are also byte aligned.

- FFS D2R message type.  Current running CR will capture no message type, but we can revisit this next meeting and also consider if any other bits are needed for the MAC header

- The length field inside MAC for SDU is not needed for R2D messages, assuming R2D MAC padding is not needed.  FFS can come back if padding is needed depending on granularity of TBS (only if needed)

- For CBRA, to avoid AS ID being occupied for unnecessary time and to keep alignment between reader and device on AS ID release, device can release AS ID upon receiving paging message with different transaction ID, no matter the paging message is for it or not.   FFS for CFRA

- FFS for need for release message.

In this contribution, we will discuss the remaining issues of data transmission of A-IoT.

Discussion

AS ID release

AS ID validation issues have been discussed and some conditions have been agreed to release the AS ID, for example, when the device receiving the CFRA paging message or CBRA paging message with a new transaction ID. The remaining issue on AS ID is whether to introduce a release message for AS ID. As comments collected by the rapporteur in Post email discussion [027], majority companies think the AS ID release message is not necessary. They understand that the current mechanisms are enough to avoid AS ID occupation indefinitely and introducing release message will bring overhead to paging message or R2D message. Other companies wonder the AS ID cannot be released if the paging period is long or no subsequent paging. From our perspective, introducing the release message to gNB reader may bring the complexity to network and it is unnecessary optimization for device, no enhancement is needed in R19.

Proposal 1: AS ID release message is not supported in R19.

Write operation response

With the LS S2-2501241 and the discussion in last RAN2 meeting, we recognised that there may be an uncertain writing time for devices with different kinds of capabilities. The device is expected to send a MAC response to the reader in the D2R occasion, which contains the NAS message if available at the D2R occasion. However, if there is no NAS message available to transmit at the D2R occasion then the response contains 0 MAC SDU and padding as needed. If the reader does not receive a response in the initial D2R resource assigned for command feedback, it needs to reallocate resources for scheduling the successful write result.

The remaining issue is whether the current D2R data format (0 SDU and padding) is sufficient to inform the reader about the “response is not ready” and request D2R scheduling in a suitable time. In our understanding, if the device wants more D2R resource for coming data transmission, it sends the more data indication with “1”, similar as handling of segmentation. If no indicator is included in D2R or the indicator is “0”, the reader may stop D2R scheduling. Of course, the subsequent D2R scheduling may not meet the right “writing finish” time. The 0 SDU and padding with more data indicator “1” D2R packet can be transmitted until the writing response is available.

Proposal 2: From RAN2’s perspective, the reader can obtain the “write” results from the devices with longer processing times through multiple D2R scheduling.

Proposal 3: If the device receives D2R scheduling before the device has finished the “write” operation, it sets SDU Length as 0, sets MDI (More Data Indicator) as “1”, absent Data SDU and fills MAC Padding for all the remaining scheduled resources.

D2R message type

RAN2 has agree only Msg1 and Msg3/data are considered as D2R message type. For Msg1, it contains Random value RN16 for CBRA and acknowledges of paging message for CFRA. The MAC header is not required for reader to identify such D2R message. For Msg3 and data, they are scheduled by the response to different R2D messages. The reader can identify the D2R message without D2R message type. The “new” case is writing command operation as we discussed above. With current progress, the more data indication and padding are included D2R packet. That is to say, the common D2R format is reused. So, the message type is not needed for D2R.

Proposal 4: D2R message type is not introduced in R19 A-IoT D2R MAC PDU.

R2D message format

For segmentation, RAN2 agree that the R2D message scheduling non-first segment (re)transmission does not include upper layer command and for the first segment and unsegmented packet (re)transmission, the “offset” (which is Received Data Size in current MAC runningCR) indicator in R2D is not present, which implies that the R2D message will either have command or offset. The remaining issue is whether define two message types or one message type with optional fields. With current progress about the R2D message type, the message types are used to identify the paging message, trigger message, Msg2, R2D data and NACK feedback. For the command case, one type of R2D message includes “command message” for the first segment (new transmission and retransmission) and unsegmented packet (new transmission and retransmission), the other type of R2D message includes “offset” for subsequent segment (new transmission and retransmission). From the devices’ perspective, the format of first R2D data message is fixed, which always include the “command message”. If the D2R data size is smaller than the resource allocated with R2D message, the more data indication is set to”0”. The subsequent R2D message can only be re-scheduling for unsegment packet, if needed. However, if the D2R data size is larger than the resource allocated with R2D message, the first segment is sent with more data indicator “1”, the coming R2D message may be flexible. It may include “offset” for subsequent segment if the first segment is successfully received; or it may include “command message” for retransmission of first segment if the first segment is unsuccessfully received. Different R2D format is needed to differentiate the two cases. Comparing the separated R2D message type, we prefer the common R2D message type with one bit indicator field.

Proposal 5: Introduce 1 bit indicator field to R2D Upper Layer Data Transfer message to indicate whether Upper Layer Data SDU or Received Data Size is present.

TBS information

According to RAN1’s LS, the TBS information for R2D is supported by higher layer signalling. For the fixed R2D message, e.g., the access trigger message, the TBS information is not needed. For the variable R2D message, the TBS information is necessary. Considering the maximum data size is 1000 bits, we propose 7 bits TBS field in R2D message, except access trigger message.

Proposal 6: The R2D TBS information is not needed for fixed size R2D message, i.e., access trigger message.

Proposal 7: 7 bits TBS field is introduced to variable size R2D message, e.g., paging message, Msg2, R2D Upper Layer Data Transfer message, NACK Feedback message.

Conclusion

According to the above discussion, the following proposals are given:

AS ID

Proposal 1: AS ID release message is not supported in R19.

Write operation response

Proposal 2: From RAN2’s perspective, the reader can obtain the “write” results from the devices with longer processing times through multiple D2R scheduling.

Proposal 3: If the device receives D2R scheduling before the device has finished the “write” operation, it sets SDU Length as 0, sets MDI (More Data Indicator) as “1”, absent Data SDU and fills MAC Padding for all the remaining scheduled resources.

D2R message type

Proposal 4: D2R message type is not introduced in R19 A-IoT D2R MAC PDU.

R2D message format

Proposal 5: Introduce 1 bit indicator field to R2D Upper Layer Data Transfer message to indicate whether Upper Layer Data SDU or Received Data Size is present.

TBS information

Proposal 6: The R2D TBS information is not needed for fixed size R2D message, i.e., access trigger message.

Proposal 7: 7 bits TBS field is introduced to variable size R2D message, e.g., paging message, Msg2, R2D Upper Layer Data Transfer message, NACK Feedback message.

Reference

[1] Post email discussion [027], Remaining A-IoT MAC open issues, Huawei