3GPP TSG RAN WG2 Meeting #131	R2-2505952

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	8.11.2

Source:	CMCC

Title:	Discussion on random access in SBFD

Document for:	Discussion

Introduction

In RAN2#130 meeting, the following agreements have been made [1]:

Agreements on MAC open issues For L3 HO and BFR cases, CSI-RS based CFRA using SBFD RO is supported from RAN2 perspective. Send LS to RAN1/4 to inform this conclusion. Msg1 repetition number fallback can be supported within SBFD RO. Once the conditions for both RO type fallback and Msg1 repetition number fallback are met, UE should perform RO type switch. FFS the Msg1 repetition number after RO type switch in this case. For RACH fallback from one RO type to another, the UE shall only be allowed to switch to an RO type that is configured with the same feature combinations. The UE is allowed to switch to an RO type that is configured with the same Msg1 repetition number. FFS on higher Msg1 repetition number, if the same is not available. Other agreements SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA. For PRACH configuration option 1, when a feature combination is configured using preamble partitioning of the RACH-ConfigCommon, using the shared ssb-SharedRO-MaskIndex configuration for Additional-ROs and Legacy-ROs. For PRACH configuration option 1, same featureCombinationPreamblesList configuration is used for both Additional-ROs and Legacy-ROs.

Then in this contribution, we would like to provide our further views on RAN2 impacts to support random access in SBFD.

Discussion

## 2.1 General

### 2.1.1 SBFD operation in LTM

In RAN2#129bis meeting, the following agreement is made in terms of the RA events that support the SBFD operation [2]:

- Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM.

During post email discussion of RAN2#130 meeting, there are following three options are on table:

- Option 1: Not to support both "early UL synchronization with an LTM candidate cell in SBFD symbols” and "RACH-based LTM in SBFD symbols".

- Option 2: Only support “RACH-based LTM in SBFD symbols".

- Option 3: Support both "early UL synchronization with an LTM candidate cell in SBFD symbols” and “RACH-based LTM in SBFD symbols".

In terms of "early UL synchronization with an LTM candidate cell in SBFD symbols”, we think it brings large spec impact, because the candidate cell need to send the RACH configuration in SBFD symbols to the source cell. Therefore, we think that "early UL synchronization with an LTM candidate cell in SBFD symbols” is not supported in R19.

For "RACH-based LTM in SBFD symbols", we observe that it has benefit to improve UE’s handover performance, that is, reduce handover latency. And we do not see many spec impacts for both RAN2 and RAN3. To be specific, RAN2 needs to add a RO type indication to LTM cell swich command MAC CE. Besides, we do not see any spec impact in RAN3, because they quotes the RAN2 IE RACH-ConfigDedicated directly within UE CONTEXT MODIFICATION REQUEST message in F1AP. Therefore, we think “RACH-based LTM in SBFD symbols" can be supported. To this end, an RO type indication should be introduced to LTM cell switch command MAC CE.

Observation 1: Supporting "early UL synchronization with an LTM candidate cell in SBFD symbols”brings large spec impact.

Observation 2: Supporting "RACH-based LTM in SBFD symbols" has benefit to improve UE’s handover performance and does not bring many spec impacts.

Proposal 1: In R19, RAN2 is asked to support “RACH-based LTM in SBFD symbols", that is, an RO type indication is introduced to LTM cell switch command MAC CE.

### 2.1.2 SBFD operation in 2-step random access procedure

In RAN1#120 meeting, the following conclusion has been made [3]:

Conclusion There is no consensus in RAN1 to support Type-2 random access procedure (2-step RACH) in SBFD symbols for SBFD-aware UEs.

Since there is no consensus whether to support 2-step RACH or not in RAN1, RAN2 is asked to confirm that 2-step RACH is not supported in R19 and study the RACH configurations for 4-step CBRA and 4-step CFRA. For a comprehensive view, the detailed analysis of the 4-step CBRA and 4-step CFRA PRACH configuration based on Option 1 with Alt 1-1 or Option 2 with Alt 2-3 is conducted in section 2.2.2 and 2.2.3 respectively.

Observation 3: There is no consensus whether to support 2-step RACH or not in RAN1.

Proposal 2: RAN2 is asked to confirm that SBFD operation in 2-step random access procedure is not supported in R19.

### 2.1.3 Determination of RACH configuration option for CFRA

In RAN1 and RAN2, both RACH configuration Option 1 with Alt 1-1 and Option 2 with Alt 2-3 is supported. For CBRA, sbfd-RACH-SingleConfig-r19 IE in the BWP-UplinkCommon indicates whether RACH configuration Option 1 is enabled or not. And if sbfd-RACH-DualConfig-r19 IE is included in the BWP-UplinkCommon, it indicates that RACH configuration Option 2 is enabled, otherwise, RACH configuration Option 2 is not enabled. Further taking the agreement that “Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell”, we think that one SBFD-aware UE can determine the RACH configuration option in one cell based on the CBRA RACH configuration option.

Proposal 3: RAN2 confirms that the SBFD-aware UE can determine the RACH configuration option in one cell based on the CBRA RACH configuration option of that cell.

## 2.2 Issues related to MSG1 repetition

In RAN2#130 meeting, it has been agreed that [1]:

Msg1 repetition number fallback can be supported within SBFD RO. Once the conditions for both RO type fallback and Msg1 repetition number fallback are met, UE should perform RO type switch. FFS the Msg1 repetition number after RO type switch in this case. For RACH fallback from one RO type to another, the UE shall only be allowed to switch to an RO type that is configured with the same feature combinations. The UE is allowed to switch to an RO type that is configured with the same Msg1 repetition number. FFS on higher Msg1 repetition number, if the same is not available.

The remaining issue is that when RO type fallback condition is met, whether RO type fallback to the resource set with higher Msg1 repetition number than the current one is allowed or not, when the resource set with the same number is not available. With the consideration that a higher repetition number helps improve UE’s uplink coverage and thereby enhances performance of random access, we do not find any reason to prohibit this. In addition to improving UE performance, this can also better ensure the flexibility of the network in configuring random access resources. Based on above analysis, we think that when RO type fallback condition is met, RO type fallback to the resource set with higher Msg1 repetition number than the current one should be allowed, when the resource set with the same number is not available.

Observation 4: Allowing RO type fallback to the resource set with higher Msg1 repetition number than the current one can ensure the random access performance of SBFD-aware UE(s) and increase the flexibility for the network in configuring random access resources.

Proposal 4: RAN2 is asked to support RO type fallback to the resource set with higher Msg1 repetition number than the current one, when the resource set with the same number is not available.

## 2.3 Issues related to power compensation

In legacy NR, when RA type is switched from 2-step to 4-step during RA procedure, one power offset (i.e. POWER_OFFSET_2STEP_RA), whcih equals to (PREAMBLE_POWER_RAMPING_COUNTER - 1)× (MSGA_PREAMBLE_POWER_RAMPING_STEP - PREAMBLE_POWER_RAMPING_STEP), is added to compensate the uplink transmission power caused by the difference in power ramping steps. However, we have also noted that the reason why only the difference in power ramp steps is considered in POWER_OFFSET_2STEP_RA is that if 2-step and 4-step random access resources are configured simultaneously on the same BWP, their target received power is the same, both being preambleReceivedTargetPower.

In SBFD, for RACH configuration Option 2, both preambleReceivedTargetPower and powerRampingStep can both be independently configured on legacy RO and additional RO. In such case, we think that considering only the compensation for the difference in power ramping steps seems inappropriate. While, we are also not sure whether it is resonable to compensate the difference in target receiving power. Furthermore, considering the urgent time budget, we think that there is little urgency or necessity to support the power compensation during RO type switching in R19.

Observation 5: In legacy NR, when RA type is switched from 2-step to 4-step during RA procedure, one power offset (i.e. POWER_OFFSET_2STEP_RA) is added to compensate the uplink transmission power caused by the difference in power ramping steps.

Observation 6: In SBFD, for RACH configuration Option 2, both preambleReceivedTargetPower and powerRampingStep can both be independently configured on legacy RO and additional RO.

Proposal 5: RAN2 is asked to determine that the power compensation is not supported during RO type switching in R19.

## 2.4 Msg3-based early indication

Msg1-based early indication in legacy ROs has been agreed not to be supported in RAN2. For Msg3-based early indication, it has been widely discussed in RAN2#128 [4] but no consensus was reached. If Msg3-based early indication is supported, dedicated LCID(s) can be allocated to SBFD-aware UEs. With that, gNB can identify a SBFD-aware UE based on the logical channel ID in Msg3 and thus schedule subsequent uplink transmission, such as: NAS messages within NAS initial access procedure, etc., in SBFD symbol(s) for SBFD-aware UE. In terms of using reserved LCID(s) in present NR, there is almost no influence on the SBFD-aware UE itself and other UEs. However, if Msg3-based early indication is not supported, gNB can only identify a SBFD-aware UE through UE capability transfer which is later than that through Msg3-based early indication.

From our point of view, RAN2 is proposed to support Msg3-based early indication for SBFD-aware UEs, because there is no impact on PRACH capacity of non-SBFD-aware UEs and can bring benefits in increasing the performance of SBFD operation.

Observation 7: With Msg3-based early indication, there is almost no impact on PRACH capacity of non SBFD-aware UEs.

Observation 8: If Msg3-based early indication is not supported, network can only identify a SBFD-aware UE through UE capability transfer which is later than that through Msg3-based early indication.

Proposal 6: RAN2 is proposed to support Msg3-based early indication for SBFD-aware UEs.

## 2.5 RA-RNTI collision

For RACH configuration Option 2, it is possible that the additional ROs configured by the additional RACH configurations overlap with the legacy ROs. Such case may happen in flexible symbols, because both legacy and additional ROs can be configured in flexible symbols. When this overlapping happens, there is a possibility that RA-RNTI collision occurs. We are open to discuss whether to solve RA-RNTI collision issue. From our point of view, there are many solutions to solve this issue, such as:

- Solution 1: It is up to network implementation to solve this.

- Solution 2: A configurable offset of symbol index (s_id) or frequency index (f_id) can be introduced.

- Solution 3: An additional offset can be introduced to the formula for computing RA-RNTI, such as:

- RA-RNTI= 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 14 × 80 × 8 × 4  (1)

- For solution 1, there is no any spec impact. However, some companies think that solution may not solve RA-RNTI collision issue efficiently at some times. Solution 2 seems to be the most flexible solutions among the above three solutions, but RAN2 needs to discuss which message should be used for indicating the configured offset and design the corresponding IE. In terms of solution 3, RAN2 needs to define a new formula for computing RN-RNTI when addition RO is used by SBFD-aware UE, but do not need to define any new message or IE. Therefore, if RA-RNTI collision issue is agreed to be addressed by RAN2, we prefer to introduce an additional offset to the formula for computing RA-RNTI in additional RO. In addition, when designing the offset value, it is necessary to avoid collisions with the legacy RA-RNTI and MSGB-RNTI, as given in formula (1).

Observation 9: There are many solutions to solve RA-RNTI collision issue:

- Solution 1: It is up to network implementation to solve this.

- Solution 2: A configurable offset of symbol index (s_id) or frequency index (f_id) can be introduced.

- Solution 3: An additional offset can be introduced to the formula for computing RA-RNTI, such as:

Observation 10: When designing the offset value in the formula for computing RA-RNTI in additional ROs, it is necessary to avoid collisions with the legacy RA-RNTI and MSGB-RNTI.

Proposal 7: If RAN2 agrees to address the RA-RNTI collision issue, introducing an additional offset to the formula for computing RA-RNTI in additional RO is preferred.

# Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on random access in SBFD:

Observation 1: Supporting "early UL synchronization with an LTM candidate cell in SBFD symbols”brings large spec impact.

Observation 2: Supporting "RACH-based LTM in SBFD symbols" has benefit to improve UE’s handover performance and does not bring many spec impacts.

Observation 3: There is no consensus whether to support 2-step RACH or not in RAN1.

Observation 4: Allowing RO type fallback to the resource set with higher Msg1 repetition number than the current one can ensure the random access performance of SBFD-aware UE(s) and increase the flexibility for the network in configuring random access resources.

Observation 5: In legacy NR, when RA type is switched from 2-step to 4-step during RA procedure, one power offset (i.e. POWER_OFFSET_2STEP_RA) is added to compensate the uplink transmission power caused by the difference in power ramping steps.

Observation 6: In SBFD, for RACH configuration Option 2, both preambleReceivedTargetPower and powerRampingStep can both be independently configured on legacy RO and additional RO.

Observation 7: With Msg3-based early indication, there is almost no impact on PRACH capacity of non SBFD-aware UEs.

Observation 8: If Msg3-based early indication is not supported, network can only identify a SBFD-aware UE through UE capability transfer which is later than that through Msg3-based early indication.

Observation 9: There are many solutions to solve RA-RNTI collision issue:

- Solution 1: It is up to network implementation to solve this.

- Solution 2: A configurable offset of symbol index (s_id) or frequency index (f_id) can be introduced.

- Solution 3: An additional offset can be introduced to the formula for computing RA-RNTI, such as:

Observation 10: When designing the offset value in the formula for computing RA-RNTI in additional ROs, it is necessary to avoid collisions with the legacy RA-RNTI and MSGB-RNTI.

Proposal 1: In R19, RAN2 is asked to support “RACH-based LTM in SBFD symbols", that is, an RO type indication is introduced to LTM cell switch command MAC CE.

Proposal 2: RAN2 is asked to confirm that SBFD operation in 2-step random access procedure is not supported in R19.

Proposal 3: RAN2 confirms that the SBFD-aware UE can determine the RACH configuration option in one cell based on the CBRA RACH configuration option of that cell.

Proposal 4: RAN2 is asked to support RO type fallback to the resource set with higher Msg1 repetition number than the current one, when the resource set with the same number is not available.

Proposal 5: RAN2 is asked to determine that the power compensation is not supported during RO type switching in R19.

Proposal 6: RAN2 is proposed to support Msg3-based early indication for SBFD-aware UEs.

Proposal 7: If RAN2 agrees to address the RA-RNTI collision issue, introducing an additional offset to the formula for computing RA-RNTI in additional RO is preferred.

Reference

- “Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others”, RAN2#130, Malta.

- “Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others”, RAN2#129bis, Wuhan, China.

- “RAN1 Chair’s Notes”, RAN1#120, Athens, Greece.

- “Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, and SBFD”, RAN2#128, Orlando, USA.