3GPP TSG-RAN WG2 Meeting #131                          R2-2505967

Bangalore, India Aug 25th – 29th , 2025

Agenda item:	8.4.3

Source: 	CMCC

Title:	Remaining issues of RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Document for:	Discussion

- Introduction

After the RAN2#130 meeting, there are still the following open issues to be discussed[1]:

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| RRC-7 | FFS on exit condition for serving cell RRM relaxation Editor’s NOTE: FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements. | Issue Type: not essential but important How to address it: can be discussed based on companies’ contribution |
| RRC-8 | FFS on whether/how to reduce the threshold number for LP-WUS/WUR Editor’s NOTE: FFS on the relationship between the thresholds for serving cell relaxation/offloading, neighboring cell relaxation and entry/exit condition of using LP-WUS, [and potential pre-condition between RRM relaxation/offloading criteria and entry/exit condition of using LP-WUS]. | Issue Type: not essential but important How to address it: can be discussed based on companies’ contribution |
| RRC-10 | FFS on low mobility criteria Editor’s NOTE: FFS on “low mobility” criteria. | Issue Type: not essential not important How to address it: can be discussed based on companies’ contribution |
| RRC-15 | FFS on impact on R16 RRM relaxation criteria (e.g. can LR can be used to evaluate Rel-16 “not-at-cell-edge” and “low mobility” criterion). As raised by Ericsson above. | Issue Type: not essential not important How to address it: can be discussed based on companies’ contribution |

- Discussion

- RRC-7: FFS on exit condition for serving cell RRM relaxation

At the RAN2#128 and RAN2#129 meetings, it was agreed that if LR thresholds are configured, the entry condition for Case #3 is when both MR and LR measurements are above the configured thresholds.

RAN2#128 Agreements - The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed.

RAN2#129 Agreements - The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements. - If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

Therefore, when the LR threshold is configured, failure to meet the entry condition means that the MR or LR is below the configured threshold. If we want to discuss whether a separate exit condition is needed, we are actually discussing whether it is reasonable to use the MR or LR below the configured threshold as the exit condition for Case #3. Our view is that, considering that RAN4 has agreed to set the MR relaxation factor for Case #3 to 16, this will significantly increase the latency of obtaining MR measurements. Therefore, when defining the entry/exit conditions for Case #3, the LR measurement results should be fully considered to improve reliability. Therefore, Case #3 should be exited when either MR or LR falls below their respective entry thresholds.

Proposal 1: No separate exit condition is needed for serving cell RRM relaxation, and the exit condition can be defined as failing to meet the entry condition, i.e., MR or LR is below the configured threshold.

- RRC-8: FFS on whether/how to reduce the threshold number for LP-WUS/WUR

In the RAN2#130 meeting, it was agreed that entry condition for serving cell RRM offloading should be higher than the threshold(s) of entry condition for R19 RRM relaxation (serving cell relaxation and neighboring cell relaxation).

RAN2#130 Agreements - The corresponding threshold(s) of entry condition for serving cell RRM offloading should be higher than the threshold(s) of entry condition for R19 RRM relaxation (serving cell relaxation and neighboring cell relaxation), if there is such configuration. Capture this in the field description. Details will be discussed in the running CR.

Therefore, when R19 RRM relaxation is configured, in order to expand the scope of application, the entry threshold for LP-WUS monitoring can be merged with the entry threshold for R19 RRM relaxation.

Proposal 2: When R19 RRM relaxation is configured, the entry threshold for LP-WUS monitoring can be merged with the entry threshold for R19 RRM relaxation.

Furthermore, considering that serving cell RRM offloading is limited to the range where the UE does not perform neighboring cell RRM measurements, when the network only configures serving cell RRM offloading, the entry thresholds for LP-WUS monitoring and serving cell RRM offloading should not be merged, as this will limit the UE's energy-saving performance.

Proposal 3: When the network only configures serving cell RRM offloading, the entry thresholds for LP-WUS monitoring and serving cell RRM offloading should not be merged, as this will limit the UE's energy-saving performance.

- RRC-10: FFS on low mobility criteria

Considering that the latency of starting MR is as high as hundreds of milliseconds, we should be very cautious about offloading serving cell measurements to LR. If a UE is in a high mobility state, it is likely to reach the cell edge quickly, so the UE should use MR to perform neighbor measurements to prepare for possible handovers. If the UE performs measurement offloading and turns off MR at this time, on the one hand, due to the extremely high startup delay of the MR, the UE is likely to miss the best handover opportunity; on the other hand, it will also cause unnecessary conversion power consumption. Therefore, the mobility of UE should be considered for serving cell measurement offloading, that is, measurement offloading should not be performed when the UE is in a high-speed state.

Proposal 4: UEs with high-mobility do not perform serving cell measurement offloading.

Similarly, considering that RAN4 defines the MR relaxation factor in R19 RRM relaxation as 16, this will significantly affect the measurement report latency and the accuracy of the serving cell and neighbor cell measurement results, which may have a negative impact on mobility management. Therefore, high-mobility UEs should not perform R19 RRM relaxation.

Proposal 5: UEs with high-mobility do not perform R19 RRM relaxation.

- RRC-15: FFS on impact on R16 RRM relaxation criteria

For R19 RRM relaxation, RAN4 agreed that MR RRM relaxation scaling factor for serving cell/neighbour cell is 16, which is significantly larger than that of legacy Rel-16 UE power saving WI (i.e., 3).

RAN4#114 Agreement: - MR RRM relaxation for serving cell/neighbour cell: scaling factor is 16.

Considering that LR cannot assist MR in performing neighbor cell measurement, in our opinion, R19 RRM relaxation should have stricter evaluation conditions than R16. Therefore, the MR threshold in the entry condition of R19 RRM relaxation should be higher than SSearchThresholdP in R16.

Proposal 6: The threshold of entry condition of R19 RRM relaxation should be higher than SSearchThresholdP in R16.

At the same time, since the relaxation factor of MR is 16, it is very reasonable to use LR to assist in evaluating the Rel-16 “not-at-cell-edge” and “low mobility” criteria.

Proposal 7: Supports using LR to evaluate the Rel-16 "not-at-cell-edge" and "low mobility" criteria, and defines the corresponding LR threshold.

Proposal 8: The LR measurement can be used to evaluate the exit condition of the Rel-16 "not-at-cell-edge" criteria. That is, the exit condition of the Rel-16 "not-at-cell-edge" criteria is defined as when the MR or LR measurement is below the threshold.

Proposal 9: LR measurements can be used to evaluate the entry/exit conditions of Rel-16 “low mobility” criteria:

- Entry condition: If the LR threshold is configured, both MR and LR measurements are below their respective thresholds.

- Exit condition: The MR or LR measurement is above its respective threshold.

- Conclusions

In this contribution, we discussed the LP-WUS measurement relaxation/offloading in RRC_IDLE/INACTIVE mode, and following is our proposals.

Proposals:

Proposal 1: No separate exit condition is needed for serving cell RRM relaxation, and the exit condition can be defined as failing to meet the entry condition, i.e., MR or LR is below the configured threshold.

Proposal 2: When R19 RRM relaxation is configured, the entry threshold for LP-WUS monitoring can be merged with the entry threshold for R19 RRM relaxation.

Proposal 3: When the network only configures serving cell RRM offloading, the entry thresholds for LP-WUS monitoring and serving cell RRM offloading should not be merged, as this will limit the UE's energy-saving performance.

Proposal 4: UEs with high-mobility do not perform serving cell measurement offloading.

Proposal 5: UEs with high-mobility do not perform R19 RRM relaxation.

Proposal 6: The threshold of entry condition of R19 RRM relaxation should be higher than SSearchThresholdP in R16.

Proposal 7: Supports using LR to evaluate the Rel-16 "not-at-cell-edge" and "low mobility" criteria, and defines the corresponding LR threshold.

Proposal 8: The LR measurement can be used to evaluate the exit condition of the Rel-16 "not-at-cell-edge" criteria. That is, the exit condition of the Rel-16 "not-at-cell-edge" criteria is defined as when the MR or LR measurement is below the threshold.

Proposal 9: LR measurements can be used to evaluate the entry/exit conditions of Rel-16 “low mobility” criteria:

- Entry condition: If the LR threshold is configured, both MR and LR measurements are below their respective thresholds.

- Exit condition: The MR or LR measurement is above its respective threshold.

- References

- Discussion summary and list of RRC open issue for LP-WUS WUR.