3GPP TSG-WG2 #131	R2-2505988

# Bengaluru, India, 25th – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | Draft | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of UE capability for network energy saving enhancement in TS 38.331 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-08-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of UE capability for network energy saving enhancement in TS 38.331 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of UE capability for network energy saving enhancement in TS 38.331 based on the following agreements: RAN2#129 Agreement： - A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features. - Use a “container” using OCTET STRING with content generated by UE when new UE Radio Paging Capabilities are introduced. This is applicable only to new Rel-19 and future capabilities added to paging capabilities. This will only work for Rel-19 Gnb RAN2#130 Agreement： - The capability for paging adaption to be included in UE-RadioPagingInfo is a separate capability from other NES features and no need to define a common capability for all NES features. - Use a “container” using OCTET STRING with content generated by UE for paging adaption capability. - The paging adaption capability is per UE. - Introduce new capability for R19 PEI in NES and follow the existing PEI capability definition to have this new capability per band. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE capability for network energy saving enhancement is not captured in 38.331. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3, 11.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.306 CR XXXX |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

### 6.3.3	UE capability information elements

#### –	UE-NR-Capability

The IE UE-NR-Capability is used to convey the NR UE Radio Access Capability Parameters, see TS 38.306 [26].

UE-NR-Capability information element

-- ASN1START

-- TAG-UE-NR-CAPABILITY-START

UE-NR-Capability ::=            SEQUENCE {

accessStratumRelease            AccessStratumRelease,

pdcp-Parameters                 PDCP-Parameters,

rlc-Parameters                  RLC-Parameters                                                        OPTIONAL,

mac-Parameters                  MAC-Parameters                                                        OPTIONAL,

phy-Parameters                  Phy-Parameters,

rf-Parameters                   RF-Parameters,

measAndMobParameters            MeasAndMobParameters                                                  OPTIONAL,

fdd-Add-UE-NR-Capabilities      UE-NR-CapabilityAddXDD-Mode                                           OPTIONAL,

tdd-Add-UE-NR-Capabilities      UE-NR-CapabilityAddXDD-Mode                                           OPTIONAL,

fr1-Add-UE-NR-Capabilities      UE-NR-CapabilityAddFRX-Mode                                           OPTIONAL,

fr2-Add-UE-NR-Capabilities      UE-NR-CapabilityAddFRX-Mode                                           OPTIONAL,

featureSets                     FeatureSets                                                           OPTIONAL,

featureSetCombinations          SEQUENCE (SIZE (1..maxFeatureSetCombinations)) OF FeatureSetCombination         OPTIONAL,

lateNonCriticalExtension        OCTET STRING (CONTAINING UE-NR-Capability-v15c0)                      OPTIONAL,

nonCriticalExtension            UE-NR-Capability-v1530                                                OPTIONAL

}

-- Regular non-critical Rel-15 extensions:

UE-NR-Capability-v1530 ::=               SEQUENCE {

fdd-Add-UE-NR-Capabilities-v1530         UE-NR-CapabilityAddXDD-Mode-v1530                            OPTIONAL,

tdd-Add-UE-NR-Capabilities-v1530         UE-NR-CapabilityAddXDD-Mode-v1530                            OPTIONAL,

dummy                                    ENUMERATED {supported}                                       OPTIONAL,

interRAT-Parameters                      InterRAT-Parameters                                          OPTIONAL,

inactiveState                            ENUMERATED {supported}                                       OPTIONAL,

delayBudgetReporting                     ENUMERATED {supported}                                       OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1540                                       OPTIONAL

}

UE-NR-Capability-v1540 ::=              SEQUENCE {

sdap-Parameters                         SDAP-Parameters                                               OPTIONAL,

overheatingInd                          ENUMERATED {supported}                                        OPTIONAL,

ims-Parameters                          IMS-Parameters                                                OPTIONAL,

fr1-Add-UE-NR-Capabilities-v1540        UE-NR-CapabilityAddFRX-Mode-v1540                             OPTIONAL,

fr2-Add-UE-NR-Capabilities-v1540        UE-NR-CapabilityAddFRX-Mode-v1540                             OPTIONAL,

fr1-fr2-Add-UE-NR-Capabilities          UE-NR-CapabilityAddFRX-Mode                                   OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1550                                        OPTIONAL

}

UE-NR-Capability-v1550 ::=               SEQUENCE {

reducedCP-Latency                        ENUMERATED {supported}                                       OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1560                                       OPTIONAL

}

UE-NR-Capability-v1560 ::=               SEQUENCE {

nrdc-Parameters                         NRDC-Parameters                                               OPTIONAL,

receivedFilters                         OCTET STRING (CONTAINING UECapabilityEnquiry-v1560-IEs)       OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1570                                        OPTIONAL

}

UE-NR-Capability-v1570 ::=               SEQUENCE {

nrdc-Parameters-v1570                   NRDC-Parameters-v1570                                         OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1610                                        OPTIONAL

}

-- Late non-critical Rel-15 extensions:

UE-NR-Capability-v15c0 ::=               SEQUENCE {

nrdc-Parameters-v15c0                    NRDC-Parameters-v15c0                                        OPTIONAL,

partialFR2-FallbackRX-Req                ENUMERATED {true}                                            OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v15g0                                       OPTIONAL

}

UE-NR-Capability-v15g0 ::=               SEQUENCE {

rf-Parameters-v15g0                      RF-Parameters-v15g0                                          OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v15j0                                       OPTIONAL

}

UE-NR-Capability-v15j0 ::=               SEQUENCE {

-- Following field is only for REL-15 late non-critical extensions

lateNonCriticalExtension                 OCTET STRING (CONTAINING UE-NR-Capability-v15t0)             OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v16a0                                       OPTIONAL

}

UE-NR-Capability-v15t0 ::=               SEQUENCE {

featureSets-v15t0                        FeatureSets-v15t0                                            OPTIONAL,

measAndMobParameters-v15t0               MeasAndMobParameters-v15t0                                   OPTIONAL,

nonCriticalExtension                     SEQUENCE {}                                                  OPTIONAL

}

-- Regular non-critical Rel-16 extensions:

UE-NR-Capability-v1610 ::=               SEQUENCE {

inDeviceCoexInd-r16                     ENUMERATED {supported}                                        OPTIONAL,

dl-DedicatedMessageSegmentation-r16     ENUMERATED {supported}                                        OPTIONAL,

nrdc-Parameters-v1610                   NRDC-Parameters-v1610                                         OPTIONAL,

powSav-Parameters-r16                   PowSav-Parameters-r16                                         OPTIONAL,

fr1-Add-UE-NR-Capabilities-v1610        UE-NR-CapabilityAddFRX-Mode-v1610                             OPTIONAL,

fr2-Add-UE-NR-Capabilities-v1610        UE-NR-CapabilityAddFRX-Mode-v1610                             OPTIONAL,

bh-RLF-Indication-r16                   ENUMERATED {supported}                                        OPTIONAL,

directSN-AdditionFirstRRC-IAB-r16       ENUMERATED {supported}                                        OPTIONAL,

bap-Parameters-r16                      BAP-Parameters-r16                                            OPTIONAL,

referenceTimeProvision-r16              ENUMERATED {supported}                                        OPTIONAL,

sidelinkParameters-r16                  SidelinkParameters-r16                                        OPTIONAL,

highSpeedParameters-r16                 HighSpeedParameters-r16                                       OPTIONAL,

mac-Parameters-v1610                    MAC-Parameters-v1610                                          OPTIONAL,

mcgRLF-RecoveryViaSCG-r16               ENUMERATED {supported}                                        OPTIONAL,

resumeWithStoredMCG-SCells-r16          ENUMERATED {supported}                                        OPTIONAL,

resumeWithStoredSCG-r16                 ENUMERATED {supported}                                        OPTIONAL,

resumeWithSCG-Config-r16                ENUMERATED {supported}                                        OPTIONAL,

ue-BasedPerfMeas-Parameters-r16         UE-BasedPerfMeas-Parameters-r16                               OPTIONAL,

son-Parameters-r16                      SON-Parameters-r16                                            OPTIONAL,

onDemandSIB-Connected-r16               ENUMERATED {supported}                                        OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1640                                        OPTIONAL

}

UE-NR-Capability-v1640 ::=               SEQUENCE {

redirectAtResumeByNAS-r16               ENUMERATED {supported}                                        OPTIONAL,

phy-ParametersSharedSpectrumChAccess-r16  Phy-ParametersSharedSpectrumChAccess-r16                    OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1650                                        OPTIONAL

}

UE-NR-Capability-v1650 ::=               SEQUENCE {

mpsPriorityIndication-r16                ENUMERATED {supported}                                       OPTIONAL,

highSpeedParameters-v1650                HighSpeedParameters-v1650                                    OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1690                                       OPTIONAL

}

UE-NR-Capability-v1690 ::=               SEQUENCE {

ul-RRC-Segmentation-r16                  ENUMERATED {supported}                                       OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1700                                       OPTIONAL

}

-- Late non-critical extensions from Rel-16 onwards:

UE-NR-Capability-v16a0 ::=               SEQUENCE {

phy-Parameters-v16a0                     Phy-Parameters-v16a0                                         OPTIONAL,

rf-Parameters-v16a0                      RF-Parameters-v16a0                                          OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v16c0                                       OPTIONAL

}

UE-NR-Capability-v16c0 ::=               SEQUENCE {

rf-Parameters-v16c0                      RF-Parameters-v16c0                                          OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v16d0                                       OPTIONAL

}

UE-NR-Capability-v16d0 ::=               SEQUENCE {

featureSets-v16d0                        FeatureSets-v16d0                                            OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v16j0                                       OPTIONAL

}

UE-NR-Capability-v16j0 ::=               SEQUENCE {

rf-Parameters-v16j0                      RF-Parameters-v16j0                                          OPTIONAL,

-- Following field is only for REL-16 late non-critical extensions

lateNonCriticalExtension                 OCTET STRING (CONTAINING UE-NR-Capability-v16k0)             OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v17b0                                       OPTIONAL

}

UE-NR-Capability-v16k0 ::=               SEQUENCE {

featureSets-v16k0                        FeatureSets-v16k0                                            OPTIONAL,

nonCriticalExtension                     SEQUENCE {}                                                  OPTIONAL

}

-- Regular non-critical Rel-17 extensions:

UE-NR-Capability-v1700 ::=               SEQUENCE {

inactiveStatePO-Determination-r17        ENUMERATED {supported}                                       OPTIONAL,

highSpeedParameters-v1700                HighSpeedParameters-v1700                                    OPTIONAL,

powSav-Parameters-v1700                  PowSav-Parameters-v1700                                      OPTIONAL,

mac-Parameters-v1700                     MAC-Parameters-v1700                                         OPTIONAL,

ims-Parameters-v1700                     IMS-Parameters-v1700                                         OPTIONAL,

measAndMobParameters-v1700               MeasAndMobParameters-v1700,

appLayerMeasParameters-r17               AppLayerMeasParameters-r17                                   OPTIONAL,

redCapParameters-r17                     RedCapParameters-r17                                         OPTIONAL,

ra-SDT-r17                               ENUMERATED {supported}                                       OPTIONAL,

srb-SDT-r17                              ENUMERATED {supported}                                       OPTIONAL,

gNB-SideRTT-BasedPDC-r17                 ENUMERATED {supported}                                       OPTIONAL,

bh-RLF-DetectionRecovery-Indication-r17  ENUMERATED {supported}                                       OPTIONAL,

nrdc-Parameters-v1700                    NRDC-Parameters-v1700                                        OPTIONAL,

bap-Parameters-v1700                     BAP-Parameters-v1700                                         OPTIONAL,

musim-GapPreference-r17                  ENUMERATED {supported}                                       OPTIONAL,

musimLeaveConnected-r17                  ENUMERATED {supported}                                       OPTIONAL,

mbs-Parameters-r17                       MBS-Parameters-r17,

nonTerrestrialNetwork-r17                ENUMERATED {supported}                                       OPTIONAL,

ntn-ScenarioSupport-r17                  ENUMERATED {gso, ngso}                                       OPTIONAL,

sliceInfoforCellReselection-r17          ENUMERATED {supported}                                       OPTIONAL,

ue-RadioPagingInfo-r17                   UE-RadioPagingInfo-r17                                       OPTIONAL,

-- R4 17-2 UL gap pattern for Tx power management

ul-GapFR2-Pattern-r17                    BIT STRING (SIZE (4))                                        OPTIONAL,

ntn-Parameters-r17                       NTN-Parameters-r17                                           OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1740                                       OPTIONAL

}

UE-NR-Capability-v1740 ::=               SEQUENCE {

redCapParameters-v1740                   RedCapParameters-v1740,

nonCriticalExtension                     UE-NR-Capability-v1750                                       OPTIONAL

}

UE-NR-Capability-v1750 ::=               SEQUENCE {

crossCarrierSchedulingConfigurationRelease-r17  ENUMERATED {supported}                                OPTIONAL,

nonCriticalExtension                            UE-NR-Capability-v1800                                OPTIONAL

}

-- Late non-critical extensions from Rel-17 onwards:

UE-NR-Capability-v17b0 ::=           SEQUENCE {

mac-Parameters-v17b0                     MAC-Parameters-v17b0                                         OPTIONAL,

rf-Parameters-v17b0                      RF-Parameters-v17b0                                          OPTIONAL,

ul-RRC-MaxCapaSegments-r17               ENUMERATED {supported}                                       OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v17c0                                       OPTIONAL

}

UE-NR-Capability-v17c0 ::=               SEQUENCE {

mac-Parameters-v17c0                     MAC-Parameters-v17c0                                         OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v17d0                                       OPTIONAL

}

UE-NR-Capability-v17d0 ::=               SEQUENCE {

featureSets-v17d0                        FeatureSets-v17d0                                            OPTIONAL,

nonCriticalExtension                     SEQUENCE {}                                                  OPTIONAL

}

-- Regular non-critical Rel-18 extensions:

UE-NR-Capability-v1800 ::=               SEQUENCE {

airToGroundNetwork-r18                   ENUMERATED {supported}                                       OPTIONAL,

eRedCapParameters-r18                    ERedCapParameters-r18                                        OPTIONAL,

ncr-Parameters-r18                       NCR-Parameters-r18                                           OPTIONAL,

softSatelliteSwitchResyncNTN-r18         ENUMERATED {supported}                                       OPTIONAL,

hardSatelliteSwitchResyncNTN-r18         ENUMERATED {supported}                                       OPTIONAL,

mt-SDT-r18                               ENUMERATED {supported}                                       OPTIONAL,

mt-SDT-NTN-r18                           ENUMERATED {supported}                                       OPTIONAL,

inDeviceCoexIndAutonomousDenial-r18      ENUMERATED {supported}                                       OPTIONAL,

inDeviceCoexIndFDM-r18                   ENUMERATED {supported}                                       OPTIONAL,

inDeviceCoexIndTDM-r18                   ENUMERATED {supported}                                       OPTIONAL,

musim-GapPriorityPreference-r18          ENUMERATED {supported}                                       OPTIONAL,

musim-CapabilityRestriction-r18          ENUMERATED {supported}                                       OPTIONAL,

dummy                                    ENUMERATED {supported}                                       OPTIONAL,

ra-InsteadCG-SDT-r18                     ENUMERATED {supported}                                       OPTIONAL,

resumeAfterSDT-Release-r18               ENUMERATED {supported}                                       OPTIONAL,

ul-TrafficInfo-r18                       ENUMERATED {supported}                                       OPTIONAL,

aerialParameters-r18                     AerialParameters-r18                                         OPTIONAL,

--R4 40-2: beam steering

ntn-VSAT-AntennaType-r18                 ENUMERATED {electronic, mechanical}                          OPTIONAL,

--R4 40-1: VSAT UE type in NTN

ntn-VSAT-MobilityType-r18                ENUMERATED {fixed, mobile}                                   OPTIONAL,

ntn-Parameters-v1820                     NTN-Parameters-v1820                                         OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1830                                       OPTIONAL

}

UE-NR-Capability-v1830 ::=               SEQUENCE {

sib19-Support-r18                        ENUMERATED {supported}                                       OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1860                                       OPTIONAL

}

UE-NR-Capability-v1860 ::=               SEQUENCE {

ntn-CHO-OnlyLocationTimeTrigger-r18      ENUMERATED {supported}                                       OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v19xy                             OPTIONAL

}

UE-NR-Capability-v19xy ::=               SEQUENCE {

ue-RadioPagingInfo-r19                   OCTET STRING (CONTAINING UE-RadioPagingInfo-r19)     OPTIONAL,

nonCriticalExtension                     SEQUENCE{}                                                   OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode ::=          SEQUENCE {

phy-ParametersXDD-Diff                   Phy-ParametersXDD-Diff                                       OPTIONAL,

mac-ParametersXDD-Diff                   MAC-ParametersXDD-Diff                                       OPTIONAL,

measAndMobParametersXDD-Diff             MeasAndMobParametersXDD-Diff                                 OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode-v1530 ::=    SEQUENCE {

eutra-ParametersXDD-Diff                 EUTRA-ParametersXDD-Diff

}

UE-NR-CapabilityAddFRX-Mode ::=          SEQUENCE {

phy-ParametersFRX-Diff                   Phy-ParametersFRX-Diff                                       OPTIONAL,

measAndMobParametersFRX-Diff             MeasAndMobParametersFRX-Diff                                 OPTIONAL

}

UE-NR-CapabilityAddFRX-Mode-v1540 ::=    SEQUENCE {

ims-ParametersFRX-Diff                   IMS-ParametersFRX-Diff                                       OPTIONAL

}

UE-NR-CapabilityAddFRX-Mode-v1610 ::=    SEQUENCE {

powSav-ParametersFRX-Diff-r16            PowSav-ParametersFRX-Diff-r16                                OPTIONAL,

mac-ParametersFRX-Diff-r16               MAC-ParametersFRX-Diff-r16                                   OPTIONAL

}

BAP-Parameters-r16 ::=                   SEQUENCE {

flowControlBH-RLC-ChannelBased-r16       ENUMERATED {supported}                                       OPTIONAL,

flowControlRouting-ID-Based-r16          ENUMERATED {supported}                                       OPTIONAL

}

BAP-Parameters-v1700 ::=                 SEQUENCE {

bapHeaderRewriting-Rerouting-r17         ENUMERATED {supported}                                       OPTIONAL,

bapHeaderRewriting-Routing-r17           ENUMERATED {supported}                                       OPTIONAL

}

MBS-Parameters-r17 ::=                   SEQUENCE {

maxMRB-Add-r17                           INTEGER (1..16)                                              OPTIONAL

}

-- TAG-UE-NR-CAPABILITY-STOP

-- ASN1STOP

UE-NR-Capability field descriptions

featureSetCombinations A list of FeatureSetCombination:s for supportedBandCombinationList in UE-NR-Capability. The FeatureSetDownlink:s and FeatureSetUplink:s referred to from these FeatureSetCombination:s are defined in the featureSets list in UE-NR-Capability.

UE-NR-Capability-v1540 field descriptions

fr1-fr2-Add-UE-NR-Capabilities This instance of UE-NR-CapabilityAddFRX-Mode does not include any other fields than csi-RS-IM-ReceptionForFeedback/ csi-RS-ProcFrameworkForSRS/ csi-ReportFramework.

Next Change

#### –	UE-RadioPagingInfo

The IE UE-RadioPagingInfo contains UE capability information needed for paging.

UE-RadioPagingInfo information element

-- ASN1START

-- TAG-UE-RADIOPAGINGINFO-START

UE-RadioPagingInfo-r17 ::=            SEQUENCE {

-- R1 29-1: Paging enhancement

pei-SubgroupingSupportBandList-r17    SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR    OPTIONAL,

...

}

UE-RadioPagingInfo-r19 ::=            SEQUENCE {

pagingAdaptation-r19                     ENUMERATED {supported}                                  OPTIONAL,

pagingAdaptionPEI-SupportBandList-r19    SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR    OPTIONAL,

...

}

-- TAG-UE-RADIOPAGINGINFO-STOP

-- ASN1STOP

Next Change

### 11.2.2	Message definitions

#### –	UERadioPagingInformation

This message is used to transfer radio paging information, covering both upload to and download from the 5GC, and between gNBs.

Direction: gNB to/ from 5GC and gNB to/from gNB

UERadioPagingInformation message

-- ASN1START

-- TAG-UE-RADIO-PAGING-INFORMATION-START

UERadioPagingInformation ::= SEQUENCE {

criticalExtensions                  CHOICE {

c1                                  CHOICE{

ueRadioPagingInformation            UERadioPagingInformation-IEs,

spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

UERadioPagingInformation-IEs ::=    SEQUENCE {

supportedBandListNRForPaging        SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR    OPTIONAL,

nonCriticalExtension                UERadioPagingInformation-v15e0-IEs                      OPTIONAL

}

UERadioPagingInformation-v15e0-IEs ::= SEQUENCE {

dl-SchedulingOffset-PDSCH-TypeA-FDD-FR1     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeA-TDD-FR1     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeA-TDD-FR2     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-FDD-FR1     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-TDD-FR1     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-TDD-FR2     ENUMERATED {supported}          OPTIONAL,

nonCriticalExtension                UERadioPagingInformation-v1700-IEs          OPTIONAL

}

UERadioPagingInformation-v1700-IEs ::= SEQUENCE {

ue-RadioPagingInfo-r17                 OCTET STRING (CONTAINING UE-RadioPagingInfo-r17)     OPTIONAL,

inactiveStatePO-Determination-r17      ENUMERATED {supported}                               OPTIONAL,

numberOfRxRedCap-r17                   ENUMERATED {one, two}                                OPTIONAL,

halfDuplexFDD-TypeA-RedCap-r17         SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR OPTIONAL,

nonCriticalExtension                   UERadioPagingInformation-v1800-IEs                   OPTIONAL

}

UERadioPagingInformation-v1800-IEs ::= SEQUENCE {

numberOfRxERedCap-r18                  ENUMERATED {one, two}                                OPTIONAL,

supportOf2RxXR-r18                     ENUMERATED {supported}                               OPTIONAL,

nonCriticalExtension                   UERadioPagingInformation-v1840-IEs                   OPTIONAL

}

UERadioPagingInformation-v1840-IEs ::= SEQUENCE {

dl-SchedulingOffset-PDSCH-TypeA-FDD-FR2-NTN-r18    ENUMERATED {supported}                   OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-FDD-FR2-NTN-r18    ENUMERATED {supported}                   OPTIONAL,

nonCriticalExtension                               SEQUENCE {}                              OPTIONAL

}

UERadioPagingInformation-v19xy-IEs ::= SEQUENCE {

ue-RadioPagingInfo-r19                 OCTET STRING (CONTAINING UE-RadioPagingInfo-r19)     OPTIONAL,

nonCriticalExtension                               SEQUENCE {}                              OPTIONAL

}

-- TAG-UE-RADIO-PAGING-INFORMATION-STOP

-- ASN1STOP

UERadioPagingInformation field descriptions

supportedBandListNRForPaging Indicates the UE supported NR frequency bands which are derived by the gNB from UE-NR-Capability.

dl-SchedulingOffset-PDSCH-TypeA-FDD-FR1 Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type A in FDD FR1.

dl-SchedulingOffset-PDSCH-TypeA-FDD-FR2-NTN Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type A in FDD FR2-NTN.

dl-SchedulingOffset-PDSCH-TypeA-TDD-FR1 Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type A in TDD FR1.

dl-SchedulingOffset-PDSCH-TypeA-TDD-FR2 Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type A in TDD FR2.

dl-SchedulingOffset-PDSCH-TypeB-FDD-FR1 Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type B in FDD FR1.

dl-SchedulingOffset-PDSCH-TypeB-FDD-FR2-NTN Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type B in FDD FR2-NTN.

dl-SchedulingOffset-PDSCH-TypeB-TDD-FR1 Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type B in TDD FR1.

dl-SchedulingOffset-PDSCH-TypeB-TDD-FR2 Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type B in TDD FR2.

halfDuplexFDD-TypeA-RedCap Indicates whether the (e)RedCap UE only supports half-duplex operation for FDD in the indicated band(s).

inactiveStatePO-Determination Indicates whether the UE supports to use the same i_s to determine PO in RRC_INACTIVE state as in RRC_IDLE state.

numberOfRxERedCap Indicates the number of Rx branches supported by an eRedCap UE.

numberOfRxRedCap Indicates the number of Rx branches supported by a RedCap UE.

supportOf2RxXR Indicates whether the UE is a 2Rx XR UE.

ue-RadioPagingInfo The field is used to transfer UE capability information used for paging. The gNB generates the ue-RadioPagingInfo-r17 and the contained UE capability information is absent when not supported by the UE. The content of ue-RadioPagingInfo-r19 is generated by the UE.

End of Change