3GPP TSG-RAN WG2 #131	R2-2506001

Bengaluru, India, 25th – 29th Aug. 2025

Source:                    	DENSO CORPORATION

Title:  	Discussion on RLC enhancements

Document for:        	Discussion and decision

Agenda Item:         	8.7.5		RLC enhancements

- Introduction

In the email discussion after RAN2#130 meeting [1], the following open issues for RLC enhancements were collected by companies.

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| RLC-10 | Whether/How to introduce any limitations or associations for the configuration of stopReTxObsoleteSDU and DL t-RxDiscard. - As raised by ZTE below | Issue Type: Not essential not important How to address it: based on companies’ contribution |
| RLC-11 | FFS how to handle the case “ if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with” - As raised by Nokia and Apple below RAN2 agreement in #130 meeting: Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting) Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work | Issue Type: Not essential but important How to address it: based on companies’ contribution |
| RLC-14 | If stopReTxDiscardedSDU is configured, when indicated from upper layer to discard a particular RLC SDU, whether the transmitting side of an AM RLC entity shall discard the corresponding RLC SDU or RLC SDU segment(s) if the RLC SDU or the RLC SDU segment(s) thereof has been submitted to lower layers. As raised by NEC below | Issue Type: Not essential not important How to address it: based on companies’ contribution |

In this contribution, we provide our views on the open issues.

- Discussion

For RLC-10, this issue may occur in cases where the transmitting entity stops retransmitting discarded SDUs, while the peer receiving entity is not configured with Rx timer-based discard. However, this can be left to network implementation. For example, in the downlink, if the gNB stops retransmissions, it would be sufficient for the gNB to ensure that the receiving UE is configured with t-RxDiscard. Conversely, in the uplink, the gNB can activate UL Rx timer-based discard to configure stopReTxDiscardedSDU at the transmitting UE.

Proposal 1:	(RLC-10) There is no need to introduce limitations or associations for the configuration of UL stopReTxObsoleteSDU and DL t-RxDiscard. This can be left to network implementation.

For RLC-11, there is a possibility that the window stalling issue remains unresolved if there is no SDU to retransmit a poll with due to discard indications from the PDCP layer. Therefore, in our opinion, in order to advance the transmitting window as soon as possible, it would be necessary to retransmit a poll with an SDU upon the expiry of t-PollRetransmit, even if the SDU has been indicated as discarded. For example, this can be achieved simply by keeping the current specification unchanged, without introducing the additional modifications highlighted in yellow.

R2-2505402, Running RLC CR [2] 5.3.3.4 Expiry of t-PollRetransmit Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements and excluding RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped as specified in clause 5.2.3.1.1); or - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling): - consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission (excluding RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped as specified in clause 5.2.3.1.1); or - consider any RLC SDU which has not been positively acknowledged for retransmission (excluding RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped as specified in clause 5.2.3.1.1). - include a poll in an AMD PDU, if any, as described in clause 5.3.3.2.

Proposal 2:	(RLC-11) The window stalling issue with polling retransmission should be addressed, i.e., UE needs to retransmit a poll with a SDU upon the expiry of t-PollRetransmit, even if the SDU has been indicated as discarded.

Proposal 3:	(RLC-11) For the window stalling issue with polling retransmission, RAN2 can keep the current Rel-18 specification related to expiry of t-PollRetransmit unchanged.

For RLC-14, considering the window stalling issue raised in RLC-11, we believe that the UE still needs to buffer RLC SDUs even if the RLC SDUs have been indicated as discard. Otherwise, even the RLC SDU to retransmit a poll with may be discarded.

Proposal 4:	(RLC-14) Considering the window stalling issue raised in RLC-11, UE still needs to buffer RLC SDUs to retransmit a poll with even if the RLC SDUs have been indicated as discard.

- Conclusion

In this contribution, we provided the following proposals.

Proposal 1:	(RLC-10) There is no need to introduce limitations or associations for the configuration of UL stopReTxObsoleteSDU and DL t-RxDiscard. This can be left to network implementation.

Proposal 2:	(RLC-11) The window stalling issue with polling retransmission should be addressed, i.e., UE needs to retransmit a poll with a SDU upon the expiry of t-PollRetransmit, even if the SDU has been indicated as discarded.

Proposal 3:	(RLC-11) For the window stalling issue with polling retransmission, RAN2 can keep the current Rel-18 specification related to expiry of t-PollRetransmit unchanged.

Proposal 4:	(RLC-14) Considering the window stalling issue raised in RLC-11, UE still needs to buffer RLC SDUs to retransmit a poll with even if the RLC SDUs have been indicated as discard.

- References

[1] 	R2-2505403, Summary of RLC open issue list for R19 XR, vivo

[2] R2-2505402, Introduction of R19 XR enhancements for RLC spec, vivo