3GPP TSG-RAN WG2 Meeting #131	R2- 2506017

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item	:		8.8.4

Source	: 	Sharp

Title	: 	Remaining issues on intended service area

Document for	:	Discussion and Decision

Introduction

In the last meeting, RAN2 reached some agreements/decisions as follows [1];

Agreements: - Do not introduce service area information specific for FSAI in SIB21. - Do not introduce service area per neighbour cell in MCCH - RAN2 assumes that the ISA can be valid for all the frequencies providing the MBS session - The new SIBxx containing MBS ISA(s) can be cell or area specific, which is up to NW implementation (No specs change).

In this paper, we discuss remaining issues on ISA.

Discussion

MBS interest indication transmission by UE within ISA

If UE is allowed to send MII from outside of the intended service area, UL resources are wasted by UEs which are not intended to receive the service. To prevent such negative impacts, the specification should limit UE behaviours for MII. Then, if the MBS service is associated to the intended service area(s), MII should not be sent by UE located outside of the intended service area.

Observation 1: If UE is allowed to send MBSInterestIndication message from outside of the intended service area, UL resources are wasted by UEs which are not intended to receive the MBS service.

Proposal 1: MBSInterestIndication message should be sent considering the intended service area.

For Rel-19 ISA capable UEs

In the last meeting, MII related topic was discussed and concluded [1] that

R2-2503560 Discussions on supporting broadcast service Fujitsu discussion Rel-19 NR_NTN_Ph3-Core Observation 1: Behaviours with respect to the initiation of MII procedure upon entering or leaving the area of broadcast service has already been defined. Proposal 1: RAN2 to confirm that, with respect to the MBS service, the term ‘broadcast service area’ refers to the ‘intended service area’. It could be captured in Stage 2 specification. => Come back in the next meeting

In addition, at the post email discussion [2], it was concluded that

FFS on the terminology misalignment between ‘broadcast service area’ vs. ‘intended service area’ and the potential impact on the MII procedure.

Base on the current RRC specification, the MII can be triggered based on the UE entering or leaving a broadcast service area. For services associated with ISA, the broadcast service area should be the area indicated by ISA(s) which is associated with service(s), and MII should be triggered based on the UE entering or leaving the ISA.

Though the term “broadcast service area” is defined in TS 23.247 and the Release 19 specifications have already been published, the relationship between the broadcast service area and ISA remains unclear and is not explicitly described.

However, RAN3 CR [3] related to NR NTN for TS38.300 was endorsed, and in this CR, there is a description that

The AMF may provide the gNB with the service area information for MBS broadcast service. This information may be signaled as cell ID list, TAI list, and/or MBS intended service area list, as defined in TS 38.413 [26]. Based on this received information, the gNB provides the intended service area information to the UEs in the relevant cell(s) as defined in TS 38.331 [12].

Therefore, it can be understood that the broadcast service area is associated to ISA, and no additional description is needed.

Proposal 2: The broadcast service area is associated to ISA in TS38.300, and no additional description is needed.

For legacy UEs

As NTN MBS supporting ISA is specified in Rel-19, we understand that if SIB20 is transmitted from Rel-19 NTN cell, legacy UEs may consider MBS services are available in the cell because there are no descriptions in legacy specifications to control MBS reception based on whether the cell is NTN cell or not.

In such a case, if all session information, i.e. both ISA related session information and no ISA related session information, are included in the MBS-SessionInfo-r17 IE and mappings are indicated in extended IE (i.e. MBS-SessionInfoList-v19xy) as the current RRC CR, legacy UEs would interpret each session included in MBS-SessionInfoList-r17 can be receivable even if the service is associated to ISA.

Observation 2: If NTN MBS supporting ISA are deployed, legacy UEs would interpret the sessions included in MBS-SessionInfo-r17 IE are receivable (regardless sessions are associated with any ISA or not).

To address the issue, two possible approaches are proposed.

The first approach is NW accepts MII triggers from legacy UEs outside the ISA, though UL resources are wasted.

The second approach is Services associated with ISA are hidden from legacy UEs, thereby preventing unintended MII triggers from legacy UEs outside the ISA.

Proposal 3: RAN2 confirms one of the following approaches for MII operation of legacy UEs.

- Approach 1: NW accepts MII triggers from legacy UEs outside the ISA.

- Approach 2: Services associated with ISA are hidden from legacy UEs.

If approach 2 is selected, to achieve the approach 2, IEs for ISA capable UEs and legacy UEs should be separated in the MCCH. Therefore, we propose the solution which both MBS-SessionInfo-r17 IE and MBS-SessionInfoList-v19xy IE are used to indicate session information. In detail, to avoid duplicate TMGI fields and to support the backward compatible behaviour (for not ISA capable UEs), we think MBS sessions indicated by MBS-SessionInfo-r17 IE should not be associated with any ISAs (i.e. whole cell is a service area), and MBS-SessionInfoList-v19xy IE includes session information which are associated with ISA, and ISA mapping information. There are advantages to splitting IE between r17 and r19 besides avoiding duplication of TMGI and compatibility with legacy.

Proposal 4: If approach 2 is selected, MBS-SessionInfoList-v19xy IE includes the ISA ID together with session information which is associated with ISA, and MBS-SessionInfo-r17 IE includes session information which is not associated with ISA.

Conclusion

In this paper, we made the following observations and proposals;

Observation 1: If UE is allowed to send MBSInterestIndication message from outside of the intended service area, UL resources are wasted by UEs which are not intended to receive the MBS service.

Proposal 1: MBSInterestIndication message should be sent considering the intended service area.

Proposal 2: The broadcast service area is associated to ISA in TS38.300, and no additional description is needed.

Observation 2: If NTN MBS supporting ISA are deployed, legacy UEs would interpret the sessions included in MBS-SessionInfo-r17 IE are receivable (regardless sessions are associated with any ISA or not).

Proposal 3: RAN2 confirms one of the following approaches for MII operation of legacy UEs.

- Approach 1: NW accepts MII triggers from legacy UEs outside the ISA.

- Approach 2: Services associated with ISA are hidden from legacy UEs.

Proposal 4: If approach 2 is selected, MBS-SessionInfoList-v19xy IE includes the ISA ID together with session information which is associated with ISA, and MBS-SessionInfo-r17 IE includes session information which is not associated with ISA.

Reference

- Draft_RAN2_130_Meeting_Report_v1.docx

- (5841) R2-25xxxx [draft] Open_issue_list_RRC_NTN_v10_Xiaomi.docx

- R3-253093 Support for Regenerative Payload and MBS broadcast in NR NTN