3GPP TSG-RAN WG2 Meeting #131	R2-2506068

Bengaluru, India, Aug. 25th – 29th, 2025

Agenda Item:	8.7.5

Source:	HONOR

Title:	Discussion on RLC enhancements

Document for:	Discussion and Decision

1. Introduction

In the summary of RLC open issue for R19 XR [1] and RLC running CR [2], the following issue is discussed for remaining-time-based RLC polling:

| RLC-11 | FFS how to handle the case “if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with” - As raised by Nokia and Apple below RAN2 agreement in #130 meeting: Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting) Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work |
| --- | --- |
| Huawei | Polling triggering: The remaining time based polling triggering in current running CR relies on the prerequisite that there is AMD PDU to transmit. The case that the remaining time based polling triggering indication is received and there is no available AMD PDU to transmit should also be considered. [Rapp] This was discussed online, and it was addressed in the running CR (same response on the running CR comments). [HW]: 5.3.3.4 is for “Expiry of t-PollRetransmit”, we don’t think “Expiry of t-PollRetransmit” clause can cover the remaining time based polling but no AMD PDU for transmission case [Rapp2] Please find the response in the running CR. |

In this contribution, we discuss the potential solutions related to this issue.

2. Discussion on open issue RLC-11

(RLC-11) FFS how to handle the case “if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with”.

RLC running CR for R19 XR 5.3.3.2 Transmission of a AMD PDU Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements and excluding RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped as specified in clause 5.2.3.1.1) after the transmission of the AMD PDU; or - if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling); or - if an indication is received from upper layer (e.g., PDCP) that the condition for remaining-time-based RLC polling has been met for an RLC SDU: - include a poll in the AMD PDU as described below.

As can be seen from the running CR, remaining-time-based RLC polling is considered only when one AMD PDU submitted for transmission. However, there may be a situation where a remaining-time-based RLC polling is received but there is no SDUs to transmit in a transmission opportunity notified by lower layer.

- If t-PollRetransmit is running, it can wait for the expiry of t-PollRetransmit to consider an SDU for retransmission which is covered by the change in RLC running CR section 5.3.3.4.

- If t-PollRetransmit is not running, there is no opportunity to consider including a poll in a PDU for retransmission.

Therefore, if t-PollRetransmit is not running, the transmitting side of an AM RLC entity shall consider including a poll in an AMD PDU for retransmission in the above situation, e.g., the RLC SDU for which the condition for remaining-time-based RLC polling has been met, or the RLC SDU with the highest SN among the RLC SDUs.

Proposal 1: (RLC-11) If t-PollRetransmit is not running, upon notification of a transmission opportunity by lower layer, the RLC TX shall consider an SDU for retransmission when an indication of remaining-time-based RLC polling is received but there is no SDUs to transmit.

3. Conclusions

Proposal 1: (RLC-11) If t-PollRetransmit is not running, upon notification of a transmission opportunity by lower layer, the RLC TX shall consider an SDU for retransmission when an indication of remaining-time-based RLC polling is received but there is no SDUs to transmit.

4. References

[1]	R2-25xxxx_Summary of RLC open issue list for R19 XR_v09_Rapp5

[2]   R2-25xxxx_RLC running CR for XR_v21_Ofinno2