3GPP RAN WG2 Meeting #131	R2-2506070

Bangalore, India, August 25th – 29th, 2025

Agenda Item:	8.9.2

Source:	HONOR

Title:	Discussion on the Store and Forward satellite operation

Document for:	Discussion and Decision

1. Introduction

In Rel-19 Non-Terrestrial Networks (NTN) for Internet of Things (IoT) WID, the following objectives were achieved:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In RAN2#130 meeting, the following agreements were made:

- In a S&F network deployment which also exhibits discontinuous coverage, existing mechanisms to handle discontinuous coverage can be leveraged (e.g. satellite assistance information, UE not needing to perform idle mode tasks when the UE determines that is out of coverage, etc.). There is no need to modify existing discontinuous coverage features due to the addition of S&F Satellite operation. FFS if we clarify in discontinuous coverage procedure in idle mode that the UE also takes into account the information about NAS configured S&F monitoring list. - Clarify in Stage 2 from Rel-18 (so in general, not only for S&F) that the values used in “satelliteId-r17” are expected to correspond to the values used in “satelliteId-r18” in other SIBs (e.g. the value used to identify the serving satellite in SIB33, SIB32 and SIB31 should be the same). Come back in the next meeting with an actual Rel-18 CR for this. FFS if in Rel-19 we need to say anything about the correspondence between the “satelliteId” IE and “S&F Monitoring List”. - Working Assumption: - In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

In this contribution, we discuss the remaining issues on Store &Forward (S&F) operation.

2. Discussion

2.1 (issue 2) Working Assumption on SF mode operation indication for neighbour cells

In RAN2#130, it has made the following working assumption:

WA: In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

In S&F mode, the service link and the feeder link are not available simultaneously. When the service link is available the UE can send UL data, the newly generated downlink data cannot be transmitted. The UE has to establish a new connection with the subsequent satellite, which will have a negative impact on the signaling overhead. Therefore, in cell reselection, the S&F capable UE can deprioritize the S&F cells. And for the non-S&F capable UE, the cells operating on S&F mode are regarded as access barred cells. From this point of view, it is necessary to provide operation mode indication of the neighbour cells.

Proposal 1: An indication whether the cell operates in S&F mode or not should be provided for neighbour cells.

Proposal 2: For S&F capable UE, the UE can deprioritize the S&F cells in cell reselection. And for non-S&F capable UE, the cells operating on S&F mode are regarded as access barred cells.

In current TS36.331, an IE(t-ServiceStartNeigh-r18) has been defined to indicate the earliest time when the area covered by the current serving cell is going to be covered by the neighbour cell(s).

t-ServiceStartNeigh-r18 TimeOffsetUTC-r17 OPTIONAL -- Need OR } t-ServiceStartNeigh Indicates the earliest time when the area covered by the current serving cell is going to be covered by the neighbour cell(s) served by the satellite indicated by satelliteId, see 5.5.3.1, 5.5.8 and 36.304 [4]. This field is only present for the NTN quasi-Earth fixed neighbour cell(s).

In addition, there is a NOTE for time-based measurement initiation, which is described as follows:

NOTE 2A: If timeMeasConfig is configured and t-Service is configured in SystemInformationBlockType3, the exact time to start measurements before t-Service is left up to UE implementation and t-ServiceStartNeigh may be used to decide when to start measurements.

If the legacy t-ServiceStartNeigh is reused for S&F mode, the S&F operation indication is needed. Nevertheless, for the legacy UE and the Rel-19 UE without S&F capability, the UE may start measurement when the network is operating S&F. If this occurs, it is meaningless for these UEs to start the measurement, and it may further lead to an incorrect reselection to one neighbour cell that operated on S&F. Therefore, we think the introduction of an additional IE is necessary.

For the neighbor cell operated on S&F, an additional IE (e.g., t-ServiceStartNeigh_s&f-r19) can be defined. It indicates the earliest time when the area covered by the current serving cell is going to be covered by the neighbour cell(s) served by the satellite that operated on S&F mode currently.

Proposal 3: The S&F timing information of neighbour cell (e.g., t-ServiceStartNeigh_s&f-r19) can be provided as satellite assistance information.

2.2 (issue 3) NAS configured list usage for idle mode operation of SF mode capable UE

In RAN2#128, the following agreements were reached:

- RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite - RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

From the previous discussion, the UE is allowed to camp on or attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list. In TS23.401, there is a note stating that the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F monitoring list there is increased probability that it will not be able to complete the NAS procedure. Therefore, it would be better for UE to prioritize the cells served by the satellites in the NAS list. To achieve this intention, the global satellite ID should be provided for neighour satellites, which will assist UE in determining whether the satellite exists in the NAS configured list.

Proposal 4: The UE can prioritize the cells served by the satellites in the NAS list.

Proposal 5: The global satellite ID should be provided for neighour satellites.

2.3 (issue 4) Paging Enhancements for SF Mode operation

For CN-initiated paging, the MME-ground sends paging message to the satellites that are going to visit the UE’s tracking areas, when the feeder link becomes available. But for UE, before starting to monitor paging, the UE needs to decide whether there is a need to monitor paging. After monitoring paging from the last time, if the satellite has never connected to MME-ground via feeder link, the UE can skip paging monitoring. Therefore, the UE can decide whether to monitor paging based on the time information about the last time the feeder link was available.

Proposal 6: The UE can decide whether to monitor paging based on the time information about the last time the feeder link was available.

When the satellite operates in S&F mode with unavailable feeder link, and had not received any paging from MME-ground before, there will be no paging for the purpose of DL data transmission during the period when the service link is valid. If there is no need to perform paging due to the update of SI messages either, a skipped paging monitoring indication can be provided via SIB for paging monitoring relaxation.

Proposal 7: A skipped paging monitoring indication can be provided via SIB for paging monitoring relaxation.

In TS23.401, the NAS configured list related text:

The S&F Monitoring List includes satellite(s) which belong to the same PLMN and indicates the satellite(s) that the UE may (re)attempt NAS procedures or receive MT data from.

We understand that, when there is MT data from CN, the CN shall initial paging via the satellites that included in the NAS configured list. Therefore, when the UE perform paging monitoring, the UE should know whether the satellite is included in the NAS configured list and determine whether to monitor paging.

Proposal 8: The UE only performs paging monitoring on the satellite which is included in the NAS configured list.

In the NAS configured list, one or more satellites are identified by global satellite identifiers. However, in the current satellite assistance information broadcast in SIB does not provide the global satellite identifier. The UE cannot get to know whether the serving satellite is in the NAS configured list. Therefore, a global satellite id should be provided.

Proposal 9: The global satellite ID should be provided for the serving satellite.

3. Conclusions

In this contribution, we discussed the issues related to Store & Forward (S&F) satellite operation with split MME onboard. Based on the discussion, the following proposals are concluded:

Proposal 1: An indication whether the cell operates in S&F mode or not should be provided for neighbour cells.

Proposal 2: For S&F capable UE, the UE can deprioritize the S&F cells in cell reselection. And for non-S&F capable UE, the cells operating on S&F mode are regarded as access barred cells.

Proposal 3: The S&F timing information of neighbour cell (e.g., t-ServiceStartNeigh_s&f-r19) can be provided as satellite assistance information.

Proposal 4: The UE can prioritize the cells served by the satellites in the NAS list.

Proposal 5: The global satellite ID should be provided for neighour satellites.

Proposal 6: The UE can decide whether to monitor paging based on the time information about the last time the feeder link was available.

Proposal 7: A skipped paging monitoring indication can be provided via SIB for paging monitoring relaxation.

Proposal 8: The UE only performs paging monitoring on the satellite which is included in the NAS configured list.

Proposal 9: The global satellite ID should be provided for the serving satellite.

4. References