3GPP TSG-RAN WG2 Meeting #131							R2-2506118

Bengaluru, India, 25 - 29 Aug, 2025

Title: 	Further Considerations on NW Side Data collection

Source: 	ZTE Corporation

Agenda item:	8.1.3

Document for:	Discussion/Decision

- Introduction

For NW side data collection, the post meeting email discussion[1] have discussed two candidate frameworks and post email discussion [2] have discussed open issue for NW side data collection, most issues has been included in both [1] and [2]. However, there are several left issues need to be resolved which has not been included in the open issue.

Besides, Considering this is the last RAN2 meeting for Rel-19 AI/ML for PHY, it is suggested to resort the cross-group impact and make the whole feature integrity and closed-loop-logic, and send the impact information to the corresponding WG.

- Discussion

Left issues

Time Stamp

The first left issue is time stamp, in RAN2#130 meeting, the agreement related to the time stamp has been achieved in below:

=> Data is collected on per data logging configuration basis and UE indicates data logging configuration ID. An indication of the “gap” is needed. “Gap” is time interval larger than the configured logging periodicity. FFS if timestamp and relative time stamp for each group is needed per “group”.

According to the discussion in RAN2#131, it is suggested to use the gap indication instead of ‘continuous relative time stamp. But the FFS is whether each data group split by gap indication shall need the time stamp and relative time stamp.

In our understanding, each group contains the continuously logged data by following the logging periodicity, and if the logging periodicity is not equivalent to CSI resource periodicity, then one logging period may contain several CSI resource occasion, UE only takes the measurement one of them in one periodicity, as a result, NW cannot deduce  the accurate timing for each logged data even though they are collected without any gap.

Observation 1: Assuming one logging periodicity contain several CSI resource occasions, NW cannot deduce the actual timing information for each logged data as the logged data may be from any one of those CSI resource occasion.

According to the observation 1, it can be seen that the time stamp is necessary for each logged data from NW perspective.

Besides, the time stamp of the first logged data in each group also can be used for NW to deduce the ‘gap’ has been occurred, there is no need to introduce extra gap indication, in this sense, each logged data shall be attached with a time stamp, the very first logged data is attached with a absolute time stamp, and the following logged data shall be attached with a relative time stamp.we propose that:

Proposal 1: For logged data attached with time information, the first logged data is attached with an absolute time stamp as long as UE starts the logging, and the following logged data is attached with relative time stamp until to the corresponding logging configuration is released.

NW side data collection in NR-DC

In the previous meeting, it is confirmed that:

RAN2 confirms that previous agreement excluded SN being configured for OAM centric data collection

However, there is no any clarification for RAN centric NW side data collection, in our understanding, the RAN centric data collection shall be as same as the OAM centric, that is ,there is no need to support to configure the NW side data collection with data logging for SCG due to the quite limited remaining time for RAN2 discussion.

Proposal 2: RAN2 confirms that, for RAN centric data collection in NR-DC scenario, the NW side data collection only can be configured for MCG.

As both OAM centirc and RAN centric NW side data collection only can be configured for MCG in NR-DC scenario, so that this conclusion needs to be captures in TS37.340 :

Proposal 3:Capture the following information in TS 37.340：

For NW side data collection with data logging in NR-DC scenario, UE only can be configured with data logging for MCG, and correspondingly, the logged data for NW side data collection only can be reported to MN (as specified in TS 38.331[4]).

Cross-WG Impact

In this subclause, what impact would be caused by the Alternative 1 and Alternative 2 [1] respectively is discussed, in order for notifying the corresponding WG in time.

Alternative 1(CSI framework basis):

In alternative 1, layer 1 framework would be reused for configuring the NW side data collection, that is , both the logging configuration and CSI resource configuration would be contained in the CSI framework.

RAN1 impact:

Normally, all the information regarding CSI framework will be captured in the layer 1 specification (i.e. TS 38.21x). Assuming the logging configuration were contained in the CSI framework, RAN2 need to clarify the following agreement achieved in RAN2 with RAN1, and then ask whether RAN1 have any concern on this agreement ;

=> As a starting point, the data logging is captured in RRC specs.

Besides, taking L1 measurement for logging data may also have RAN1 impact somewhat, for example, the CPU occupation for CSI measurement for logging data.

Observation 2: For CSI framework based NW side data collection, RAN2 need inform RAN1 of the below information for their further discussion

- The data logging related configuration is configured in the CSI framework, but the data logging related behavior is specified in RAN2.

- The CSI resource configuration used for logging data is configured in the CSI framework.

RAN3 impact:

In this alternative, DU is responsible to generate the NW side data collection configuration while the CU is responsible to retrieve the logged data from UE, for NW side data collection, the below RAN3 impact can be foreseen:

- The first impact: the CU-DU interaction is needed for CU to retrieve the logged data as long as the DU configures the NW side data collection configuration to a UE.

- The second impact: the CU-DU interaction is needed for DU to configure the event configuration for the event triggered data logging.

- The third impact: the CU-DU interaction is needed for DU to de-configure the NW side data collection configuration if CU receiving the UAI from UE to indicate the lower power state.

Observation 3: For CSI framework based NW side data collection, RAN2 need inform RAN3 of the below information for their further discussion:

- In gNB split case, the CU-DU interaction is needed for CU to trigger the UE information procedure for retrieving the logged data.

- In gNB split case, the CU-DU interaction is needed for DU to configure the L3 event configuration for the event triggered data logging.

- In gNB split case, the CU-DU interaction is needed for DU to de-configure the NW side data collection configuration if CU receiving the UAI that indicating low power state.

Above all, assuming alternative 1 is the way to go after all, we need to send an LS to RAN1 and RAN3 to inform them of all above information related agreement.

Proposal 3: Assuming CSI framework is the way to go for NW side data collection, RAN2 need to send an LS to RAN1 and RAN3 for NW side data collection containing the below agreements achieved in previous RAN2 meetings for them to start the further discussion:

RAN2#130 => As a starting point, the data logging is captured in RRC specs. RAN2#129bis =>Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication RAN2#129 1. The UE reports to the network when the power state is low. We will not specify how the UE determines low power state. The network should de-configure the data collection (this can be captured in stage 2). 2. Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not. L1 measurement event triggered will not be supported. FFS what to log RAN2#127bis - UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection. FFS of details of the message

Alternative 2(Layer 3 framework basis):

In alternative 2, layer 3 framework would be used for configuring the NW side data collection, that is , the logging configuration would be contained in a layer 3 configuration which independents with the CSI framework.

RAN1 impact:

As the data logging configuration has been included in the layer 3 configuration, it is nature that data logging related behavior shall be captured in RAN2, i.e. TS 38.331. But the RAN1 impact still can be foreseen, that is, UE shall take the L1 measurement for logging data. .

Observation 4: For Layer 3 framework based NW side data collection, RAN2 needs inform RAN1 of the below information for their further discussion:

- UE shall take the Layer 1 measurement, and log the corresponding measurement result  according to the logged configuration.

RAN3 impact:

In this alternative, at least the below impact can be foreseen:

- For configuring the NW side data collection, CU-DU interaction is needed for CU to obtain the CSI reference signaling resource Id to configure the NW side data collection to the UE.

Observation 5: For Layer 3 framework based NW side data collection, RAN2 needs inform RAN3 of the below information for their further discussion:

- In gNB split case, the CU-DU interaction is needed for CU to acquire the CSI resource configuration in terms of NW side data collection.

Above all, assuming alternative 2 is the way to go after all, we need to send an LS to RAN1 and RAN3 to inform them of all above information.

Proposal 4: Assuming Layer 3 framework is the way to go for NW side data collection,  RAN2 need to send an LS to RAN1 and RAN3 for NW side data collection containing the below agreements achieved in previous RAN2 meetings for them to start the further discussion:

RAN2#127： => As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support. RAN2#127bis - UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection. FFS of details of the message

- Conclusion

In this paper, we provide our views on the left issues of NW side-side data collection, and the information those are needed to be informed to the RAN1 and RAN3. We have the following observations, and proposals:

Left Issue:

Observation 1: Assuming one logging periodicity contain several CSI resource occasions, NW cannot deduce the actual timing information for each logged data as the logged data may be from any one of those CSI resource occasion.

Proposal 1: For logged data attached with time information, the first logged data is attached with an absolute time stamp as long as UE starts the logging, and the following logged data is attached with relative time stamp until to the corresponding logging configuration is released.

Proposal 2: RAN2 confirms that, for RAN centric data collection in NR-DC scenario, the NW side data collection only can be configured for MCG.

Proposal 3: Capture the following information in TS 37.340：

For NW side data collection with data logging in NR-DC scenario, UE only can be configured with data logging for MCG, and correspondingly, the logged data for NW side data collection only can be reported to MN (as specified in TS 38.331[4]).

Cross-WG Impact from NW side data collection

CSI framework based alternative：

Observation 2: For CSI framework based NW side data collection, RAN2 need inform RAN1 of the below information for their further discussion

1: Containing the logging configuration in the CSI framework, but the logging data behaviour would be specified in RAN2.

2: UE shall perform the L1 measurement for logging the data.

Observation 3: For CSI framework based NW side data collection, RAN2 need inform RAN3 of the below information for their further discussion:

- In gNB split case, the CU-DU interaction is needed for CU to start the UEInformationRequest/UEInforamtion.

- In gNB split case, the CU-DU interaction is needed for DU to configure the event configuration to support event triggered data logging.

- In gNB split case, the CU-DU interaction is needed for DU to de-configure the NW side data collection configuration if CU receiving the UAI that indicating low power state.

Proposal 3: Assuming CSI framework is the way to go for NW side data collection, RAN2 needs to send an LS to RAN1 and RAN3 for NW side data collection containing the below agreements achieved in previous RAN2 meetings for them to start the further discussion:

RAN2#130 => As a starting point, the data logging is captured in RRC specs. RAN2#129bis =>Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication RAN2#129 1. The UE reports to the network when the power state is low. We will not specify how the UE determines low power state. The network should de-configure the data collection (this can be captured in stage 2). 2. Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not. L1 measurement event triggered will not be supported. FFS what to log RAN2#127bis - UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection. FFS of details of the message

Layer 3 framwork based alternative:

Observation 4: For Layer 3 framework based NW side data collection, RAN2 needs inform RAN1 of the below information for their further discussion:

- UE shall perform the Layer 1 measurement for logging the data.

Observation 5: For Layer 3 framework based NW side data collection, RAN2 needs inform RAN3 of the below information for their further discussion:

- In gNB split case, the CU-DU interaction is needed for CU to acquire the CSI resource configuration in terms of NW side data collection.

Proposal 4: Assuming Layer 3 framework is the way to go for NW side data collection,  RAN2 needs to send an LS to RAN1 and RAN3 for NW side data collection containing the below agreements achieved in previous RAN2 meetings for them to start the further discussion:

RAN2#127： => As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support. RAN2#127bis - UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection. FFS of details of the message

- References

- [1]:R2-25xxxxx   Report of email discussion [POST130][031][AI PHY] NW side data collection, Ericsson/ZTE

- [2]:R2-25xxxxx   RRC open issues AIML PHY_v21_Rapp.docx      Ericsson

- Text proposal

Text proposal for proposal 3

----------------------------------------------- Change on TS 37.340 ----------------------------------------------------

  - 7.2	 Measurements

If the measurement is configured to the UE in preparation for the Secondary Node Addition procedure described in clause 10.2, the Master node should configure the measurement to the UE.

In case of the intra-secondary node mobility described in clause 10.3, the SN should configure the measurement to the UE in coordination with the MN, if required.

/omit for short/

For MUSIM operation, when the UE is configured to operate in NR-DC in Network A (as described in TS 38.300 [3]), the MN indicates the per-UE MUSIM gap configuration to the SN.

For NW side data collection in NR-DC, UE only can be configured with data logging for MCG by MN, and correspondingly, the logged data reporting for NW side data collection only can be triggered and reported to the MN(as specified in TS 38.331[4]).

----------------------------------------------- Change on TS 37.340 ----------------------------------------------------

Draft LS for CSI framework based NW side data collection

1. Overall Description: Regarding the handling of logged data collected for NW-side AI/ML model trainingRAN2 has made the following agreements: RAN2#131 =>logging configuration is introduced as a new list of configurations under CSI-MeasConfig Note: It will be captured as long as RAN2 makes the final decision in RAN2#131 meeting RAN2#130 => As a starting point, the data logging is captured in RRC specs. RAN2#129bis =>Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication RAN2#129 1. The UE reports to the network when the power state is low. We will not specify how the UE determines low power state. The network should de-configure the data collection (this can be captured in stage 2). 2. Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not. L1 measurement event triggered will not be supported. FFS what to log RAN2#127bis - UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection. FFS of details of the message RAN2 assumes above agreements may cause the RAN1 impact as the data logging related configuration is configured in CSI-MeasConfig and data logging related behavior will be captured in RAN2 . RAN2 assume above agreements may cause the RAN3 impact, especially in gNB split case. 2. Actions: To RAN1 ACTION: RAN2 respectfully asks RAN1 to take the agreement into account, and provide concern, if any, on RAN2 agreement. To RAN3 ACTION: RAN2 respectfully asks RAN3 to take the agreement into account, and provide concern, if any, on RAN2 agreement.

Draft LS for L3 framework based NW side data collection

1. Overall Description: Regarding the handling of logged data collected for NW-side AI/ML model trainingRAN2 has made the following agreements: RAN2#131: => logging configuration is introduced in a new L3 measurement framework, at the same level as MeasConfig and CellGroupConfig Note: It will be captured as long as RAN2 make a decision in RAN2#131 meeting RAN2#127： => As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support. RAN2#127bis - UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection. FFS of details of the message RAN2 assumes above agreements may cause the RAN1 impact as UE shall take the L1 measurement for logging the corresponding measurement result. . RAN2 assume above agreements may cause the RAN3 impact as the measurement resource configuration for data logging is configured in the CSI-MeasConfig, while the data logging related configuration is configured as the same level as MeasConfig and CellGroupConfig in RRC. 2. Actions: To RAN1 ACTION: RAN2 respectfully asks RAN1 to take the agreement into account, and provide concern, if any, on RAN2 agreement. To RAN3 ACTION: RAN2 respectfully asks RAN3 to take the agreement into account, and provide concern, if any, on RAN2 agreement.