3GPP TSG RAN WG2 Meeting #131            	                      R2-2506142

Bangalore, India Aug 25th – 29th, 2025

Agenda Item:	8.10.2

Source:	Xiaomi

Title:	Discussion on Rel-19 SON/MDT open issues

Document for:	Discussion and Decision

Introduction

In the post RAN2 #130 meeting [Post130][605][SONMDT] Running NR RRC CR (Ericsson) [1], the RRC Running CR captured previous RAN2 agreements pertains to Rel-19 SON/MDT WI, and some issues are left which need to be brought for online decision.

In this contribution, we discuss some RRC left issues and summarize them for discussion, e.g., whether to indicate the correlation of SHR and SPR, how to handle the scenario of CHO with candidate SCG with complementary CHO only configuration.

Discussion

RRC open issues

Open issue RRC-1

In the post email discussion [1], the RRC-1 open issue shows several options to be down-selected in RAN2.

[Open issue RRC-1] For correlation of SHR and SHR generated for a CHO with Candidate SCG, RAN2 discuss and agree to one of the following options - Correlation indication in RRC_Complete message (a flag beside availability indication) - Correlation indication inside the SHR and SPR - Logging C-RNTI of PCell and timeSinceSPR in SPR - The duplicated information in correlated SHR and SPR is sufficient. no further information is needed for correlation.

Among the four options above, we prefer Option d. In our view, there is no need to introduce such extra correlation indications (Options a, b, c) to add signaling overhead. This is because the network always optimizes based on cell-based measurements and information in SON reports; it can bind a pair of SHR and SPR together when SHR and SPR both contain the same pair of cell IDs (Pcell/PScell), and the same source/neighbour PCell/PScell measurements with configured with CHO with candidate SCGs, which can indicate the alignment of SHR and SPR are generated at the same time clearly. Thus, no needs to introduce additional a, b, c mechanisms for this. The duplicated information in correlated SHR and SPR is sufficient.

Proposal 1: (RRC-1) For whether to introduce correlation indicator, RAN2 to agree with option d and no specs impacts.

- Option d. The duplicated information in correlated SHR and SPR is sufficient. no further information is needed for correlation.

Open issue RRC-2

Regarding whether to consider the scenario where a complementary CHO-only configuration is set up after a CHO with candidate SCGs configuration, some companies have pointed out that this scenario was proposed in the Rel-18 mobility Work Item (WI) to ensure that at least the PCell handover can be executed. In the 38.331 specifications, there is no restriction on how the network configures complementary CHO-only configurations for the same UE that has already been configured with candidate SCG configurations. Companies are interested in the case where CHO-only configurations are set up after candidate SCG configurations. They believe that MRO for this scenario can help the network determine which CHO/CPAC execution condition is properly configured. Companies propose enhancing support for the two cases below:

- Case 1: Neither of the two conditions of CHO with candidate SCGs is fulfilled

- Case 2: One of the two conditions of CHO with candidate SCGs is fulfilled

To analyze these two cases, the RAN2#127bis agreements for CHO with candidate SCGs are listed below, and Table 1 is provided to analyze the scenario where both CHO with candidate SCGs and CHO-only configurations are configured.

- UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. - Include the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report.

Table 1: Analysis of cases when both CHO with candidate SCGs and CHO-only are configured

| Cases (configured with both CHO with candidate SCGs and CHO-only) | outcome | NW’s position |
| --- | --- | --- |
| 1. Neither condition is met | RLF | NW need to optimize both CHO and CPAC configurations due to RLF failure. |
| 2a. Only CHO condition is met | CHO-only handover successfully | NW knows CHO-only condition is successfully executed and it is highly likely that CPAC condition is not met, NW doesn’t need to do the optimization due to PCell HO successfully. |
| 2b. Only CPAC condition is met | RLF, UE reports RLF report to NW and set firstFulfilledConfig to “cpc” | NW need to optimize CHO configuration. |

For Case 1: This is not an issue unique to the scenario where both CHO with candidate SCGs and CHO-only configurations are configured. According to the current 38.331 running CR, when a UE is configured with CHO with candidate SCGs, only the first fulfilled execution condition is reported to the network. The situation where neither of the two conditions is met has not been considered in the current running CR. Therefore, RAN2 only needs to extend the firstFulfilledConfig IE to support a "neither" indication, so that the network can know when neither condition is fulfilled.

For Cases 2a and 2b: Current agreements already cover this scenario. When the CPAC condition is fulfilled (e.g., during CPAC execution), the UE will set firstFulfilledConfig to "cpc", and the network will know that the CHO condition was not met before RLF occurs. When the CHO condition is fulfilled first, the CHO-only configuration will be executed successfully. The network will then know that the CPAC condition was not met in time (otherwise, the CHO with candidate SCG would have been executed successfully). However, the network does not need to perform optimization in this case because the CHO-only configuration was executed successfully that satisfy network’s requirements.

Therefore, there is no need to consider the scenario where a complementary CHO configuration is configured after a CHO with candidate SCG configuration. Instead, RAN2 needs to consider enhancing the firstFulfilledConfig IE in the 38.331 running CR to support the "neither" indication when both the CHO and CPAC conditions are not fulfilled.

Proposal 2: (RRC-2) RAN2 do not need to do optimization for the scenario of CHO with candidate SCG with complementary CHO only configuration.

Proposal 3: (RRC-2) RAN2 needs to consider to extend firstFulfilledConfig IE in 38.331 running CR to support “neither” in case of both CHO and CPAC conditions are not fulfilled.

Conclusion

In this contribution, we propose:

Proposal 1: (RRC-1) For whether to introduce correlation indicator, RAN2 to agree with option d and no specs impacts.

- Option d. The duplicated information in correlated SHR and SPR is sufficient. no further information is needed for correlation.

Proposal 2: (RRC-2) RAN2 do not need to do optimization for the scenario of CHO with candidate SCG with complementary CHO only configuration.

Proposal 3: (RRC-2) RAN2 needs to consider to extend firstFulfilledConfig IE in 38.331 running CR to support “neither” in case of both CHO and CPAC conditions are not fulfilled.

Reference

[1] [Post130][605][SONMDT] Running NR RRC CR (Ericsson)