3GPP TSG-RAN WG2 Meeting #131	R2-2506149

Bangalore, India Aug 25th – 29th, 2025

Agenda item:	8.6.3

Source:	Samsung

Title:	Remaining Issues of L1 Event Triggered Measurement Report

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

# Introduction

In this contribution, we will further address the remaining essential open issues including post email discussion [POST130][117] and conclusion on the coexistence with mTRP, stage-3 identified open issues.

# Discussion

## Coexistence with mTRP with LTM L1 event triggered MR

During the post email discussion [POST130][117], companies provided the input to support the coexistence between L1 event triggered MR and mTRP (source cell). It seems majority companies want to support this feature and companies discussed how the UE selects the serving beam to be included in the MR MAC CE when mTRP is configured in the serving cell. Below options were considered.

- UE should use the best beam of the two “current beams” for event evaluation

- It is up to the UE which of the two beams it uses for L1 event evaluation

- Network indicates which beam the UE shall use

- Specify to use one of the two TCI states (e.g., just use the first indicated TCI state)

- Other

Among above options, majority companies preferred option a) because selecting the best source beam approach seems good in terms of performance enhancements and solution is not really complex. However, we are not sure if option a) really simple enough to resolve all details i.e. we observed there are some more issues.

- Issue 1: There are two mTRP modes i.e. mTRP with the single DCI/multiple DCI

  - mTRP with the single DCI

    - Only one TRP (e.g. the first TRP) is used for TRP’s DCI but data is sent from/to each/both TRP(s).. So, if the best beam is located in the second TRP, the beam used for this mTRP operation especially for PDCCH beam is not used for evaluation of L1 MR MAC CE.

    - Not sure if anything additionally needs to be included in MR MAC CE (since from the UE point of view, it is considered as TCI state change, which is same/similar to non-mTRP case).

  - mTRP with the multi-DCI

    - Option a) is applied without any specification impact

    - coresetPoolIndex or TRP information may be included in MR MAC CE if NW want to know the TRP used for evaluation when only mTRP with multiple DCI is configured

- Issue 2: Some additional UE complexities

  - From UE perspective, UE always needs to measure beams for both TRPs to identify the best beam of the serving cell so it would require the additional UE complexity.

- Issue 3: How to handle if the current serving beam is changed during the TTT is running

  - In non-mTRP case, if the current serving beam is changed during TTT running, RAN2 agreed that UE continues the TTT if the new beam is still met the condition.

  - Similar as non-mTRP case, RAN2 needs to make an agreement of the serving beam change chase on the mTRP operation.

RAN2#128 agreements

- TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

As we provided above remaining issues if option a) is applied, there are still some specification impacts so we think the simplest solution would be accepted if this coexistence between L1 event triggered MR and mTRP (source cell) is supported at this late stage. We think Option d) is the simplest option without any specification impact and UE complexity is not increased in this option.

Proposal 1:	Specify to use one of the two TCI states (e.g., just use the first indicated TCI state) to support coexistence between L1 event triggered MR and mTRP (source cell).

## Handling of LTM L1 event triggered MR during beam failure

In RAN2#127, it was agreed that the current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. If the UE has triggered event triggered L1 MR for LTM and the current beam fails, the event triggered L1 MR Procedure may be cancelled, as the MR is based on old beam, when the MR reaches the network. We may also note that if the UE is requested to report the serving beam measurements, the measured value is reported without RSRI and after beam failure (and subsequent recovery), the received measurements at the network may still be based on the old beam. The network may interpret it as the measurements from (new) current beam. A simple way to solve this is to cancel the MR, but it can be also argued that MR can be sill helpful to the NW (e.g. NW can perform LTM early after BF is recovered). So we propose to discuss the following options for this case.

Proposal 2:   RAN2 to discuss the behavior of any trigged L1 MR during beam failure and recovery on the SpCell:

a. Cancel the triggered L1 MR.

b. Send the triggered L1 MR.

Another issue is related to handling of TTT. As there could be a long delay of measurement due to beam failure and recovery and the decision based on the quality of measurements of the (old) current beam during this period may not be suitable. For handling TTT, then there could be two options during BFR- Either reset the TTT for event triggered L1 measurement report triggering condition evaluation or continue the TTT if the condition is satisfied with the new beam.

Proposal 3:RAN2 to discuss the behavior of TTT for L1 MR during beam failure and recovery on the SpCell:

a. UE resets TTT for L1 MR.

b. TTT is continued if the condition is satisfied with the new beam.

## LTM2 reporting

Reporting of serving beam measurements

In the MAC running CR, the UE includes RSRPserving if the UE is configured with reportCurrentBeam.

RSRPserving: This field indicates the measured quantity based on SS/PBCH block or CSI-RS (i.e. the L1-RSRP) for current RS of serving cell as described in TS 38.215 [24], if UE is configured to report the measurement result of current RS of the serving cell by reportCurrentBeam. The length of the RSRPserving field is 7 bits;

In the RRC running CR, there is no conditional presence for LTM2 which indicates that allowReportAnyBeam will be always set.

LTM-EventTriggeredReportContent-r19 ::=     SEQUENCE {

maxNumberOfReportedBeams-r19                   INTEGER (1..16),

allowReportAnyBeam-r19                         ENUMERATED {enabled}                                                OPTIONAL, -- Need R

reportCurrentBeam-r19                          ENUMERATED {enabled}                                                OPTIONAL, -- Need R

...}

For LTM2, since the triggered beam is the current beam, its measurements need to be always included. There could be two ways to do this. Either the UE can always report measurement result of current RS of the serving cell irrespective of the current beam for event LTM2 or the network can always configure the UE with reportCurrentBeam in the LTM2 configuration.

Proposal 4:	RAN2 to discuss below two options for reporting current beam measurements in LTM2

- UE reports the current beam measurements irrespective of reportCurrentBeam.

- Network always configures reportCurrentBeam in the LTM-EventTriggeredReportContent.

Truncated L1 MR

Regarding truncated L1 MR, there were below agreements.

In RAN2#129,

15.	Truncated MR MAC CE includes the triggered beam information:

- One report config ID (event config ID)

- At least one triggered beam id (SSBRI or CRI) + L1 RSRP

In RAN2#130,

Working assumption:

5. If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE.

Since it was also agreed that other beams can be reported for LTM2 based on network configuration, truncated event triggered L1 MR is also applicable for LTM2. For LTM2, as the triggered beam and serving beam are same, truncated MR MAC CE can be sent with report conifg ID and L1 RSRP of serving beam when  the grants are not sufficient for sending information from any other beam. If the grants are sufficient for sending information from other beams along with the serving beam, both can be included.

Proposal 5:	For LTM2, truncated event triggered L1 MR can be sent with ReportID and serving beam L1 RSRP when the grants are not sufficient for sending information from any other beam.

# Conclusion

Based on the discussion, we have the following proposals.

Proposal 1:	Specify to use one of the two TCI states (e.g., just use the first indicated TCI state) to support coexistence between L1 event triggered MR and mTRP (source cell).

Proposal 2:   RAN2 to discuss the behavior of any trigged L1 MR during beam failure and recovery on the SpCell:

a. Cancel the triggered L1 MR.

b. Send the triggered L1 MR.

Proposal 3:RAN2 to discuss the behavior of TTT for L1 MR during beam failure and recovery on the SpCell:

a. UE resets TTT for L1 MR.

b. TTT is continued if the condition is satisfied with the new beam.

Proposal 4:	RAN2 to discuss below two options for reporting current beam measurements in LTM2

- UE reports the current beam measurements irrespective of reportCurrentBeam.

- Network always configures reportCurrentBeam in the LTM-EventTriggeredReportContent.

Proposal 5:	For LTM2, truncated event triggered L1 MR can be sent with ReportID and serving beam L1 RSRP when the grants are not sufficient for sending information from any other beam.

# References

[1]	R2-250xxxx		Report from [POST130][117][MOB] 	Nokia.