3GPP RAN WG2 Meeting #131	R2-2506156

# Bangalore, India, August 25th – 239th, 2025

Agenda Item:	8.9.2

Source:	InterDigital, Inc.

Title:	Store and Forward open issues.

Document for:	Discussion

# Introduction

A revised WID was approved to further enhance the support of NB-IoT and eMTC [1]

The following objective is provided

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In RAN2#127 the following was agreed:

Agreements:

1.	RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options)

2.	RAN2 will consider both single satellite pass and multiple satellite pass scenarios

3.	RAN2 will consider both MO and MT data within scope

4.	UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

5.	RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

7.	RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

In RAN2#127bis the following was agreed:

Agreements:

1.	The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication (we can come back on the exact name when putting this in the spec, if needed)

2.	RAN2 assumes that if an indication that “the cell is operating in S&F mode” is not provided in a NTN cell, the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal/”not-S&F” mode). RAN2 assumes there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but indication that “the cell is operating in S&F mode” is not provided.

3.	An S&F explicit capability indication by the serving cell, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the indication that “the cell is operating in S&F mode”, is not needed

4.	If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure

5.	When present, the “S&F operation” indication has two possible settings:

‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

In RAN2#128 the following was agreed.

Agreements:

1.	Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

2.	It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable

3.	“S&F operation” indication is provided in SIB1

4.	RAN2 assumes that AS signalling is not needed to indicate the architecture option (Split MME or Full CN).

5.	RAN2 waits for further progress in SA2, if any, before working on RAN2 aspects of a CIoT-UP solution for SF operation

6.	RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite

7.	RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

In RAN2#129 the following was agreed:

Agreements:

- RAN2 currently assumes that a S&F mode changes does not necessarily imply a MME change

- The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information)

- Use absolute UTC time to broadcast the (S&F-> normal mode) time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time).

- A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply

In RAN2#129bis the following was agreed:

Agreements:

1.	For UEs supporting S&F, the UE AS indicates the information on transition time (if any) from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1.

2.	We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS

3.	We don’t introduce any new release cause related to S&F operation

In RAN2#130 the following was agreed:

Agreements:

1.	When the S&F operation indication is present, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the normal mode will start; otherwise, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the S&F mode will start.

2.	In a S&F network deployment which also exhibits discontinuous coverage, existing mechanisms to handle discontinuous coverage can be leveraged (e.g. satellite assistance information, UE not needing to perform idle mode tasks when the UE determines that is out of coverage, etc.). There is no need to modify existing discontinuous coverage features due to the addition of S&F Satellite operation. FFS if we clarify in discontinuous coverage procedure in idle mode that the UE also takes into account the information about NAS configured S&F monitoring list.

3.	Clarify in Stage 2 from Rel-18 (so in general, not only for S&F) that the values used in “satelliteId-r17” are expected to correspond to the values used in “satelliteId-r18” in other SIBs (e.g. the value used to identify the serving satellite in SIB33, SIB32 and SIB31 should be the same). Come back in the next meeting with an actual Rel-18 CR for this. FFS if in Rel-19 we need to say anything about the correspondence between the “satelliteId” IE and “S&F Monitoring List”.

Working Assumption:

1.	In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

# Discussion

## 2.1 S&F Operation mode

According to the agreements from RAN2#127bis meeting:

5.	When present, the “S&F operation” indication has two possible settings:

‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

The behaviour described in the above agreement is identical to the behaviour used for feature specific barred/no barred indication. Idle mode UEs, when determining cell status and cell reservations according to 38.304 [3] section 5.3.1 may access the cell if the value is “not barred” and treat the cell as barred if the value is “barred”. UEs in RRC_CONNECTED don’t use the IE.

Observation 1: The agreed behaviour for “S&F operation” is identical to “cellBarred” and feature specific “cellBArred” IEs.

For consistency with all of the other IEs of this type, we propose that the IE is named in line with the other IEs of this type, and the behaviour in 38.304 is aligned with the other IEs of this type and captured in section 5.3.1 along with the others.

Proposal 1: To align with existing IEs of the same type and behaviour, “S&F operation” IE is named “cellBarredSandF” and has the value “barred” or “not barred”.

Typically, when a cell is barred (either using the legacy cellBarred IE or using a feature specific IE) then the UE is required to exclude the cell for cell selection/reselection for 300s. Note that there are some conditions whereby the UE “may” exclude the cell for up to 300s for power saving purpose, but these conditions are typically MIB or SIB1 reception failure, i.e. the error case. Conditions whereby the cell is explicitly barred result in the UE “shall” exclude the cell from cell selection/reselection for 300s (see 38.304 [3] section 5.3.1)

In most cases, cell barring occurs due to abnormal situations, and the status is not updated very frequently. Hence, UE does not need to re-check the status for 5 minutes to allow for power saving.

However, in the case of S&F operation, it is likely that the mode itself is likely to be updated more frequently than other cellBarred IEs. Applying 5 minutes barring may limit the amount of time a cell is usable, if the UE determined based on the status being barred and S&F operation being on. For example, the UE may determine that the cell is barred, but the gNB then changes S&F operation to normal operation sooner than 5 minutes. Similarly, if the gNB does not change the status for longer than 5 minutes, then the UE may need to perform SIB reception and determine the status again unnecessarily.

One additional reason for S&F mode to have an indicated timing other than fixed 5 minutes is related to the satellite movement. Due to the satellite movement, nature of the orbit and the number of satellites in the constellation, a satellite may likely go out of coverage for a UE in about 5 minutes. Thus, based upon the deployment, e.g., orbit and constellation, the operators may choose a suitable time after which the UE may need to check the S&F status/indication.

The fixed 5 minute timer may not be appropriate if the remaining time of S+F operation is used, but rather the UE should bar according to the remaining time parameter so that it may check the status when the mode changes (In this case we may also need to discuss whether this applies to both cases - by checking the legacy cellBarred when the mode changes from S&F to normal, and by checking the cellBarredSandF when the mode changes from normal to S&F).

Proposal 2: If the UE is barred due “cellBarredSandF” (or whatever the name is) = “barred” the UE uses the value of the remaining time parameter to determine how long to bar the cell.

In the previous meeting, it was agreed:

1.	When the S&F operation indication is present, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the normal mode will start; otherwise, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the S&F mode will start.

It is still not clear how a UE supporting S+F handles the legacy t_Service. Currently UE treats t_Service as end of coverage of that cell, and takes the appropriate actions. Legacy UE will always behave that way, however for S+F UE we now have 2 conflicting requirements – t_ModeSwitching and t_Service needs to be handled.

In order for legacy UE to behave correctly in an S+F cell, the legacy parameter t_Service would have to be equal to t_ModeSwitching (because legacy UE doesn’t support S+F then this is the end of service for that UE).

Proposal 3: RAN2 assumes that the legacy t-Service will be set to S+F mode switch time in order that legacy UE interprets this as end of the service time.

Even without the above assumption, a network without such restriction could still set t_Service and t_ModeSwitching to the same value, hence the S+F UE needs to know which requirement to follow. With the agreements in the last meeting, it is not possible to set t_Service to the mode switching time (for legacy UEs) and t_ModeSwitching to a different time representing mode switch. The agreement from the previous meeting therefore needs to be revisited. We either need to handle this case in order that both legacy UEs and S+F UEs do have the correct behaviour (correct behaviour is that legacy UEs treat t_Service as end of service, and S+F UEs use t_Service and t_ModeSwitching to determine end of service and mode switch), or we need to accept that an S+F UE being provided with t_ModeSwitching needs to ignore t_Service (meaning that S+F UE does not support the functionality supported by legacy UEs!)

Proposal 4: Clarify/revisit the agreement from the previous meeting “When the S&F operation indication is present, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the normal mode will start; otherwise, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the S&F mode will start.”

Option 1: S+F UE ignores t_Service if t_ModeSwitching indicates the time when S+F operation starts.

Option 2: S+F UE uses t_Service to determine the mode switch time, and t_ModeSwitching to determine the end of service time, if t_ModeSwitching indicates the time when S+F operation starts.

## 2.2 Cell reselection prioritization.

In the last meeting, it was proposed that the UE may prioritize frequency(ies)/cell(s) corresponding to the satellite(s) indicated in the MME-configured satellite list, and that this proposal will be discussed again in this meeting.

In the SA2 specification [5] the following has been specified:

NOTE 7: How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one.

However, as correctly observed in [6], the RAN2 specification [7], clearly specifies that the UE shall perform cell reselection evaluation taking into account the configured priorities, thresholds, and offsets according to the normative procedure. This essentially means that the UE cannot determine to prioritise S+F cells or carriers according to the note in 23.401, because the normative requirements in 36.304 do not allow for that.

Observation 2: According to the current 36.304 the UE is not allowed to prioritize cells or carriers corresponding to the S+F monitoring list provided by NAS, therefore the note in 23.401 (“How UE behaves when receiving the S&F Monitoring List is up to UE implementation.”) appears to be meaningless without introducing changes to the cell reselection evaluation procedure in RAN2.

It was agreed in the previous meeting:

Working Assumption:

1.	In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

For eMTC, allowing UE to prioritize carriers associated with the S+F monitoring list is straightforward. Similar prioritization exists already for the cases of CSG, HSDN, sidelink, V2X, IAB, MBMS. A similar sentence just needs to be added for S+F, e.g. as follows:

If the UE is capable of S+F and is operating in S+F mode, the UE may consider frequencies associated with satellite IDs within S+F monitoring list to be the highest priority

Proposal 5: For eMTC, if the UE is operating in S+F mode, the UE may consider frequencies associated with satellite IDs within S+F monitoring list to be the highest priority.

For NB-IoT, absolute priority-based cell reselection is not supported. The UE shall reselect to the best ranked cell, even for inter-frequency cells. The thinking at the time was that NB-IoT UEs will always benefit from being camped on the best cell according to radio quality measurements, and that in order to keep complexity to a minimum, support of absolute priorities was deemed as not essential.

Introduction of absolute priority-based cell reselection for NB-IoT would be a significant effort for both UE and NW implementation. For ranking based cell reselection, prioritization can be realised by applying an offset. A precedent for this has been set for the case of SC-PTM which applies the Qoffsetscptm to all frequencies providing SC-PTM.

A similar solution could be used for the case of S+F for NB-IoT, i.e. to introduce a configurable offset which is applied to frequencies associated with the S+F monitoring list satellite IDs.

Qoffsetsandf This specifies the offset to be used for cell re-selection for S+F operation for NB-IoT UEs. The same offset is applicable to all frequencies associated with satellite IDs within S+F monitoring list.

Proposal 6: For NB-IoT, introduce an offset to apply to the cell ranking criteria R. If the UE is operating in S+F mode, the UE applies the offset to frequencies associated with satellite IDs within S+F monitoring list to be the highest priority.

# Conclusion

In this contribution we discuss support of PWS and make the following proposals for discussion:

S&F Operation mode

Observation 1: The agreed behaviour for “S&F operation” is identical to “cellBarred” and feature specific “cellBArred” IEs.

Proposal 1: To align with existing IEs of the same type and behaviour, “S&F operation” IE is named “cellBarredSandF” and has the value “barred” or “not barred”.

Proposal 2: If the UE is barred due “cellBarredSandF” (or whatever the name is) = “barred” the UE uses the value of the remaining time parameter to determine how long to bar the cell. FFS the case of changing from normal to S&F mode.

Proposal 3: RAN2 assumes that the legacy t-Service will be set to S+F mode switch time in order that legacy UE interprets this as end of the service time.

Proposal 4: Clarify/revisit the agreement from the previous meeting “When the S&F operation indication is present, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the normal mode will start; otherwise, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the S&F mode will start.”

Option 1: S+F UE ignores t_Service if t_ModeSwitching indicates the time when S+F operation starts.

Option 2: S+F UE uses t_Service to determine the mode switch time, and t_ModeSwitching to determine the end of service time, if t_ModeSwitching indicates the time when S+F operation starts.

Cell reselection prioritization

Observation 2: According to the current 36.304 the UE is not allowed to prioritize cells or carriers corresponding to the S+F monitoring list provided by NAS, therefore the note in 23.401 (“How UE behaves when receiving the S&F Monitoring List is up to UE implementation.”) appears to be meaningless without introducing changes to the cell reselection evaluation procedure in RAN2.

Proposal 5: For eMTC, if the UE is operating in S+F mode, the UE may consider frequencies associated with satellite IDs within S+F monitoring list to be the highest priority.

Proposal 6: For NB-IoT, introduce an offset to apply to the cell ranking criteria R. If the UE is operating in S+F mode, the UE applies the offset to frequencies associated with satellite IDs within S+F monitoring list to be the highest priority.

# References

- RP-242397 Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”

- 36300-i00 , “E-UTRA/E-UTRAN;Overall description;Stage 2”

- 36304-i30, “User Equipment (UE) procedures in idle mode”

- R2-2500309,	“Discussion on Store & Forward satellite operation”	OPPO

- 23.401, “Technical Specification Group Services and System Aspects”

- R2-2502053,	“Discussion on RAN2 impacts for the support of S&F operation”,	CATT

- 36.304, “User Equipment (UE) procedures in idle mode”