3GPP TSG-RAN WG2 Meeting #131bis 	 R2-2506832

Prague, Czech Republic, October 13th-17th, 2025

Agenda item:	9.7.2

Source:	vivo

Title:	Discussion on Support of IMS Voice over NB-IoT NTN

Document for:	Decision

# Introduction

In the RAN#108 meeting, a new WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4 was approved [1]. This WID was further revised in the RAN#109 meeting [2]. The objectives of this WID are listed as follows,

The aim of this work item is to specify the enhancement of NB-IoT-NTN to support IMS voice call over GSO with the following objectives: - Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2] - Coordination with SA is required - The objective can be updated with further details based on the outcome of the above down-selection - Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1] - Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN [RAN2] - Support of necessary modifications for emergency call for voice over NB-IoT-NTN [RAN2] - Study and if feasible, specify UE transmit power higher than PC1 (e.g. up to 37dBm) for NB-IoT-NTN [RAN4] - Note: The enhancements to support voice over NB-IoT-NTN via GSO in this work item may be also applicable to NGSO cases without additional specification enhancements - Note: Coordination with SA4 is expected

Regarding the support for IMS voice over NB-IoT NTN, SA4 is conducting a feasibility study on Ultra Low Bitrate Speech Codec (FS_ULBC) for GEO voice services. An LS in [3] has been sent to RAN to seek confirmation on the RAN assumptions. The specific questions regarding RAN2 are outlined below,

# To SA2 and RAN2: ACTION: SA4 kindly asks SA2 and RAN2 to comment on - the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option, and - specifically, whether a packet overhead of 1 byte of MAC header is realistic.

In the RAN2#131 meeting, RAN2 had a brief discussion on that SA4 LS, and achieved the following agreement,

R2-2505068 LS on the RAN simulation assumptions for ULBC (S4-251584; contact: Qualcomm) SA4 LS in Rel-20 FS_ULBC To:RAN1, RAN2, RAN4, SA2, CT1 Cc:SA1 - - Qualcomm thinks that we can try to respond with a simple answer on header sizes. - Vivo thinks we should wait as there will be discussions. Samsung thinks that the header sizes will not change so we can at least respond on that. - ZTE thinks that in theory we can provide an answer like, if we go with CP vs. UP what would be the header size without deciding. - Vivo, Lenovo, Huawei and Xiaomi think that there are a lot of aspects to consider. Lenovo doesn’t think it is urgent to provide a precise answer. Noted We will begin this work in RAN2#131bis and will attempt to answer to SA4 after some initial RAN2 work.

Additionally, SA2 has been working on the support for IMS voice over NB-IoT NTN connected to EPC over several meetings. An LS [4] has been sent to RAN to ask some questions that will assist SA2 in evaluating the alternative solutions captured in TS 23.700-19. The specific questions regarding RAN2 are listed below,

Question 1 (To RAN2): Does RAN2 have any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC? Question 3 (To RAN2): Can RAN2 confirm whether scheduling methods for support of IMS voice over NB-IoT NTN can handle the voice packets of different sizes changing dynamically? Question 6 (To RAN2): Is it feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20? Question 9 (To RAN2): Can RAN2 provide feedback on whether it is possible to define a new SRB that will be used to carry voice media? Question 10 (To RAN2): If the answer to the above question is yes, can RAN2 confirm whether such SRB(s) are defined, whether it is technically feasible to support and configure RLC UM for such SRB(s) for example: in NB-IoT deployments over GEO satellite, when SRBs are used to carry voice media? To RAN2, SA4, SA3, CT1, SA1, RAN1: ACTION: SA2 requests the respective WGs in “TO” to answer the questions addressed to them as per above and provide any other feedback they think is necessary.

Recently, another SA4 LS on SPS has been sent to RAN [5]. The specific questions regarding RAN2 are given below,

SA4 understands that RAN1/2 have not yet started the work on designing SPS for NB-IoT NTN. In SA4 one company proposed the 120ms and 240ms bundling periods, which means that the UE would send a voice packet and receive a voice packet every 120ms or 240ms. Some companies have concerns about potential SPS operation, because 120 and 240 do not divide 10240. # To RAN2: ACTION: SA4 kindly asks RAN2 to confirm whether the 120ms and/or 240ms bundling periods would be valid for the SPS operation in NB-IoT NTN, and provide feedback, if any.

In this contribution, to well address questions from SA2/SA4 LSs and advance the down-selection progress between CP and UP solutions for voice support over NB-IoT NTN, we will focus on discussing the following four parts from the RAN2 perspective. Specifically, we will provide detailed analysis and proposed LS responses to SA2/SA4, present detailed analysis and comparison of candidate CP and UP solutions, and give our down-selection recommendation.

- Part 1: SA4 LS on RAN assumption;

- Part 2: SA2 LS on support of IMS voice;

- Part 3: SA4 LS on SPS;

- Part 4: Down-select between CP and UP solutions.

# Discussion of Part 1: SA4 LS on RAN assumption

Question: Whether a packet overhead of 1 byte of MAC header is realistic

Fig. 1: Example of MAC PDU

According to TS 36.321, a MAC header comprises one or more MAC PDU sub-headers, and each MAC PDU sub-header corresponds to either a MAC SDU, a MAC CE, or padding, as shown in Fig. 1. A MAC PDU sub-header includes the header fields R/F2/E/LCID/(R/R/eLCID)/(F)/(L), with an overhead of 1-4 bytes. In NB-IoT, the eLCID-related function is not supported. Thus, the minimum size of the MAC PDU sub-header for NB-IoT can be 1 byte when the L field is absent. However, as specified in the LTE MAC specification, the L field shall be present in the MAC PDU sub-header except for some special cases (e.g., the last sub-header in the MAC PDU and fixed-sized MAC CE), considering that the L field plays an important role in MAC (de)multiplexing function.

For the use case of GEO voice, we believe parallel data transmission of different bearers (DRBs, DRB, and SRB) may exist. If parallel data transmission (e.g., DRB carrying IMS signalling and DRB carrying voice packet) is allowed, the MAC (de)multiplexing function should be used, and then multiple MAC sub-headers in the MAC header are needed. In addition, from the MAC perspective, there may exist the requirement that MAC SDU and MAC CE (e.g., SPS confirmation MAC Control Element) should be multiplexed. In the context of the aforementioned cases, a 1 byte MAC header is not realistic. Moreover, due to the talk-silence period transition and codec adjustment, different lengths of MAC SDU will exist during the ongoing IMS call. At least from these aspects, there are significant limitations to using 1 byte of the MAC header. It is suggested that RAN2 responds to SA4 that a packet overhead of 1 byte of MAC header for the voice packets is not realistic based on the current MAC spec.

Proposal 1: RAN2 responds to SA4 that a packet overhead of 1 byte of MAC header for voice packets is not realistic based on the current MAC spec.

Question: The respective overall packet overhead for UP/CP and IP/Non-IP, and the preferred option

In the NB-IoT CP architecture, service data transmission is performed via SRB1bis, with the PDCP entity bypassed. Consequently, the PDCP protocol overhead for the CP architecture is 0 bytes. RLC AM is the only applicable RLC mode for SRB1bis. Additionally, AM RLC SN for NB-IoT is 10 bits. With a 10-bit AM RLC SN, the fixed part of the AMD PDU header is 2 bytes in length (with no RLC concatenation or segmentation).

In the NB-IoT UP architecture, service data transmission occurs via DRB mapped to either RLC AM or RLC UM, with a PDCP entity used. For the NB-IoT UP case, an UM RLC entity uses a 5-bit SN. When a 5-bit SN is used, the fixed part of the UMD PDU header is 1 byte in length (with no RLC concatenation and segmentation). When a 10-bit RLC SN is used, the fixed part of the AMD PDU header is 2 bytes in length. Regarding the PDCP protocol header, only PDCP data PDU with short PDCP SN (7-bit) is supported for NB-IoT, which has a fixed header size of 1 byte.

Regarding the RRC overhead, the UP architecture incurs 0-byte RRC overhead. For CP architecture, UL data transmission in RRC connected state is performed via the ULInformationTransfer-NB message, which maps to the uplink DCCH logical channel (as shown in the following ASN.1 structure). Next, we will demonstrate how the RRC overhead for ULInformationTransfer-NB message is calculated.

To identify the message type of ULInformationTransfer-NB message, 5 bits are needed for the two-level CHOICE structure in UL-DCCH message  (i.e., 1 bit for outer-level CHOICE, and 4 bits for inner-level c1 CHOICE). Within the ULInformationTransfer-NB message, 1 bit is required for CHOICE, and 2 bits are required for two optional fields in SEQUENCE. Thus, the total RRC overhead for UL data transmission is 1 byte.

For CP architecture, DL data transmission in RRC connected state is performed via the DLInformationTransfer-NB message, which is mapped to downlink DCCH logical channel (as shown in the following ASN.1 structure). To identify the message type of DLInformationTransfer-NB message, 4 bits are needed for the two-level CHOICE structure in UL-DCCH message (i.e., 1 bit for outer-level CHOICE, and 3 bits for inner-level c1 CHOICE). Within the DLInformationTransfer-NB message, 2 bits are needed for rrc-TransactionIdentifier (i.e., value range is (0..3)), 1 bit is needed for CHOICE, and 2 bits are needed for two optional fields in SEQUENCE. Consequently, the total RRC overhead for DL data transmission is 10 bits. Considering that both the NAS PDU carried in the RRC message and the RRC message itself are always multiples of 8 bits in length, the RRC overhead, after ASN.1 PER encoding in this case, is 2 bytes.

Based on the analysis above, we have the following observation:

Observation 1: In Legacy NB-IoT, the L2 protocol header size for service data transmission is specified as follows,

- For CP architecture, the RRC overhead is 1 byte for UL and 2 bytes for DL, whereas for UP architecture, the RRC overhead is 0 bytes;

- For UP architecture, the PDCP protocol header has a size of 1 byte (i.e., PDCP SN=7bits), whereas for CP architecture, the PDCP protocol overhead is 0 bytes;

- With no RLC concatenation and segmentation, the AM RLC protocol header has a fixed size of 2 bytes (AM RLC is the only applicable RLC mode for the CP architecture), whereas the UM RLC protocol header has a fixed size of 1 byte;

- Except for certain special cases (e.g., the last sub-header in the MAC PDU), the MAC protocol header has a fixed size of 2 bytes.

To answer the SA4 question regarding the overall protocol overhead for the voice packet, it is suggested that RAN2 respond to SA4 with the L2 protocol overhead for the voice packet based on Observation 1 above, i.e., based on the current NB-IoT specifications.

Proposal 2: RAN2 responds to SA4 with the L2 protocol header size for service data based on the current specifications for NB-IoT.

Regarding the SA4 question about the RAN2 preferred options of CP and UP solutions, it is suggested that RAN2 respond to SA4 that RAN2 is working on down-selection solutions and has not yet concluded on the option.

Proposal 3: RAN2 responds to SA4 that RAN2 is working on the down-selection of solutions and has not yet concluded on the option.

# Discussion of Part 2: SA2 LS on support of IMS voice

Q1: Does RAN2 have any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC?

In SA2 LS [4], when the ROHC mechanism is used, it is mentioned that if enough consecutive packets are lost or erroneously decompressed, the compressor will transit to a lower compression state (e.g., FO or IR state). Some SA2 companies indicated that the number of consecutive lost packets that will trigger the compressor to fall back from the SO state to the FO state is around 16, and the number of consecutive lost packets that will trigger the compressor to fall back from the FO state to the IR state is around 64. SA2 asks whether RAN2 has any observation on the downward state transition of the compressor. From the RAN2 perspective, it should be taken into mind that ROHC function is located in the PDCP layer and 3GPP RAN only has responsibility for the configuration and application of ROHC function. According to the TS 36.323, it is stated that the implementation of the functionality of the ROHC framework and of the functionality of the supported ROHC profiles is not covered in the 3GPP specification. Therefore, there is no way that RAN2 has accurate knowledge of the number of consecutive lost packets that will trigger the downward state transition of the compressor, which is up to the detailed configuration of the ROHC framework.

Observation 2: 3GPP RAN only takes responsibility for the configuration and application of the ROHC function, while the implementation of the functionality of the ROHC framework and of the functionality of the supported ROHC profiles is not in the scope of RAN.

Proposal 4: For SA2 Q1, RAN2 responds to SA2 that 3GPP RAN only takes responsibility for the configuration and application of the ROHC function, while the implementation of the functionality of the ROHC framework falls outside of RAN2's scope.

Q3: Can RAN2 confirm whether scheduling methods for support of IMS voice over NB-IoT NTN can handle the voice packets of different sizes changing dynamically?

From the perspective of efficient radio resource scheduling, the Semi-Persistent Scheduling (SPS) mechanism is more suitable for the transmission of IMS voice packets, given the periodic characteristic of IMS voice service. Currently, a single SPS configuration is primarily used to transmit fixed-size packets. For the scenario where packet sizes vary due to talk/silence transitions during an IMS service call, two SPS configurations should be introduced to transmit packets generated in talk and silence periods, respectively. However, whether separate SPS configurations for the talk and silence period are actually necessary depends on the design of the voice codec in SA4. For example, if the voice codec bit rate is extremely low, the difference in packet size between the talk and silence periods is negligible. Under such circumstances, a single SPS configuration is enough. In case where packet sizes vary due to occasional RoHC state change (e.g., FO->SO state), the dynamic grant mechanism can serve as a supplementary means to transmit packets when SPS resources are insufficient. For example, for DL data transmission, it is up to the NW implementation to provide dynamic scheduling to the UE when the NW finds that the SPS resource is not sufficient for the voice packet. For UL data transmission, NW can provide dynamic scheduling to the UE based on BSR or segmentation info in the RLC layer. However, this approach may bring additional delay and impact system capacity, which requires further evaluation to determine whether it is acceptable or not. In summary, to accommodate the voice packet with different sizes, one of the candidate solutions could be enabling two SPS configurations with dynamic scheduling used as a supplementary means. Considering that the related standardization work will begin in 2026, to advance the progress of SA2 discussion, RAN2 may first respond to SA2 and confirm that the RAN scheduling method can handle the voice packets of different sizes.

Observation 3: To handle the voice packet with different sizes, e.g., due to talk/silence transition, two SPS configuration mechanisms with dynamic scheduling used as a supplementary means can be one of the candidate solutions.

Proposal 5: For SA2 Q3, RAN2 responds to SA2 with the confirmation that RAN scheduling can handle the voice packets of different sizes.

SA2 Q6: Is it feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20?

In legacy NB-IoT, the support for up to 2 DRBs for UE supporting CIoT UP optimizations is an optional UE capability (i.e., multipleDRB). The main motivation for this is to reduce the required size for the associated UE hardware and memory (along with the limited and simple nature of supported NB services), thereby lowering the UE cost. Consequently, the limitation that the capability of an NB-IoT UE supports a maximum of 2 DRBs is mainly driven by the UE cost and not by technology restrictions. Therefore, in response to SA2’s question, we think it is technically feasible to support more than 2 DRBs for UE accessing NB-IoT. In SA2 LS [4], it is mentioned that if more than 2 DRBs (e.g., 3 DRBs) are supported, then the UE can support simultaneous voice and other services even during the call. Considering the limited radio resource in the GEO scenario, RAN2 understands there is no need for UE to support simultaneous data transmission of voice and other services in the Uu interface.

Proposal 6: For SA2 Q6, RAN2 responds to SA2 that it is technically feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20.

SA2 Q9: Can RAN2 provide feedback on whether it is possible to define a new SRB that will be used to carry voice media?

SA2 Q10: If the answer to the above question is yes, can RAN2 confirm whether such SRB(s) are defined, whether it is technically feasible to support and configure RLC UM for such SRB(s) for example, in NB-IoT deployments over GEO satellite, when SRBs are used to carry voice media?

Regarding the SA2 Q9 and Q10, there are related questions. Hence, we will analyze these questions together.

In legacy RAN design, SRBs are typically used to transmit RRC messages and NAS messages. Compared with service data, these signaling messages require higher reliability guarantees and scheduling while placing less emphasis on the low-latency requirement. To ensure reliability, SRBs are mapped to AM RLC, with this mapping implemented as a hard-coded design in the RAN specification. Even in NB-IoT CP architecture where SRB is extended to transmit NB-IoT service data (e.g., periodic reporting of an Intelligent water meter) carried in NAS message, this design is still tailored to service with relaxed delay characteristics. Such a latency-tolerant framework makes SRB unsuitable for transmitting voice packets, which demand strict real-time performance. Moreover, in legacy NB-IoT, the default values for SN field length used by an AM RLC entity are 10 bits, which is the only RLC SN length supported for AM RLC, and at least needs a two-byte RLC header overhead. Compared with UM RLC with 5-bit RLC SN, SRB with AM RLC will bring more protocol overhead. By contrast, in legacy communication systems, voice packets are intentionally mapped on the DRB with UM RLC. This design directly addresses the real-time characteristic of voice services: UM RLC adopts a pull-based receipt window and omits the retransmission mechanism of AM RLC, minimizing transmission latency to meet the strict delay requirements of voice communication. In addition, UM RLC adopts a short RLC SN length (i.e., 5 bits), which reduces the protocol overhead. In summary, SRBs’ inherent design (hard-coded AM RLC mapping, latency tolerance, long RLC SN length) and application scope (signaling, delay-relaxed services) make them incompatible with voice packet transmission. Instead, DRBs with UM RLC are the suitable choice to ensure voice services’ real-time performance.

Proposal 7: For SA2 Q9 and 10, RAN2 responds to SA2 that, as per legacy RAN design, the voice packet is mapped on DRB with UM RLC considering real-time characteristic of voice service, while SRB with AM RLC is used to transmit packets with relaxed latency requirements. SRB is not preferred for voice packets from the RAN2 point of view.

# Discussion of Part 3: SA4 LS on SPS

In legacy NB-IoT, only UL SPS for BSR is supported, with periodicity divided by 10240ms as below.

SR-SPS-BSR-Config-NB-r15 ::= CHOICE {

release								NULL,

setup								SEQUENCE {

semiPersistSchedC-RNTI-r15			C-RNTI,

semiPersistSchedIntervalUL-r15		ENUMERATED {sf128, sf256, sf512, sf1024,

sf1280, sf2048, sf2560, sf5120}

}

As mentioned in SA4 LS [5], for 120ms and 240ms bundling periods that are not divided by 10240ms, the UE would send a voice packet and receive a voice packet every 120ms or 240ms. Taking the 120ms bundling period as an example, to match the service periodicity as much as possible, NW may configure SPS with a periodicity of 128ms. However, the periodicity misalignment between the data service periodicity and the SPS periodicity will cause additional delay, as shown in Fig.2 below. Especially, as time elapses, the additional cumulative delay will significantly increase.

Fig.2 Example of UL data transmission under periodicity misalignment

To solve the periodicity misalignment issue, some candidate solutions are listed as follows,

- Option 1: Adjustment of SPS resource by RRC reconfiguration (as per current specification)

    - When the additional delay becomes acceptable, NW sends RRC reconfiguration to adjust the SPS resource. It should be noted that this is only supported in NB-IoT UP architecture. How to adjust the SPS resource in NB-IoT CP architecture is currently unknown;

    - The disadvantage of this solution is that NW should adjust the SPS resource by RRC message whenever the additional delay is unacceptable, which will cause additional signalling overhead.

- Option 2: Usage of short SPS periodicities (e.g., shorter periodicities than those available in legacy NB-IoT)

    - Shorter SPS periodicity (e.g.120ms) can be introduced to match service data periodicity;

    - The disadvantage of this solution is that radio resource waste will occur if the SPS periodicity is smaller than the data service periodicity. Additionally, if the SPS periodicity that can not be divided by 10240ms is introduced (e.g., 120ms), a reference SFN, which is used for UE to determine the correct starting SFN, may be needed when NW configures the SPS resource.

- Option 3: Multiple SPS configurations

    - Multiple SPS configurations with different offsets are introduced to accommodate the service data periodicity as much as possible.

    - The disadvantage of this solution is that UE is required to support multiple active SPS configurations simultaneously, which will complex UE behaviour. Radio resource waste will occur as well.

- Option 4: More efficient adjustment of SPS resource timing in the UE

    - Instead of RRC reconfiguration, UE autonomously adjusts the SPS resource based on, e.g., knowledge of voice service pattern.

Considering that there are many candidate solutions to solve the periodicity misalignment issue, which are not easy to reach a conclusion in a short time, it is proposed that the related SA4 LS is postponed until RAN2 begins the standardization work on SPS.

Proposal 8: RAN2 postpones the SA4 LS on SPS until it initiates the standardization work on SPS objective.

# Discussion of Part 4: Down-select between CP and UP solutions

## Background in SA

### 5.1.1	SA1 requirement

SA1 has studied the use cases of IMS voice call using GSO satellite access, and the results are documented in TR 22.887. Normative service requirements and KPIs on IMS voice call using GSO satellite access are captured in TS 22.261, as shown in Table 1 below.

It can be found that the GEO scenario demands a low data rate (1~3kbps) for voice service. In this context, the original payload size of the voice packet generated in the application layer is relatively small compared to the total packet size of the voice packet transmitted over the Uu interface. This characteristic makes protocol overhead reduction particularly beneficial, as excessive overhead would account for a significant proportion of the limited transmission resources. Furthermore, radio resources in GEO scenarios are inherently limited, which results in a limited PHY data rate. This constraint further emphasizes the necessity of protocol overhead reduction. Therefore, protocol overhead should be one of the critical criteria for evaluating and selecting feasible and practical solutions for supporting the GEO voice service.

Table 1: Performance requirements for satellite access

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Overall user density | Activity factor | UE speed | UE type | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IMS voice call using GEO (note 7) | [1-3] kbit/s | [1-3] kbit/s | N/A | N/A | N/A | N/A | TBD | Handheld | Call set up time (note 8) ≤30 s (note 9) |
| Note 7: All the values defined for IMS voice call using GEO satellite access are generic for both regular and emergency calls. Note 8: call set up time refers to [56]; Note 9: 30s is the upper bound that is derived based on the user’s patience suggestions (30s) in [57]; |  |  |  |  |  |  |  |  |  |  |

Observation 4: Due to the low data rate 1~3kbps for the GEO voice service, from RAN’s perspective, protocol overhead should be taken as one of the criteria for down-selecting solutions.

### 5.1.2	SA2 candidate solutions

SA2 has been working on supporting GEO voice over NB-IoT for several meetings, and the candidate solutions have been documented in SA2 TR 23.700-19, which can serve as a reference for RAN2’s work.

From the perspective of PDN connection type (i.e., IP or non-IP), data type (i.e., IP or non-IP), and bearer type (i.e., DRB or SRB) for voice packet and IMS signalling, these candidate solutions are summarized as follows,

Table 2: Candidate solutions for supporting GEO voice over NB-IoT

| Solution | PDN number and type | Voice packet |  |  | IMS Signalling |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | IP/Non-IP | CP/UP | Default/dedicated EPS bearer, via DRB/SRB | IP/Non-IP | CP/UP | Default/dedicated EPS bearer, via DRB/SRB |
| #1 | 1 (IP type) | IP | UP | Default, DRB | IP | UP | Default, DRB |
| #2 | 1 (IP type) | IP | UP | Dedicated, DRB | IP | UP | Default, DRB |
| #3 | 1 (IP type) | Non-IP | UP | Dedicated, DRB | IP | UP | Default, DRB |
| #4 | 2 (non-IP type) | Non-IP | UP | Dedicated, DRB | Non-IP | UP | Default, DRB |
| #5 | 1 (non-IP type) | Non-IP | CP | Dedicated, SRB | Non-IP | CP | Default, SRB |
| #6 | 1 (IP type) | IP | CP | Dedicated, SRB | IP | CP | Default, SRB |
| #7 | 1 (IP type) | IP | CP& UP* | Default, SRB->DRB | IP | CP | Default, SRB |
| #8 | 1 (IP type) | IP | UP | Dedicated, DRB | IP | CP | Default, SRB |
| #9 | 1 (IP type) | IP | UP | Dedicated, DRB | IP | UP** | Default, SRB->DRB |
| #10 | 2 (IP and IP type) | Non-IP (without IP header) | UP | Dedicated, DRB | IP (without IP header) | CP | Default, SRB |
| #11 | 2 (IP and IP type) | Non-IP | UP | Dedicated, DRB | IP | CP | Default, SRB |
| * S1-U transfer ** only SIP INVITE via SRB |  |  |  |  |  |  |  |

It can be observed that there are 11 candidate solutions for supporting the GEO voice service. As noted in clause 3.2.1 above, from RAN’s perspective, protocol overhead should be one of the important criteria for evaluating and down-selecting these solutions. Taking this perspective into account and focusing specifically on the protocol overhead associated with voice packet transmission, these 11 candidate solutions can be categorized into four types: UP IP, CP non-IP, UP non-IP, and CP IP-based solutions.

Observation 5: As specified in SA2 TR 23.700-19, there are 11 candidate solutions in total for supporting the GEO voice service. From the perspective of protocol overhead for voice packets, these solutions are categorized into four types: UP IP, CP non-IP, UP non-IP, and CP IP based solutions.

## Details of candidate solutions

Based on the observations above, in the following, we will provide a detailed analysis for UP IP, CP non-IP, UP non-IP, and CP IP-based solution, respectively.

Per analysis in part 1, a 1-byte MAC sub-header is only applied in some special cases (e.g., the last sub-header in the MAC PDU and fixed-sized MAC CE), and is not realistic for GEO voice. Thus, it should be noted that the MAC PDU sub-header with a fixed size of 2 bytes (including L fields) is taken as an example in the following analysis.

Additionally, per analysis in part, the RRC overhead for data transmission is 1 byte for UL and 2 bytes for DL. Thus, it should be noted that the RRC overhead with 2 bytes is taken as an example in the following analysis.

### 5.2.1	UP IP based solution

For the UP IP-based solution, the original protocol stack for the voice packet is illustrated in Fig. 3-1.

In the legacy NB-IoT UP architecture, the ROHC mechanism between UE and eNB is already supported, except for RTP-related ROHC profiles (e.g., ROHC profile 0x0001 and 0x0101). For GEO voice, NB-IoT ROHC mechanism can be enhanced to support compression and decompression of RTP/UDP/IP protocol header, taking into consideration the requirement of low data rate (e.g., 1~3kbps) as specified in SA1 TS 22.261. According to the ROHC specification (e.g., RFC 3095), the RTP/UDP/IP protocol header can be compressed to 3 bytes in ROHC SO state, as depicted in Fig. 3-2.

As mentioned in SA2 LS [4], the packet size of the voice packet may vary due to the different ROHC compression states (e.g., different ROHC compression states will result in a compressed packet with a different packet size). Therefore, RAN scheduling should efficiently handle the variable packet size. Based on the initial consideration, this can be handled by SPS configuration with dynamic scheduling served as a complementary means.

Fig. 3-1 Original protocol header size                          Fig. 3-2 Compressed protocol header size (UE-eNB ROHC)

Observation 6: For UP IP based solution, the existing NB-IoT ROHC mechanism between UE and eNB should be enhanced to compress the protocol overhead (i.e., RTP/UDP/IP) of the GEO voice packet, and RAN scheduling should be able to efficiently handle the packet with variable packet size generated in different ROHC compression states.

### 5.2.2	UP non-IP based solution

According to the description of solution 3 in SA2 TR 23.700-19, the PDN for voice packet is IP type (e.g., IPv4, IPv6) PDN connection. Then, the original protocol stack for voice packet is the same as UP IP based solution, as illustrated in Fig. 4-1.

In UP Non-IP based solution, voice packet will use non-IP type transmission. Specifically, the UE sends the voice packet with RTP info (e.g., 1 byte SN), whereas UDP/IP is removed considering static characteristics. When the PGW receives the uplink voice packet with non-IP type transmission, the PGW converts the voice packet with RTP info to voice data with a full RTP/UDP/IP header. The compressed protocol header size for voice packet is shown in Fig.4-2. It should be noted that the exact length of the reduced RTP header needs further discussion by SA2.

In the legacy mechanism, for IP based S1 bearer, NW will apply ROHC mechanism to handle the RTP/UDP/IP protocol header to improve the efficiency of the radio resource. Under UP non-IP based solution, NW shall be aware that the ROHC mechanism is not applied. To achieve this, explicit/implicit indication should be provided by CN, which depends on RAN3/SA2 work. For example, given a specific QCI is allocated to the GEO voice service, NW will not configure ROHC function for DRB carrying GEO voice service when S1-bearer setup for IMS service voice is requested by MME.

Fig. 4-1 Original protocol header size                                    Fig. 4-2 Compressed protocol header size

Observation 7: For UP non-IP based solution (without IP/UDP header), how to perform RTP protocol overhead reduction is up to SA2 work. However, RAN should be indicated by CN that ROHC function is not applied in case UP non-IP based solution is used.

### 5.2.3	CP IP based solution

In NB-IoT CP architecture, uplink user data are transmitted in a NAS message, which is transmitted in a UL RRC container. Thus, the original protocol stack for voice packet is illustrated in Fig. 5-1.

In the legacy NB-IoT, ROHC function between UE and MME is already supported, except for RTP related ROHC profiles. For GEO voice, NB-IoT ROHC mechanism can be enhanced to support compression and decompression of RTP/UDP/IP protocol header between UE and MME. In Rel-19, CT1 has supported NAS header with 7 bytes for the connected UE [5]. In this context, the compressed protocol header size for voice packet is shown in Fig.3-2. It should be noted that whether further NAS header reduction is up to SA/CT1 discussion.

For CP IP based solution, AS protocol overhead is higher than UP based solution. RAN2 may need to discuss whether further AS overhead reduction (e.g., RRC protocol overhead) is needed, in order to satisfy the requirement of low data rate (i.e., 1~3kbps). In addition, the same as UP IP solution, the packet size of voice packet may vary due to the different ROHC compression states.

Observation 8: For CP IP based solution, RAN2 may need to further AS overhead reduction (e.g., RRC protocol overhead reduction), to satisfy the requirement of low data rate, and RAN scheduling should be able to efficiently handle the packet with variable packet size generated in different ROHC compression state between UE and MME.

Fig. 5-1 Original protocol header size                  Fig. 5-2 Compressed protocol header size (UE-MME ROHC)

### 5.2.4	CP non-IP based solution

According to the description of solution 3 in SA2 TR 23.700-19, the PDN for the voice packet is a non-IP PDN connection. Then, the UDP/IP protocol header is not present. The original protocol stack for the voice packet is illustrated in Fig.6-1. Same with UP non-IP-based solution, the exact length of the reduced RTP header needs further SA discussion. In addition, the same as the CP IP based solution, whether further reducing RRC overhead needs RAN2 discussion.

Fig. 6-1 Original protocol header size                                      Fig. 6-2 Compressed protocol header size

Observation 9: For CP non-IP based solution (without IP/UDP header), how to perform RTP protocol overhead reduction is up to SA2 work, and RAN2 may need to further AS overhead reduction (e.g., RRC protocol overhead reduction), to satisfy the requirement of low data rate.

## Comparison of candidate solutions

### 5.3.1	Comparison of protocol overhead

Based on the analysis in sub-clause 5.2, the comparison of protocol overhead for different solutions is shown in Fig. 7. It can be clearly observed that UP based solutions have lower protocol overhead, compared with CP based solutions. Even if the AS layer protocol overhead of CP-based solutions is optimized (e.g., reducing RRC protocol overhead to 0 bytes), their overall protocol overhead remains higher than that of UP based solutions. This is primarily due to the inherent existence of the NAS protocol header in CP-based solutions, specifically, a minimum NAS protocol overhead of 7 bytes.

Fig. 7 Comparison of protocol overhead for different solutions

Observation 10: The overall protocol overhead of UP based solution is lower than that of CP based solution which has NAS protocol overhead with a minimum size of 7 bytes.

### 5.3.2	Comparison of Potential RAN impacts

In the legacy TN, IMS currently operates over the user plane, i.e., one DRB for IMS signalling (QCI=5) and another DRB for voice packet (QCI=1). For the legacy NB-IoT UP architecture, a maximum of two DRBs is supported. For GEO voice scenarios, we think these two DRBs can be used to transmit IMS signalling and voice, respectively, the same as the TN mechanism. An FFS here is whether the extension of the maximum number of supported DRB (e.g., 3 DRB) should be supported, as asked by SA2 LS [4].

For NB-IoT CP architecture, only SRB1bis is used. Given the distinct QoS characteristics of voice packets, a new SRB should be introduced for voice packets to differentiate them from legacy NAS messages. Another FFS here is whether an additional new SRB should be introduced specifically for IMS signalling. Additionally, the legacy NB-IoT architecture does not support the RRC reconfiguration message. For GEO voice scenarios, however, this message must be introduced to establish or modify SRBs for IMS voice services.

Based on the above analysis, the potential RAN impacts of different solutions are summarized in Table 3. It can be clearly observed that for UP based solution, the existing RAN function framework can be reused, meaning that no new function needs to be introduced. In contrast, for CP based solutions, new RAN function framework (e.g., RRC reconfiguration, SRB management for IMS voice service) must be introduced in NB-IoT CP architecture, which is nearly equivalent to that of the UP solution.

Table 3: Comparison of RAN impacts for different solutions

|  | RB management | RRC reconfiguration | Overhead reduction |
| --- | --- | --- | --- |
| UP IP | Legacy - Reuse existing DRB management FFS - Extension of maximum number of supported DRB | Legacy | ROHC - Introduce RTP related ROHC configuration and capability FFS - Scheduling enhancement due to varying ROHC header size (UE-RAN ROHC) |
| UP non-IP | Legacy - Reuse existing DRB management FFS - Extension of maximum number of supported DRB | Legacy | FFS - RAN impact to support RTP reduction |
| CP IP | New - Introduce new SRB for IMS voice service, FFS for IMS signalling - QoS management for SRB - The mapping of IMS voice service to SRB | New - Introduce RRC reconfiguration | FFS - Further RRC overhead reduction - Scheduling enhancement due to varying ROHC header size (UE-MME ROHC) |
| CP non-IP | New - Introduce new SRB for IMS voice service FFS for IMS signalling - QoS management for SRB - The mapping of IMS voice service to SRB | New - Introduce RRC reconfiguration | FFS - Further RRC overhead reduction |

Observation 11: For UP based solutions, the existing RAN Function framework can be reused, while new RAN functions (e.g., RRC reconfiguration, SRB management for IMS voice service) should be introduced in NB-IoT CP architecture to support CP based solutions.

## Down-selection recommendation

Based on the comparison of different solutions from the perspective of overall protocol overhead and potential RAN impacts, it is suggested that RAN first select the UP solution. Considering the variable packet size generated due to the ROHC mechanism, RAN can further discuss the potential scheduling solutions and then down-select between UP IP and UP non-IP based solutions.

Proposal 9: From the RAN2 perspective, RAN2 selects the UP solution with a further down-selection between UP IP and UP non-IP.

# Conclusion

In this contribution, we have analyzed SA2/SA4 LSs, and provided a detailed comparison among candidate solutions for supporting GEO voice from the perspectives of overall protocol overhead and potential RAN impacts. All the observations and proposals are listed below,

Part 1: SA4 LS on RAN assumption

Proposal 1: RAN2 responds to SA4 that a packet overhead of 1 byte of MAC header for voice packets is not realistic based on the current MAC spec.

Observation 1: In Legacy NB-IoT, the L2 protocol header size for service data transmission is specified as follows,

- For CP architecture, the RRC overhead is 1 byte for UL and 2 bytes for DL, whereas for UP architecture, the RRC overhead is 0 bytes;

- For UP architecture, the PDCP protocol header has a size of 1 byte (i.e., PDCP SN=7bits), whereas for CP architecture, the PDCP protocol overhead is 0 bytes;

- With no RLC concatenation and segmentation, the AM RLC protocol header has a fixed size of 2 bytes (AM RLC is the only applicable RLC mode for the CP architecture), whereas the UM RLC protocol header has a fixed size of 1 byte;

- Except for certain special cases (e.g., the last sub-header in the MAC PDU), the MAC protocol header has a fixed size of 2 bytes.

Proposal 2: RAN2 responds to SA4 with the L2 protocol header size for service data based on the current specifications for NB-IoT.

Proposal 3: RAN2 responds to SA4 that RAN2 is working on the down-selection of solutions and has not yet concluded on the option.

Part 2: SA2 LS on support of IMS voice

Observation 2: 3GPP RAN only takes responsibility for the configuration and application of the ROHC function, while the implementation of the functionality of the ROHC framework and of the functionality of the supported ROHC profiles is not in the scope of RAN.

Proposal 4: For SA2 Q1, RAN2 responds to SA2 that 3GPP RAN only takes responsibility for the configuration and application of the ROHC function, while the implementation of the functionality of the ROHC framework falls outside of RAN2's scope.

Observation 3: To handle the voice packet with different sizes, e.g., due to talk/silence transition, two SPS configuration mechanisms with dynamic scheduling used as a supplementary means can be one of the candidate solutions.

Proposal 5: For SA2 Q3, RAN2 responds to SA2 with the confirmation that RAN scheduling can handle the voice packets of different sizes.

Proposal 6: For SA2 Q6, RAN2 responds to SA2 that it is technically feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20.

Proposal 7: For SA2 Q9 and 10, RAN2 responds to SA2 that, as per legacy RAN design, the voice packet is mapped on DRB with UM RLC considering real-time characteristic of voice service, while SRB with AM RLC is used to transmit packets with relaxed latency requirements. SRB is not preferred for voice packets from the RAN2 point of view.

Part 3: SA4 LS on SPS

Proposal 8: RAN2 postpones the SA4 LS on SPS until it initiates the standardization work on SPS objective.

Part 4: Down-select between CP and UP solutions

Observation 4: Due to the low data rate 1~3kbps for the GEO voice service, from RAN’s perspective, protocol overhead should be taken as one of the criteria for down-selecting solutions.

Observation 5: As specified in SA2 TR 23.700-19, there are 11 candidate solutions in total for supporting the GEO voice service. From the perspective of protocol overhead for voice packets, these solutions are categorized into four types: UP IP, CP non-IP, UP non-IP, and CP IP based solutions.

Observation 6: For UP IP based solution, the existing NB-IoT ROHC mechanism between UE and eNB should be enhanced to compress the protocol overhead (i.e., RTP/UDP/IP) of the GEO voice packet, and RAN scheduling should be able to efficiently handle the packet with variable packet size generated in different ROHC compression states.

Observation 7: For UP non-IP based solution (without IP/UDP header), how to perform RTP protocol overhead reduction is up to SA2 work. However, RAN should be indicated by CN that ROHC function is not applied in case UP non-IP based solution is used.

Observation 8: For CP IP based solution, RAN2 may need to further AS overhead reduction (e.g., RRC protocol overhead reduction), to satisfy the requirement of low data rate, and RAN scheduling should be able to efficiently handle the packet with variable packet size generated in different ROHC compression state between UE and MME.

Observation 9: For CP non-IP based solution (without IP/UDP header), how to perform RTP protocol overhead reduction is up to SA2 work, and RAN2 may need to further AS overhead reduction (e.g., RRC protocol overhead reduction), to satisfy the requirement of low data rate.

Observation 10: The overall protocol overhead of UP based solution is lower than that of CP based solution which has NAS protocol overhead with a minimum size of 7 bytes.

Observation 11: For UP based solutions, the existing RAN Function framework can be reused, while new RAN functions (e.g., RRC reconfiguration, SRB management for IMS voice service) should be introduced in NB-IoT CP architecture to support CP based solutions.

Proposal 9: From the RAN2 perspective, RAN2 selects the UP solution with a further down-selection between UP IP and UP non-IP.

# Reference

- RP-251867, New WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4, vivo.

- RP-252473, Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4, vivo.

- R2-2505068/S4-251584, LS on the RAN simulation assumptions for ULBC.

- R2-2506747/S2-2507636, LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC.

- R2-2506754/S4aA250258, LS on bundling period and SPS for ULBC.

- R2-2506706/C1-255650, Reply LS to SA4 on the RAN simulation assumptions for ULBC.

# Annex 1: Draft Reply LS to SA4

3GPP TSG RAN WG2 Meeting #131bis	R2-250xxxx

Prague, Czech Republic, October 13th-17th, 2025

Title:	Draft Reply LS on RAN Simulation Assumptions for ULBC

Response to:	R2-2505068/S4-251550

Release:		Release 20

Work Item:		IoT_NTN_Ph4-Core

Source:		vivo [to be RAN WG2]

To:		SA WG4,

Cc:		RAN WG1, CT WG1, SA WG2

Contact Person:

Name:	Xiaodong Yang

E-mail Address:	yangxiaodong5g@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: 	none

1. Overall Description:

RAN2 would like to thank SA4 for their LS on the RAN simulation assumptions for ULBC.

Regarding SA4 questions, RAN2’s responses are as follows,

SA4 kindly asks SA2 and RAN2 to comment on

- the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option, and

RAN2 answer: RAN2 is working on the down-selection solutions between CP and UP solutions and has not yet concluded on the option. And based on the current NB-IoT specification, the L2 protocol overhead for service data is specified as follows,

- For CP architecture, the RRC overhead is 1 byte for UL and 2 bytes for DL, whereas for UP architecture, the RRC overhead is 0 bytes;

- For UP architecture, the PDCP protocol header has a size of 1 byte (i.e., PDCP SN=7bits), whereas for CP architecture, the PDCP protocol overhead is 0 bytes;

- With no RLC concatenation and segmentation, the AM RLC protocol header has a fixed size of 2 bytes (AM RLC is the only applicable RLC mode for the CP architecture), whereas the UM RLC protocol header has a fixed size of 1 byte;

- Except for certain special cases (e.g., the last sub-header in the MAC PDU), the MAC protocol header has a fixed size of 2 bytes.

SA4 kindly asks SA2 and RAN2 to comment on

- specifically, whether a packet overhead of 1 byte of MAC header is realistic.

RAN2 answer: A packet overhead of 1 byte of MAC header for voice packets is not realistic based on the current MAC specification.

2. Actions:

To SA4:

ACTION: RAN2 kindly inform SA4 to take the above response into account.

3. Dates of Next RAN2 Meetings:

RAN2#132				17th to 21st November 2025			Dallas, US

# Annex 2: Draft Reply LS to SA2

3GPP TSG RAN WG2 Meeting #131bis	R2-250xxxx

Prague, Czech Republic, October 13th-17th, 2025

Title:	Draft Reply LS on the support of IMS voice over NB-IoT NTN connected to EPC

Response to:	R2-2506747/S2-2507636

Release:	Release 20

Work Item:	IoT_NTN_Ph4-Core

Source:	vivo [to be RAN WG2]

To:	SA WG2,

Cc:	RAN WG1, CT WG1

Contact Person:

Name:	Xiaodong Yang

E-mail Address:	yangxiaodong5g@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN2 would like to thank SA2 for their LS on the support of IMS voice over NB-IoT NTN connected to EPC.

Regarding SA2 questions, RAN2’s responses are as follows,

SA2 Q1: Does RAN2 have any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC?

RAN2 answer: In NB-IoT UP architecture, 3GPP RAN only takes responsibility for the configuration and application of ROHC function, while the implementation of the functionality of the ROHC framework falls outside of RAN2's scope.

SA2 Q3: Can RAN2 confirm whether scheduling methods for support of IMS voice over NB-IoT NTN can handle the voice packets of different sizes changing dynamically?

RAN2 answer: RAN2 confirms that RAN scheduling can handle the voice packets of different sizes.

SA2 Q6: Is it feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20?

RAN2 answer: It is technically feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20.

SA2 Q9: Can RAN2 provide feedback on whether it is possible to define a new SRB that will be used to carry voice media?

RAN2 answer: As per legacy RAN design, the voice packet is mapped on DRB with UM RLC considering real-time characteristic of voice service, while SRB with AM RLC is used to transmit packets with relaxed latency requirements. SRB is not preferred for voice packets from the RAN2 point of view.

SA2 Q10: If the answer to the above question is yes, can RAN2 confirm whether such SRB(s) are defined, whether it is technically feasible to support and configure RLC UM for such SRB(s) for example, in NB-IoT deployments over GEO satellite, when SRBs are used to carry voice media?

RAN2 answer: Same answer as for Q9.

2. Actions:

To SA2:

ACTION: RAN2 kindly inform SA2 to take the above response into account.

3. Dates of Next RAN2 Meetings:

RAN2#132				17th to 21st November 2025			Dallas, US