3GPP TSG-RAN WG2 Meeting #131bis	R2-2506851

Prague, Czech Republic, Oct. 13th - 17th, 2025

Agenda Item:	10.3.3

Source:	OPPO

Title:	Discussion on the RAN2-related 6G security aspects

Document for:	Discussion, Decision

# Introduction

According to the RAN WG-level 6G SID [1] as quoted below, RAN2 needs to study the AS-level security aspects in alignment with SA3.

RAN 6G SID [1]: (2) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

According to the SA3 6G SID [2] as quoted below, SA3 also needs the RAN inputs to complete the 6G security study.

SA3 6G SID [2]: This study aims to define a security and privacy architecture and procedures for 6G mobile networks for improvement of existing services and support of new services, to meet the 6G system requirements as defined by 3GPP SA1 and TSG RAN for the system architecture defined by 3GPP SA2. The study will work towards goals endorsed at TSG#107(Mar 2025) to "create lean and streamlined standards for 6G, e.g. by dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, avoiding excessive configurations, etc. Any exception to the above shall be well justified." The study includes the following high level work tasks, and the conclusion will consolidate the 6G security specifications among all work tasks: WT#1: Study security and privacy aspects for overall 6G system architecture (ref: SP-250806). NOTE1: The above WT contains security aspects that do not fit into any other tasks, e.g. security aspects that affect both the RAN and Core Network, enhancements to the security architecture and security aspects related to work from other groups. WT#2: Study security and privacy aspects of 6G RAN architecture (including all UE – RAN interactions and ref: RP-251881). WT#3 : Study security and privacy aspects of 6G UE to core network interactions. WT#4: Study enhancements to Core Network security including endpoint security at transport and application layers, internal and external interfaces as well as end to end roaming security taking roaming intermediary into account. NOTE2: For roaming, coordination with GSMA is required. NOTE3: In the above WTs, the study will cover possible security enhancements of the procedures from previous generations and new security aspects that arise from work in other working groups (including the developments regarding different particular verticals and deployments).

In this contribution, we provide our understandings on the required coordination between RAN and SA3 for the 6G security requirements, and the corresponding RAN2 standard works.

# Discussion

## RAN2 6G security functions/requirements inherited from 5G

### 5G AS security functions reused for 6G

3GPP TS 38.331 [8]: ### 5.3.6 Counter check #### 5.3.6.1 General Figure 5.3.6.1-1: Counter check procedure The counter check procedure is used by the network to request the UE to verify the amount of data sent/ received on each DRB. More specifically, the UE is requested to check if, for each DRB, the most significant bits of the COUNT match with the values indicated by the network. NOTE: The procedure enables the network to detect packet insertion by an intruder (a 'man in the middle').

3GPP TS 38.323 [9]: ## 5.8 Ciphering and deciphering … ## 5.9 Integrity protection and verification …

According to the 5G AS procedures, the AS-level security functions include:

- RRC counter check procedure [8]

- Integrity protection and cyphering for SRBs (i.e. dedicated RRC messages) at PDCP layer [9]

- Integrity protection and cyphering for DRBs at PDCP layer [9]

From our understanding, the above AS-level security functions of 5G can be reused for 6G, so that the Rel-20 security study in RAN2 can focus on the extra 6G enhancements of security. For the integrity protection and cyphering of SRBs and DRBS, keeping the security function at the PDCP layer can reduce the number of key changes, as when the PDCP anchor is not changed, the security key refresh is not always performed during handover. By implementing the count check procedure via RRC signalling, the DRB does not have to be configured with the integrity protection, so that the extra signalling overheads (i.e. MAC-I) and the processing load over the DRB SDUs can be reduced. On the other hand, as the 5G UE only re-establishes the RRC connection when the SRB fails the integrity verification, and drops the PDCP PDU of DRB which fails the integrity verification, the gNB by using the count check procedure can detect the maliciously inserted packets of the DRB.

Observation 1: In 5G, the integrity protection and cyphering for SRB and DRB at the PDCP layer reduces the number of key changes during handover. The count check procedure helps the gNB to detect maliciously inserted packets of the DRB.

Proposal 1: RAN2 confirms the following 5G security functions reused in 6G AS: 1) Count check procedure via RRC; 2) Integrity protection and cyphering for SRB and DRB at PDCP layer

### 5G RAN security requirements reused for 6G

SA3 6G SID in [2]:

According to the endorsed SA3 6G SID as highlighted above, the security studies in SA3 need to take into account the requirements from other working groups, including “the architecture related requirements from RAN”.

Observation x.1: The SA3 6G security study needs to consider the architecture related requirements from RAN.

3GPP TR 38.913 [3] provided by RAN WGs: ## 10.12 Security and Privacy related requirement relevant for Radio Access The RAN design for the Next Generation Radio Access Technologies shall ensure support for integrity and confidentiality protection of radio signalling messages, including messages between RAN and Core network nodes. The RAN design for the Next Generation Radio Access Technologies shall ensure the ability to support integrity and confidentiality protection of user plane messages, including messages between RAN and Core network nodes, with the use of such security to be configurable during security set-up. The RAN design for the Next Generation Radio Access Technologies shall ensure support for the allocation and use of identities to provide user privacy, e.g. reduce the need for sending any permanent identities in the clear. The RAN design for the Next Generation Radio Access Technologies shall ensure the efficient establishment of RAN security mechanisms. The RAN design for the Next Generation Radio Access Technologies shall ensure resilience against jamming. NOTE1: Security and Privacy-related system requirements are reflected in 3GPP TR 33.899 [19]. This TR includes security areas on "RAN security" and "Privacy security", which is a possible source of security and privacy related requirements for the Radio Access.

According to the 3GPP TR 38.913 [3] provided by RAN WGs, RAN provided the security-related requirements from the radio access point of view, which is based on the study outcomes from both RAN and SA3 (e.g. 3GPP TR 33.899 [3]).

From our understanding, the 5G security requirements as quoted above for the radio access should also be applicable for 6G. The security requirements inherited from 5G include:

- Requirement 1: The integrity protection and cyphering for SRB, which is used to carry both dedicated RRC message and NAS message.

- Requirement 2: The integrity protection and cyphering for DRB, which is used to carry both user plane data and the NAS message (over UP).

- Requirement 3: Avoid using permanent UE ID, by using temporary UE ID (e.g. C-RNTI or I-RNTI) instead.

- Requirement 4: Efficient establishment of RAN security context, e.g. avoid frequent key change during mobility, efficient temporary UE ID allocation and efficient detection of integrity protection.

- Requirement 5: Resilience against jamming, e.g. detecting the jamming signal via measurement and reducing the interference impact caused by the jamming signal.

According to the above requirements, the requirements of 1+2 require the SA3 to design the proper solutions for the integrity protection and cyphering for both SRB and DRB, and the requirements of 3+4+5 can be realized by the RAN solutions (e.g. temporary UE ID allocation and radio signal measurement). The details on which message/data via which SRB/DRB needs integrity protection and/or cyphering can be discussed later-on based on the solutions provided by both SA3 and RAN, so that the system is able to “ensure the efficient establishment of RAN security mechanisms”. For example, the 5G integrity protection applied for each PDCP SDU causes lots of signalling overheads and UE processing, especially for the traffic with a large number of small packets. In 6G, we can consider to concatenate multiple PDCP SDUs to a single PDCP PDU, which is used as a single input to create one MAC-I.

Observation x.2: The “Security and Privacy related requirement relevant for Radio Access” provided by RAN is based on the study outcomes from both RAN and SA3.

### Mobility impacts on security

3GPP TS 38.300: # 13 Security ## 13.1 Overview and Principles … Figure 13.1-1: 5G Key Derivation … ## 13.2 Security Termination Points … ## 13.3 State Transitions and Mobility …

According to the 5G TS 38.300, the RAN2-led stage-2 specification needs to clarify the AS-level security functions, including key derivation, security termination points and key generation for state transitions and mobility. The security section in the RAN2 stage-2 specification is based on the inputs from SA3 and the solutions (e.g. how to handle PDCP COUNT wrap around and how to handle the failed integrity check) from RAN.

Figure 1: Relation between L2 reset and key change

Since SA3 are planning to discuss some enhanced solutions for 6G key derivation (e.g. more keys for different RAN nodes including the key differentiation amongst CU-CP/CU-UP, multiple CU-UP(s) and DU), so as to provide more robust 6G security for the UE. However, as the security key refresh impacts the mobility procedure (e.g. L2 reset) as explained above, we think that RAN2 should inform SA3 that any key refresh at the UE will cause the UE L2 reset, which leads to the packet loss buffered at the UE L2. This is the reason why the UE in 5G performs key refresh for KgNB (i.e. no partial key refresh for KUPenc, KUPint, KRRCenc and KRRCint) and resets L2 whenever the CU-CP or CU-UP serving the UE is changed. Here, we have some detailed descriptions on how the security key refresh impacts the UE L2 buffer. As illustrated in the above figure, the relation between L2 reset and key change can be described as follows:

- Step 1: When the CU-UP key from PDCP#1 and PDCP#2 (or the CU-CP key from PDCP#5) changes, the UE needs to re-establish PDCP#1 and PDCP#2 (or PDCP#5) to update the key configuration of PDCP.

- Step 2: As the key configuration of PDCP is changed, the UE needs to stop the reception and transmission of the old packets using the old key. Otherwise, the transmission resource will be wasted, as the old packets encrypted by the old key is not able to be decrypted.

  - To stop the reception and transmission of the old packets using the old key, the UE needs to reset PDCP, RLC and MAC, which flushes all HARQ buffer (i.e. dropping the MAC PDU) in MAC. The packets from all RBs in the same MAC entity are dropped.

Figure 2: Potential issue of 6G key derivation

In 5G, if the CU is not changed, when one DRB is offloaded from one CU-UP to another CU-UP, the UE does not need to refresh its KgNB and does not result in L2 reset. In 6G, if SA3 introduces multiple CU-UP keys, the UE needs to change the key of a DRB#x when the DRB#x is offloaded from one CU-UP entity to another CU-UP entity. However, the partial key change should be avoided due to the UE L2 reset, and the UE should refresh KgNB, instead of refresh one set of (KUPenc, KUPint) of CU-UP. SA3 can then take into account the RAN2 concerns while designing the 6G security solutions.

3GPP TR 38.804 [5]: # 11 Security Security key refresh is not performed at every mobility procedure (i.e. handover), at least for the case of mobility where the PDCP anchor point is not changed. NOTE: It is to be confirmed by SA3 considering whether it has any implication on the inputs for key derivation, e.g. PCI. For DC between LTE and NR where the master RAT is LTE, S-KeNB is derived from KeNB of the master node.

3GPP TS 33.501 [6]: ##### 6.9.2.3.4 UE handling The UE behaviour is the same regardless if the handover is intra-gNB-CU, intra ng-eNB, Xn, or N2 with the exception that during intra-gNB-CU handover, the UE may retain the same key based on an indication from the gNB. The UE behaviour is also same in case of conditional handover, as specified in TS 38.300 [52], i.e., the UE shall use the parameters of the selected target cell in KNG-RAN* derivations.

According to the 3GPP 5G TR 38.804 drafted by RAN2 as quoted above, RAN2 decided that security key refresh is not performed at every mobility procedure when the PDCP anchor point is not change, so as to provide more efficient RAN handover procedure by reducing the number of security key refresh and the L2 reset caused by the security key refresh. According to the RAN requirements for the security key refresh, SA3 specified that “during intra-gNB-CU handover, the UE may retain the same key based on an indication from the gNB”, so that the security key refresh is not always performed when the PDCP anchor is not changed. We consider that the same 5G requirement regarding the security key refresh should be reused for 6G, to reduce the impacts (e.g. the packet loss caused by the security key refresh) on the UE mobility.

Observation 2: The RAN2 stage-2 specification needs to clarify the security functions, including key derivation, security termination points and key generation for state transitions and mobility. The UE refreshes KgNB and resets L2 whenever a security key (i.e. KUPenc, KUPint, KRRCenc and KRRCint) changes, i.e. no partial key change in 5G. According to 3GPP TR 38.804 of 5G, RAN decided that security key refresh is not always performed during handover when the PDCP anchor point is not changed, and SA3 followed the RAN2 requirement for the security key refresh.

Proposal 2: RAN2 informs SA3 the following 5G mobility requirements reused for 6G security: 1) The security key change during mobility should be minimized to avoid the packet loss due to L2 reset; 2) Security key change is not always performed at handover when the PDCP anchor point is not changed.

### Migration architecture /mobility solution /RRC state impacts on security

3GPP TS 37.340 [7] # 9 Security related aspects …

3GPP TS 33.501 [6]: ## 6.10 Dual connectivity …

Regarding the support of DC (Dual Connectivity) in 6G, we could have both inter-RAT DC (i.e. MN and SN from different RAT(s)) and intra-RAT DC (i.e. MN and SN from the same RAT). As the SN security key is derived from the MN security key, the security impacts caused by the DC architecture are as follows:

- 6G DC option 1: 6G MN + 6G SN. SA3 needs to define the 6G “Security mechanisms and procedures for DC”, including the SN key derivation and the coordination between MN and SN.

- 6G DC option 2: 6G MN + another-RAT SN. The 6G “Security mechanisms and procedures for DC” as defined for Option 1 is expected to be reused in SA3, by replacing 6G SN with another-RAT SN.

- 6G DC option 3: Another-RAT MN + 6G SN. The “Security mechanisms and procedures for DC” as defined for another RAT (e.g. 5G NR DC) is expected to be reused, by replacing another-RAT SN with 6G SN.

As whether to support 6G DC is to be decided by the RAN plenary, we consider that once RAN decided to support 6G DC, RAN should inform SA3 which DC architecture is supported for 6G, so that SA3 can design the corresponding security solutions for different 6G DC architectures.

3GPP TS 33.501 [6]: #### 6.10.2.4 Security mechanism and procedures for SCPAC … ## 6.17 Security mechanism and procedures for L1/L2 Triggered Mobility …

According to the 3GPP TS 33.501 as quoted above, different mobility procedures (e.g. SCPAC (Subsequent Conditional PSCell Addition or Change) and LTM (L1/L2 Triggered Mobility)) require different security mechanisms and procedures. When RAN decides to support different mobility procedures, RAN should send an LS to SA3 to inform them which mobility solution is supported in 6G.

3GPP TS 33.501 [6]: ### 6.8.2 Security handling at RRC state transitions #### 6.8.2.1 Security handling at transitions between RRC_INACTIVE and RRC_CONNECTED states … #### 6.8.2.2 Key handling during mobility in RRC_INACTIVE state …

According to the 3GPP TS 33.501 as quoted above, SA3 needs to specify how the AS security context is managed at different RRC states and at RRC state transition. When RAN decides which/how many RRC state is supported in RAN, RAN should inform the corresponding decisions to facilitate the SA3 standard work.

Observation 3: In 5G, the support of different migration architectures / mobility solutions (e.g. SCPAC or LTM) / RRC states (e.g. RRC_INACTIVE) requires different SA3 standard efforts.

Proposal 3: RAN2 informs SA3 once the migration architecture(s) / mobility solution(s) /RRC state(s) are decided.

## RAN2 6G new security functions/requirements different from 5G

A summary of SA3 6G security proposals impacting RAN2, according to the tdocs submitted in Section “6.3 New 6G SIDs/WIDs” in SA3#123 [10]: - WT#2.1 (RAN related generic security): Study security and privacy aspects of 6G RAN architecture. - WT#2.2 (Lower layer protection): Study security and privacy aspects of radio lower layers and define a security mechanism for vulnerable lower layer signalling including protection of MAC-CE. - WT#2.3 (SIB protection): Study security for SIBs to avoid FBS (False Base Station) attacks - WT#2.4 (Msg 3/5 protection): Study security for messages (e.g. Msg3/Msg5) exchanged prior to AS Security Mode Command - WT#2.5 (RAN UP security enhancements): Study enhanced user plane security for 6G to enable finer granularity control for the UP security and support UP key isolation for all the CU-UPs for a 6G RAN node in case of CU-CP/CU-UP RAN split architecture.

According to the 6G SID discussion in SA3#123 meeting, the work tasks related to the AS security as quoted above were discussed. Compared with the 5G security schemes, we consider that the extra RAN2 impacts of the 6G security solutions include MAC CE security, SIB security, Msg3/Msg5 security and multiple-sets of UP keys (given that 5G UE has only-one set of UE UP keys (KUPenc, KUPint)). Once SA3 decides to support the extra 6G security direction(s), RAN2 can start to design the corresponding RAN2 solutions, including signalling format (e.g. input variables like COUNT and (MAC) PDU format), configuration procedure, verification handling and so on).

Observation 4: Compared with 5G, the potential extra 6G RAN-related security impacts as currently being discussed in SA3 include: 1) MAC CE security; 2) SIB security; 3) Msg3/Msg5 security; 4) Finer granularity of the UP security, i.e. support UP key isolation for different CU-UP(s).

For the above 6G enhancements of security, RAN2 needs to consider how the security related parameters (e.g. COUNT and/or MAC-I) is added in MAC layer and RRC layer (e.g. CCCH RRC message), and how the verification procedure is performed by the UE. As the 6G enhancements of security are still ongoing in SA3 and the 6G CP/UP study is also ongoing in RAN2, we think that the 6G CU/UP design should have sufficient flexibility to add the extra 6G RAN-related security enhancements.

Proposal 4: The 6G CU/UP design should have sufficient flexibility to add the 6G RAN-related security enhancements.

From our understanding, the MAC CE security issue was identified since 5G, e.g. the NCC security issue identified for the LTM Cell Switch Command MAC CE. However due to the limited time for the Rel-19 mobility work item, the security issue of the LTM Cell Switch Command MAC CE was not resolved in Rel-19 5G. In 6G, the MAC CE security should be prioritized to resolve the already-revealed risks for the unprotected MAC CE(s). Additionally, we consider that one another security risk which should be considered in 6G is the protection of SIB, so that the UE communication blocked by the potential fake SIBs can be avoided. In the following sections, we provide more details on the use cases for the MAC CE security and the SIB security. In addition, we think that another issue which 6G RAN2 design should consider is the per-PDCP-SDU MAC-I generation issue, and we think that a proper packet concatenation solution should be considered to reduce the signalling overhead and the processing load of MAC-I.

### MAC-I processing issue in PDCP

3GPP TS 24.501: #### 9.11.4.7 Integrity protection maximum data rate .. 8; 7; 6; 5; 4; 3; 2; 1. Integrity protection maximum data rate IEI; octet 1. Maximum data rate per UE for user-plane integrity protection for uplink; octet 2. Maximum data rate per UE for user-plane integrity protection for downlink; octet 3. Figure 9.11.4.7.1: Integrity protection maximum data rate information element

According to the 3GPP TS 38.323 [9], when DRB integrity protection is configured, each PDCP SDU with a separate SN creates a separate MAC-I, which causes heavy processing load at the UE and lots of signalling overhead. In 5G, according to the 3GPP TS 24.501 [16], due to the UE processing limitation, an extra UE capability indication (i.e. “Integrity protection maximum data rate”) is introduced to differentiate the UE processing capability for DRB integrity protection at different data rate (i.e. one for 64 kbps and one for full data rate). The capability as quoted above means that if the DRB integrity protection is configured, some low-cost UE implementation may only be able to support up-to 64 kbps data rate. In 6G, when multiple PDCP SDUs are concatenated to a single PDCP PDU with single SN and single MAC-I, the UE processing load can be reduced for calculating MAC-I, and the signalling overhead of separate SN and MAC-I for each PDCP SDU is also reduced.

Observation 5: In 5G, the MAC-I generated per PDCP SDU causes extra signalling overheads and UE processing (i.e. impacting the UE’s transmission bit rate).

Proposal 5: More than one packets can be used to generate a single MAC-I.

### MAC CE security

In 5G, MAC CE have gained extensive utilization due to its unique advantages in control signalling, e.g. low latency and low header consumption. However, MAC CEs allowing the transmission of plaintext contents are neither ciphered nor integrity protected. The unprotected MAC CE(s) carries essential IEs (e.g., Beam index ID, Serving Cell ID, cell switch command and etc.), potential tampering attacks may affect radio quality or further lead to unavailability of system services. The attacker may estimate the approximate location of the UE based on some certain IEs (e.g., C-RNTI, Serving Cell ID, Beam index ID etc.) carried by MAC CE. Potential trackability and link-ability attacks of the UE may lead to the disclosure of privacy.

Spoofing during LTM: 3GPP TS 38.321 [14]: Vulnerable NCC in Enhanced LTM Cell Switch Command MAC CE Figure 6.1.3.75a-1: Enhanced LTM Cell Switch Command MAC CE Endorsed running CR 3GPP TS 33.501 [15]: ## 2.1 Key Issue #1: Security aspects of inter-CU LTM Handover … ### 2.1.2 Security threats An attacker may manipulate the timing advance information, and cause the desynchronization between the UE and the BS. The Target Configuration ID may be tampered with, leading to connection failure between UE and the target cell. Furthermore, if the NCC, keySetChangeIndicator or the chosen algorithms of the target gNB are carried by the LTM Cell Switch Command MAC CE, they could be tampered, if the UE has accepted the altered NCC value, this may result in out-of-sync, key mismatch or security negotiation failure between the UE and target gNB.

Spoofing during beam management: 3GPP TS 38.321 [14]: Vulnerable beam indication Figure 6.1.3.15-1: TCI State Indication for UE-specific PDCCH MAC CE

Eavesdropping of UE ID, Cell ID, TCI state and TA Command: 3GPP TS 38.321 [14]: Exposure of the UE location Figure 6.1.3.4-1: Timing Advance Command MAC CE Figure 6.1.3.75a-1: Enhanced LTM Cell Switch Command MAC CE Figure 3: UE location calculation based on TA

In general, the lack of MAC layer security may make L1/L2 vulnerable of spoofing and eavesdropping attacks [17] (as also illustrated above), such as:

- Spoofing during LTM: Attackers can tamper with sensitive information transmitted in the Enhanced LTM cell switch command MAC CE, e.g. candidate configuration index and NCC as illustrated above, leading to handover failure. According to the LS [13] and the endorsed running CR [15] from SA3 for Rel-19 mobility item, SA3 indicated that “the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure”, and “the Target Configuration ID may be tampered with, leading to connection failure between UE and the target cell”.

- Spoofing during beam management: Attackers can tamper with sensitive information transmitted in the TCI state indication MAC CE, such as beam index, potentially causing beam failure, RLF and data transmission interruption.

- Eavesdropping of UE ID, Cell ID, TCI state and TA Command: Parameters once considered not privacy-sensitive may become exposed to privacy risks, especially if they are combined in ways that can be used to derive privacy information, as illustrated above. Some examples are:

  - Eavesdropping of C-RNTI + Cell ID: C-RNTI is related to a certain UE within the cell in a period of time, and Cell ID indicates the identity of the serving cell to which the UE is currently connected. If the C-RNTI remains unchanged for an extended period within the serving cell, an attacker could continuously track the UE’s location information in that cell by monitoring the binding relationship between the C-RNTI and Cell ID.

  - Eavesdropping of C-RNTI + Cell ID + TA Command + Beam index ID: The TA value in TA Command reflects the signal propagation delay between the user equipment and the base station, which is proportional to the distance between the user equipment and the base station. By using the TA value, the user’s location can be narrowed down into a circle area around the base station as illustrated above. Combined with the Beam index ID, an attacker can further determine the user location relative to the base station (e.g., northeast direction to a TRP), further reducing the positioning range to a sector-shaped area. If multiple historical TA values / beam index(es) associated with the C-RNTI are tracked, the attacker may reconstruct the movement trajectory of a certain user corresponding to a certain C-RNTI in a period of time.

Since the SA3 solutions for MAC CE security depend on which/what fields will be introduced in 6G, RAN2 should also inform SA3 the MAC CE contents causing security issue. According to the analysis given above, we think that the following contents [17] included in MAC CE can cause security issue:

- Security parameters (e.g. NCC via LTM Cell Switch MAC CE)

- UE ID (e.g. C-RNTI and UE Contention Resolution ID)

- User’s location-related information

  - TA values (e.g. via RAR/TAC MAC CE)

  - Cell ID (e.g. Serving Cell ID or PCI or Candidate Cell ID)

  - Beam information (e.g. TCI-state / QCL-Info indication / SSB index)

Observation 6: The unprotected MAC CE leads to more security attack scenarios and higher probability of handover failure / radio link failure / beam failure / UE privacy exposure. SA3 needs to know which/what MAC CE contents causing security issue are included in 6G.

Proposal 6: RAN2 informs the SA3 the MAC CE contents (e.g. security parameters; UE ID and User’s location-related information) causing security issue.

### SIB security

Figure 4: Security threat of fake SIBx

In 5G, the system information works without security protection, and the AS security is not activated before the establishment of SRB1. According to the SA3 study on the FBS (False Base Station) as illustrated above, without the security protection for the system information, it may happen that the UEs camping on a real cell receives rogue SI message or previous SI message from an attacker, which induces the UE reselecting to incorrect neighbouring cells or responding to inexistent PWS message. The detailed descriptions of use cases for SIB security thread can be found in 3GPP TR 33.809 [11]. Apparently, attacks mounted by a false base station is a major security flaw in the existing 5G system. Additionally, the SA1 6G study also proposed to increase the level of security between the RAN entities and the UE during the initial network selection procedure against FBS, as described in 3GPP TR 22.870 [12]. In 6G, we think that it is a good opportunity to address this concern, without considering the backward compatibility (e.g. how to introduce the SIB security without impacting the legacy UE/network).

3GPP TS 38.331 [8]: NOTE 2: The physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2976 bits.

3GPP TR 33.809 [11]: ## 5.2 Key Issue #2: Security protection of system information … ## 6.0 Mapping between key issues and solutions Solutions; Key Issues. 1; 2; 3; 4; 5; 6; 7. Solution #1: Protection for the UE Capability Transfer; x. Solution #2: Protection of RRCReject message in RRC_INACTIVE state; x. Solution #3: Protection of uplink UECapabilityInformation RRC message; x. Solution #4: Enriched measurement reports; x. Solution #5: Mitigation against the authentication relay attack; x. Solution #6: Avoiding UE connecting to false base station during HO; x. Solution #7: Verification of authenticity of the cell; x. Solution #8: Network detection of nearby false base stations from call statistics and measurements; x. Solution #9: Using symmetric algorithm with assistance of USIM and home network; x; x. Solution #10: Protection on the unicast message based on ECDH; x. Solution #11: Certificate based solution against false base station; x; x. Solution #12: ID based solution against false base station; x; x. Solution #13: Protecting RRCResumeRequest against MiTM; x. Solution #14: Shared key based MIB/SIBs protection; x. Solution #15: Mitigation against the authentication relay attack with different PLMNs; x. Solution #16: Protection of RRC Reject Message; x. Solution 17: Integrity protection of the whole RRCResumeRequest message; x. Solution #18: Avoiding UE connecting to False Base Station during Conditional Handover; x. Solution #19: AS security based MIB/SIBs integrity information provided by gNB; x. Solution #20: Digital Signing Network Function (DSnF); x. Solution #21: Certificate based solution against false base station for Non-Public Networks; x; x. Solution #22: Detecting false base stations based on UE positioning measurements; x. Solution #23: Cryptographic CRC to avoid MitM relay nodes; x; x; x. Solution #24: UE&Network-assisted UE avoidance and Network detection of FBS; x. Solution #25: Detection of Man-in-the-Middle false base stations; x. Solution #26: KI#2 with PKC-based and without tight time synchronization; x. Solution #27: Short-lived asymmetric key-based solution for protecting system information; x. One candidate solution of SIB security: Figure 6.7.2.1-1: System Information verification using Digital Signatures

On the other hand, as the security parameters (e.g. verification fields for integrity protection) could range from tens of bits to thousands of bits (which may/may not impact the 6G cell coverage) depending on the security solution(s) [11] selected by SA3, the SA3 also needs the RAN inputs regarding whether it is feasible to include the extra bits of security parameters in SIB. Given that the maximum number of bits in SIB1 or SI in 5G is 2976 bits, as the transmission of SIB1 or SI does not support RLC segmentation (i.e. only TM mode RLC is supported for SIB1 and SI [8].). RAN2 could also send some guidance/requirements to SA3, e.g. to minimize the number of security bits included in SIB1 or SI.

Observation 7: Without SI security protection, the UE may reselect to incorrect neighbouring cells or respond to inexistent PWS message due to attacks from a false base station, as captured in 3GPP 5G TR 33.809 and 3GPP 6G TR 22.870. As the addition of the security parameters ranging from tens of bits to thousands of bits may/may not impact the coverage of the 6G cell, SA3 needs to know whether the extra bits for SIB security can be supported by 6G RAN, and RAN can inform SA3 the transmission requirements of SIB.

Proposal 7: RAN2 studies whether it is feasible to support the SIB security in 6G, in alignment with SA3.

# Conclusion

For the 6G security, we consider that with the increasing use of more advanced tools (e.g. AI and quantum computer) by the attackers, the 6G cellular network can be more vulnerable to various attacks. In order to make the 6G security more robust and efficient, we think that a tight coordination between RAN and SA is needed, so that the RAN can use the SA3 inputs for the initial design (e.g. (MAC) PDU format) of 6G RAN, and SA3 can take the RAN inputs for the security design to avoid the security impacts on the RAN procedures (e.g. mobility).

5G AS security functions reused for 6G

Observation 1: In 5G, the integrity protection and cyphering for SRB and DRB at the PDCP layer reduces the number of key changes during handover. The count check procedure helps the gNB to detect maliciously inserted packets of the DRB.

Proposal 1: RAN2 confirms the following 5G security functions reused in 6G AS: 1) Count check procedure via RRC; 2) Integrity protection and cyphering for SRB and DRB at PDCP layer

Mobility impacts on security

Observation 2: The RAN2 stage-2 specification needs to clarify the security functions, including key derivation, security termination points and key generation for state transitions and mobility. The UE refreshes KgNB and resets L2 whenever a security key (i.e. KUPenc, KUPint, KRRCenc and KRRCint) changes, i.e. no partial key change in 5G. According to 3GPP TR 38.804 of 5G, RAN decided that security key refresh is not always performed during handover when the PDCP anchor point is not changed, and SA3 followed the RAN2 requirement for the security key refresh.

Proposal 2: RAN2 informs SA3 the following 5G mobility requirements reused for 6G security: 1) The security key change during mobility should be minimized to avoid the packet loss due to L2 reset; 2) Security key change is not always performed at handover when the PDCP anchor point is not changed.

DC architecture /mobility solution /RRC state impacts on security

Observation 3: In 5G, the support of different migration architectures / mobility procedures (e.g. SCPAC or LTM) / RRC states (e.g. RRC_INACTIVE) requires different SA3 standard efforts.

Proposal 3: RAN2 informs SA3 once the migration architecture(s) / mobility solution(s) /RRC state(s) are decided.

New 6G security enhancements in AS

Observation 4: Compared with 5G, the potential extra 6G RAN-related security impacts as currently being discussed in SA3 include: 1) MAC CE security; 2) SIB security; 3) Msg3/Msg5 security; 4) Finer granularity of the UP security, i.e. support UP key isolation for different CU-UP(s).

Proposal 4: The 6G CU/UP design should have sufficient flexibility to add the 6G RAN-related security enhancements.

MAC-I processing issue in PDCP

Observation 5: In 5G, the MAC-I generated per PDCP SDU causes extra signalling overheads and UE processing (i.e. impacting the UE’s transmission bit rate).

Proposal 5: More than one packets can be used to generate a single MAC-I.

MAC CE security

Observation 6: The unprotected MAC CE leads to more security attack scenarios and higher probability of handover failure / radio link failure / beam failure / UE privacy exposure. SA3 needs to know which/what MAC CE contents causing security issue are included in 6G.

Proposal 6: RAN2 informs the SA3 the MAC CE contents (e.g. security parameters; UE ID and User’s location-related information) causing security issue.

SIB security

Observation 7: Without SI security protection, the UE may reselect to incorrect neighbouring cells or respond to inexistent PWS message due to attacks from a false base station, as captured in 3GPP 5G TR 33.809 and 3GPP 6G TR 22.870. As the addition of the security parameters ranging from tens of bits to thousands of bits may/may not impact the coverage of the 6G cell, SA3 needs to know whether the extra bits for SIB security can be supported by 6G RAN, and RAN can inform SA3 the transmission requirements of SIB.

Proposal 7: RAN2 discusses whether it is feasible to support the SIB security in 6G, in alignment with SA3.

# Reference

[1] RP-251881, “New SID: Study on 6G Radio”

[2] SP-251132, “Study on Security for the 6G System”

[3] 3GPP TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”

[4] 3GPP TR 33.899, “Study on the security aspects of the next generation system”

[5] 3GPP TR 38.804, “Study on New Radio Access Technology; Radio Interface Protocol Aspects”

[6] 3GPP TS 33.501, “Security architecture and procedures for 5G system”

[7] 3GPP TS 37.340, “Multi-connectivity; Overall Description; Stage 2”

[8] 3GPP TS 38.331, “NR; Radio Resource Control (RRC) protocol specification”

[9] 3GPP TS 38.323, “NR; Packet Data Convergence Protocol (PDCP) specification”

[10] Meeting report of SA3#123.

[11] 3GPP TR 33.809, “Study on 5G security enhancements against False Base Stations (FBS)”

[12] 3GPP TR 22.870, “Study on 6G Use Cases and Service Requirements; Stage 1”

[13] S3-251124, “Reply LS on security handling for inter-CU LTM in non-DC cases”

[14] 3GPP TS 38.321, “NR; Medium Access Control (MAC) protocol specification”

[15] S3-252285, “Living document on NR mobility enhancement”, NR_Mob_Ph4_Sec

[16] 3GPP TS 24.501, “Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3”

[17] Yuanyuan Yang∗, Jin Cao∗, Ruhui Ma∗, Sheng Li∗, and Hui Li∗, ∗State Key Laboratory of Integrated Services Networks, School of Cyber Engineering, Xidian University, China, arXiv:2506.09502v2 [cs.CR] 12 Jun 2025, “The Security Overview and Analysis of 3GPP 5G MAC CE”