3GPP TSG-RAN WG2 Meeting #131-bis	R2-2506857

Prague, Czech Republic, 13th –17th October, 2025

Agenda Item:	10.3.2

Source:	Huawei, HiSilicon

Title:	Energy efficient and unified RRC state modelling

Document for: Discussion and Decision

1	Introduction

6GR study scope includes at least below:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. - Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In this contribution:

- We first analyse RRC state design lessons from 5G and how that could guide the 6G study, in section 2.1;

- Then, with the above justifications, we discuss the first target for 6G CONNECTED state, in section 2.2:

  - Both short latency and power saving are critical targets for 6G CONNECTED state, see 2.2.1;

  - How to achieve energy saving for both UE and NW sides in 6G CONNECTED state, see 2.2.2 and 2.2.3;

  - How to achieved short latency status switch within 6G CONNECTED state, see 2.2.4;

  - To summary, single 6G CONNECTED state may meet the targets sufficiently, see 2.2.5.

- Furthermore, we discussed the second target: simplified RRC states modelling, in section 2.3

  - The 6G IDLE state for the UE’s initial power-on phase is demonstrated, see 2.3.1;

  - And, the 6G CONNECTED state is suitable to accommodate more UEs, see 2.3.2;

  - Then, with such clean RRC modelling, the UE management and identification can be simplified, see 2.3.3.

  - In summary, with the above proposed 6G CONNECTED state and initial transient 6G IDLE state, we conclude the essential RRC state modelling for simplification, in section 2.3.4.

2	Discussion

2.1	5G lessons for 6G justifications

- Foundational question: Which RRC state is the BEST CHOICE for UE with no data, in terms of energy efficiency and latency performance?

In NR, three RRC states have been defined: RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED, among which RRC_INACTIVE is the additional state introduced compared with LTE. The intention for introducing three states is to balance the performance between UE power consumption and latency to access to the network. Figure 2.1-1 below shows the current RRC states clearly:

Figure 2.1-1 NR RRC state transition

2.1.1	Transition latency from Inactive to Connected is not short enough

In NR, when the UE resumes from RRC_INACTIVE to RRC_CONNECTED, the transition latency includes the random access latency and the RRC processing latency. Note that the RRCResumeRequest message is transmitted via the random access procedure (e.g., Msg3), so the RRC processing combined with the random access procedure. In order to have a clear analysis, in the following discussion, random access latency includes the Msg1 to Msg4 transmission and reception latency, while the RRC processing latency includes the latency related to the RRCResume message processing.

Figure 2.1.1-2 Resume procedure from RRC_INACTIVE to RRC_CONNECTED

Paging latency:

For MT traffic, the UE monitors paging message, once its paging is received, and initiates CBRA to access the network. To be able to receive paging, the UE is required to periodically monitor PDCCH scrambled by P-RNTI in paging occasions, and then decoding Paging message in the corresponding PDSCH. NR introduced a paging early indication (PEI) for UE power saving, where the typical delay value is around 50ms, including 20ms for transition time from Deep Sleep, 20ms for reception of 2 SSB bursts and PEI, and 10ms for monitoring of paging occasions.

Random access latency:

For 4-step CBRA procedure, the typical random access delay value is around 20~30 ms when there is no collision, which includes SSB detection (~10 ms) if the UE has no available SSB measurement results, RACH occasion waiting, RAR reception (~10 ms), and Msg3 and Msg4 transmission and reception (~10 ms). If collision happens, the random access delay value is longer, e.g., more than 30 ms. With the 2-step CBRA, although the delay is shorter than 4-step CBRA, collision may still happen which increases the random access delay.

RRC procedure latency:

In NR, RRC state transition comes with RRC procedures. For example, for the RRC resume procedure, the RRCResumeRequest message is included either in Msg3 (4-step RA) or in MsgA (2-step RA) of the CBRA procedure. If the contention resolution and UE context retrieval procedures are successful, the UE processes the RRCResume message sent from the gNB. The UE applies the configuration and resumes the RRC connection. According to the RRC procedure delay requirement in TS 38.331, this RRC resume processing delay is around 6 ms or 10 ms. In inter-gNB resume scenario, 10 ms should be added for the UE context retrieval procedure on the Xn interface.

To some extends, the NR separate RRC states constrains the even further latency reduction due to the key bottleneck of the RRC/L3-level procedure. If the real short-latency switch is the target, the lower layer switch is necessary to remove the constrain/limitation caused by RRC/L3 kind of procedure.

Even further latency before actual data transmission:

After the UE has entered RRC_CONNECTED, before the user can resume actual data transmission, the UE needs to perform L1 measurements, e.g., CSI acquisition, and to report the measurement results, which may involve a resource waiting time for the reporting. In another word, there is a slow-start time for the case when the UE enters RRC— a “last step latency”; it should be included in the overall transition latency when evaluating actual UE experience during the state transition procedure.

Inevitable latency even with SDT:

Upon applying SDT in inactive state, an overall latency may include the paging latency, CBRA latency (if it is RA-SDT, otherwise the CG occasion delay in CG-SDT), and RRC procedure latency, and thus SDT does not change significantly to reduce the state transition latency in order to transmit data.

Remark 1: The state transition from NR RRC INACTIVE to RRC CONNECTED includes a transition latency, which is primarily constrained by RRC-level procedure, paging reception and contention-based random access.

Based on above analysis, we observe that:

- There is still long state transition latency from NR RRC_INACTIVE to RRC_CONNECTED, which is not suitable for the traffic with some user experienced latency requirement. Hence, for latency-sensitive scenarios, the network may prefer to keep the UE in the NR RRC_CONNECTED state.

- NR RRC_INACTIVE may at most achieve the similar power-saving like NR RRC_IDLE for a short period of time. However, for power-saving-critical scenarios, the network may prefer to release the UE directly to the NR RRC_IDLE state for deeper sleep.

In addition, other practical issues are observed which may be the problem for easy use of NR RRC_INACTIVE:

- It impacts the core network, which requires an upgrade of CN (e.g., operations related to paging);

- There is still some network complexity to maintain the UE context and RANU management, compared to NR RRC_IDLE.

Eventually, it becomes somehow clear why NR RRC_INACTIVE is not that widely commercialized. In summary, we have following observations about 5G inactive state, which will motivate the 6G re-design for RRC state modelling.

Observation 1:	The 5G inactive state (including SDT) is nearly not commercialized. Inactive state may be not the best choice for UE in no-data status, since it is not as power saving as idle and still faces state transition latency.

As a result, it is better to use singe RRC connected state to satisfy both energy efficiency and short latency, to be addressed in 2.2.

2.1.2	Power saving features are not simple enough

Various UE power saving features for NR RRC_IDLE/INACTIVE state and NR RRC_CONNECTED state were introduced in different 5G releases, as listed in Table 2.1.2-1.

Table. 2.1.2-1. UE power saving features in 5G

|  | 5G Non-CONNECTED state | 5G CONNECTED state |
| --- | --- | --- |
| Rel-15 | - I-DRX | - C-DRX - Overheating UAI - BWP adjustment |
| Rel-16 | - RRM Measurement relaxation | - PDCCH-based WUS (DCP) - UE power saving UAI - SCell dormancy |
| Rel-17 | - Paging early indication (PEI) - Assistance TRS - Inactive SDT | - RLM/BFD measurement relaxation - PDCCH skipping/SSSG switching |
| Rel-18/19 | - LP-WUS (LP-WUS based paging with RRM offloading/relaxation) | - LP-WUS (LP-WUS triggered PDCCH monitoring) |

However, the commercialization of these UE power saving features in 5G is limited. One reason is that most of the features are not 5G Day 1 features, and they were gradually introduced in subsequent releases. Thus it takes time to deploy them in commercial networks, which may require careful evaluation on a risk of backward compatibility issues. Besides, due to the enhancements through different releases, some of the functions are duplicated, such as PEI and LP-WUS for paging, DCP and LP-WUS for PDCCH monitoring.

These power saving features also involve different RRC states, which makes it difficult for operators to manage RRC states and effectively achieve UE power saving. Therefore, one unified/common UE power saving feature may help improve this situation in 6G.

In Rel-19, the LP-WUS for RRC_IDLE and RRC_INACTIVE was introduced, in order to minimize the paging monitoring power. In the Rel-19 procedure, the UE first monitors the LP-WUS, then monitors paging DCI and receives paging message on the main radio if its LP-WUS is detected. This multiple-steps procedure still consumes quite amount of UE power. In addition, the LP-WUS and paging DCI are not UE dedicated message, which means some UEs are waken up even if they are not the target UEs of the paging, this may lead to unnecessary power consumption.

Moreover, the data transmission operations are also intertwined with UE power-saving operations (e.g., the C-DRX and data transmission in RRC_CONNECTED, I-DRX and SDT in RRC_INACTIVE). A more streamlined design, however, should prioritize UE power saving during no-data periods, while finishing data transmission promptly to revert to power-saving mode afterward.

Observation 2:	5G UE power saving features have limited commercialization, possibly because:

The diversity of UE power saving features brings complexity;

Several features have the redundant benefits in the same RRC state;

Several similar features are defined for the redundant benefits in different RRC states.

Hence, the unified/consolidated UE power saving design can be considered at the beginning of 6G; for example, some schemes are provided in section 2.2.2. In addition, the UE power saving design and data transmission design may be decoupled as two status/modes, that is described below in section 2.2.4.

2.2	Comprehensive 6G CONNECTED: both short-latency and power saving

2.2.1	Requirement and target for 6G CONNECTED

In NR, RRC_INACTIVE state intends for a UE to transition to it from RRC CONNECTED state when there is no data transmission for some time, but if not and keeping the UE always in RRC CONNECTED will consume extra or additional power. To enhance this situation, a target for 6GR RRC state design should be to able to achieve UE power saving when there are no data while the UE is able to begin data transmission with a short delay.

Regardless the RRC state modelling assumptions, the UE’s operations can be considered at least two statuses or scenarios: when a UE has no data to be transmitted and when the UE has occasional data transmission. From user experience perspective, it would be desired to gain more power saving while no data are transmitted, meanwhile it may still be able to perform fast switch to data transmission as needed from a type of a sleep condition. Compared to 5G NR design, one fundamental target for 6G would be to achieve both short latency from “no-data” with a power saving mode to “data transmission” status and effective power saving while no-data being for a while, rather than a trade-off between the two scenarios.

One straightforward idea would be to support both power saving and short-latency data transmission in 6G CONNECTED state.

Proposal 1:	A key requirement for RRC state design is short-latency switching from “no-data” to “data transmission” status/operations, at the same time the UEs in no-data status can still achieve extreme power saving.

The following sub-sections investigate the feasibility to fulfil the above requirement by 6G CONNECTED state.

2.2.2	Unified/consolidated UE power saving

For 6G CONNECTED state, the basic idea for UE power saving in the time domain is to reduce the unnecessary DL monitoring such as PDCCH monitoring, and keep the UE in “low-power/sleep mode” as much as possible. The ideal behaviour is that the UE performs the UL/DL transmission when data arrives, and enters “low-power/sleep mode” once the data transmission completes. This is also the motivation for an introduction of C-DRX, DCP, PDCCH skipping and LP-WUS in 5G, and (LP-)WUS is the one efficient technique for UE power saving in the time domain. For example, the (LP-)WUS scheme without necessary to start an OnDuration timer with Connected DRX mode may achieve better performance, i.e., for both UE/network power saving (when there is no data for transmission) and reduced data transmission latency, compared with C-DRX, DCP and PDCCH skipping.

Figure. 2.2.2-1. 5G UE power saving features for “No-Data” and “Data transmission”

In one sense, all 5G existing UE power saving features in inactive state and connected state can be consolidated as single common power saving feature.

As discussed above, the UE may achieve maximum power saving when it is able to use low-power receiver for its wake-up and RRM measurement, the cycled and DCI based wake-up can be replaced. Namely, PEI, DCP, I-DRX and C-DRX are somehow merged into this WUS mechanism (based on either low-power receiver or main-radio), as a single design. Moreover, if the group-based WUS from the NR design can become UE dedicated WUS in connected state, the UE power consumption can be even lower, by avoiding or reducing the false-alarm wake-up.

Proposal 2:	In 6G CONNECTED state, RAN2 studies the unified UE power saving while in no-data status, by consolidating various candidate UE power saving features (from different 5G RRC states) into one common design.

2.2.3	Network energy saving aspect

C-DRX with fixed On-Off time pattern is introduced as basic UE power saving feature in 5G Day 1. However, as the service becomes increasingly diverse, the C-DRX pattern may not match the UE traffic perfectly. That’s why DCP, PDCCH skipping and LP-WUS, aiming for different time period granularity, are further introduced to make the On-Off time pattern more dynamic. On the network side, cell DTX/DRX operation for network energy saving is configured in a semi-static manner. If considering some improvement, it can be studied to use a dynamic way to inform the UE that the cell has been awakened for urgent communications or services. That means a flexible operation for the network.

Remark 2:	It is worthy to study easier network operation to configure a proper parameter (e.g., cycle) for random traffic pattern for energy saving purpose.

On the other hand, if the data transmission for the network is very frequent, it harms the energy saving for the network. In 5G, there was limited consideration of energy saving taking into account both the network and the UE, so these objectives tend to contradict each other. To improve this, 6G design should aim to balance efficient data transmission and energy efficiency jointly for the network and the UE. For example, for non-real-time service, the data do not need to be transmitted immediately when they arrive. It is beneficial to aggregate DL/UL data in one-shot transmission via dynamic wake-up, to maximize the network and UE sleep duration. At the same time, the user experience will not be impacted as long as the transmission delay is not larger than user’s QoS delay budget.

In the 6G study, it can be considered whether the dynamic wake-up for transmission outperforms cycle-based periodic wake-up, in terms of UE power saving and network energy saving.

Observation 3:	Lacking joint consideration of network energy saving and UE power saving may result in that UE and network energy saving most often conflict with each other.

Proposal 3:	In 6G CONNECTED state, RAN2 studies flexible/dynamic manner for the network to aggregate the traffic/data across UEs, for a better balance of efficient data transmission and energy efficiency for the UE and network.

2.2.4	Fast switch within 6G CONNECTED

In the connected state, the UE does not perform data transmission all the time. During some period of time, the UE has no data to transmit or receive, which can be called “no-data” status. In this no-data status, the UE can reduce power consumption, e.g., by monitoring only necessary wake-up signals and relaxed RRM measurement. Once the data transmission begins, the UE should switch to the “data-transmission” status with very short delay.

Figure 2.2.4-1 Fast switch between modes within 6G CONNECTED

There could be some initial descriptions about those two statuses (or two modes) in 6G CONNECTED.

- “no-data low-power mode”: including RRM measurement, wake-up signal monitor (e.g., for data arrival), cell switch as needed, etc. Also, it is better if the UE can use the low-power-radio when there is no data for power saving purpose.

- “data transmission mode”: including occasional data transmission on MR (via various data rate and suitable BWP bandwidth), handover procedure, measurement report, network reconfiguration, UL sync (e.g., preamble) and DL sync, etc.

As UE can enjoy power saving in above “no-data low-power mode”, as clarified in section 2.2.2. Then, it is necessary to achieve short-latency for switching from this mode to the “data-transmission mode”.

Corresponding to the analysis in section 2.1.1, reducing the paging latency, random access latency and RRC processing latency are the key aspects to enable fast switch.

Reducing paging latency:

- For MT traffic, one-step paging/wake-up is a promising direction, in which the UE monitors a UE dedicated WUS signal (e.g., a UE dedicated sequence). This UE dedicated WUS signal replaces the NR group based WUS + Paging DCI + Paging PDSCH procedure, which can achieve faster mode switch. Therefore, UE dedicated wake-up resource can enable direct "paging" in connected state, without any paging message reception.

Reducing random access latency:

- One way to reduce random access latency is to use CFRA during the mode switch procedure. In order to support CFRA for most UEs, 6GR should consider a larger preamble sequence pool, so that most UEs can be allocated a dedicated preamble. Therefore, UE dedicated RACH resource can enable fast access.

Reducing RRC procedure latency:

- During the mode switch procedure, if both the UE and the network maintain the UE context, the UE continues to use the previous configuration and starts to transmit data. In this procedure, RRC involvement can be avoided or minimized, to reduce the RRC processing latency, as long as the UE can be identified via lower layer procedure.

Fast switch during cell switch:

- If there are ongoing data transmission in the connected state, the handover procedure can be used, where 6GR will further reduce the interruption during handover procedure.

- When cell switch happens in the connected state while UE is in no-data status, one way is to perform a short latency for transition to the "data transmission" status for the cell switching. To keep low power consumption in this case during the mobility, more studies may be required, e.g., how the WUS or preamble validation is performed in a candidate cell for faster cell switching, how the UE maintains the TA, and how the network manages the UE context?

- In addition to the intra-cell fast mode switch, inter-cell fast mode switch can also be considered. When the UE dedicated sequence pool becomes large enough, dedicated preamble/WUS can be provided for multiple cells, which means even the UE moves to another cell in “no-data mode”, the new cell can wake up the UE by the WUS sequence and determine the UE identity by the preamble, which achieves fast mode switch to “data-transmission mode”. However, comparing with the intra-cell case, one key aspect we need to consider in the inter-cell case is that how to manage the UE context and how to re-configure some necessary updates (e.g., security related parameters) to the UE. RAN2 can further study this inter-cell case.

Therefore, in order to support the fast wake-up, fast random access and fast cell switch as mentioned above, it is worth to study fast UE identification. For example, if network can identify a particular UE by its dedicated preamble for UL traffic arrival and if the UE can be identified by a dedicated WUS for DL traffic arrival, this switch notification can be really fast. Of course, this requires larger the sequence pool for WUS and preamble.

Remark 3:	Lower-layer switch procedure and fast UE identification via UE dedicated sequence/time/frequency resource can enable the fast switch from no-data low-power mode to UL/DL data transmission mode.

In summary, a possible way to fundamentally reduce the "no-data" to "data transmission" latency will be using lower layer procedure and UE dedicated resource to early identify the UE. Therefore, it is proposed:

Proposal 4:	In 6G CONNECTED state, RAN2 studies the fast UE identification (e.g., configuring UE dedicated ID/sequences from a larger pool of UE ID/sequences) and lower-layer transition procedure, to enable the short-latency switching to UL/DL data transmission status.

2.2.5	Summary of 6G CONNECTED

Based on the discussion from section 2.2.2 to 2.2.4, the proposed single 6G CONNECTED state can achieve short-latency switching from “no-data” to “data transmission” status. In addition, it enables UE power saving when UE is in no-data status.

This single 6G CONNECTED state meets the 6G RRC design target as described in section 2.2.1 by considering the 5G lessons as mentioned in section 2.1.1.

Therefore, we believe this is one important direction for RAN2 to study: to achieve both short latency and power saving, by incorporating two modes (i.e., "no-data low-power" mode and "data transmission" mode) within a single 6G CONNECTED state, rather than relying on two separate RRC states. Namely that one RRC connected state can cover the functions and benefits of legacy separate RRC_INACTIVE and RRC_CONNECTED. The simplification benefits are further discussed in section 2.3 on top of this 6G CONNECTED design.

Proposal 5:	RAN2 aims to achieve both short latency and power saving, by incorporating two modes (i.e., "no-data low-power" mode and "data transmission" mode) within a single 6G CONNECTED state, rather than relying on two separate RRC states.

2.3	Simplified RRC state modelling for UE management

2.3.1	Initial transient 6G IDLE state

According to the analysis in section 2.2, the UE can achieve fast switch from “no-data mode” to “data-transmission mode” if we introduce the UE dedicated WUS sequence and UE dedicated CFRA preamble sequence. However, the UE can be allocated these UE dedicated sequences only after initial access. For example, when the UE is initially powered on, the UE does not have those UE dedicated sequences, so we still need to use the CBRA-based RRC connection setup procedure to move to connected state. In addition, after the successful initial access, even though the UE is configured with a dedicated preamble sequence, the network may determine that the UE will not have data service for a long time. In this case, the network has the flexibility to release the UE into IDLE state and re-allocate the sequence to other UEs.

Comparing with the NR RRC_IDLE state, the 6G IDLE state can use additional UE power saving technologies to further reduce power consumption, e.g., using low lower radio to measure the neighbour cell RS for the cell selection/re-selection procedures when suitable.

Hence, during the 6GR study, it is understood that the idle state is still needed, targeting the case when UE is in initial power-on phase.

Observation 4:	6G IDLE state is still needed for the UE’s initial power-on phase.

2.3.2	Enable more UEs in 6G CONNECTED state

The 6G CONNECTED state as described in section 2.2 can satisfy both the requirements of energy efficiency and short latency for data transmission, without any trade-off or contradiction between these two requirements. Consequently, the network can keep the UE in connected state, if both power saving and user experience are important.

Thus, if network resources allow it, the network can keep more UEs in CONNECTED state. Even in terms of the occupied network resources, it is possible that the UEs in "low-power no-data" mode require much less resource as the one in “data transmission” mode.

For the same amount of network resources (network capacity), if it is supposed to support N UEs in the NR RRC_CONNECTED state, the network can support {m*(N-L)+ L} UEs in the proposed 6G connected state, where m UEs in "low-power no-data mode" require similar amount of resources as one UE in “data transmission” mode, and there are L UEs in "data transmission" mode for a period of time.

For maximumly high-load scenarios, the network can still release UEs to the 6G IDLE state, specifically those UEs with latency-insensitive services.

In any case, by using the 6G IDLE state as a fallback and leveraging the 6G CONNECTED state to accommodate more UEs, the number of UEs accessing the network will be higher compared to the NR RRC state design.

Remark 4:	Given that 6G CONNECTED state can achieve both energy efficiency and short latency performance, 6GR design can/should target more UEs staying in 6G CONNECTED state.

Proposal 6:	6GR design should maximize UEs in 6G CONNECTED state, rather than releasing them to 6G IDLE state.

2.3.3	Simplify UE management and identification

According to the above analysis, the UE may stay in 6G CONNECTED state for longer time and more UEs will be in that state. Since it is connected state, both the UE and the network maintain the UE context, so there is no need to re-allocate UE ID (e.g., C-RNTI) when the UE switches from no-data mode to data transmission mode, which simplifies the UE management procedure. In addition, the UE dedicated WUS resource and UE dedicated preamble help to achieve fast UE identification by physical layer sequence detection method. This is more than fast, while it is also simplifying the UE identification. Namely that this is one-step UE identification, rather than WUS followed by paging for DL and preamble followed by contention resolution for UL.

If the WUS and preamble can be valid in neighbour cells, it can also achieve the usage of UE ID for UE context management/retrieval at the network side (e.g., similar to I-RNTI).

Therefore, the single connected state can avoid: RNTI reallocation, paging after WUS, contention resolution after preamble, separate ID for UE context management. This significantly simplifies the UE management for the network.

2.3.4	Summary of RRC state modelling

In 5G, certain control plane features such as system information acquisition, access control, paging and cell (re)selection, are almost common between the NR RRC_IDLE and RRC_INACTIVE. Therefore, when RAN2 investigates these control plane functionalities for 6GR, it should be sufficient to only consider connected state and idle state [1].

In addition, if the latency and power saving requirements can be achieved by the proposed 6G CONNECTED state, it is not necessary to support any other RRC state (like 5G RRC_INACTIVE), which can help avoiding redundant features.

Furthermore, from the perspective of UE management complexity at the network side, e.g., context, resources and state transition management, the two RRC states reduces those procedure, where only 2 state transitions are possible. If another RRC state would be introduced, the number of possible transitions would be increased such as e.g., the 5 state transition cases in the 5G design [2].

Observation 5:	Simple RRC state modelling simplifies the network operation for UE management. 6GR should avoid introducing unnecessary RRC state, i.e., each additional RRC state should be first well justified.

Based on the analyses in previous sections, we propose to consider only a 6G CONNECTED state and a 6G IDLE state.

Proposal 7:	6GR design should simplify RRC state modelling: starting from or focusing on essential RRC states, i.e., 6G CONNECTED state and 6G IDLE state.

# 3	Conclusion

This contribution makes the following proposals:

Lessons and 6G justifications

Observation 1:	The 5G inactive state (including SDT) is nearly not commercialized. Inactive state may be not the best choice for UE in no-data status, since it is not as power saving as idle and still faces state transition latency.

Observation 2:	5G UE power saving features have limited commercialization, possibly because:

The diversity of UE power saving features brings complexity;

Several features have the redundant benefits in the same RRC state;

Several similar features are defined for the redundant benefits in different RRC states.

Observation 3:	Lacking joint consideration of network energy saving and UE power saving may result in that UE and network energy saving most often conflict with each other.

Target 1: 6G CONNECTED: both short-latency and power saving

- Target for 6G CONNECTED

Proposal 1:	A key requirement for RRC state design is short-latency switching from “no-data” to “data transmission” status/operations, at the same time the UEs in no-data status can still achieve extreme power saving.

- Consolidated UE power saving

Proposal 2:	In 6G CONNECTED state, RAN2 studies the unified UE power saving while in no-data status, by consolidating various candidate UE power saving features (from different 5G RRC states) into one common design.

- Network energy saving aspect

Proposal 3:	In 6G CONNECTED state, RAN2 studies flexible/dynamic manner for the network to aggregate the traffic/data across UEs, for a better balance of efficient data transmission and energy efficiency for the UE and network.

- Fast switch aspect within 6G CONNECTED

Proposal 4:	In 6G CONNECTED state, RAN2 studies the fast UE identification (e.g., configuring UE dedicated ID/sequences from a larger pool of UE ID/sequences) and lower-layer transition procedure, to enable the short-latency switching to UL/DL data transmission status.

- Summary of 6G CONNECTED

Proposal 5:	RAN2 aims to achieve both short latency and power saving, by incorporating two modes (i.e., "no-data low-power" mode and "data transmission" mode) within a single 6G CONNECTED state, rather than relying on two separate RRC states.

Target 2: Simplified RRC states modelling

Observation 4:	6G IDLE state is still needed for the UE’s initial power-on phase.

Proposal 6:	6GR design should maximize UEs in 6G CONNECTED state, rather than releasing them to 6G IDLE state.

Observation 5:	Simple RRC state modelling simplifies the network operation for UE management. 6GR should avoid introducing unnecessary RRC state, i.e., each additional RRC state should be first well justified.

Proposal 7:	6GR design should simplify RRC state modelling: starting from or focusing on essential RRC states, i.e., 6G CONNECTED state and 6G IDLE state.

# 4	References

- R2-2506856 Discussion on 6G control plane, Huawei, HiSilicon, 3GPP RAN2 meeting #131bis

- RP-252138 Consideration on UE management, Huawei, HiSilicon, 3GPP TSG RAN Meeting #109