3GPP TSG-RAN WG2 Meeting #131bis	R2-2506875

Prague, Czech Republic, 13-17 Oct 2025

Agenda Item:	9.2.2

Source:	NEC

Title:	Discussion on Topology 2 with intermediate UE as Reader

Document for:	Discussion, Decision

# Introduction

This paper discusses the issues and solutions for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader.

# Discussion

## Architecture Assumption

Rel-20 Ambient IoT Phase 2 WID aims to specify a single architecture by down-selection between RRC-based architecture and UP-based architecture, in coordination with SA2. SA/SA2 concluded to support only RRC-based architecture. According to the conclusion of TR23.700-13, the protocol stack studied by SA2 for Topology 2 is depicted by Figure-1 as below.

Figure-1: Protocol Stack for RRC based architecture

With the RRC based architecture, the messages between the UE Reader and the AIoTF are delivered using RRC between UE and NG-RAN node, and NGAP between gNB and AMF, and using an SBI interface between AMF and AIoTF.

Proposal-1: RRC is used to convey the messages between the UE Reader and the AIoT Functionality for AIoT device over Uu interface.

## Applicability of Rel-19 Topology -1 solution to Rel-20 Toplogy-2

In Rel-19, the AS protocol stack for A-IoT radio interface between the AIoT device and the Base Station AIoT reader contains A-IoT MAC layer and A-IoT physical layer as shown below in Figure-2. The AS layer control information and data are handled by A-IoT MAC layer and A-IoT physical layer. For A-IoT radio interface, there is no differentiation between the control plane and the user plane.

In Rel-20, Ambient IoT radio interface is between the AIoT device and an Intermediate UE AIoT reader.

Figure-2: AS protocol stack for A-IoT

Proposal-2: Rel-19 AS protocol stack over Ambient IoT radio interface applies to Rel-20 Ambient IoT radio interface between the AIoT device and an Intermediate UE AIoT reader.

In addition, we assume that the functionalities of specified Rel-19 A-IoT MAC layer and A-IoT physical layer can be taken as the baseline for Rel-20 AIoT Topology-2.

## UE Reader Selection

To support Topology-2, an intermediate UE should be selected as the AIoT UE reader before the establishment of Topology-2 communication architecture. A normal Rel-20 UE cannot easily act as AIoT UE reader due to its lack of support of Ambient IoT radio interface. We assume a UE reader should be normal plus its additional capability of AIoT reader function.

From the UE capabilities report perspective, the UE should report its support and preference to act as an AIoT UE reader to the network. Different readers may support different inventory frequencies, and the inventory frequency supported by the reader may be different from the inventory frequency supported by the AIoT device as well. To execute the inventory related procedure between the device and the reader, the inventory frequency used by the device and the reader should match with each other.

Proposal-3: UE reader capability e.g. inventory frequencies should be considered during UE reader selection.

Other than the UE reader capability, the first discussion is which type of UE reader, RRC connected state or RRC Inactive/Idle, can be chosen as the UE reader, although the WID clearly states that only the UE in RRC connected state can act UE reader. If the RRC Inactive/Idle UE can be chosen as the UE reader, a paging procedure may be specified to wake up the UE to allow it to enter RRC connected state.

Proposal-4: RAN2 to confirm if RRC Inactive/Idle UE can be chosen as the UE reader, or only RRC connected state can be chosen as the UE reader.

In addition to UE reader capability, the UE should subscribe the special AIoT reader operation with the CN before acting as a real UE reader. The CN may provide its authorization based on the UE reader’s subscription before the UE reader can be chosen. An interesting discussion can be which node gNB or CN make a final decision on the UE reader. This discussion may be also dependent on which type of UE can be chosen.

As the AIoT communication involves both R2D transmission and D2R reception from UE reader perspective, it is interesting to discuss if the transmitting UE reader can be decoupled from the receiving UE reader. If the transmitting and receiving can be decoupled, two separate UE Readers, one for transmitting R2D signal and the other for receiving D2R signal should be supported for Topology-2.

Proposal-5: RAN2 to discuss if transmitting UE reader and receiving UE reader can be different UE readers for AIoT device in case of Topology-2.

## Data forwarding over Uu for AIoT Device

A distinguishing issue for Topology -2 is how to forward UL/DL data by the Intermediate UE (as the reader) in case of data/message transmission. A full signalling based data transmission path is assumed for the whole data transmission path. From the Intermediate UE perspective, it should be in RRC connected mode to perform such data forwarding. During UL Data forwarding, the Intermediate UE can use a specific Uu RRC message (carried by SRB) to forward the A-IoT device’s UL data received at its A-IoT MAC layer from Ambient IoT air interface. For DL data, the Intermediate UE can receive the A-IoT device’s data via a specific Uu RRC message (carried by SRB) from the Base Station (i.e., NG-RAN node), and then deliver such data to at its A-IoT MAC layer for further transmission over Ambient IoT air interface.

In practice, one UE reader may server multiple AIoT devices for example for the purpose of inventory. This means the UE may need forward traffic for different AIoT devices very often, which may be not very efficient.

One possibility is that the intermediate UE may assemble all of the data segments for the same data payload for the Ambient IoT device in case there is data segmentation for D2R traffic for the AIoT device. In addition,  the intermediate UE may receive multiple data payload coming from different Ambient IoT device. The intermediate UE may aggregate all or part of the data payloads into one RRC message to be sent to NG RAN node.

Proposal-6: RAN2 to discuss if data aggregation for same Ambient IoT device is supported at the UE reader in case there is data segmentation for D2R traffic for the AIoT device.

Proposal-7: RAN2 to discuss if data aggregation for different Ambient IoT device is supported at the UE reader.

When the UE reader receives the D2R traffic from the Ambient IoT device, there is no identity information that can be resolved by the UE reader, since the AIoT device ID is included in the upper layer data. However the UE reader can distinguish the AIoT devices via the AS ID assigned during access procedure. It would be interesting to discuss if the UE reader needs to identify the D2R traffic to the NG RAN node when it forwards the traffic over Uu. Otherwise the NG RAN node does not know which traffic it belongs to. For the similar reason, there may be a need for the identification for R2D transmission also.

In addition, for command service in Rel-19, the BS reader (i.e., NG-RAN node) may assign the NGAP ID for the follow-up command. When we consider the UE reader, the similar question is whether and how to identify the A-IoT device or the D2R traffic of the A-IoT device over the Uu interface.

Proposal-8: RAN2 to discuss if the identification of the D2R/R2D message over Uu interface is needed during data forwarding towards gNB by UE reader.

Proposal-9: RAN2 to discuss which node (either NG-RAN node or UE reader) should assign the identity if the identification of the D2R/R2D message over Uu interface is needed for data forwarding.

## Resource allocation to UE Reader

In the Work Item description, it is said that the AIoT radio resource allocation to the UE Reader shall be via Uu RRC signaling. In addition, the A-IoT radio resource of the UE reader is only valid within the serving cell. At the start of the WID, RAN2 needs to discuss the framework for AIoT radio resource allocation.

The assumption here is the NG RAN node (i.e., Base Station) is in full control of the resources used for the communication between the AIoT devices and the intermediate UE reader. The resource to be allocated for the intermediate UE reader may include the resources for both AIoT access and data transmission for AIoT devices after their successful access. It can be subject to semi-static resource allocation and dynamic resource allocation.

The resources for random access can be seen as a shared resource pool and may be shared by multiple AIoT devices that may access the intermediate UE as the reader. These resources may be mapped to the Access Occasions that the intermediate UE as the reader will announce in its R2D AIoT paging message.

Proposal-10: RAN2 to assume the resource allocated for AIoT access be shared by multiple AIoT devices that may access the UE reader.

The resources for AIoT device data transmission can be dedicated resource for each AIoT device. This means the NG RAN node need to allocate dedicated resource for each AIoT device. Alternatively, these resources can be another shared resource pool assigned by the NG RAN node, which may be shared by multiple AIoT devices. Then the intermediate UE reader may allocate a specific resource to a specific AIoT device based on the resources within the shared resource pool for AIoT data transmission when the AIoT device successfully access the reader.

Proposal-11: RAN2 to discuss if the resource allocated for AIoT data transmission is based on a dedicated resource for each AIoT device or a shared resource pool for multiple AIoT devices that successfully accessed the UE reader.

In case of Contention free access for AIoT devices, the abovementioned two resource pools may be combined into a single resource pool, since the AIoT device can generate the data transmission in its very first message to the UE reader.

# Conclusion and Proposal

We have the following proposals:

Proposal-1: RRC is used to convey the messages between the UE Reader and the AIoT Functionality for AIoT device over Uu interface.

Proposal-2: Rel-19 AS protocol stack over Ambient IoT radio interface applies to Rel-20 Ambient IoT radio interface between the AIoT device and an Intermediate UE AIoT reader.

Proposal-3: UE reader capability e.g. inventory frequencies should be considered during UE reader selection.

Proposal-4: RAN2 to discuss if RRC Inactive/Idle UE can be chosen as the UE reader, or only RRC connected state can be chosen as the UE reader.

Proposal-5: RAN2 to discuss if transmitting UE reader and receiving UE reader can be different UE readers for AIoT device in case of Topology-2.

Proposal-6: RAN2 to discuss if data aggregation for same Ambient IoT device is supported at the UE reader in case there is data segmentation for D2R traffic for the AIoT device.

Proposal-7: RAN2 to discuss if data aggregation for different Ambient IoT device is supported at the UE reader

Proposal-8: RAN2 to discuss if the identification of the D2R/R2D message over Uu interface is needed during data forwarding towards gNB by UE reader

Proposal-9: RAN2 to discuss which node (either NG-RAN node or UE reader) should assign the identity if the identification of the D2R/R2D message over Uu interface is needed for data forwarding.

Proposal-10: RAN2 to assume the resource allocated for AIoT access be shared by multiple AIoT devices that may access the UE reader.

Proposal-11: RAN2 to discuss if the resource allocated for AIoT data transmission is based on a dedicated resource for each AIoT device or a shared resource pool for multiple AIoT devices that successfully accessed the UE reader.

# Reference

[1] RP-251885 Rel-20 Ambient IoT Phase 2 WID

[2] TR23.700-13 Study on Architecture support of Ambient power-enabled Internet of Things