3GPP TSG RAN WG2#131bis	R2-2506908

Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda Item:	9.7.2

Source:	CMCC

Title:	Discussion on UP solution vs. CP solution for voice support over NB-IoT-NTN

Document for:	Discussion, Discussion

Introduction

A new R20 work item Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4 is agreed in RAN #108 to support IMS voice call over GSO for NB-IoT-NTN.

The aim of this work item is to specify the enhancement of NB-IoT-NTN to support IMS voice call over GSO with the following objectives: - Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2] - Coordination with SA is required - The objective can be updated with further details based on the outcome of the above down-selection - Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1] - Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN [RAN2] - Support of necessary modifications for emergency call for voice over NB-IoT-NTN [RAN2] - Study and if feasible, specify UE transmit power higher than PC1 (e.g. up to 37dBm) for NB-IoT-NTN [RAN4] - Note: The enhancements to support voice over NB-IoT-NTN via GSO in this work item may be also applicable to NGSO cases without additional specification enhancements - Note: Coordination with SA4 is expected

Based on the WID scope, to support voice over NB-IoT-NTN, we need to down select between CP and UP solutions firstly. In this contribution, we would like to provide some analysis to help down select between CP and UP solutions for support IMS voice call over GSO for NB-IoT-NTN.

Discussion

In the SA4 LS [2], there is a question to RAN2 as following:

# To SA2 and RAN2: ACTION: SA4 kindly asks SA2 and RAN2 to comment on - the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option, and - specifically, whether a packet overhead of 1 byte of MAC header is realistic.

From RAN2 perspective, the first bullet is related to SA2, and RAN2 believes SA2 would provide corresponding feedback. Then for second bullet, a packet overhead of 1 byte of MAC header may be not realistic due to that 1 byte MAC subheader is only used for the last subheader in the MAC PDU and subheaders for fixed sized MAC control elements as mentioned in the TS 36.321.

Observation 1: A packet overhead of 1 byte of MAC header may be not realistic due to that 1 byte MAC subheader is only used for the last subheader in the MAC PDU and subheaders for fixed sized MAC control elements as mentioned in the TS 36.321.

And in SA2 #170 meeting, they sent an LS [3] also including a series of questions, where the following questions are for RAN2:

Question 1 (To RAN2): Does RAN2 have any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC? Question 3 (To RAN2): Can RAN2 confirm whether scheduling methods for support of IMS voice over NB-IoT NTN can handle the voice packets of different sizes changing dynamically? Question 6 (To RAN2): Is it feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20? Question 9 (To RAN2): Can RAN2 provide feedback on whether it is possible to define a new SRB that will be used to carry voice media? Question 10 (To RAN2): If the answer to the above question is yes, can RAN2 confirm whether such SRB(s) are defined, whether it is technically feasible to support and configure RLC UM for such SRB(s) for example: in NB-IoT deployments over GEO satellite, when SRBs are used to carry voice media?

Regarding the question in the SA2 LS, we could consider to provide the following response:

For Q1, it may be difficult for RAN2 to provide observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC.

For Q3, from our perspective, RAN2 could discuss solutions to handle the voice packets of different sizes changing dynamically, for example, combination SPS mechanism and dynamically scheduling.

For Q6, it is feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20, however, the existing UP solution could already maximum 2 DRBs based on UE capability, which may be sufficient to support IMS voice over NB-IoT NTN.

For Q9 it may be not possible to define such a new SRB, which would bring significant specification impact. Then there is no reply to Q10.

Proposal 1: For the response to SA4, RAN2 could provide the following reply based on the above discussion:

To SA4: A packet overhead of 1 byte of MAC header is not realistic.

To SA2:

Reply to Q3: RAN2 could discuss solutions to handle the voice packets of different sizes changing dynamically.

Reply to Q6: It is feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20, however, existing maximum 2 DRBs may be sufficient to support IMS voice over NB-IoT NTN.

Reply to Q9: It is not possible to define a new SRB used to carry voice media.

And in the following we provide detail analysis for whether CP solution or UP solution is better from our perspective.

Per the SA2 discussion progress, they have already output TR 23700-19, which includes UP based solutions, CP based solutions and CP/UP combination solutions for support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC. And they will determine a final solution before the normative work to align with RAN scope. For coordination, some analysis and discussion work are also needed from RAN perspective.

As in the existing specification, for Control Plane CIoT EPS optimisation, only SRB1bis could be used to transmit NAS message. To support voice over CP solution, maybe new RB is needed to be introduced to carry voice packets and RB handling behaviour needs to be further discussed subsequently. Further different from SRB1 and DRB, SRB1bis has no PDCP entity, which would also lead to security issue unexpected.

Then for User Plane CIoT EPS optimisation, the maximum 2 DRBs is supported based on UE capability. If adopt UP solution, legacy transmission solution based on DRB could be reused, and there is no security issue either.

Besides, based on the description of rlc-Config in TS 36.331[5]: RLC AM is the only applicable RLC mode for SRB1 and SRB2, CP based solutions have to use RLC AM mode for IMS voice over GSO for NB-IoT-NTN even for the new SRB. However, due to the huge RTT delay of IoT NTN GEO scenario (around 500~600ms), RLC AM mode will have severe impact to user experience due to ARQ and Status Report mechanisms. Otherwise, CP based solutions need to consider applying RLC UM mode for the new RB which may need further discussion in RAN2.

Figure 1 Protocol stack for CP solution                     Figure 2 Protocol stack for UP solution

Observation 2: For CP solution, new RB is needed to be introduced to carry voice packets and RB handling behaviour needs to be further discussed subsequently.

Observation 3: For CP solution, there may be security issue due to no PDCP entity.

Observation 4: For CP solution, there may be RLC mode issue since AM mode is the only applicable RLC mode for SRBs.

And considering the packet size of voice service is fixed and the arrival interval meets certain rules of real-time services. Per the WID scope, SPS for voice packets transmission could be considered. And the existing UP solution already supports SPS for BSR, while CP solution does not support SPS entirely due to no RRC reconfiguration message. Therefore to support SPS for voice service, enhancements could be discussed based on legacy solution for UP solution while new mechanism needs to be considered for CP solution.

And we provide an example enhancement based on legacy SPS for UP solution, to resolve possible silence period during voice service, if the occurrence frequency of silence period is not so high, eNB could suspend the SPS resources for the UE, and schedule the subsequent silence period by dynamic scheduling, which could help to avoid the unnecessary occupation of radio resources during the SPS cycle, and once silent period return to the normal voice packets transmission/reception, continue to use the SPS resource.

It could be seen that UP solution has smaller specification impact compared to CP solution as mentioned above. Therefore, it is better to support UP solution for voice call over GEO.

Observation 5: To support SPS for voice service, enhancements could be discussed based legacy solution for UP solution while new mechanism needs to be considered for CP solution.

Proposal 2: It is proposed to select UP solution to support IMS voice call over GSO.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on IMS voice call over GSO for NB-IoT-NTN and have the following observations and proposals:

Observation 1: A packet overhead of 1 byte of MAC header may be not realistic due to that 1 byte MAC subheader is only used for the last subheader in the MAC PDU and subheaders for fixed sized MAC control elements as mentioned in the TS 36.321.

Proposal 1: For the response to SA4, RAN2 could provide the following reply based on the above discussion:

To SA4: A packet overhead of 1 byte of MAC header is not realistic.

To SA2:

Reply to Q3: RAN2 could discuss solutions to handle the voice packets of different sizes changing dynamically.

Reply to Q6: It is feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20, however, existing maximum 2 DRBs may be sufficient to support IMS voice over NB-IoT NTN.

Reply to Q9: It is not possible to define a new SRB used to carry voice media.

Observation 2: For CP solution, new RB is needed to be introduced to carry voice packets and RB handling behaviour needs to be further discussed subsequently.

Observation 3: For CP solution, there may be security issue due to no PDCP entity.

Observation 4: For CP solution, there may be RLC mode issue since AM mode is the only applicable RLC mode for SRBs.

Observation 5: To support SPS for voice service, enhancements could be discussed based legacy solution for UP solution while new mechanism needs to be considered for CP solution.

Proposal 2: It is proposed to select UP solution to support IMS voice call over GSO.

Reference

- RP-251867 New WI: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4

- S4-251584 LS on the RAN simulation assumptions for ULBC

- S2-2507636 LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC

- 3GPP TR 23.700-19 V1.0.0 Study on Integration of satellite components in the 5G architecture; Phase 4 (Release 20)

- 3GPP TS 36.331 V18.0.0 Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification (Release 18)