3GPP TSG-RAN2#131bis	Tdoc R2-2507072

Prague, Czech Republic, 2025-10-13 - 2025-10-17

Agenda Item:	10.3.2

Source:	Ericsson

Title:	Controlling the 6G access stratum

Document for:	Discussion, Decision

# 1	Introduction

In this document we present our view on the control plane functions of the 6G Radio Interface. We analyse problems observed in previous generations and suggest high level principles to avoid those in 6GR.

# 2	Discussion

## 2.1	Paging

Improved network energy efficiency is an important ambition with the 6G design. An important means to achieve this is to further reduce the mandatory (always on) DL signal transmissions. It is also important to do this in a way that allows base station hardware to benefit from deep sleep. As transition to and from deep sleep state requires some time, sufficiently long periods with no DL transmissions (or UL monitoring) at all are needed to efficiently leverage the deep sleep energy saving potential.

Improved network energy efficiency is an important ambition in 6G and enabling efficient leveraging of base station deep sleep is an important means to achieve this.

SSBs constitute a DL signal that cannot be avoided (but its intervals may be increased, and proposals exist to increase it to 160 ms). The length of DL transmission-free periods is thus ultimately limited by the interval between two SS bursts.

Sufficiently long periods without DL transmissions and UL monitoring are needed to efficiently leverage the base station deep sleep energy saving potential, and such periods are ultimately limited by the intervals between SS burst transmissions.

To allow as long DL transmission-free periods as possible, other tasks preventing deep sleep should to the extent possible be gathered close to the SS burst transmissions. Among other network tasks, this applies also to paging.

Further decreasing of DL transmission-free periods can be avoided by gathering network tasks close to the SS bursts.

To this end, network energy saving should be facilitated e.g. by allowing the network to collect paging occasions close after to the SS bursts instead of distributing them uniformly over time. This principle was introduced in release 19 Network Energy Saving (NES), but it was achieved by tweaking the paging occasion algorithm that was designed based on the principle of uniform distribution.

The 6G paging mechanism should be designed to improve network energy efficiency.

## 2.2	System information

System information in NR is divided into two parts, minimum system information and other system information. The former consists of the MIB and the SIB1 and the latter consists of the other SIBs, which are broadcasted in SI messages. The SIB1 includes information for the UE to camp in a cell, to perform access to the cell (including Unified Access Control information) and information for the UE to acquire the other system information in the cell, i.e. the SI scheduling information.

With all this information in SIB1 the message can however become very large. This leads to issues with detection performance and resource utilization, which should be avoided in 6GR. The resource utilization is especially a problem when there is a need for beam sweeping in many directions and with multiple repetitions. It can then be considered that not all the information in SIB1 may need to be provided with the same periodicity, e.g. the SI scheduling information may not need to be provided as often as the information for camping and accessing in the cell.

Large SIB1 size is a problem especially when there is a need for beam-sweeping in many directions and multiple repetitions

In NR, the broadcast of the other system information (SIB2 and above) is done in different SI messages which are transmitted in SI windows with the same size. The SI windows are continuous and in each SI window only a single SI message (i.e. with the same content) can be transmitted. This leads to that the broadcast of system information in NR needs to be spread over time.

As discussed in the previous section, an important aspect of 6G is to support an improved network energy efficiency. To achieve this, it is important to allow a base station to have long periods without any transmissions. In 6GR we should therefore also have a scheduling of system information that supports that all the system information can be transmitted within a limited time frame, thus allowing long periods without any transmissions in between. This is not possible with the 5G SI scheduling mechanism where the SI message transmissions need to be spread out. By transmitting the system information within a limited time frame, the SI acquisition time for the UE can also be reduced and thereby possibly also the corresponding UE energy consumption.

The scheduling of system information in 6GR should also be flexible to avoid that the broadcast needs to be performed at certain occasions, which might not be suitable from a resource point of view. The SI scheduling mechanism in NR has restrictions, which prevents the network from broadcasting SI messages when it is most suitable from a radio resource point of view. In some cases, there are periods where no SI message at all can be transmitted. An enhancement for the latter was introduced in Rel-17 with support of a configured (instead of implicit) SI window position for an SI message. This added flexibility should be assumed as a baseline for 6GR.

To enable an improved network energy efficiency and to allow a better resource utilization, RAN2 should study how to do SI scheduling in 6GR in an energy and resource efficient way.

The system information scheduling mechanism in NR prevents network energy savings and an efficient resource utilization for broadcast of system information.

The 6GR scheduling of system information should be done in a more energy and resource efficient way than in NR.

## 2.3	RRC states and connection management

NR defined three RRC states: RRC_IDLE, RRC_CONNECTED, and RRC_INACTIVE. The first two states were inherited from LTE. RRC_INACTIVE was a new state introduced in NR with similar power consumption as RRC_IDLE but with reduced signalling and access delay when transitioning to RRC_CONNECTED.

This RRC state model has worked well in NR and can be kept in 6GR. We see no strong reason to merge RRC_IDLE and RRC_INACTIVE as the states have different UE behaviour and there will be a need to distinguish between them in procedure text (e.g. RRC_IDLE UEs don’t have a stored UE context). We also don’t think RRC_INACTIVE can be replaced with RRC_CONNECTED as the UE power consumption will likely continue to be higher in RRC_CONNECTED in practice. Additionally, the gNB is more limited in the number of UEs it can keep in RRC_CONNECTED state.

6GR can use the same RRC states as 5G (RRC_IDLE, RRC_INACTIVE, RRC_CONNECTED).

In NR, RRC_INACTIVE has seen limited uptake mainly due to the slow migration to NR SA and lack of UE support for the RRC_INACTIVE state. Among operators that have deployed NR SA, there is strong interest in RRC_INACTIVE, and the feature is starting to be commercially deployed. For 6GR, we should aim for RRC_INACTIVE to be deployed from day 1 and replace RRC_IDLE as the main state where the UE spends most of its time whenever it does not have ongoing data transmission.

However, there are a few shortcomings with RRC_INACTIVE that should be addressed in 6GR:

- Complex RNA configuration: In NR, the RAN Notification Area (RNA) is used for location tracking and paging of UEs in RRC_INACTIVE and can be configured in three ways: as a cell list, as a list of RAN areas, and as a list of tracking areas. We should revisit if all these configuration options are really needed and how the RNA configuration can be simplified for the operator.

- Requirement on Xn connectivity: In NR, a gNB needs to have Xn connectivity to all gNBs within and neighboring the RNA to be able to page and retrieve the UE context for a UE in RRC_INACTIVE. For RNAs spanning more than one gNB, setting up all these Xn connections is difficult and the gNB may not always be dimensioned to handle such many simultaneous connections. Also, Xn link breaks cannot be completely avoided and needs to be handled. For 6G, we should relax the Xn connectivity requirement by allowing paging and context fetching to be relayed via the CN when Xn is not available. This would be similar to how NG-based handover is used as fallback when Xn-handover is not available. For paging another alternative could be to let the CN be responsible for paging similar as in RRC_IDLE, i.e. there would be a common CN paging mechanism for both RRC_IDLE and RRC_INACTIVE UEs. This latter alternative is in line with the SA 6G SI objective of avoiding “duplication of functionality in RAN and CN” (see SP-250806); however, its impact on performance needs to be studied.

- Too short I-RNTI: In NR, the I-RNTI is used to identify the last serving gNB and the UE context therein. However, the gNB ID part of the I-RNTI is too short to uniquely identify the last serving gNB and hence careful planning of the gNB IDs is needed to avoid/minimize conflicts. For 6GR, we should extend the I-RNTI and instead use the global gNB ID (same identity as is broadcasted in SIB1) as gNB identifier. As this may require the size of Msg3 in the random access procedure to be increased, this aspect needs to be discussed in the context of the study for random access and requires collaboration with RAN1.

Study methods to address shortcomings and improve RRC_INACTIVE, such as simplified RNA configuration, relaxed requirement on Xn connectivity, and improved I-RNTI format.

## 2.4	Random access

The core principle of the random-access procedure in 6GR is that it should support all 6G UEs from the very beginning. In NR, issues emerged when lower-end devices (e.g., RedCap) introduced after the baseline UE in the first release (Rel-15) required additional mechanisms such as MSG1 indication. This revealed the need for improvements to better accommodate diverse UEs. In 6GR, the aim is to ensure the random-access procedure works seamlessly for all UEs and features from day one.

6GR should support a common random-access procedure for all UEs (including lowest-complexity UEs).

2-step RA has been introduced in NR, and it can reduce 1 RTT compared to 4-step RA. It has been argued that it is useful to reduce delays.

- For large cells, there is a need to configure guard period/band, e.g., MSGA-PUSCH must have gaps to cater unknown TA. In addition, there is a configurable offset between preamble and MSGA-PUSCH, and it takes value of 1 slot to 32 slots; there must be resources reserved for MSGA-PUSCH. These lead to capacity loss and gNB receiver complexity and 2-step RA has therefore not been implemented nor deployed to date.

- Some argue 2-step RA may be useful in small cells which do not need TA adjustment and so there is no capacity loss due to the need to configure guard period, but then it is not clear why PRACH, which was designed to derive timing advance, is needed in MSGA.

It was argued that 2-step RA is useful for operation in the unlicensed spectrum, in which when UE tries to access the spectrum, there is a chance of collision and thus delay. However, it is not clear yet whether unlicensed spectrum operation (which has not seen any business success) would be supported from day 1. The other use case is NTN, in which the propagation delay of one Round Trip is large. For NTN, the UE would perform TA pre-compensation and so there is no need either of MSGA-PRACH to derive the timing.

To summarize, 2-step RA in NR suffers from spectrum inefficiency for large cells with a need of TA adjustment. For small cells without TA adjustment, it is unclear if MSGA-PRACH is needed. On the other hand, it is clear 4-step RA should be the baseline random access type in 6GR.

4-step RA can be the baseline for 6GR study, while further discussion on the use case of 2-step RA is needed.

In NR, both CBRA and CFRA are supported. CBRA is used, among other things, for initial access when the UE is first powered up. CFRA is used in cases where the network knows that the UE would perform random access right after or within a short time after receiving the configuration, e.g., PDCCH order or handover command. Since a reserved preamble is used for the UE, CFRA would not result in preamble collision, and thus give a short random-access delay. In addition, the preamble will not be locked/reserved for a too long time and leading to resource waste. Thus, we believe both types should be considered in the study.

Both CBRA and CFRA should be considered for 6GR study.

In NR, a UE can perform early indication in MSG1, and it is done by partitioning both in preamble sequences and the time/frequency resources used for random access. It is possible to indicate the following to the network:

- Which DL beams the UE has detected and have a good quality,

- Which beam is suitable for beam failure recovery (BFR),

- SI request,

- Random access is associated with a particular network slice,

- A need of MSG1 repetition,

- UE initiating 2-step RA,

- Redcap UE needing a special MSG2 scheduling,

- Msg3 size/repetition related:

  - preamble group A/B for different msg3 size,

  - coverage enhancement for msg3,

  - small data transmission for large msg3 size

- CFRA preamble for, e.g., PDCCH order, handover

In theory, the UE may indicate in MSG1 any combination of the above, but that would bring issues for PRACH capacity since the indication is done by partitioning the resources. Thus, in Rel-17, to reduce the number of partitions, Rel-17 RACH Indication and Partitioning allows a set of RACH resources to be configured for a feature combination, including one or more above features.

With a new generation and the benefit of hindsight regarding which features have been implemented, deployed, and found useful in practice, 3GPP RAN2 should first instead question the need and the benefit to have a MSG1 indication. For example, SI request is not latency critical and does not need an indication in MSG1; very few occurrences of BFR are observed in real deployment and, even in those occurrences, there are no benefits to indicate in MSG1; 2-step RA has not been implemented nor deployed; there are multiple indications related with MSG3 size that seem to be related to each other (e.g., Type A/B and SDT) and there is a chance to clean them up and unify the indication in 6GR.

With that said, it should be studied which features or UE capabilities would benefit from indication in MSG1 for 6GR. At least, information for MSG2 scheduling and contents to include in the MSG2 (e.g., MSG3 size, MSG3 repetition) seem to be useful. One example is the coverage level for MSG2/MSG3, and another example is whether a UE supports more advanced capabilities than the lowest end device so that network can have a different MSG2 and MSG3 scheduling strategy for these devices to improve spectrum efficiency. RAN2 can further study the need to indicate in these cases.

The Msg1 indication in NR became unnecessarily complicated by adding extensions over time.

In NR, Msg3 can be scheduled with different sizes in the Msg2 scheduling, as RAR contains an UL grant. But the CCCH size design in NR took the worst-case cell edge assumption, and was decided to be 56 bits and 72 bits in order to support the targeted coverage level in the first release of NR. It turned out to be insufficient to uniquely identify the UE context, see Proposal 4. It is worthwhile to emphasize that the MSG3 repetition introduced in the coverage enhancement WI in Rel-17 can be applied in 6GR to support a larger MSG3 TBS than it was in NR, assuming the same coverage target. Thus, we don’t see any hurdles to support a larger MSG3 size, assuming coverage enhancements techniques are part of 6GR day 1 features. What RAN2 needs to discuss is the benefit of a larger MSG3 size, how large the size is, and what needs to be transmitted in MSG3. One clear example is the long I-RNTI value, and RAN2 can further discuss other information that is beneficial to be included in MSG3.

Large MSG3 size in the cell edge is beneficial from RAN2 point of view.

With the above discussions, we propose that:

Study a common random-access procedure for all UEs (including lowest-complexity UEs), with a baseline assumption of 4-step RA, CBRA/CFRA, a simplified and generic Msg1 indication framework, and a larger MSG3 size in the cell edge.

## 2.5 	Spectrum aggregation

Cellular deployments typically consist of multiple carriers over several frequency bands, ranging from sub-GHz FDD carriers to midband TDD carriers and in some cases also mmW FR2 carriers. Spectrum aggregation is a key feature to ensure users get access to the required amount bandwidth to ensure good end user performance. From operator perspective, spectrum aggregation is a key tool to ensure user load is efficiently distributed among the deployed carriers in the system.

NR supports two forms of spectrum aggregation: Carrier Aggregation (CA) and Dual Connectivity (DC). DC was envisioned to support deployments with relaxed backhaul latencies and as a tool for 4G to 5G migration. However, DC has several weaknesses, difficult to tune flow control, poor coverage due to uplink power split, complexity affecting both implementation and specifications, market fragmentation, delayed 5GCN launch, etc. Therefore, and in line with the 6G SID, we support a single framework for spectrum aggregation in 6GR, based on enhanced carrier aggregation.

Among others, we see the following areas of improvement for carrier aggregation in 6GR:

- Avoid scheduling dependencies between carriers and strict DL/UL scheduling timelines. The strict time lines in NR between PDSCH and PUCCH and the Downlink Assignment Index (DAI) linking the UL and DL scheduling across carriers became very complex with the introduction of carrier aggregation with multiple numerologies, and combined with advanced MIMO schemes and different UEs supporting different ACK/NACK timing it resulted in complex restrictions as to when HARQ feedback is scheduled in UL, leading to reduced scheduling flexibility and reduced performance. In 6GR, solutions for asynchronous feedback should be studied to overcome this. See section 2.5 of [1].

- Relaxed requirements on timing alignment between carriers to support flexible deployments and possibilities for aggregation of non-collocated carriers, e.g. where carriers served by non-collocated radio units (RU) are aggregated by carrier aggregation in a single baseband unit (BBU).

- Reduced setup latency for adding additional carriers. Traffic is often bursty and a challenge in 5G has been to setup and activate additional SCells in time before the buffer is emptied to ensure good user performance and effective load balancing between deployed carriers. Several enhancements were introduced during later 5G releases, like SCell activation/deactivation, early measurements, dormant BWP, etc., but none has completely solved the problem. UE power savings in idle/inactive state often limits the possibilities for early measurements and a challenge for 6GR will be to find solutions for fast setup of additional carriers that is acceptable from UE power consumption point of view in idle/inactive state.

- Flexible combining of UL/DL carriers to support increased coverage of high band carriers, which are often limited by the uplink. The best carrier for the downlink may not be the best carrier for the uplink. Being able to flexibly combine a high carrier downlink with a low carrier uplink would allow increasing the coverage of high band carriers.

- Improved resiliency. In 5G CA, the PCell became a single point of failure, as RLF declared on the PCell caused the UE to trigger RRC re-establishment, regardless of the status of SCells. In 6GR, resilience can be improved by letting the UE declare RLF only when radio link problem is observed on all configured component carriers.

Study a single framework for spectrum aggregation based on carrier aggregation, enhanced with fast setup of additional carriers, flexible combining of UL/DL carriers and improved resiliency.

# 3	Conclusion

In this paper we have reached these observations:

Observation 1	Improved network energy efficiency is an important ambition in 6G and enabling efficient leveraging of base station deep sleep is an important means to achieve this.

Observation 2	Sufficiently long periods without DL transmissions and UL monitoring are needed to efficiently leverage the base station deep sleep energy saving potential, and such periods are ultimately limited by the intervals between SS burst transmissions.

Observation 3	Further decreasing of DL transmission-free periods can be avoided by gathering network tasks close to the SS bursts.

Observation 4	Large SIB1 size is a problem especially when there is a need for beam-sweeping in many directions and multiple repetitions

Observation 5	The system information scheduling mechanism in NR prevents network energy savings and an efficient resource utilization for broadcast of system information.

Observation 6	6GR should support a common random-access procedure for all UEs (including lowest-complexity UEs).

Observation 7	4-step RA can be the baseline for 6GR study, while further discussion on the use case of 2-step RA is needed.

Observation 8	Both CBRA and CFRA should be considered for 6GR study.

Observation 9	The Msg1 indication in NR became unnecessarily complicated by adding extensions over time.

Observation 10	Large MSG3 size in the cell edge is beneficial from RAN2 point of view.

We propose:

Proposal 1	The 6G paging mechanism should be designed to improve network energy efficiency.

Proposal 2	The 6GR scheduling of system information should be done in a more energy and resource efficient way than in NR.

Proposal 3	6GR can use the same RRC states as 5G (RRC_IDLE, RRC_INACTIVE, RRC_CONNECTED).

Proposal 4	Study methods to address shortcomings and improve RRC_INACTIVE, such as simplified RNA configuration, relaxed requirement on Xn connectivity, and improved I-RNTI format.

Proposal 5	Study a common random-access procedure for all UEs (including lowest-complexity UEs), with a baseline assumption of 4-step RA, CBRA/CFRA, a simplified and generic Msg1 indication framework, and a larger MSG3 size in the cell edge.

Proposal 6	Study a single framework for spectrum aggregation based on carrier aggregation, enhanced with fast setup of additional carriers, flexible combining of UL/DL carriers and improved resiliency.

# 3	References

R2-2507071, “User plane: Let's keep it simple!”, Ericsson