3GPP TSG-RAN WG2 Meeting #131bis	                               R2-2507094

Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda Item:	8.6.3

Source: 	OPPO

Title:  	MAC open issues for R19 mobility

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the remaining MAC open issues for R19 mobility.

2	Discussion

2.1 MAC-O01

In MAC spec section 5.4.4, the following procedure is captured:

As long as at least one SR is pending, the MAC entity shall for each pending SR: 1> if the MAC entity has no valid PUCCH resource configured for the pending SR; and 1> if there is no ongoing RACH-less LTM cell switch; and 1> if rach-LessHO is not configured: 2> initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel the pending SR.

In legacy, an SR is considered as pending once it is triggered. If there is valid PUCCH resource configured for the pending the SR, the UE will trigger SR with the associated PUCCH resources, otherwise, the UE will initiate RACH.

In 5.x.4, the highlighted part is not needed as legacy SR procedure already specified the UE behaviour on RACH triggering.

2> else:

3>	if the dedicated SR configuration for L1 measurement report MAC CE transmission is configured:

4>	trigger the SR using the dedicated SR configuration for L1 measurement report;

3>	else:

4>	initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel the pending SR;

3>	stop the periodical reporting timer for this ltm-CSI-ReportConfigId, if running.

To avoid the duplication, we propose to change is as:

2> else:

3>	if the dedicated SR configuration for L1 measurement report MAC CE transmission is configured:

34>	trigger the SR using the dedicated SR configuration for L1 measurement report;

3>	else:

4>	initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel the pending SR;

3>	stop the periodical reporting timer for this ltm-CSI-ReportConfigId, if running.

- RAN2 to adopt the Text Proposal provided in Annex.

2.2 MAC-O02

In earlier meeting, it has agreed that NW can configure a dedicated SR configuration for MR MAC CE transmission. And the legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

There are still open issues related to SR handling which need to be further studied. In current spec, there are some notes to clarify the SR/PUCCH resource handling for the cases that PUCCH resources are overlapped among the triggered SRs.

- Case 1: non-BFR SR overlapping

Except for SR for SCell beam failure recovery, the selection of which valid PUCCH resource for SR to signal SR on when the MAC entity has more than one overlapping valid PUCCH resource for the SR transmission occasion is left to UE implementation.

- Case 2: BFR SR& non-BFR SR

When the MAC entity has pending SR for SCell beam failure recovery and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery of a BFD-RS set) overlapping with PUCCH resource for SCell beam failure recovery for the SR transmission occasion, the MAC entity considers only the PUCCH resource for SCell beam failure recovery as valid. When the MAC entity has pending SR for beam failure recovery of a BFD-RS set of Serving Cell and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery) overlapping with PUCCH resource for beam failure recovery of that BFD-RS set for the SR transmission occasion, the MAC entity considers only the PUCCH resource for beam failure recovery of that BFD-RS set as valid.

- Case 3 Scell BFR SR & BFD-RS set BFR SR

When the MAC entity has PUCCH resource for pending SR for SCell beam failure recovery overlapping with PUCCH resource for pending SR for beam failure recovery of a BFD-RS set for the SR transmission occasion, it is up to UE implementation to select PUCCH resource for SCell beam failure recovery or PUCCH resource for beam failure recovery of a BFD-RS set.

For L1 event triggered measurement report, there may be similar issue. When the MAC entity has PUCCH resource for pending SR for L1 event triggered MR and the MAC entity has one or more PUCCH resources overlapping with PUCCH resource for “other triggered SR”, which PUCCH resource is considered as valid.

In general, there are three ways:

- Option 1: left to UE implementation.

- Option2: MAC entity considers only the PUCCH resource for L1 event triggered MR MAC CE as valid.

- Option3: if “other triggered SR” is for BFR, the MAC entity considers only the PUCCH resource for beam failure recovery as valid.

- When the MAC entity has PUCCH resource for pending SR for event triggered L1 MR and the MAC entity has one or more PUCCH resources overlapping with PUCCH resource for “other triggered SR”, UE selects PUCCH resource based on:

- Option 1: left to UE implementation.

- Option2: MAC entity considers only the PUCCH resource for L1 event triggered MR MAC CE as valid.

- Option3: if “other triggered SR” is for BFR, the MAC entity considers only the PUCCH resource for beam failure recovery as valid.

2.3 MAC-O03

In last meeting, RAN2 agreed to stop the CLTM TAT timer if the CLTM candidate is released. One more issue to be further studied is how to handle CLTM TAT if the CLTM execution condition is released but candidate configuration is kept, i.e., NW reconfigures a CLTM candidate cell to an LTM candidate cell.

According to current running CR, if NW triggers a LTM cell switch execution to a CLTM candidate, UE will start PTAG TAT timer regardless the running CLTM TAT. That is, upon release of the CLTM execution condition, the running CLTM TAT does not impact the NW triggered CLTM execution. Thus we prefer to stop the corresponding CLTM TAT to save UE consumption.

Upon release of the CLTM execution condition, the UE stops CLTM TAT if running even if the corresponding LTM candidate configuration is kept.

3	Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1	RAN2 to adopt the Text Proposal provided in Annex.

Proposal 2	When the MAC entity has PUCCH resource for pending SR for event triggered L1 MR and the MAC entity has one or more PUCCH resources overlapping with PUCCH resource for “other triggered SR”, UE selects PUCCH resource based on:

-	Option 1: left to UE implementation.

-	Option2: MAC entity considers only the PUCCH resource for L1 event triggered MR MAC CE as valid.

-	Option3: if “other triggered SR” is for BFR, the MAC entity considers only the PUCCH resource for beam failure recovery as valid.

Proposal 3	Upon release of the CLTM execution condition, the UE stops CLTM TAT if running even if the corresponding LTM candidate configuration is kept.

4	Annex

Text proposal

- 5.x.4	Measurement report

For the event triggered L1 measurement reporting, for each ltm-CSI-ReportConfigId included in the LTM-CSI-ReportConfig, the MAC entity shall:

1>	if at least one L1 measurement report has been triggered as specified in 5.x.3 and not cancelled:

2>	if UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization:

3>	instruct the Multiplexing and Assembly procedure to generate the L1 measurement report MAC CE associated with the ltm-CSI-ReportConfigId as befined in clause 6.1.3.x according to the measurement report information in the MR_LIST;

3> if reportAmount is configured in the LTM-EventTriggeredPeriodicReport by RRC:

4> if at least one L1 measurement report associated with the ltm-CSI-ReportConfigId has been triggered due to the reason other than the expiry of the periodical reporting timer and the L1 measurement report is not cancelled:

5>	set the MR_SENT_COUNTER to 0 for this ltm-CSI-ReportConfigId;

4> increment the MR_SENT_COUNTER associated with the ltm-CSI-ReportConfigId by 1;

4> stop the periodical reporting timer, if running;

4> if reportAmount is configured in RRC, and the MR_SENT_COUNTER as defined for this ltm-CSI-ReportConfigId is less than reportAmount:

5> restart the periodical reporting timer with the value of reportInterval for this ltm-CSI-ReportConfigId as defined within the corresponding LTM-CSI-reportConfig;

3> include the SSBRI or CRI in the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId, if any, into the BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId;

3> clear the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId;

3> clear the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId;

3>	if the BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId is empty:

4>	remove the measurement reporting entry within the MR_LIST for this ltm-CSI-ReportConfigId;

4>	stop the periodical reporting timer for this ltm-CSI-ReportConfigId, if running;

3> cancel the triggered L1 measurement report;

2>	else if the UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the Truncated L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization:

3>	instruct the Multiplexing and Assembly procedure to generate the Truncated L1 measurement report MAC CE associated with the ltm-CSI-ReportConfigId as defined in clause 6.1.3.x according to the measurement report information in the MR_LIST by selecting the RS(s) based on a decreasing order of the priority for the type of beam: RS(s) in BEAM_ENTERING_LIST, RS(s) in BEAM_LEAVING_LIST, RS(s) in BEAM_REPORTED_LIST, and other RS(s) not in these three lists;

2> else:

3>

trigger the SR for L1 measurement report;

3>	stop the periodical reporting timer for this ltm-CSI-ReportConfigId, if running.