# 3GPP TSG RAN WG2 Meeting #131b      	                           R2-2507116
Prague, Czech Republic, Oct. 13 – 17, 2025

Agenda item:	8.5.3

Source:	Apple

Title:	User plane open issues on Rel-19 NES

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

By RAN2#131 [1], RAN2 has completed Rel-19 NES. In this contribution, we discuss the following user plane open issues on Rel-19 NES:

- Issue 1: RA-RNTI for PRACH adaptation (described in LS R1-2506587)

- Issue 2: whether "initialDownlinkBWP-RedCap" can be configured before SIB1 request procedure (e.g. acquired by UE earlier) and can be used for PDCCH monitoring. If so, can the network know that for a redcap UE performing an OD-SIB1 request.

# 2 Discussion

## 2.1 Issue 1: RA-RNTI for PRACH adaptation (described in LS R1-2506587)

The key contents of LS from RAN1 (R1-2506587) can be found as follows:

RAN1 thanks RAN2 for providing the LS on RA-RNTI for PRACH adaptation. From RAN1 perspective, there is no proper place in RAN1 specifications to capture the impact on f_id since f_id is not defined in RAN1 specifications. The index nRA in RAN1 specification is an index for physical signal generation and cannot be directly used for determination of f_id. RAN1 kindly requests RAN2 to take the above into account, as well as the below RAN1 agreements, with regards to the impact on f_id of valid RO in TS38.321: Agreement For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - When an additional RO is overlapped with legacy valid RO in both time and frequency domain, the additional RO is invalid before the SSB-RO mapping - Note: the overlapped RO for legacy resource is not impacted - FFS: Clarification on configuration of legacy ROs Agreement - Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs) - FFS: When there is no configuration of Msg1-FDM - Separate configuration of number of SSB per RO is supported 2. Actions: RAN1 kindly requests RAN2 to take the above into account for further work.

In RAN2#130 [2], the following agreements were made:

No need to extend the RA-RNTI space for additional PRACH occasions, i.e. no RAN2 impact for RA-RNTI calculation. RAN2 assumes that f_id for the additional PRACH resources is reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs.

RAN2 understand there is no MAC impact.

Send an LS to RAN1 to inform RAN2 agreement.

The definition of RA-RNTI is defined in TS38.321 [3], which is copied as below, where f_id is the index of the PRACH occasion in the frequency domain which has a value range between 0 and 7.

TS 38.321 Section 5.1.3 Random Access Preamble transmission The RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted or the RA-RNTI associated with the last valid PRACH occasion in the set of PRACH occasions (as specified in TS 38.213 [6]) for Msg1 repetition, is computed as: RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 [8] for μ = {0, 1, 2, 3}, and for μ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier).

We understand that the RAN2#130 agreement was made with the assumption that RAN1 can capture the agreement of reinterpretation of f_id in RAN1 specification. However, since RAN1 conclude that there is no proper place in RAN1 specifications to capture the impact on f_id, we think there are two WFs for RAN2:

- WF-1: Follow RAN2#130 agreement that no MAC spec changes on RA-RNTI calculation

- WF-2: Add description of f_id for additional PRACH resources in TS 38.321, e.g., “for PRACH occasion configured by RACH-ConfigSIB1, f_id is the index of the PRACH occasion in the frequency domain plus the number of PRACH occasion(s) configured by RACH-ConfigCommon (0 ≤ f_id < 8)”.

We suggest RAN2 to discuss which WF to adopt:

Proposal 1: On f_id of the additional PRACH resources for RACH adaptation, RAN2 down-select between the following two WFs:

- WF-1: Follow RAN2#130 agreement to not pursue MAC spec changes on RA-RNTI calculation.

- WF-2: Add description of f_id of additional PRACH resources in TS 38.321, e.g., “for PRACH occasion configured by addlRACH-Config-Adapt, f_id is the index of the PRACH occasion in the frequency domain plus the number of PRACH occasion(s) configured by RACH-ConfigCommon (0 ≤ f_id < 8)”.

## 2.2 Issue 2: whether "initialDownlinkBWP-RedCap" can be configured before SIB1 request procedure (e.g. acquired by UE earlier) and can be used for PDCCH monitoring. If so, can the network know that for a redcap UE performing an OD-SIB1 request.

The issue can be described as follows:

| OPPO | For the issue touched by Sharp001 and ERI005: 1> if initialDownlinkBWP-RedCap is configured: 2> if the Random Access procedure was initiated for SI request or SIB1 request (as specified in TS 38.331 [5]) and the Random Access Resources for SI request have been explicitly provided by RRC, and if the selected carrier is SUL carrier: 3> monitor the PDCCH on the BWP configured by initialDownlinkBWP. 2> else: 3> monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap. 1> else: 2> monitor the PDCCH on the BWP configured by initialDownlinkBWP. There seem some unclear point to support OD-SIB1 request / response via Redcap-specific BWP: 1) Currently there is no Redcap-specific OD-SIB1 request configuration specified in SIBx, is this paragragh to hint there might be OD-SIB1 response may rely on PDCCH configured in Redcap-specific initial DL BWP? 2) If so, i.e., when initialDownlinkBWP-RedCap is configured (after SIB1 acquisition), UE is supposed to rely on PDCCH configured on initialDownlinkBWP-RedCap. Then from network perspective, upon reception of a SIB1 request, it has no idea whether it is from a UE has acquired SIB1, or a UE has not. And then it is hard to decide which PDCCH to use for response, the one for initialDownlinkBWP or the one for initialDownlinkBWP-RedCap. |
| --- | --- |

First, the existing procedure text in TS 38.321 was agreed in P3 of R2-2208785 [4] in RAN2#119e [5]. Its reasonings can be summarized as follows:

- If initialDownlinkBWP-RedCap is configured and CBRA is triggered, Redcap UE in IDLE/INACTIVE state monitors RAR in initialDownlinkBWP-RedCap, irrespective of whether sending Msg1 in initialUplinkBWP-RedCap or initialUplinkBWP.

- If Redcap UE in IDLE/INACTIVE state triggers Msg1-based SI request in NUL, it monitors RAR in initialDownlinkBWP-RedCap because NW can identify RedCap UE if it transmits Msg1 in initialUplinkBWP-RedCap via dedicated resource si-RequestConfigRedCap-r17.

- However, if Redcap UE in IDLE/INACTIVE state triggers Msg1-based SI request in SUL, it monitors RAR in initialDownlinkBWP because NW can’t identify whether the UE is the RedCap UE (since there is no separated configuration for Msg1-based SI request for the RedCap UE).

Observation 1: In legacy, if initialDownlinkBWP-RedCap is configured and CBRA is triggered, Redcap UE in IDLE/INACTIVE state monitors RAR in initialDownlinkBWP-RedCap, irrespective of whether sending Msg1 in initialUplinkBWP-RedCap or initialUplinkBWP.

Observation 2: In legacy, if Redcap UE in IDLE/INACTIVE state triggers Msg1-based SI request in SUL, it monitors RAR in initialDownlinkBWP because NW can’t identify whether the UE is the RedCap UE (since there is no separated configuration for Msg1-based SI request for the RedCap UE).

Then, let’s get back to issue 2 of OD-SIB1 for RedCap UE. We think that the following two cases need to be discussed:

- Case 1: if initialDownlinkBWP-RedCap is configured and OD-SIB1 is triggered in SUL

In this case, we think the RedCap UE can follow legacy behavior to monitor RAR in initialDownlinkBWP because NW can’t identify whether the UE is the RedCap UE.

- Case 2: if initialDownlinkBWP-RedCap is configured and OD-SIB1 is triggered in NUL

We think this case is different from legacy OD-SIB case, where NW can identify whether the UE is the RedCap UE if reception of Msg1 in initialUplinkBWP-RedCap via dedicated resource si-RequestConfigRedCap-r17. However, OD-SIB1 doesn’t introduce separate SIB1-RequestConfig to send Msg1 in initialUplinkBWP-RedCap, which follows conclusion of RAN1#120 [6].

Conclusion - The parameter “ssb-PeriodicityServingCell” is not included in the UL-WUS configuration for FDD system. - RA-RNTI is not included in the UL-WUS configuration for FDD system. - Cell barring information is not included in the UL-WUS configuration for FDD system. - Cell selection information is not included in the UL-WUS configuration for FDD system. - Accessible UE types is not included in the UL-WUS configuration for FDD system. - CarrierBandwidth (for MPR calculation) is not included in the UL-WUS configuration for FDD system. - InitialUplinkBWP is not included in the UL-WUS configuration for FDD system. - TotalNumberOfRA-Preambles is not included in the UL-WUS configuration for FDD system. - SIB1-RequestResourceRedCap is not included in the UL-WUS configuration for FDD system. - SupplementaryUL is not included in the UL-WUS configuration

Observation 3: If initialDownlinkBWP-RedCap is configured and OD-SIB1 is triggered in NUL, different from legacy OD-SIB procedure, NW can’t identify whether the UE is the RedCap UE because OD-SIB1 doesn’t introduce separate SIB1-RequestConfig to send Msg1 in initialUplinkBWP-RedCap (RAN1#120 conclusion).

As NW can’t identify whether the UE triggering OD-SIB1 in NUL is RedCap UE, NW can only send response in initialDownlinkBWP, i.e., the RedCap UE can only monitor RAR in initialDownlinkBWP.

Thus, based on above analysis, we can see that the Redcap UE monitors the PDCCH in initialDownlinkBWP, irrespective of whether OD-SIB1 request is triggered in NUL or SUL. We draft a TP in Appendix.

Proposal 2a: If initialDownlinkBWP-RedCap is configured and Redcap UE has acquired OD-SIB1-Config earlier, the Redcap UE monitors the PDCCH in initialDownlinkBWP, irrespective of whether OD-SIB1 request is triggered in NUL or SUL.

Proposal 2b: RAN2 adopt the TP in Appendix.

# 3 Conclusion

In this contribution, we discuss User Plane open issues on Rel-19 NES. Our observations are:

Observation 1: In legacy, if initialDownlinkBWP-RedCap is configured and CBRA is triggered, Redcap UE in IDLE/INACTIVE state monitors RAR in initialDownlinkBWP-RedCap, irrespective of whether sending Msg1 in initialUplinkBWP-RedCap or initialUplinkBWP.

Observation 2: In legacy, if Redcap UE in IDLE/INACTIVE state triggers Msg1-based SI request in SUL, it monitors RAR in initialDownlinkBWP because NW can’t identify whether the UE is the RedCap UE (since there is no separated configuration for Msg1-based SI request for the RedCap UE).

Observation 3: If initialDownlinkBWP-RedCap is configured and OD-SIB1 is triggered in NUL, different from legacy OD-SIB procedure, NW can’t identify whether the UE is the RedCap UE because OD-SIB1 doesn’t introduce separate SIB1-RequestConfig to send Msg1 in initialUplinkBWP-RedCap (RAN1#120 conclusion).

    - Based on above observations, our proposals are:

Proposal 1: On f_id of the additional PRACH resources for RACH adaptation, RAN2 down-select between the following two WFs:

- WF-1: Follow RAN2#130 agreement to not pursue MAC spec changes on RA-RNTI calculation.

- WF-2: Add description of f_id of additional PRACH resources in TS 38.321, e.g., “for PRACH occasion configured by addlRACH-Config-Adapt, f_id is the index of the PRACH occasion in the frequency domain plus the number of PRACH occasion(s) configured by RACH-ConfigCommon (0 ≤ f_id < 8)”.

Proposal 2a: If initialDownlinkBWP-RedCap is configured and Redcap UE has acquired OD-SIB1-Config earlier, the Redcap UE monitors the PDCCH in initialDownlinkBWP, irrespective of whether OD-SIB1 request is triggered in NUL or SUL.

Proposal 2b: RAN2 adopt the TP in Appendix.

# 4 References

[1] RAN2#131, Chair Notes.

[2] RAN2#130, Chair Notes.

[3] TS 38.321-v18.0.0, Medium Access Control (MAC) protocol specification, 2023-12.

[4] R2-2208785, [AT119-e][114][RedCap] MAC corrections (vivo) – 2nd round, vivo.

[5] RAN2#119e, Chair Notes.

[6] RAN1#120, Chair Notes.

# Appendix (38.321 TP of Proposal 2)

### 5.15.1	Downlink and Uplink

In addition to clause 12 of TS 38.213 [6], this clause specifies requirements on BWP operation.

A Serving Cell may be configured with one or multiple BWPs, and the maximum number of BWP per Serving Cell is specified in TS 38.213 [6].

The BWP switching for a Serving Cell is used to activate an inactive BWP and deactivate an active BWP at a time. The BWP switching is controlled by the PDCCH indicating a downlink assignment or an uplink grant, by the bwp-InactivityTimer, by RRC signalling, or by the MAC entity itself upon initiation of Random Access procedure or upon detection of consistent LBT failure on SpCell. Upon RRC (re-)configuration of firstActiveDownlinkBWP-Id and/or firstActiveUplinkBWP-Id for SpCell except for PSCell when SCG is deactivated (see clause 5.29) or activation of an SCell, the DL BWP and/or UL BWP indicated by firstActiveDownlinkBWP-Id and/or firstActiveUplinkBWP-Id respectively (as specified in TS 38.331 [5]) is active without receiving PDCCH indicating a downlink assignment or an uplink grant. Upon RRC (re-)configuration of firstActiveDownlinkBWP-Id for PSCell when SCG is deactivated, the DL BWP is switched to the firstActiveDownlinkBWP-Id as specified in TS 38.331 [5]. The active BWP for a Serving Cell is indicated by either RRC or PDCCH (as specified in TS 38.213 [6]). For unpaired spectrum, a DL BWP is paired with a UL BWP, and BWP switching is common for both UL and DL.

For each SCell a dormant BWP may be configured with dormantBWP-Id by RRC signalling as described in TS 38.331 [5]. Entering or leaving dormant BWP for SCells is done by BWP switching per SCell or per dormancy SCell group based on instruction from PDCCH (as specified in TS 38.213 [6]). The dormancy SCell group configurations are configured by RRC signalling as described in TS 38.331 [5]. Upon reception of the PDCCH indicating leaving dormant BWP, the DL BWP indicated by firstOutsideActiveTimeBWP-Id or by firstWithinActiveTimeBWP-Id (as specified in TS 38.331 [5] and TS 38.213 [6]) is activated. Upon reception of the PDCCH indicating entering dormant BWP, the DL BWP indicated by dormantBWP-Id (as specified in TS 38.331 [5]) is activated. The dormant BWP configuration for SpCell or PUCCH SCell is not supported.

BWP for SRS for positioning Tx frequency hopping can be configured for a Serving Cell in TS 38.331 [5]. BWP for SRS Tx frequency hopping is considered as activated when it is configured. BWP switching is not applicable for BWP for SRS Tx frequency hopping.

For each activated Serving Cell configured with a BWP, the MAC entity shall:

1>	if a BWP is activated and the active DL BWP for the Serving Cell is not the dormant BWP and the Serving Cell is not the PSCell of deactivated SCG:

2>	transmit on UL-SCH on the BWP;

2>	transmit on RACH on the BWP, if PRACH occasions are configured;

2>	monitor the PDCCH on the BWP;

2>	transmit PUCCH on the BWP, if configured;

2>	report CSI for the BWP;

2>	transmit SRS on the BWP, if configured;

2>	receive DL-SCH on the BWP;

2>	(re-)initialize any suspended configured uplink grants of configured grant Type 1 on the active BWP according to the stored configuration, if any, and to start in the symbol according to rules in clause 5.8.2;

2>	if lbt-FailureRecoveryConfig is configured:

3>	stop the lbt-FailureDetectionTimer, if running;

3>	set LBT_COUNTER to 0;

3>	monitor LBT failure indications from lower layers as specified in clause 5.21.2.

1>	if a BWP is activated and the active DL BWP for the Serving Cell is dormant BWP:

2>	stop the bwp-InactivityTimer of this Serving Cell, if running.

2>	not monitor the PDCCH on the BWP;

2>	not monitor the PDCCH for the BWP;

2>	not receive DL-SCH on the BWP;

2>	not report CSI on the BWP, report CSI except aperiodic CSI for the BWP;

2>	not transmit SRS on the BWP;

2>	not transmit on UL-SCH on the BWP;

2>	not transmit on RACH on the BWP;

2>	not transmit PUCCH on the BWP;

2>	clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively;

2>	suspend any configured uplink grant Type 1 associated with the SCell;

2>	if configured, perform beam failure detection and beam failure recovery for the SCell if beam failure is detected;

2>	if the SCell is configured as a scheduled cell in MC-DCI-SetOfCells and with the search space for DCI to schedule multiple cells (as specified in TS 38.213 [6]) of the same searchSpaceId as the serving cell in which MC-DCI-SetOfCells containing the SCell is configured:

3>	not monitor the PDCCH for scheduling multiple cells (as specified in TS 38.213 [6]) for the set of cells in MC-DCI-SetOfCells including the SCell.

1>	if a BWP is deactivated or the Serving Cell is PSCell of deactivated SCG:

2>	not transmit on UL-SCH on the BWP;

2>	not transmit on RACH on the BWP;

2>	not monitor the PDCCH on the BWP;

2>	not transmit PUCCH on the BWP;

2>	not report CSI for the BWP;

2>	not transmit SRS on the BWP;

2>	not receive DL-SCH on the BWP;

2>	clear any configured downlink assignment and configured uplink grant of configured grant Type 2 on the BWP;

2>	suspend any configured uplink grant of configured grant Type 1 on the inactive BWP.

Upon initiation of the Random Access procedure on a Serving Cell, after the selection of carrier for performing Random Access procedure as specified in clause 5.1.1, the MAC entity shall for the selected carrier of this Serving Cell:

1>	if PRACH occasions are not configured for the active UL BWP:

2>	if the UE is an (e)RedCap UE; and

2>	if initialUplinkBWP-RedCap is configured:

3>	switch the active UL BWP to BWP indicated by initialUplinkBWP-RedCap.

2>	else:

3>	switch the active UL BWP to BWP indicated by initialUplinkBWP.

2>	if the Serving Cell is an SpCell:

3>	if the UE is an (e)RedCap UE; and

3>	if initialDownlinkBWP-RedCap is configured:

4>	switch the active DL BWP to BWP indicated by initialDownlinkBWP-RedCap.

3>	else:

4>	switch the active DL BWP to BWP indicated by initialDownlinkBWP.

1>	else:

2>	if the Serving Cell is an SpCell:

3>	if the active DL BWP does not have the same bwp-Id as the active UL BWP:

4>	switch the active DL BWP to the DL BWP with the same bwp-Id as the active UL BWP.

1>	stop the bwp-InactivityTimer associated with the active DL BWP of this Serving Cell, if running.

1>	if the Serving Cell is SCell:

2>	stop the bwp-InactivityTimer associated with the active DL BWP of SpCell, if running.

1>	perform the Random Access procedure on the active DL BWP of SpCell and active UL BWP of this Serving Cell.

If the MAC entity receives a PDCCH for BWP switching of a Serving Cell, the MAC entity shall:

1>	if there is no ongoing Random Access procedure associated with this Serving Cell; or

1>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in clauses 5.1.4, 5.1.4a, and 5.1.5):

2>	cancel, if any, triggered consistent LBT failure for this Serving Cell;

2>	perform BWP switching to a BWP indicated by the PDCCH.

If the MAC entity receives a PDCCH for BWP switching for a Serving Cell(s) or a dormancy SCell group(s) while a Random Access procedure associated with that Serving Cell is ongoing in the MAC entity, it is up to UE implementation whether to switch BWP or ignore the PDCCH for BWP switching, except for the PDCCH reception for BWP switching addressed to the C-RNTI for successful Random Access procedure completion (as specified in clauses 5.1.4, 5.1.4a, and 5.1.5) in which case the UE shall perform BWP switching to a BWP indicated by the PDCCH. Upon reception of the PDCCH for BWP switching other than successful contention resolution, if the MAC entity decides to perform BWP switching, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure after performing the BWP switching; if the MAC decides to ignore the PDCCH for BWP switching, the MAC entity shall continue with the ongoing Random Access procedure on the Serving Cell.

Upon reception of RRC (re-)configuration for BWP switching for a Serving Cell while a Random Access procedure associated with that Serving Cell is ongoing in the MAC entity, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure after performing the BWP switching.

Upon reception of RRC (re-)configuration for BWP switching for a Serving Cell, cancel any triggered consistent LBT failure in this Serving Cell.

The MAC entity shall for each activated Serving Cell configured with bwp-InactivityTimer:

1>	if the defaultDownlinkBWP-Id is configured, and the active DL BWP is not the BWP indicated by the defaultDownlinkBWP-Id, and the active DL BWP is not the BWP indicated by the dormantBWP-Id if configured; or

1>	if the UE is neither a RedCap nor an eRedCap UE, and if the defaultDownlinkBWP-Id is not configured, and the active DL BWP is not the initialDownlinkBWP, and the active DL BWP is not the BWP indicated by the dormantBWP-Id if configured; or

1>	if the UE is an (e)RedCap UE, and if the defaultDownlinkBWP-Id is not configured, and initialDownlinkBWP-RedCap is not configured, and the active DL BWP is not the initialDownlinkBWP; or

1>	if the UE is an (e)RedCap UE, and if the defaultDownlinkBWP-Id is not configured, and initialDownlinkBWP-RedCap is configured, and the active DL BWP is not the initialDownlinkBWP-RedCap:

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received on the active BWP; or

2>	if a PDCCH addressed to G-RNTI or G-CS-RNTI configured for multicast indicating downlink assignment is received on the active BWP; or

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received for the active BWP; or

2>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers; or

2>	if a MAC PDU is received in a configured downlink assignment for unicast or MBS multicast:

3>	if there is no ongoing Random Access procedure associated with this Serving Cell; or

3>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in clauses 5.1.4, 5.1.4a and 5.1.5):

4>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

2>	if the bwp-InactivityTimer associated with the active DL BWP expires:

3>	if the defaultDownlinkBWP-Id is configured:

4>	perform BWP switching to a BWP indicated by the defaultDownlinkBWP-Id.

3>	else:

4>	if the UE is a (e)RedCap UE; and

4>	if initialDownlinkBWP-RedCap is configured:

5>	perform BWP switching to the initialDownlinkBWP-RedCap.

4>	else:

5>	perform BWP switching to the initialDownlinkBWP.

NOTE:	If a Random Access procedure is initiated on an SCell, both this SCell and the SpCell are associated with this Random Access procedure.

1>	if a PDCCH for BWP switching is received, and the MAC entity switches the active DL BWP:

2>	if the defaultDownlinkBWP-Id is configured, and the MAC entity switches to the DL BWP which is not indicated by the defaultDownlinkBWP-Id and is not indicated by the dormantBWP-Id if configured; or

2>	if the UE is neither a RedCap nor an eRedCap UE, and if the defaultDownlinkBWP-Id is not configured, and the MAC entity switches to the DL BWP which is not the initialDownlinkBWP and is not indicated by the dormantBWP-Id if configured; or

2>	if the UE is an (e)RedCap UE, and if the defaultDownlinkBWP-Id is not configured, and initialDownlinkBWP-RedCap is not configured, and the MAC entity switches to the DL BWP which is not the initialDownlinkBWP; or

2>	if the UE is an (e)RedCap UE, and if the defaultDownlinkBWP-Id is not configured, and initialDownlinkBWP-RedCap is configured, and the MAC entity switches to the DL BWP which is not the initialDownlinkBWP-RedCap:

3>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

Upon initiation of the Random Access procedure, after selection of the carrier for performing Random Access procedure as specified in clause 5.1.1, if the UE is an (e)RedCap UE in RRC_IDLE or RRC_INACTIVE mode, the MAC entity shall:

1>	if initialUplinkBWP-RedCap is configured for the selected carrier:

2>	perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP-RedCap.

1>	else:

2>	perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP.

1>	if initialDownlinkBWP-RedCap is configured:

2>	if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]) and the Random Access Resources for SI request have been explicitly provided by RRC, and if the selected carrier is SUL carrier; or

2>	if the Random Access procedure was initiated for SIB1 request (as specified in TS 38.331 [5]) and the Random Access Resources for SIB1 request have been explicitly provided by RRC:

3>	monitor the PDCCH on the BWP configured by initialDownlinkBWP.

2>	else:

3>	monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap.

1>	else:

2>	monitor the PDCCH on the BWP configured by initialDownlinkBWP.