3GPP TSG-RAN WG2 Meeting #131bis	R2-2507159

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.7.3

Source:	Nokia, Nokia Shanghai Bell

Title:	UP open issues

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we address several open issues identified during PDCP, RLC, and MAC open issue list discussion.

# 2	PDCP

## 2.1	N001 – PDCP SN gap report

During the last review, we pointed out that one agreement seems to be missing from #131 meeting:

(RLC-12) No special handling is needed in R19 for PDCP SN gap report during UE mobility. It can be left to UE implementation whether to re-send the gap report after HO.

Rapporteur explained that this doesn’t need to be specified because we agreed on no additional handling. But our understanding is that, from the current specification, the UE cannot tell whether re-sending is even allowed (Rather it could understand that re-sending is not allowed). It was also argued that it is sufficient to capture it in the meeting minutes, but we don’t think that it is a proper way to capture our agreement because the agreement was not a confirmation of how it is stated in the specification, rather to allow some additional freedom beyond what has been stated in the specification. Therefore, the agreement needs to be captured in the clause 5.16.1.

Proposal 1: [PDCP-N001] The agreement on RLC-12 from the RAN2#131 meeting is captured in the clause 5.16.1 of the PDCP specification as follows:

## 5.16 SN gap report ### 5.16.1 Transmit operation For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers. NOTE: It is up to UE implementation whether to re-send a PDCP SN gap report when upper layer requests a PDCP entity re-establishment (e.g., based on a previously submitted PDCP SN gap report not having been successfully delivered before PDCP entity re-establishment). If a PDCP SN gap report is triggered for these discarded PDCP SDU(s), the transmitting PDCP entity shall: - compile a PDCP SN gap report as indicated below by: - setting the FDC field to the smallest COUNT value among the COUNT values associated with these discarded PDCP SDU(s); - if more than one PDCP SDU is discarded: - allocating a Discard Bitmap field of length in bits equal to the number of COUNT values from and not including the first of these discarded PDCP SDU(s) up to and including the last of these discarded PDCP SDU(s), rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first; - setting in the discard bitmap field as '1' for these discarded PDCP SDU(s); - setting in the discard bitmap field as '0' for all other PDCP SDU(s). - submit the PDCP SN gap report to lower layers as specified in clause 5.2.1 for Uu interface. NOTE: It is up to UE implementation how to limit the frequency of PDCP SN gap reporting.

# 3	RLC

The rapporteur proposes the followings for RLC open issues, and we discuss RLC-X01, V01, N01, N02, H01, and V02.

Proposal 1: Following open issues are suggested to be discussed based on companies’ contributions: RLC-X01, RLC-V01, RLC-N01, RLC-N02.

Proposal 2: Proponent companies are invited to provide proposals on the following issues, which Rapporteur understands there is no problem: RLC-E01, RLC-H02, RLC-S01.

Proposal 3: Following open issues are suggested to be discussed during the CR phase: RLC-H01, RLC-V02.

## 3.1	RLC-X01

In the previous meeting, RAN2 has agreed that the PDCP provides the DSR data indication for a PDCP Data PDU for each threshold, and RLC calculates the delay-reporting data volume based on the DSR indication. Accordingly, the definition of delay-reporting and non-delay-reporting RLC SDU were removed from RLC specification.

During open issue discussion, it was asked whether non-delay-reporting RLC SDU should be kept in the RLC specification. The concern seems to be that an RLC SDU that could be considered as non-delay-reporting RLC SDU from RLC side may not be considered as such from the PDCP perspective.

Our understanding is that when PDCP SDU is delivered to RLC, it is not physically moved; 'delivery' of PDCP SDU to RLC would only mean that PDCP header is added. The order of PDCP SDUs in the transmission buffer remains unchanged and no new data will be added at RLC. Therefore, we see no case where the DSR data indication for an RLC SDU, determined at the PDCP layer for a certain threshold, would not hold from RLC perspective.

Proposal 2: [RLC-X01] RAN2 confirm that there is no case where the DSR data indication determined at the PDCP layer is not sufficient to evaluate the delay-reporting data volume in RLC.

## 3.2	RLC-V01

When t-RxDiscard expires, the RLC entity stops waiting to receive the obsolete RLC SDUs, and the corresponding RLC SDUs will be positively acknowledged in the STATUS report. However, the PDCP t-Reordering may be still running and waiting for receiving the outdated SDUs from the RLC layer, this can lead to unnecessary delays in packet delivery. To resolve this issue, it was proposed to indicate the SN of the obsoleted RLC SDU to the receiving PDCP entity.

In our view, this issue would happen only rarely, since 1) t-Reordering would be shorter according to latency requirement while t-RxDiscard could be larger considering HARQ latency, and 2) t-Reordering may already have been running for quite some time before the t-RxDiscard. Even when it happens, i.e., t-RxDiscard expires while t-Reordering is running, in practice, large misalignment is unlikely. In addition, the suggested solution would not work since PDCP does not know which RLC SN is allocated to PDCP PDU, meaning that RLC SN is of no use from PDCP point of view. Thus, we see no need of additional handling for this issue.

Proposal 3: [RLC-V01] No optimization is required for the case where t-RxDiscard expires while t-Reodering is still running.

## 3.3	RLC-N01

During email discussion, we suggested to update the RX_Next to address the impact by introducing t-RxDiscard:

a) RX_Next – Receive state variable This state variable holds the value of the SN of the first RLC SDU not delivered to upper layers, but still waited for and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next or t-RxDiscard expires.

Rapporteur agreed with the intention, but it was suggested to further discuss the text because the original text from us was considered potentially affecting the legacy behaviour when t-RxDiscard is not configured. One proposed alternative was to use the structure: ‘if t-RxDiscard is configured, […]. Otherwise, […]’.

In our view, when t-RxDiscard is not configured,

- 	'the SN following the last in sequence completely received RLC SDU' and 'the SN of the first RLC SDU not delivered to upper layers, but still waited for' actually result in pointing to the same SN.

-	t-RxDiscard does not expire.

Therefore, the legacy behavior remains unchanged with the originally proposed text. In contrast, if we use 'if t-RxDiscard, […]. Otherwise, […].' structure, different sentences with the same meaning may end up written as if they were intended to point different SN values, which makes the NOTE confusing only with redundant texts.

Proposal 4: [RLC-N01] The definition of the RX_NEXT in clause 7.1 is updated as below:

a) RX_Next – Receive state variable This state variable holds the value of the SN of the first RLC SDU not delivered to upper layers, but still waited for and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next or t-RxDiscard expires.

## 3.4	RLC-N02

The NOTE was introduced in Clause 5.3.3.3 to capture the agreement from RAN2#130:

NOTE: When all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

The intent of the NOTE is to avoid unnecessary polling after expiry of t-PollRetransmit by allowing UE to stop and reset the t-PollRetransmit.  However, it seems that we overlooked the downside of including NACK part, especially when there is an unACKed RLC SDU still under transmission. In this case, the transmitting side is waiting for ACK/NACK after retransmission or before discard, and hence, the transmitter should be able to keep polling.

In addition, the current NOTE does not clarify that the NOTE should be applied only when stopReTxDiscardedSDU is configured, which may mislead that t-PollRetransmit may stop and reset based on the discard indication from PDCP even when stopReTxDiscardedSDU is not configured.

Proposal 5: [RLC-N02] The NOTE in clause 5.3.3.3 is updated as below:

NOTE: If stopRetxDiscardedSDU is configured, when all RLC SDUs with SNs up to and including POLL_SN are already positively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

## 3.5	RLC-H01

During the post email discussions for RAN2#130, the following paragraph and NOTE have been introduced in clause 5.2.3.1.1

If stopReTxDiscardedSDU is configured, when indicated from upper layer to discard a particular RLC SDU (see TS 38.323 [4]), the transmitting side of an AM RLC entity shall not consider the corresponding RLC SDU or RLC SDU segment(s) for transmission or retransmission if the RLC SDU or the RLC SDU segment(s) thereof has been submitted to lower layers. NOTE Y: The corresponding RLC SDU or RLC SDU segment(s) above includes RLC SDU or RLC SDU segment(s) which have been already considered or pending for transmission or retransmission.

The issue mentioned is the case for discarding RLC SDUs that are not yet transmitted which is already covered by the SDU Discard procedures in clause 5.4. Also, it was suggested that the NOTE should be moved to the normative text.

Our understanding is that although the case for discarding SDUs that are not yet transmitted is covered by SDU Discard Procedures, the paragraph in discussion is under the “General” section of the Transmit Operations clause (5.2.3.1.1), where we can still mention about both transmitting and retransmitting SDUs for simplicity and completeness. We would also prefer to keep the NOTE as is since the normative text mentions all RLC SDUs for which a discard indication has been received from the upper layers.

Proposal 6: [RLC-H01] The paragraph and the NOTE in the clause 5.2.3.1.1 can be kept as it is.

## 3.6	RLC-H02

In clause 5.3.4, it is specified how to construct a STATUS PDU. During post email discussion, it was argued that the current text is redundant considering the case where RX_NEXT becomes higher than RX_Higest_Status. It was suggested to split the structure for a case when a STATUS PDU is triggered by expiry of t-RxDiscard and not.

However, as the issue itself explained, the current procedural text allows skipping the first part related to NACK_SN when RX_Next is higher than RX_Highest_Status, and then the STATUS PDU will only include ACK_SN based on the updated RX_Next. Thus, we see nothing is broken with the current text.

Proposal 7: [RLC-H02] No need for a specification change to address the case when RX_Next > RX_Highest_Status.

## 3.7	RLC-V02

In RAN2#131 meeting, it was proposed to specify that “the pending remaining time based RLC retransmission should be cancelled when the RLC entity receives a positively ACK for the corresponding RLC SDU”. However, it has not be captured due to lack of common understanding.

During the post email discussion, it was first captured as a normative text in clause 5.2.3.1.1. It was then suggested to capture it as a NOTE instead, since it was considered as a clarification rather than a new normative behavior. Still, it was considered by some companies that even a NOTE is unnecessary, since remaining-time-based retransmission is also a retransmission, and adding such a NOTE could cause confusion rather than clarification. We tend to agree that differentiation of normal retransmission and remaining-time-based retransmission is not essential in general. However, given that the terminology of ‘remaining-time-based retransmission’ is used in the specification, we see no harm to clarify it in a NOTE.

Proposal 8: [RLC-V02] Specify in a NOTE in clause 5.2.3.1.1 that the pending remaining-time-based RLC retransmission is cancelled when the RLC entity receives a positive ACK for the corresponding RLC SDU.

NOTE X: When receiving a positive acknowledgement for an RLC SDU with SN = x, the transmitting side of an AM RLC entity should cancel any pending remaining-time-based retransmissions for the corresponding RLC SDU or RLC SDU segment(s).

# 4	MAC

## 4.1	C.2 – Congestion Relief indication

During email discussion, it was proposed that the gNB sends the congestion relief indication – using codepoint 0 – to the UE so that the UE query the bit rate immediately when the congestion is relieved.

Once congestion is relieved, the network will send another UL bit rate – possibly with higher bit rate – and the UE can infer from that information that the congestion has been resolved. Since the trigger of the query is left to the implementation, it is still possible to trigger and send the query when congestion is relieved. Thus, we don’t see the need of explicit indication.

Proposal 9: [MAC-C.2] There is no need of explicit relief indication.

## 4.4	C.3 – Fi index for rate control and rate control query

In UL rate control MAC CE, Fi field indicates the presence of bit rate for the i-th QoS flow, where the index i is determined based on PDU Session ID and QoS Flow Identifier of the QoS flows configured to support UL rate control.

From RRC, there are two separate lists: ul-RateControlConfigList and ul-RateQueryConfigList and these list may contain different sets of QoS flows. This raises one question of whether the current text of Fi is clear that Fi field in UL rate control MAC CE for command (in downlink) is based on ul-RateControlConfigList and Fi field in UL rate control MAC CE for query (in uplink) are based on ul-RateQueryConfigList.

The current text states that the index i is derived from the PDU session ID and QFI of the QoS flows configured to support UL rate control, as specified in the clause 5.18.40. However, the clause 5.18.40 does not provide any hint on how to interpret or set the Fi field when different list of QoS flows are given. Therefore, we see that the current description of Fi is not entirely unambiguous. A simple fix would be to explicitly refer to the relevant RRC parameters in the text.

Proposal 10: [MAC-C.3] Add the relevant RRC parameters – ul-RateControlConfgList or ul-RateQueryConfigList – to the description of Fi in clause 6.1.3.86:

- Fi: This field indicates the presence of bit rate for the i:th QoS flow. The index i is the ascending order of the value defined by PDU Session ID × 64 + QoS Flow Identifier, where PDU Session ID (specified in clause 5.6.9 in TS 23.501 [32]) and QoS Flow Identifier (specified in clause 5.7.3 in TS 23.501 [32]) are those of the QoS flows configured to support UL rate control indicated by ul-RateControlConfigList or ul-RateQueryConfigList (as specified in clause 5.18.40). The Fi field set to 1 indicates that a bit rate for the i:th QoS flow is included in the MAC CE. The Fi field set to 0 indicates that no bit rate information for the i:th QoS flow is included. The Fi field is reserved and set to 0, if the number of QoS flows configured to support UL rate control is less than i;

# 3	Conclusion

In this contribution, we have discussed the open issues in user plane with corresponding proposals mentioned below:

PDCP:

Proposal 1: [PDCP-N001] The agreement on RLC-12 from the RAN2#131 meeting is captured in the clause 5.16.1 of the PDCP specification as follows:

## 5.16 SN gap report ### 5.16.1 Transmit operation For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers. NOTE: It is up to UE implementation whether to re-send a PDCP SN gap report when upper layer requests a PDCP entity re-establishment (e.g., based on a previously submitted PDCP SN gap report not having been successfully delivered before PDCP entity re-establishment). If a PDCP SN gap report is triggered for these discarded PDCP SDU(s), the transmitting PDCP entity shall: - compile a PDCP SN gap report as indicated below by: - setting the FDC field to the smallest COUNT value among the COUNT values associated with these discarded PDCP SDU(s); - if more than one PDCP SDU is discarded: - allocating a Discard Bitmap field of length in bits equal to the number of COUNT values from and not including the first of these discarded PDCP SDU(s) up to and including the last of these discarded PDCP SDU(s), rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first; - setting in the discard bitmap field as '1' for these discarded PDCP SDU(s); - setting in the discard bitmap field as '0' for all other PDCP SDU(s). - submit the PDCP SN gap report to lower layers as specified in clause 5.2.1 for Uu interface. NOTE: It is up to UE implementation how to limit the frequency of PDCP SN gap reporting.

RLC:

Proposal 2: [RLC-X01] RAN2 confirm that there is no case where the DSR data indication determined at the PDCP layer is not sufficient to evaluate the delay-reporting data volume in RLC.

Proposal 3: [RLC-V01] No optimization is required for the case where t-RxDiscard expires while t-Reodering is still running.

Proposal 4: [RLC-N01] The definition of the RX_NEXT in clause 7.1 is updated as below:

a) RX_Next – Receive state variable This state variable holds the value of the SN of the first RLC SDU not delivered to upper layers, but still waited for and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next or t-RxDiscard expires.

Proposal 5: [RLC-N02] The NOTE in clause 5.3.3.3 is updated as below:

NOTE: If stopRetxDiscardedSDU is configured, when all RLC SDUs with SNs up to and including POLL_SN are already positively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

Proposal 6: [RLC-H01] The paragraph and the NOTE in the clause 5.2.3.1.1 can be kept as it is.

Proposal 7: [RLC-H02] No need for a specification change to address the case when RX_Next > RX_Highest_Status.

Proposal 8: [RLC-V02] Specify in a NOTE in clause 5.2.3.1.1 that the pending remaining-time-based RLC retransmission is cancelled when the RLC entity receives a positive ACK for the corresponding RLC SDU.

NOTE X: When receiving a positive acknowledgement for an RLC SDU with SN = x, the transmitting side of an AM RLC entity should cancel any pending remaining-time-based retransmissions for the corresponding RLC SDU or RLC SDU segment(s).

MAC:

Proposal 9: [MAC-C.2] There is no need of explicit relief indication.

Proposal 10: [MAC-C.3] Add the relevant RRC parameters – ul-RateControlConfgList or ul-RateQueryConfigList – to the description of Fi in clause 6.1.3.86:

- Fi: This field indicates the presence of bit rate for the i:th QoS flow. The index i is the ascending order of the value defined by PDU Session ID × 64 + QoS Flow Identifier, where PDU Session ID (specified in clause 5.6.9 in TS 23.501 [32]) and QoS Flow Identifier (specified in clause 5.7.3 in TS 23.501 [32]) are those of the QoS flows configured to support UL rate control indicated by ul-RateControlConfigList or ul-RateQueryConfigList (as specified in clause 5.18.40). The Fi field set to 1 indicates that a bit rate for the i:th QoS flow is included in the MAC CE. The Fi field set to 0 indicates that no bit rate information for the i:th QoS flow is included. The Fi field is reserved and set to 0, if the number of QoS flows configured to support UL rate control is less than i;