3GPP TSG RAN WG2 Meeting #131bis                                                       R2-2507185

# Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda item:	10.2

Source:	Ofinno

Title:	UE capability framework and key features for 6G

Document for:	Discussion and decision

# Introduction

This document discusses the UE capabilities and key features for 6G system based on the agreed 6G SID [1].

# Discussion

The foundational approach for 6G is driven by lessons learned from 5G, particularly the need to move beyond incremental solutions that resulted in fragmented capability definitions and interoperability issues. A core architectural principle endorsed across the industry is the commitment to creating lean and streamlined standards by dimensioning an appropriate set of functionalities and minimizing unnecessary optionality or configuration complexity [2]. This focused effort aims to define a 6G system that is scalable, unified, and forward-compatible for a broad range of devices and services from its first release.

## UE capabilities

From a RAN2, architectural, and protocol standpoint, a foundational design principle for 6G involves creating a single, unified Radio Access Technology (RAT) that natively accommodates a wide spectrum of device types from its initial release. The 6G UE capability framework should overcome the complexities and market fragmentation caused by the thousands of optional features seen in NR. The system must natively support an even broader range of devices, including low-performance types (e.g., LPWA, RedCap) up to high-performance UEs (e.g., XR, aerial/vehicular communications). To ensure strong economies of scale and streamlined signaling, the framework should aim to define a mandatory common baseline of features that all UEs (or specific groups) must support. This approach focuses on achieving a simple, unified framework with minimal signaling overhead and forward compatibility for diverse devices.

To realize this vision, the 6G architecture and protocols will integrate several key principles. A modular protocol design for the RRC, NAS, and user plane will be a cornerstone, featuring an anchor baseline protocol for the lowest-capability devices with additional, implementation-friendly modules for higher-capability UEs. This structure is intended to optimize Layer 2 processing, support parallel processing in hardware, and facilitate scalable implementation for all device categories. Moreover, informed by 5G's challenges with excessive feature options, the 6G UE capability framework will be significantly simplified by minimizing optional features, reducing configurability, and defining a meaningful, globally aligned set of mandatory features for different device profiles to establish a strong, common baseline.

A critical enabler for this streamlined approach is that 6G UEs will be non-backward compatible with 5G NR radio protocols, a design choice that liberates 6G from the legacy constraints of 5G device design and allows for a higher-performing mandatory feature set from the start. This ensures all 6G devices can natively support crucial day-one functionalities like advanced energy saving and robust uplink coverage. The protocol framework must also be forward-compatible, allowing for the introduction of new device types or dynamic capability updates in future releases with minimal service interruption and without necessitating major architectural changes.

6G UE capability framework aims to be scalable, unified, and forward-compatible for the broad range of devices and services while being simplified (in comparison to NR), such as, by defining clear/mandatory baseline of features and reducing the optionality and overhead associated with its capability reporting.

- Study a 6G UE capability framework simple, unified, with minimal overhead and forward compatible for the broad range of devices (from low performance UEs (e.g., LPWA, RedCap) to high performance UEs (e.g., eMBB, URLLC)).

## Energy efficiency

Energy saving is considered a key driver for 6G system design, necessitating a joint, harmonized, and mutually beneficial framework for reducing power consumption in both the UE and the network. The mechanisms proposed for study include the efficient and coordinated use of DRX/DTX operations, wake-up signals (WUS), and adaptive operation of common channels/signals.

This focus on energy efficiency should be integral to the entire system design, ensuring that E2E energy consumption is reduced and that consumption scales appropriately with the scheduled data amount. Network Energy Saving (NES) schemes should aim for a leaner carrier concept, where essential signals like SSB/SIB1 are transmitted only when requested by the device. This unified approach targets significant power savings while maintaining high performance, contributing to the mobile industry's sustainability goals.

6G system should aim for a joint, harmonized, and mutually beneficial framework for energy saving from the first version while considering the trade-offs network and UE.

A general principle to maximize the "Less ON, More OFF" operation across all network components and UEs in any RRC state might be the key driver. The techniques learnt in NR included:

- Adaptive/On-Demand Common Channels (minimizing frequency and the "always ON" Signals). The static and continuous transmission of "always-on" signals like Synchronization Signal Blocks (SSB) and System Information Blocks (SIB1) consumes substantial energy, especially at low or zero traffic load. 6G may study the support/enhancement of the on-demand provisioning mechanisms (OD-SSB/OD-SIB1) to enable deep sleep states for gNBs. Increasing the SSB periodicity (e.g., from 20 ms in 5G to for example 160 ms in 6G) may provide significant NES gains with a potential trade-off in performance and with the necessity of a redesign of the common channels to allow reducing/adapting their transmissions.

- DRX/DTX Coordination. Discontinuous Reception (DRX) and Discontinuous Transmission (DTX) for the UE (C-DRX, paging) and the network (Cell DTX/DRX) are essential tools to reduce the monitoring and transmission activity. In 6G, DTX/DRX like feature should be studied for any RRC state and expanded to cover more cell-wide signals (e.g., suspending SSB/SIB transmission during non-active periods). Moreover, 6G framework should aim to study the harmonized/unified design of Cell DTX/DRX and UE DRX/DTX to align as much as possible the sleep and wake cycles to maximize the coordinated E2E energy savings.

- Wake-Up Signals (WUS) and Receivers (WUR): Low-Power Wake-Up Signals (LP-WUS) and their associated receivers (LP-WUR) also need to be studied for 6G UE power saving. WUS enables the UE's Main Receiver (MR) to remain in a deep sleep state for longer, only waking up when triggered by the low-power signal. 6G studies must enhance WUS/WUR design to achieve improved coverage, reduced footprint, and extended applicability to connected mode operations (e.g., replacing C-DRX), potentially using OFDM-based WUR designs to balance complexity, coverage, and power saving gains. Furthermore, the concept of a UE-initiated wake-up signal (UL-WUS) could also be studied to trigger the network to exit deep sleep and transmit signals like SSB or SIB1, ensuring the network only transmits when needed.

Summarizing 6G should aim to simplify energy saving mechanism(s) targeting to enhance UE and network.

- Study 6G energy saving mechanism(s) to reducing power consumption in UE and network considering joint operations between DRX/DTX, WUS, and adaptive/on-demand common channel/signals.

## AI/ML

The following is RAN2 study objective on AI/ML in 6G [1]:

(8) AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…). NOTE: lead WG depends on the use case.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures ; Data collection and data management, in coordination with SA WGs

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

RAN2 should minimize overlapping discussions with Rel-20 AI/ML related work while at the same time incorporate the AI/ML learnings of NR study and work in the first study/release of 6G. The 6G AI/ML framework should be designed as an AI-native, end-to-end (E2E) system that fundamentally integrates intelligence into the architecture across the UE, RAN, and CN, moving away of the add-on or overlay approach of NR. This native integration is anticipated to drive substantial improvements in radio performance, operational efficiency, and energy savings, while also enabling new revenue streams through services like AI-as-a-Service (or Data-as-a-Service) and Network for AI. It is critical to note that while AI/ML is a key driver for development, the 6G system must always be able to operate without any AI/ML functionality as its operation is explicitly not mandatory.

From a RAN2 perspective, a major architectural challenge is establishing a unified data framework or a dedicated data plane for managing massive volume of heterogeneous data, which includes AI data, sensing data, and operational data. This unified framework must support data collection (LCM), management, and transfer, potentially utilizing a dedicated data plane distinct from the traditional control and user planes. Efficient data handling is crucial to ensure data visibility, controllability, and user consent across the system without creating bottlenecks in traditional control or user planes.

The initial study of AI/ML in RAN2 should commence by identifying high-priority use cases that impact the protocol stack, potentially including enhancements to mobility management, beam management, and positioning, leveraging the work already completed in 5G-Advanced. The framework must be designed to be extensible and future-proof to accommodate unforeseen AI applications, potentially supporting concepts like on-device reinforcement learning and AI Agents that facilitate autonomous decision-making.

AI/ML is a key driver to study/develop 6G system without being a mandatory operation (i.e., 6G system should also operate without AI/ML).

- For 6G AI/ML discussion, start discussing new use cases to be considered for 6G study considering 5G related discussions on AI/ML PHY and AI/ML Mob.

## NTN

For 6G, Non-Terrestrial Networks (NTN) are envisioned as an integral component of the system from Day 1, aimed at fulfilling the IMT-2030 vision of ubiquitous connectivity. Unlike 5G, where NTN support was added in later releases with some fragmentation, 6G system aims to harmonize and integrate the design for TN and NTN from the start of the first release to achieve seamless, ubiquitous, and resilient connectivity. In 5G, NTN support was introduced in a fragmented manner, requiring significant hardware and protocol modifications, which created market inefficiencies and limited adoption. To overcome these shortcomings, 6G aims to natively embed NTN considerations (e.g., long propagation delays, high Doppler, and coverage continuity) into the fundamental design (e.g., waveforms, frame structures, initial access, and HARQ) aiming to ensure/provide a single, harmonized radio framework across TN and NTN. This unified approach aims to maximize commonality, simplify device implementation, and in theory enable any capable 6G UE to access NTN when TN coverage is unavailable, potentially supporting mobility, and multi-connectivity across both domains. By treating NTN as an integral part of the system rather than a retrofitted extension, 6G may improve its deployment, interoperability, and service consistency, while also addressing global coverage, disaster recovery, and 3D mobility use cases in a cost-efficient and scalable manner.

From a RAN2 perspective, the study should concentrate on ensuring this seamless integration is reflected in the higher-layer protocols. This involves investigating procedures for efficient mobility between TN and NTN, managing the unique protocol challenges of satellite communication such as large propagation delays and high Doppler shifts, and ensuring that features like energy saving are effective in NTN contexts. The ultimate goal is to create a single, cohesive 6G system where the distinction between terrestrial and non-terrestrial access becomes transparent to the end-user.

- Study how to harmonize and integrate the design for TN and NTN (as part of the first release of 6G) discussing maximization of commonality, mobility, and multi-connectivity.

## Sensing / ISAC

(9) Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914]

- PHY functions and proc. for sensing technology (e.g., waveform. reference signals, measurement feedback)

- Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases

- Aspects of integration with communication services

- higher layer procedures and protocol aspects [RAN2]

- RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs

Note: RAN1 identify detailed requirements, if triggered by TSG RAN

6G Sensing objective is captured as follows in SID [1]:

Integrated Sensing and Communication (ISAC) is identified as a key new usage scenario and a foundational capability for 6G, expanding the network's role beyond simple data transmission to that of a sensor for the physical environment. This capability is expected to enable a wide array of new services and revenue opportunities, including environmental monitoring, object detection, and the creation of digital twins of the physical world. Furthermore, sensing data can be utilized to enhance the communication network's own performance, for instance, by providing environmental awareness to improve beam management and mobility.

The 6G system will support a flexible framework for ISAC, enabling various sensing modes including monostatic, bistatic, and multi-static operations that involve both network nodes (gNBs) and UEs. The initial study should begin with a limited set of high priority use cases, such as UAV detection and automotive sensing, to guide the initial design. A fundamental architectural consideration is the degree of integration between sensing and communication; the design could range from sharing infrastructure and spectrum to sharing waveforms and even specific signals, involving a trade-off between sensing performance and the impact on communication efficiency.

From a RAN2 standpoint, while the foundational PHY layer design is being studied in RAN1, it is important to consider the higher-layer procedures and protocol aspects required to support ISAC. This includes defining mechanisms for configuring sensing operations, reporting sensing measurements, and managing the large volume of sensing data, potentially through a dedicated data plane or a new data service framework. Given the dependency on the physical layer design, it is prudent for RAN2 to monitor progress in RAN1 before defining detailed RAN2-specific solutions, although architectural implications such as the need for a sensing function in the network and protocols for sensing data transfer can be considered at an early stage.

- Wait for RAN1 progress on sensing PHY study before studying impacts on higher layer procedures and protocols.

# Conclusion

The observations captured are the following:

Observation 1.	6G UE capability framework aims to be scalable, unified, and forward-compatible for the broad range of devices and services while being simplified (in comparison to NR), such as, by defining clear/mandatory baseline of features and reducing the optionality and overhead associated with its capability reporting.

Observation 2.	6G system should aim for a joint, harmonized, and mutually beneficial framework for energy saving from the first version while considering the trade-offs network and UE.

Observation 3.	AI/ML is a key driver to study/develop 6G system without being a mandatory operation (i.e., 6G system should also operate without AI/ML).

The proposals captured are the following:

Proposal 1.	Study a 6G UE capability framework simple, unified, with minimal overhead and forward compatible for the broad range of devices (from low performance UEs (e.g., LPWA, RedCap) to high performance UEs (e.g., eMBB, URLLC)).

Proposal 2.	Study 6G energy saving mechanism(s) to reducing power consumption in UE and network considering joint operations between DRX/DTX, WUS, and adaptive/on-demand common channel/signals.

Proposal 3.	For 6G AI/ML discussion, start discussing new use cases to be considered for 6G study considering 5G related discussions on AI/ML PHY and AI/ML Mob.

Proposal 4.	Study how to harmonize and integrate the design for TN and NTN (as part of the first release of 6G) discussing maximization of commonality, mobility, and multi-connectivity.

Proposal 5.	Wait for RAN1 progress on sensing PHY study before studying impacts on higher layer procedures and protocols.

# Reference

- 3GPP RP-252912, “Study on 6G Radio, SID,” September 2025.

- 3GPP RP-250766, “Lean and Streamlined 6G Standards,” March 2025.