3GPP TSG-RAN2 #131bis	R2-2507210

Prague, Czech Republic, October 13 – October 17, 2025

Agenda Item	: 8.2.2 (Ambient_IoT_solutions)

Source	: LG Electronics Inc.

Title	: Discussion on open issues for AIoT

Document for	: Discussion and Decision

Introduction

In this contribution, we show our views on open issues for AIoT.

Discussion

  - Permanent disable command

According to the TS 24.369, the permanent disable procedure is captured as follows.

- 5.3.4 Permanent disable command procedure - 5.3.4.1 General The purpose of the permanent disable command procedure (see example in figure 5.3.4.1.1) is to permanently disable the communication capability of the AIoT device as specified in 3GPP TS 23.369 [2]. AIoT device AIOTF PERMANENT DISABLE COMMAND PERMANENT DISABLE COMPLETE

In addition, how to disable the communication capability of the AIoT device is captured in TS 23.369 as follows.

- 5.2.2.3 Permanent Disable Command An AIoT Device may be permanently disabled. A permanently disabled AIoT Device does not respond to the Inventory Procedure, as described in clause 6.2.2. An AIoT Device is permanently disabled by the Permanent Disable command sent by an AIOTF to the AIoT Device. The Permanent Disable command is sent to an AIoT Device when an authorized AF uses the Permanent Disable command service operations as described in clause 5.2.2.1, or if the network determines to disable the AIoT Device. The Permanent Disable command is sent in the Command Request step and a response is sent in the Command Response step of the Command procedure described in clause 6.2.3. The AIoT Device responds indicating whether the Permanent Disable command was successful. NOTE: The trigger conditions when a network determines to disable the AIoT Device depends on operator and implementation policy. When the AIoT Device has received and verified a Permanent Disable command, it shall no longer respond to the inventory procedure.

According to SA2 and CT1 specification, the device shall not respond to the inventory procedure if the device is permanently disabled. In other words, the device shall not imitate the random access procedure if the device is permanently disabled.

Observation 1. Even if a device receives a paging message associated with the device, the device shall not imitate the random-access procedure if the device is permanently disabled.

According to AIoT MAC specification, the A-IOT device initiates the random access procedure when the paging message without paging ID is received regardless of whether the device is enabled or disabled as highlighted in yellow in below.

Upon receiving the A-IoT Paging message, the A-IoT MAC entity shall: 1> if the Access Type field in the A-IoT Paging message indicates CBRA: 2> if the device has no stored Transaction ID; or 2> if the value of the Transaction ID field is different from the stored Transaction ID; or 2> if the value of the Transaction ID field is the same as the stored Transaction ID, and the previous procedure was determined as failed for this Transaction ID as specified in clause 5.5: 3> release the stored AS ID, if any; 3> store the received value in Transaction ID field, if the device has no stored Transaction ID, or replace the previously stored Transaction ID with the current received value, if the value of the Transaction ID field is different from the stored Transaction ID; 3> if the Paging ID Presence Indication field indicates Paging ID field is absent: 4> consider the device is selected and indicate it to the upper layers; 3> else: 4> forward the content of the Paging ID field to the upper layers; 4> if the upper layers indicate that the Paging ID is matched: 5> consider the device is selected; 3> if the device is selected: 4> initiate Contention-Based Random Access procedure as specified in clause 5.3.1;

Observation 2. The A-IOT device initiates the random-access procedure when the paging message without paging ID is received even when the device is permanently disabled.

In order to not respond to paging message when the device is disabled, we think that there are two options as follows.

- Option 1. The device does not monitor the PRDCH when the device is disabled.

- Option 2. The device does not respond to only paging message when the device is disabled

In our understanding, the device can receive the “Permanent Disable Command” again if the response to the “Permanent Disable Command” is not successfully transmitted. This is because the reader may transmit the “Permanent Disable Command” unnecessarily if the reader does not receive the response of the “Permanent Disable Command “. Thus, we think that Option2 is more reasonable way than the Option 1.

Proposal 1. The device does not respond to only paging message after the device is permanently disabled.

For the above proposal, our text proposal is as follows.

- 5 A-IoT MAC procedures 5.1 General The clause describes the A-IoT MAC procedures. When the device is powered on, the device starts monitoring PRDCH for an R2D message, as specified in TS 38.291 [2], in order to perform the corresponding A-IoT MAC procedures. Once the device has executed the Permanent Disable Command, as specified in TS 23.369 [4], the device considers that it is permanently disabled. If the device has not executed the Permanent Disable Command before, the device considers that it is enabled. … 5.2 A-IoT paging Upon receiving the A-IoT Paging message, the A-IoT MAC entity shall: 1> if the Access Type field in the A-IoT Paging message indicates CBRA: 2> if the device has no stored Transaction ID; or 2> if the value of the Transaction ID field is different from the stored Transaction ID; or 2> if the value of the Transaction ID field is the same as the stored Transaction ID, and the previous procedure was determined as failed for this Transaction ID as specified in clause 5.5: 3> release the stored AS ID, if any; 3> store the received value in Transaction ID field, if the device has no stored Transaction ID, or replace the previously stored Transaction ID with the current received value, if the value of the Transaction ID field is different from the stored Transaction ID; 3> if the Paging ID Presence Indication field indicates Paging ID field is absent: 4> consider the device is selected and indicate it to the upper layers; 3> else: 4> forward the content of the Paging ID field to the upper layers; 4> if the upper layers indicate that the Paging ID is matched: 5> consider the device is selected; 3> if the device is selected and the device is enabled: 4> initiate Contention-Based Random Access procedure as specified in clause 5.3.1; 1> else (i.e., the Access Type field in the A-IoT Paging message indicates CFA): 2> release the stored AS ID, if any; 2> release the stored Transaction ID, if any; 2> forward the content of the Paging ID field to the upper layers; 2> if the upper layers indicate that this Paging ID is matched: 3> consider the device is selected; 3> if the device is enabled: 4> initiate Contention-Free Access procedure as specified in clause 5.3.2.

  - Paging ID length

In SA2 LS (S2-2507793), SA2 ask whether extending the AIoT Device permanent ID to 256bit and 496bit is feasible or not to RAN2. In this LS, it is stated that the size of paging ID can be up to 600bit. Thus, we look into that the larger paging ID can be included in the paging message.

According to AIoT MAC specification, the maximum size of the paging message is up to 432bit.

- R2D message type field: 3bit

- R2D TBS field: 7bit

- AT field: 1bit

- Transaction ID field: 6bit

- Paging ID Presence Indication field: 1bit

- Paging ID length field: 8bit

- Number of access occasion field: 4bit

- D2R Scheduling Info field: 18bit

- Paging ID field: up to 256bit

- K field: 1bit

- 128bit security parameter: 128bit

Since it is assumed that the maximum R2D message size is 1000bit, there is a room to accommodate 600bit paging ID in the paging message. However, since the paging length can be indicated up to 256bit, the paging length field should be extended to 10bit.

Proposal 2. The paging length field should be extended to 10bit to support the larger paging ID.

  - No data available case

In last RAN2 meeting, how to handle no data available case was discussed, and the following agreements are made.

Agreements on no data available due to delay in NAS 1 The reader determines no data available case by SDU length 0. As more data indication is mandatory, the device sets this bit to "0". 2 The reader, in response to 0 SDU in the device’s MAC response, may send a follow-up R2D Upper Layer Data Transfer message at a later time to schedule another D2R Upper Layer Data Transfer message from the device. 3 The follow-up R2D Upper Layer Data Transfer message includes the Received Data Size field with the Received Data Size field set to value 0, without including the original command. 4 RAN2 would like to check if there is a case where NAS doesn’t provide a response at all. If this case exists, RAN2 will discuss this issue.

Based on the above agreement, RAN2 need to check whether there is a case where NAS layer does not provide a response at all. According to TS24.369, the NAS layer ignores a NAS message if the NAS message is that NAS message is too short or NAS message is unknown or unforeseen message type.

6.2 Message too short or too long 6.2.1 Message too short When a message is received that is too short to contain a complete message type information element, that message shall be ignored, c.f. 3GPP TS 24.007 [3]. 6.2.2 Message too long Editor's note: Maximum AIoT NAS message length is FFS and to be defined by RAN3. 6.3 Unknown or unforeseen message type If the AIoT device or the network receives a AIoT NAS message with message type not defined for the AIoT NAS protocol or not implemented by the receiver, it shall ignore the AIoT NAS message. NOTE: A message type not defined for the AIoT NAS protocol in the given direction is regarded by the receiver as a message type not defined for the AIoT NAS protocol, see 3GPP TS 24.007 [3].

As shown above, there is a case where the NAS layer does not respond to a message at all, i.e., the NAS error case. In addition, as previously discussed in RAN2, there is case where the NAS layer cannot generate the NAS message due to NAS processing delay, i.e., the NAS processing delay case.

From the reader’s point of view, even if the D2R message with SDU length = 0 and the MDI = 0 is received, it is difficult to distinguish whether the device has no data due to the NAS error or the NAS processing delay. In addition, without knowing this information, the reader unnecessarily schedules the uplink grant to the device even for the NAS error case. Thus, the device should indicate why the SDU length = 0 is indicated in the R2D message.

In order to indicate the NAS error case and NAS processing delay case, we think that there are three options as follows.

- Option 1. The estimated time when the NAS command response will be ready is included in the R2D message.

- Option 2. Introduce a new field (1bit) to indicate whether the uplink is needed later for the NAS processing delay case or uplink scheduling is not needed for NAS error case.

- Option 3. If the SDU field is set to 0, the MDI field is set to 0 for the NAS error case and the MDI field is set to 1 for the NAS processing delay case, i.e., revert to the previous agreement.

For Option 1, the device provides information on when the D2R message is ready. Using this information, the reader can schedule the D2R message on time. In this option, if the estimated time is set to zero, the reader identifies that the device has no data due to an error. Otherwise, if the estimated time is not zero, the reader schedules the D2R message on time. However, since this option requires an estimation of the time when the D2R message is ready, the feasibility of the estimation of the time is questionable due to the AIoT device's limited processing capacity.

The Option 2 and Option 3 are similar solutions. The only difference is whether to indicate the NAS error and the NAS processing delay using the new field or the MDI field. However, during the [POST130][027][AIoT] email discussion, many companies had a concern to reuse the MDI field with the different purposes because the motivation for introducing the MDI field is to indicate whether the data in the MAC PDU is the last segment or not. In addition, for Option 2, one reserved bit in the R2D message format can be used for the new field. Thus, even if new 1bit field is introduced, there is no problem on the signaling overhead.

In our view, since it would be encouraged for the simple implementation that one single field should be used only for single purpose, we prefer to have option 2 to indicate whether the uplink is needed later for the NAS processing delay case or uplink scheduling is not needed for NAS error case.

Proposal 3. Introduce new field (1bit) to indicate whether the uplink is needed later for the NAS processing delay case or uplink scheduling is not needed for the NAS error case.

Conclusion

Based on the above discussion, we made following proposals and observations.

Observation 1. Even if a device receives a paging message associated with the device, the device shall not imitate the random-access procedure if the device is permanently disabled.

Observation 2. The A-IOT device initiates the random-access procedure when the paging message without paging ID is received even when the device is permanently disabled.

Proposal 1. The device does not respond to only paging message after the device is permanently disabled.

Proposal 2. The paging length field should be extended to 10bit to support the larger paging ID.

Proposal 3. Introduce new field (1bit) in to indicate whether the uplink is needed later for the NAS processing delay case or uplink scheduling is not needed for the NAS error case.