3GPP TSG-RAN2 #131bis	R2-2507212

Prague, Czech Republic, October 13 – October 17, 2025

Agenda Item	: 9.2.2 (Ambient_IoT_Solutions_Ph2)

Source	: LG Electronics Inc.

Title	: Discussion on Topology 2 for AIoT

Document for	: Discussion and Decision

Introduction

In this contribution, we show our view on the topology 2 in AIoT

Discussion

As stated in WID, the RRC-based architecture is used for topology 2, and the following figure shows an architecture for the topology 2 in the TR23.700-13.

As shown above figure, the gNB is responsible for the AIoT reader (i.e., intermediate UE) controllably. This means that when the AIoT procedure is initiated, the gNB can be aware of the number of targeted devices for the inventory procedure and the type of procedure. In this sense, the AIoT radio resources should be controlled by the gNB. In summary, the gNB should provide the AIoT radio resources based on the knowledge of the details of the procedure to the UE reader using RRC signalling.

Proposal 1:	The gNB allocates the AIoT radio resource to the UE reader using RRC signalling.

Then, RAN2 need to discuss how to allocate the A-IOT radio resources by the gNB.

During the SI phase, how to allocate the AIoT radio resources between the UE reader and the AIoT devices was discussed, and the following candidate two options were proposed.

- Option 1: The UE reader receives the AIoT radio resources configuration in RRC signalling. The AIoT radio resources remain valid until the network releases them explicitly.

- Option 2: The UE reader receives the AIoT radio resources configuration in RRC signalling, which configures a time period in which the corresponding resource can be used. The UE reader considers that the AIoT radio resources remain valid for that time period, unless the resource configuration is explicitly released by the network.

In our view, each option can be used for a different case. Option 1 can be used for the case where the gNB is not aware of the number of targeted devices for the inventory procedure, i.e., the inventory procedure is targeted to the unknown devices or all devices, and Option 2 can be used for the case where the gNB is aware of the number of targeted devices for the inventory procedure. Thus, we suggest to introduce both options for AIoT resources allocation.

Proposal 2. The following AIoT resource allocation methods should be supported.

- The UE reader receives the AIoT radio resources configuration in RRC signalling. The AIoT radio resources remain valid until the network releases them explicitly.

- The UE reader receives the AIoT radio resources configuration in RRC signalling, which configures a time period in which the corresponding resource can be used. The UE reader considers that the AIoT radio resources remain valid for that time period, unless the resource configuration is explicitly released by the network.

Lastly, RAN2 need to further discuss how to handle the allocated AIoT radio resources for RLF and handover. In our understanding, since RLF or handover is associated with Uu interface, the allocated AIoT radio resources can be used even when the UE detects the RLF or UE performs the handover.

More specifically, for RLF case, if the UE reader detects the RLF, the UE reader initiates the RRC Connection Reestablishment. In this case, the allocated AIoT radio resources can be used until the RRC Connection Reestablishment is completed or failed. In addition, for handover case, even if the UE reader receives the handover command containing the AIoT radio resources for the target gNB from the source gNB, the UE reader can use the AIoT radio resources received from the source gNB until the handover is completed to the target gNB.

Thus, considering the above explanation, we propose that the allocated AIoT radio resources should be used before completion of the handover or the RRC Connection Reestablishment procedure.

Proposal 3. The allocated AIoT radio resources are considered as valid before completion of the handover or the RRC Connection Reestablishment procedure.

Conclusion

Based on the above discussion, we made following proposals and observations.

Proposal 1:	The gNB allocates the AIoT radio resource to the UE reader using RRC signalling.

Proposal 2. The following AIoT resource allocation methods should be supported.

- The UE reader receives the AIoT radio resources configuration in RRC signalling. The AIoT radio resources remain valid until the network releases them explicitly.

- The UE reader receives the AIoT radio resources configuration in RRC signalling, which configures a time period in which the corresponding resource can be used. The UE reader considers that the AIoT radio resources remain valid for that time period, unless the resource configuration is explicitly released by the network.

Proposal 3. The allocated AIoT radio resources are considered as valid before completion of the handover or the RRC Connection Reestablishment procedure.