3GPP TSG RAN WG2 #131-bis	R2-2507366

Prague, Czechia, 13th – 17th October 2025

Agenda item	: 10.4

Source	: Jio Platforms Limited

Title	: Fundamentals of 6G Mobility

Document for	: Discussion

# Introduction

The new SID “Study on 6G Radio” [1] has been agreed in RAN#108 and revised [2] in RAN#109 meeting and objective 4 of the study item is as shown below:

- Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

This document provides our perspective on determining the Mobility framework in the first release of 6G.

# Discussion

Mobility is fundamental function of wireless cellular networks which helps users to maintain connectivity during movement from one place to another one. It has significantly evolved from LTE to 5G NR.

NR is having legacy L3 measurement based handovers as well as conditional , Dual active protocol state and Lower layer triggered mobility. Each new method is having it’s own pros and cons. The use case of each type of mobility is different so all of them are not fully implemented by the vendors.

The overview of the mobility procedures introduced in 5G are summarized below:

L3 based Handover (Xn/N2)

This is base line mobility profile which is taken since Release 15. This is defined via Xn interface (gNB to gNB) and N2 interface ( gNB to AMF) for intra frequency as well as Inter frequency. Positive aspect of this type is that since it is legacy so it has been easily adopted for implementation. But major drawback is that user plane interruption time is not meeting the use cases related to time critical services as well as low latency devices.

As a baseline of NR , L3 based handover is globally accepted and implemented despite the long user plane interruption time.

Conditional Handover (CHO)

This is introduced in Release 16. Conditional Handover (CHO) is a handover procedure that is executed only when the configured execution condition(s) are met . As per this feature network sends the pre-evaluated target cells information to UE and then further UE measures the target cells and execute the handover. Major positive aspect of this handover is that it can minimize the handover failures since UE already has pre-configured conditional target cell configuration. However, it is still prone to interruption time as it improves the robustness but not interruption time (the UE may still be exposed to interruption before HO execution). Another drawback is that signalling load is increased due to conditional reconfiguration during HO region.

Despite the improved robustness, CHO has not seen a good commercial interest as it comes with additional signaling load and interruption time.

Dual Active Protocol State (DAPS)

DAPS Handover is a handover procedure introduced in Release 16 that maintains the source gNB connection after reception of RRC message for handover and until releasing the source cell after successful random access to the target gNB. Major positive aspect is that it is having almost zero interruption time and packet loss because UE maintains DL reception from both source and target cell along with the UL transmission towards source cell during handover execution zone. Major drawback is that UE side complexity is increased due to simultaneously Rx from two different nodes while maintaining Tx towards source cell.

DAPS provides better user interruption time but it increases the UE level complexity to monitor source and target cells in DL simultaneously.

L1/L2 Layer triggered Mobility (LTM)

3GPP Release 18 has introduced a new type of L1/L2-triggered mobility procedure (LTM), where cell switch is based on L1 measurements instead of L3 and triggered by MAC CE. Major positive aspect is that it is having close-to-zero interruption time and packet loss due to early TA acquisition and slimmer HO trigger mechanism via MAC CE. Major drawback is that UE side complexity is increased due to L1/L2 based measurements. Besides, Release 18 LTM is limited to intra-gNB (both intra-DU and inter-DU); inter-gNB LTM is not allowed.

Rel-18 LTM cuts HO latency via early TA and MAC-CE-triggered execution but introduces substantial L1 reporting overhead and is constrained to intra-gNB scenarios.

Enhancement in L1/L2 Layer triggered Mobility (LTM)

Rel-19 extends LTM from Rel-18’s intra-gNB to inter-gNB (inter-CU), prioritizes Xn-based operation, and reuses Rel-18 RRC/LTM structures. Early sync is supported (DL via CSI-RS; PDCCH-ordered early RACH), and early TA follows the Rel-18 baseline (no RAR-based TA for non-conditional). For C-LTM, TA is signalled by the source via MAC CE with per-candidate TA timers, leading to RACH-less execution when TA is valid at CG, otherwise RACH-based.

The R19 design delivers low-latency mobility across gNBs with predictable execution (pre-delivered configs + MAC-CE trigger) and keeps implementation friction low by reusing Rel-18 modeling; early DL/UL sync and early TA enable faster, cleaner switches in challenging radio. On the flip side, cross-CU coordination raises complexity, beam-level reporting can still tax uplink/control resources, TA validity/timer management introduces new tuning and failure edges, and the no-RAR-TA rule for non-conditional cases reduces flexibility in some deployments, so benefits hinge on careful parameterization and ecosystem readiness.

Rel-19 makes low-latency LTM feasible across gNBs via early sync/TA and reuse of Rel-18 structures, but added cross-CU coordination, beam-reporting load, and TA-timer management can limit broad, plug-and-play adoption.

In practice, 5G mobility taught a clear lesson: the simple, mature L3 handover won broad deployment despite interruption, while lower-latency options (CHO, DAPS, LTM) delivered technical gains at the cost of extra signaling, UE/control complexity, and narrower applicability—slowing wide rollout. Even with Rel-19’s broader LTM, cross-CU coordination and timer/beam tuning keep plug-and-play elusive. The market rewarded predictability and operational simplicity over theoretical best-case latency. To establish an optimal foundation for 6G mobility, RAN2 should start by rigorously assessing 5G mobility frameworks. The primary objective of this assessment is to clearly identify the lessons learned—specifically, which features achieved market success, and which introduced complexity with limited real-world adoption.

To establish an optimal foundation for 6G mobility, RAN2 should start by rigorously assessing 5G mobility frameworks. The primary objective of this assessment is to identify the lessons learned—specifically, which features achieved market success, and which introduced complexity with limited real-world adoption.

Another consideration on 6GR is the overhead reduction on RRC configuration. In 5G, some of the mobility frameworks allow only complete RRC configuration whereas some other mobility frameworks allow full and delta configurations. The merit of the full and delta configuration is clear from technical perspective. RAN2 should discuss whether to support the full and delta configuration as unified framework for any kind of mobility framework in the first release of 6G.

RAN2 should discuss whether to support the full and delta configuration as unified framework for any kind of mobility framework in the first release of 6G.

While assessing the base line mobility frameworks for 6GR, RAN2 may discuss possible approaches. As a simple approach RAN 2 may study features which have given more practical benefits and accordingly adopt.

3GPP Release 17 onwards it is possible to configure dual measurement trigger quantities along with existing measurement event but it is only limited to conditional HO scenario only. Secondly even in CHO also it is allowed to measure in HO execution state only.

In current NR deployments, event-triggered measurements (such as A3 and A5) typically rely on a single quantity (e.g., RSRP, RSRQ, or SINR). However, single-quantity triggering can be unreliable under fluctuating interference, coverage holes, or fast fading conditions where SINR can degrade abruptly even when RSRP appears strong, resulting in ping-pong handovers or delayed event detection.

Major benefit of this type of configuration for all the mobility events will provide UE to filter out measurement properly. UE will send L3 based measurement report more precise manner which will improve network’s mobility performance. In fact, RAN 2 should start discussion to create a more comprehensive decision metric to trigger handover which can be composed of more than one triggering quantity such as:

- Combine a primary quantity (e.g., RSRP) with a secondary quantity (e.g., SINR or RSRQ).

- Trigger measurement events only when both conditions are satisfied at UE

- Allow optional weighting, hysteresis, or AND/OR logic to adjust sensitivity.

RAN 2 should start discussion to create a more comprehensive decision metric to trigger handover which can be composed of more than one triggering quantities.

In current NR deployments, node configures multiple frequency  measurement objects and UE uses these  configurations to measure respective frequency. Depending on UE implementation, UE may or may not measure frequency in sequence in which it is shared in measurement configuration. This sometimes creates a scenario where 2nd frequency RF condition is much better than that of 1st defined frequency . But since UE measures 1st frequency and finds criteria meeting, it will initiate Measurement Report corresponding to 1st frequency and accordingly the handover will be triggered (but to the 2nd best cell).

In the idle mode mobility (cell reselection procedure) 3GPP has already defined the different set of priority levels to consider other than serving frequency for measurements. UE moves to other cells based on the target cell priority. The same can be extended for connected mode mobility also.

RAN 2 should start discussion to avoid above scenario by giving flexibility to incorporate “Priority” for connected mode mobility similar to Idle mode mobility in 5G.

# Conclusion

Observation 1: As a baseline of NR, L3 based handover is globally accepted and implemented despite the long user plane interruption time.

Observation 2: Despite the improved robustness, CHO has not seen a good commercial interest as it comes with additional signaling load and interruption time.

Observation 3:	 DAPS provides better user interruption time but it increases the UE level complexity to monitor source and target cells in DL simultaneously.

Observation 4:	 Rel-18 LTM cuts HO latency via early TA and MAC-CE-triggered execution but introduces substantial L1 reporting overhead and is constrained to intra-gNB scenarios.

Observation 5:	 Rel-19 makes low-latency LTM feasible across gNBs via early sync/TA and reuse of Rel-18 structures, but added cross-CU coordination, beam-reporting load, and TA-timer management can limit broad, plug-and-play adoption.

Proposal 1: To establish an optimal foundation for 6G mobility, RAN2 should start by rigorously assessing 5G mobility frameworks. The primary objective of this assessment is to identify the lessons learned—specifically, which features achieved market success, and which introduced complexity with limited real-world adoption.

Proposal 2:	 RAN2 should discuss whether to support the full and delta configuration as unified framework for any kind of mobility framework in the first release of 6G.

Proposal 3:	 RAN 2 should start discussion to create a more comprehensive decision metric to trigger handover which can be composed of more than one triggering quantities.

# References

3GPP RAN#108, RP-251881, New SID: Study on 6G Radio

3GPP RAN#109, RP-252912, Revised SID: Study on 6G Radio