3GPP RAN WG2 Meeting #131bis	R2-2507476

Prague, Czech Republic, October 13th – 17th 2025

Agenda Item:	8.1.2.2

Source:	InterDigital

Title:	Discussion on UE-sided data collection-related RILs: [E040], [X003], [X004]

Document for:	Discussion, Decision

# Introduction

A RAN2 objective for AI/ML for Air Interface Work Item [1] is to design a general framework for one-sided AI/ML, including the signaling and protocol aspects of Life Cycle Management (LCM) to enable functionality selection, activation, deactivation, switching and fallback.

A list of remaining issues related to implementation of the UE data collection procedure has been provided as part of the AIML ASN.1 review [2] [3]. This contribution discusses these remaining issues, specifically those related to UE-sided data collection.

# Remaining Issues marked ‘ToDo’

The following issues and descriptions have been extracted from the AIML issues list [2] [3] as part of the ASN.1 review, and listed as “ToDo” by the TS 38.331 specification Rapporteur.

## [E040] Repeat UE-side data collection configuration

As described in [2], the current procedural text in 5.7.4.3 allows the UE to request a candidate UE-side data collection configuration in UAI (received in otherConfig) even if it already has a corresponding UE-side data collection configuration (received in CSI-ReportConfig) for that candidate configuration.  It is noted this is not the desired behaviour should be fixed (e.g., via one of the solutions below):

- Change only the procedural text to make it clear that the UE does not request candidate configurations for which it already has a corresponding UE-side data collection configuration;

- Change the ASN.1 (and corresponding procedures) so the network can indicate which candidate configuration a real UE-side data collection configuration corresponds to.

Though this is the intended behaviour, it seems that the above issue can be solved by proper implementation. For example, by implementation the UE can determine whether the current set of configurations satisfy it’s data collection needs, and if so there seems little reason to request additional data collection configurations (even if not explicitly forbidden from doing so). Also, even if there is a redundant request, the network can determine a configuration has already been provided, and it can simply ignore the request for additional configuration by it’s own implementation. Since explicit procedural text is not needed to support the intended behaviour, it is proposed this be left to implementation and [E040] be marked as “PropReject”.

Proposal 1:	[E040] is marked as “PropReject”.

## [X003] Redundant inclusion of dataCollectionStart

[X003] explains that since the presence of dataCollectionCandidateConfigList can implicitly indicate UE is about to collect data, there is no need to introduce a separate indication. On the other hand, start indication is useless without dataCollectionCandidateConfigList, since NW is not aware of the required configuration. If UE has already obtained the required configuration for data collection, UE doesn’t need to indicate start indication alone. It is proposed that dataCollectionStart can therefore be removed.

The need of dataCollectionStart may depend on the availability of dataCollectionCandidateConfigList. For example, if dataCollectionCandidateConfigList is already provided to the UE, indicating a preferred configuration implicitely notifies the network the UE would like to start data collection. In this case (unless the UE would like to postpone data collection and store the configuration for later, which doesn’t seem a strong use case) there seems no reason to also indicate dataCollectionStart along with the preferred candidate.

However, if dataCollectionCandidateConfigList is not provided to the UE, it may be useful to allow the UE to send dataCollectionStart alone to indicate a desire to start data collection, prompting the network to provide the candidate list to the UE. Since the UE has already indicated it’s preference to  start data collection, when the UE subsequently indicates it’s preferred data collection configuration it does not need to again include dataCollectionStart. This may be captured as a RAN2 understanding, with any possible specification impact left FFS.

Proposal 2:	[X003] RAN2 understands UE may send a start indication alone to request the dataCollectionCandidateconfiglist (if not already provided). UE does not re-send start indication when subsequently indicating the preferred configuration.

## [X004] Candidate data collection configuration

Currently, the inference related configuration and candidate data collection configuration is provided via a list in OtherConfig. However, with a list, NW has to provide the full list upon modification or release for a certain entry. ToAddMod structure can be used to reduce the signalling upon modification or release for a certain entry. [X004] proposes to change the current structure of candidate data collection configuration to ToAddMod structure, along with corresponding UE behaviour.

This issue is like [C083] and [C084], where it is similarly noted the current structure requires additional signally overhead to make even a minor change to the list. As noted by the proponent, the current implementation can be resolved by adoption of an AddModList structure (which has obvious benefits from a flexibility and signaling overhead perspective). It is proposed that [X004] be marked as “PropAgree”.

Proposal 3:	[X004] is marked as “PropAgree”.

# Conclusion

In this contribution the following proposals are made regarding remaining open issues identified during the AIML ASN.1 review:

Proposal 1:	[E040] is marked as “PropReject”.

Proposal 2:	[X003] RAN2 understands UE may send a start indication alone to request the dataCollectionCandidateconfiglist (if not already provided). UE does not re-send start indication when subsequently indicating the preferred configuration.

Proposal 3:	[X004] is marked as “PropAgree”.

# References

RP-243244 – WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface – Qualcomm

AIML Comments file v029 – Rel-19 CR ASN.1 Review (2025-09)

AIML Review file v029  – Rel-19 CR ASN.1 Review (2025-09)