3GPP RAN WG2 Meeting #131bis	R2-2507505

Prague, Czech Republic, Oct. 13th – 17th, 2025

Agenda Item:	8.4.4

Source:	InterDigital, Inc.

Title:	Open issues on LP-WUS

Document for:	Discussion

# 1. Discussion

As outcome of the open issue discussion [1], RAN2 needs to discuss further regarding open issues.

- 38304-1: Discuss whether “low mobility” criterion based on LR measurements needs to be introduced for LP-WUS.

- 38304-3: FFS the impact on legacy low-mobility criteria for MR when the UE exits serving cell measurement offloading case.

(38.304-1)

In the last meeting, RAN2 agreed not to support low mobility for Rel-19 measurement relaxation/offloading also LR measurement is not used for Rel-16 measurement relaxation. However, when a UE is not allowed to measure with MR during a long period due to the offloading condition, this may have negative impacts on cell reselection or paging performance. From our view, it would be beneficial that a UE can evaluate for Rel-16 low mobility criterion with available LR measurements.

Proposal 1: [38304-1] Support to evaluate low mobility criterion with LR measurements.

(38.304-3)

In the last meeting, RAN2 discussed this issue and postponed [2]. After the UE exits the serving cell measurement offloading, the UE can trun on MR and restart the evaluation for Rel-16 low mobility criterion. It would be similar behavior that a UE restarts the evaluation for low mobility criterion after a period of reselection procedure according to the current 38.304 specification. From our view, no specification impact is foreseen on the low mobility criterion when UE exits RRM measurement offloading.

Proposal 2: [38304-3] Do not consider the specification impact on low-mobility criteria when a UE exits measurement offloading.

# 2. Conclusion

Proposal 1: [38304-1] Support to evaluate low mobility criterion with LR measurements.

Proposal 2: [38304-3] Do not consider the specification impact on low-mobility criteria when a UE exits measurement offloading.

# 3. References

[1] R2-2506862, List of open issues for LP-WUS 38.304 CR (CATT).

[2] R2-2506247, [AT131][204][LPWUS] Proposals for RRC-10/38304-6, RRC-15/38304-2, 38304-12 (CATT)