3GPP TSG-RAN WG2 Meeting #131bis	R2-2507538

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda Item:	8.8.2

Source:			ASUSTeK

Title:	Discussion on closest reference location reporting

Document for: 	Discussion and Decision

- Introduction

In this contribution, we would like to discuss the RIL issues H252 and S024/C005/Z252 in the RIL review for NR NTN [1].

- Discussion

For the closest reference location reporting, the current RRC spec [2] is quoted as below:

For reporting via RRCConfigurationComplete: 2> if the UE is configured in this RRCReconfiguration message to provide location information for assisted SMTC configuration in RRC_CONNECTED state: 3> include referenceLocationReport; For reporting via UAI: 1> if transmission of the UEAssistanceInformation message is initiated to provide location information for assisted SMTC configuration in RRC_CONNECTED state according to 5.7.4.2; 2> include the referenceLocationReport with a number of closest reference locations to the current UE’s position determined by closestLocsToReport;

As described in H252, the closest reference locations can be reported in both RRCConfigurationComplete message and UAI with the same format. However, the descriptions of including referenceLocationReport are not aligned currently, and the number of reference locations is only added for UAI.

As mentioned by the rapporteur, the RRCReconfiguration message contains OtherConfig with closestLocsToReport to configure location information for assisted SMTC configuration. As a result, the UE could also apply closestLocsToReport when including the referenceLocationReport in the RRCConfigurationComplete message. To align the description of closest reference location reporting in both RRCConfigurationComplete message and UAI, a clarification could be added in the RRC spec.

Proposal 1: [H252] Align the description for including closest reference location report in RRCReconfigurationComplete and UAI as following:

2> if the UE is configured in this RRCReconfiguration message to provide location information for assisted SMTC configuration in RRC_CONNECTED state: 3> include the referenceLocationReport with a number of closest reference locations to the current UE’s position determined by closestLocsToReport;

On the other hand, in S024, C005 and Z252, the UE behavior for closest reference location reporting was discussed, and several text proposals were provided as below:

- S024:

2>	if the UE is configured in this RRCReconfiguration message to provide location information for assisted SMTC configuration in RRC_CONNECTED state:

3>	include referenceLocationReport, if available;

- Option 1 of C005:

A UE capable of providing location information for assisted SMTC configuration in RRC_CONNECTED state shall initiate the procedure upon determining that the closest reference location(s) have changed compared with the last reported values.

- Z252:

A UE capable of providing location information for assisted SMTC configuration in RRC_CONNECTED state shall initiate the procedure upon being configured to do so and the first report has not been sent, and upon determining that the closest reference location(s) have changed compared with the last reported values.

The relevant agreements in the last meeting are quoted as below:

3.	UE Assistance Information message will include information to the NW to (re)configure the most relevant SMTCs for the area where the UE is located.

4.	The procedure for UE assisted SMTC configuration in connected mode is follows:

-	For a UE indicating the relevant capability (FFS the name of the capability and it this is merged with some other capability) the network configures the UE via reconfiguration message with a list of reference locations and a parameter N, thereby configuring the UE to report the N closest reference locations (FFS whether also a distance threshold is indicated). If the list of reference locations is not provided, the list of reference locations provided in system information is reused.

-	The UE reports an indication of the N closest reference locations via UE assistance information, e.g. bitmap or list of indices of the locations.

-	The UE can report the N closest reference locations via the RRCReconfigurationComplete message.

-	The network configures the relevant SMTC(s). The maximum number of configured SMTCs is according to legacy UE capability in terms of SMTC support (same as legacy).

-	The UE provides an update on the set of the N closest reference locations, if changed, via UE assistance information. Upon receiving an update, the network reconfigures the relevant SMTC(s), if changed.

It was agreed that the UE reports the closest reference location via UAI and the UE can report the closest reference location via the RRCReconfigurationComplete message. Based on the current RRC spec, if the UE receives a RRCReconfiguration message configuring the OtherConfig with Assisted-SSB-MTC-Config, the UE would report the closest reference location via RRCReconfigurationComplete message. In our understanding, there is no issue to report via RRCReconfigurationComplete. However, the UE would also initiate a transmission of UAI to report the closest reference location upon receiving the configuration of the OtherConfig, which is redundant. As a result, the text proposal provided by C005 is preferred.

Proposal 2: [S024/C005/Z252] Adopt the proposed change of C005 (i.e., remove the “upon being configured to do so” triggered condition under the UAI).

- Conclusion

We discuss the open issues closest reference location reporting and have the following proposals:

Proposal 1: [H252] Align the description for including closest reference location report in RRCReconfigurationComplete and UAI as following:

2> if the UE is configured in this RRCReconfiguration message to provide location information for assisted SMTC configuration in RRC_CONNECTED state: 3> include the referenceLocationReport with a number of closest reference locations to the current UE’s position determined by closestLocsToReport;

Proposal 2: [S024/C005/Z252] Adopt the proposed change of C005 (i.e., remove the “upon being configured to do so” triggered condition under the UAI).

- Reference

- NTN Comments file v022

- NTN Review file v022