3GPP TSG-RAN WG2 Meeting #131bis	R2-2507544

Prague, Czech Republic, Oct. 13th-17th

Agenda Item:	10.4

Source: 	ASUSTeK

Title:  	Discussion on 6G Mobility design

Document for:	Discussion and Decision

- Introduction

In 6G SID [1], one objective is to study Mobility in 6GR:

- Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

In this contribution, we’d like to provide analysis for baseline Mobility design in 6G.

- discussion

6G Mobility in RRC_CONNECTED

Before studying 6GR mobility design in RRC_CONNECTED state, we first look into current 5G mobility. Currently, in addition to traditional RRC-signaling-initiated handover, 5G supports various alternatives to perform mobility in RRC_CONNECTED, which were established from different work items for different services:

  - DAPS handover

  - Conditional handover (CHO)

  - L1/L2-triggered Mobility (LTM) or conditional LTM

  - RACH-based and RACH-less handover

In the previous RAN1 meeting[2], it was agreed to study a scalable 6G design and study supported device types:

Agreement Study a scalable 6GR design for diverse device types, considering aspects: - What should be commonly applicable to all 6G device types - FFS: add-on features dedicated to specific device types, if any Agreement Study the device types from physical layer perspective to be supported by 6GR, subject to further discussion and confirmation in RAN.

From a scalability point of view, it is undesirable to introduce various mobility mechanisms since not all supported devices may contain the capability to support all mechanisms, and it is also not encouraged to provide multiple options to one functionality. Therefore, we propose not to support all mobility mechanisms from NR in 6G.

Proposal 1: For CONNECTED state mobility, considering the complexity of supporting various mobility mechanisms, not all mobility mechanisms specified in NR should be supported for 6G.

Regarding which mobility mechanisms to consider as a starting point for 6G, we propose to prioritize mechanisms which consumes less interruption time (e.g., CHO and LTM). In addition, we propose to continue supporting RACH-less mechanisms besides RACH-based mobility to further reduce time consumption.

Proposal 2: Aiming for less interruption time, (RACH-less/RACH-based) CHO and LTM specified in NR are considered as a starting point for 6G CONNECTED state mobility.

6G Mobility design in CHO and LTM enhancement

To further reduce latency and enhance accuracy and efficiency for 6G, first we propose to leverage AI for pre-configuration and resource reservation for CHO and LTM in 6G.

Proposal 3: Using AI to enhance the accuracy of pre-configuration and reserved resources for CHO and LTM should be studied in 6G.

Second, with UE-initiated L1/L2 measurement reporting, measurement triggering and reporting can be more efficient and accurate for NW to provide a more dynamic and suitable response to achieve efficient mobility procedures. In addition, AI can also be leveraged to enhance efficiency of measurement and reporting for mobility, which should be studied in 6G as well.

Proposal 4: Aiming for triggering mobility efficiently, UE initiated L1/2 measurement reporting should be considered as starting point for 6G.

Proposal 5: Using AI to enhance efficient measurement and reporting for mobility should be studied in 6G.

- Conclusion

We have the following observations and proposals for 6GR protocol design:

Proposal 1: For CONNECTED state mobility, considering the complexity of supporting various mobility mechanisms, not all mobility mechanisms specified in NR should be supported for 6G.

Proposal 2: Aiming for less interruption time, (RACH-less/RACH-based) CHO and LTM specified in NR are considered as a starting point for 6G CONNECTED state mobility.

Proposal 3: Using AI to enhance the accuracy of pre-configuration and reserved resources for CHO and LTM should be studied in 6G.

Proposal 4: Aiming for triggering mobility efficiently, UE initiated L1/2 measurement reporting should be considered as starting point for 6G.

Proposal 5: Using AI to enhance efficient measurement and reporting for mobility should be studied in 6G.

- Reference

[1] RP-251881

[2] Draft Report of 3GPP TSG RAN WG1 #122 v0.1.0