3GPP TSG-RAN WG2 Meeting #131bis	R2-2507581

Prague, Czech Republic, Oct 13th - 17th, 2025

Agenda item:		8.18.2

Source:		Samsung

Title:			RRC corrections on LTE-based 5G Broadcast

Document for:	Discussion & Decision

# 1 Introduction

In this paper, we address a missed scenario where the last MTCH has not applied time-interleaving but other scheduled MTCH services have time-interleaving enabled.

# 2 Discussion

M and/or N value for last MTCH service (residual space) may be less than other preceeding MTCH services of the time-interleaved MCH. Further, RAN1 parameter list also allows to have the time-interleaving disabled for the last MTCH service.

RAN 1 parameter list:

| pmch-TimeInterleaving-N-lastMTCH | New |  | This field indicated an additional optional TFI configuration for the last MTCH service (residual space) | {2,4,8,16} | The absent of this field indicates time-interleaving for these MTCHs is disabled, i.e., N=1 |
| --- | --- | --- | --- | --- | --- |

However, RRC spec is ambiguous as does not address/support the case 3

- Case 1: Last MTCH service applies same M and N as other scheduled MTCH services of the MCH (i.e., pmch-TimeInterleaving-N-lastMTCH is absent)

- Case 2: Last MTCH service applies M and/or N lesser than other scheduled MTCH services of the MCH (i.e., pmch-TimeInterleaving-N-lastMTCH is present)

- Case 3: Last MTCH has not applied time-interleaving but other scheduled MTCH services have time-interleaving enabled (i.e., pmch-TimeInterleaving-N-lastMTCH = 1 indicated)

We propose to address this missed case 3 as follows:

Proposal 1: RAN2 to adopt TP1 to address the missed case where:

- Last MTCH has not applied time-interleaving but other scheduled MTCH services have time-interleaving enabled (i.e., pmch-TimeInterleaving-N-lastMTCH = 1 indicated)

TP1

pmch-TimeInterleavingConfig-r19		SEQUENCE {

pmch-TimeInterleavingM-r19			ENUMERATED {sf4, sf8, sf16, sf32},

pmch-TimeInterleavingN-r19			ENUMERATED {n2, n4, n8, n16},

pmch-TimeInterleavingM-LastMTCH-r19	ENUMERATED {sf4, sf8, sf16, sf32}	OPTIONAL,	-- Need OR

pmch-TimeInterleavingN-LastMTCH-r19	ENUMERATED {n1, n2, n4, n8, n16}			OPTIONAL,	-- Need OR

pmch-SoftBufferSizeParameters-r19	PMCH-SoftBufferSizeParameters-r19,

pmch-CyclicShiftAlpha-r19 			ENUMERATED {alpha1, alpha2, alpha3}	OPTIONAL 	-- Need OR

}					OPTIONAL, -- Need OR

pmch-TimeInterleavingConfig Presence of the field indicates time interleaving is enabled as specified in TS 36.212 [22] and TS 36.213 [23].

pmch-TimeInterleavingM Indicates the separation, in number of MBSFN subframes not containing MCCH and MSI, between two successive transmissions of the same TB (except for the last MTCH service if pmch-TimeInterleavingM-LastMTCH is present) as specified in in TS 36.212 [22] and TS 36.213 [23] when time interleaving is enabled. Value sf4 indicates 4 subframes, value sf8 indicates 8 subframes and so on.

pmch-TimeInterleavingM-LastMTCH Indicates the separation, in number of MBSFN subframes not containing MCCH and MSI, between two successive transmissions of the same TB for the last MTCH service (residual space) as specified in TS 36.212 [22] and TS 36.213 [23] when time interleaving is enabled. Value sf4 indicates 4 subframes, value sf8 indicates 8 subframes and so on. If this field is absent and pmch-TimeInterleavingN-LastMTCH is absent, pmch-TimeInterleavingM applies also for the last MTCH service.

pmch-TimeInterleavingN Indicates the TBS scaling factor (except for the last MTCH service if pmch-TimeInterleavingN-LastMTCH is present) as specified in in TS 36.212 [22] and TS 36.213 [23] when time interleaving is enabled. Value n2 indicates scaling factor 2, value n4 indicates scaling factor 4 and so on.

pmch-TimeInterleavingN-LastMTCH Indicates the TBS scaling factor for the last MTCH service (residual space) as specified in in TS 36.212 [22] and TS 36.213 [23] when time interleaving is enabled. Value n2 indicates scaling factor 2, value n4 indicates scaling factor 4 and so on. Value n1 indicates time interleaving is not applied for last MTCH service. If this field is absent, pmch-TimeInterleavingN applies also for the last MTCH service.

# 3 Conclusion

Based on the discussion above, request RAN2 to consider the following proposal:

Proposal 1: RAN2 to adopt TP1 to address the missed case where:

- Last MTCH has not applied time-interleaving but other scheduled MTCH services have time-interleaving enabled (i.e., pmch-TimeInterleaving-N-lastMTCH = 1 indicated)