3GPP TSG RAN WG2 #131			R2-2507583

Prague, Czech Republic, October 13th – 17th, 2025

Agenda Item:	10.2

Source:	BT Plc, Orange, T-Mobile USA, Deutsche Telekom, Turkcell, Verizon, KDDI, Vodafone

Title:	Guidelines for 6G AI_ML for the air interface model delivery options

Document for:	Discussion/Decision

# Introduction

In the Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2 [1], the following is captured where RAN2 is the leading working group

- Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

Due to time constraints over the AI/ML for the air interface WID, RAN2 has had no time to properly study the model transfer.

In RAN2, the first discussion related to model transfer was done during RAN2#130 where the following TDocs where discussed.

R2-2504379	Discussion on AIML model transfer delivery	CMCC,China Telecom,CATT,ZTE,Apple,Samsung,Xiaomi	discussion	Rel-19	NR_AIML_air-Core

R2-2504641	Operators views for the AI_ML model delivery options	BT Plc, Turkcell, T-Mobile, Deutsche Telekom, Orange	discussion	Rel-19	R2-2504635 [2]

Both documents where noted

R2-2504379 Discussion on AIML model transfer delivery CMCC,China Telecom,CATT,ZTE,Apple,Samsung,Xiaomi discussion Rel-19 NR_AIML_air-Core Proposal 1: There is no requirements on controllability or visibility for UE-sided model transfer/delivery case y, since the model trained in OTT server can be sent to the UE directly - Tmobile disagrees and thinks its important to have visibility. How would you manage something for which you have no knowledge of. CMCC is not sure how to have visibility as this is going over OTT. Qualcomm thinks that the whole monitoring procedure is sufficient to check the performance of UE. - Mediatek thinks that we shouldn’t decide whether we will have same of different tunnel. Proposal 2: It is proposed to de-prioritize case z1 for one-sided model transfer/delivery, since there is no strong requirement on visibility and controllability for UE-sided model transfer/delivery from the perspective of operator. Proposal 3: It is proposed to follow case y for UE-sided model transfer/delivery in R19, i.e. the UE-sided model is delivered over the top, which is transparent to 3GPP. Noted R2-2504635 Operators views for the AI_ML model delivery options BT Plc, Turkcell, T-Mobile, Deutsche Telekom, Orange discussion Rel-19 => Revised in R2-504641 R2-2504641 Operators views for the AI_ML model delivery options BT Plc, Turkcell, T-Mobile, Deutsche Telekom, Orange discussion Rel-19 R2-2504635 Proposal 1: Solution 4a, OTT without 3GPP impact, captured on [1] is no further discussed in RAN2 Proposal 2: One-sided dataset/model transfer solution shall follow below principles: • A1 - Size: from RAN2 point of view, aim to support various sizes of model parameter transfer (FFS on model size); • A2 - Continuity: service continuity of model transfer/delivery during UE mobility needs to be supported; • A3 - Controllability: NW decides on if and when to transfer/delivery over the air interface; • A4 - Latency: relaxed latency requirement and infrequent update; • A5 - Visibility: dataset and model parameters to be visible and understandable by MNO; - Huawei asks which case you are referring to. BT explains is the model transfer to the UE. Qualcomm asks how do you control the equivalent of 1a over the top. BT explains that this beyond 1a. - Qualcomm asks how does it work, if the network configures you with inference config and you don’t have model you have to download. - Samsung, Apple, Xiaomi, Oppo, thinks that the discussion should focus on 2sided model and doesn’t see how the network knows whether the model is trusted. Apple explains that RAN1 already concluded to focus on 2sided. - Nokia, BT and Ericsson think controllability is important as the UE shouldn’t download a model if it is going to idle. Also it is strange how RAN1 can decide on data transfer that is not in their expertise in scope. Proposal 3: Adopt additional list of requirements for one-sided and two-sided model transfer/delivery. • Activation: once the AI/ML one-sided or two-sided model is transferred to NW and/or to UE, MNO should be capable to activate and deactivate them at any point in time • Model transfer/delivery: traffic should be transferred at a different priority, e.g., lower than user traffic. • Differentiability: model transfer/delivery traffic should be differentiated from other user traffic. • Security: there should be a guarantee that models are transferred securely, in a NW-aware manner, such that untrusted models cannot be downloaded. • Addressability: Models need to be addressable such that the UE can request the transfer/delivery of a specific one • Controllability: the NW is in control of if and when to transfer/deliver a model Noted

Due to the absence of any prior study in RAN2, a paper [3] co-signed by 11 operators, BT, T-Mobile USA, Deutsche Telekom, Turkcell, Orange, TIM, AT&T, Verizon, Docomo, KPN, and KDDI, and one RAN vendor, Nokia, was submitted for discussion at RAN2#131. Unfortunately, despite the high number of co-signers, no online time was allocated to discuss it.

Given the number of MNOs that consider this topic highly relevant for their networks, it is requested that the 6G 3GPP AI for air interface study includes model transfer.

# Discussion

## 3GPP solution for model transfer

3GPP RAN has spend a considerable amount of time discussing non-3GPP solutions during the AI/ML for the air interface SI and WI.

For 6G definition, it is expected that 3GPP discussion are focus on 3GPP solutions. Otherwise, valuable TUs are wasted and the workload increases without a justified reason.

- During 5G specification, 3GPP forum has used to many TUs to discuss non-3GPP solutions for the AI/ML for the air interface.

Since the OTT solution for AI/ML for the air interface has no impact on 3GPP specifications, there is no need to further discuss. Instead, 3GPP should focus all their efforts to study solutions that bring benefit and support future growth.

- RAN2 should focus the AI/ML for the air interface study on 3GPP solutions only.

## AI model transfer requirements

For operators, it is essential to have a standardized solution for AI model transfer. Managing the mobile network is part of the operators’ core business. Leaving the model transfer unspecified (OTA) may negatively impact network KPIs for several reasons. For example, the AI/ML model could be transferred during peak hours, introducing unnecessary congestion into the network. UEs at the cell edge or with low bandwidth configurations should skip the model transfer, as it would severely affect their power consumption. This operational knowledge is specific to the MNO, and an OTA solution is not aware of such conditions..

- The radio conditions when the model transfer is initiated are unknown for an OTT server.

There may be emergency situations in a given area, and it is well-known that such events lead to network congestion. If first responders are using the 3GPP network, the congestion becomes even more severe, leaving very limited resources available for other users. The knowledge of the emergency situation is restricted to the MNO. Therefore, attempting to transfer the AI model to UEs within the affected area would further exacerbate the congestion issue.

- Leaving transfer/delivery of the AI models unspecified by 3GPP will have harmful risks, including negative impacts on MNOs network KPIs.

These are just a few examples that highlight the importance of operator control over the decision of when the model should be transferred. Currently, both the model size and the periodicity of transfers are unknown. Given this uncertainty, MNOs will be required to adopt a cautious approach and assume model sizes could reach hundreds of megabytes.

The periodicity of model transfers to UEs is also undefined, meaning models could be transferred daily, weekly, monthly, or at other intervals. Moreover, the criteria used to determine when a model should be transferred remain outside the scope of 3GPP.

In summary, the MNO cannot control the model size, the transfer periodicity, or the criteria triggering the transfer. At the same time, the model owner cannot control network KPIs, congestion levels, UE radio configurations, or the radio conditions at the time of transfer. For all these reasons, it is essential that the MNO retains control over the AI model transfers.

The 6G Study item [4] should ensure this topic is studied in deep, considering it was skipped during 5G specification period.

- 3GPP AI model transfer solutions for the air interface should be deeply studied by 3GPP as part of 6G Study Item.

The principles identified by operators during the 5G specification needs to be addressed during the 6G study.

- The AI model transfer solution shall consider at least the below requirements:

- Size: from RAN2 point of view, aim to support various sizes of the model parameter transfer (FFS on model size);

- Continuity: service continuity of model transfer during UE mobility needs to be supported;

- Controllability: NW decides on if and when to transfer over the air interface;

- Latency: relaxed latency requirement and infrequent update;

## Principles to transfer/delivery the AI model

AI/ML functionality has the potential to optimize procedures by improving key aspects such as energy consumption and signaling overhead. To achieve this, well-established and tested procedures may be replaced. For example, traditional measurements could be avoided, along with the associated work already covered by RAN4 and RAN5 to ensure expected behavior. Instead, decisions will be made based on predictions. As with any prediction, there are inherent error margins. Well-trained models can reduce these margins to a level where expected performance is maintained. However, it is important to note that this represents the best-case scenario.

- AI/ML models may require deactivation if the end-to-end performance is impacted

Model traffic should be prioritized lower than user traffic to avoid impacting service quality. During peak hours or congestion, transferring large models could degrade user experience. Assigning lower priority ensures network stability and preserves resources for critical user and control-plane traffic.

Model transfer should be distinguishable from regular user traffic to enable optimized handling. Differentiation allows operators to apply specific policies, monitor performance, and manage resources effectively, ensuring that model delivery does not interfere with latency-sensitive or high-priority services.

Secure, network-aware model transfer is essential to prevent unauthorized or malicious models from being downloaded. Only trusted models should be delivered, ensuring integrity and protecting both the UE and network from potential threats or performance degradation caused by unverified AI behavior.

Models should be uniquely addressable so that UEs can request specific versions. This enables targeted updates, supports model lifecycle management, and avoids unnecessary transfers. Addressability ensures that only relevant models are delivered, reducing overhead and improving operational efficiency

- RAN2 needs to perform the study based on at least the following principles for the AI model transfer:

- Model transfer/delivery: traffic should be transferred at a different priority, e.g., lower than user traffic.

- Differentiability: model transfer/delivery traffic should be differentiated from other user traffic.

- Security: there should be a guarantee that models are transferred securely, in a NW-aware manner, such that untrusted models cannot be downloaded.

- Addressability: Models need to be addressable such that the UE can request the transfer/delivery of a specific one

# Conclusions

In this TDoc, we have made the following observations:

Observation 1	During 5G specification, 3GPP forum has used to many TUs to discuss non-3GPP solutions for the AI/ML for the air interface.

Observation 2	The radio conditions when the model transfer is initiated are unknown for an OTT server.

Observation 3	Leaving transfer/delivery of the AI models unspecified by 3GPP will have harmful risks, including negative impacts on MNOs network KPIs.

Observation 4	AI/ML models may require deactivation if the end-to-end performance is impacted

For those reasons, we propose the following:

Proposal 1	RAN2 should focus the AI/ML for the air interface study on 3GPP solutions only.

Proposal 2	3GPP AI model transfer solutions for the air interface should be deeply studied by 3GPP as part of 6G Study Item.

Proposal 3	The AI model transfer solution shall consider at least the below requirements:

	Size: from RAN2 point of view, aim to support various sizes of the model parameter transfer (FFS on model size);

	Continuity: service continuity of model transfer during UE mobility needs to be supported;

	Controllability: NW decides on if and when to transfer over the air interface;

	Latency: relaxed latency requirement and infrequent update;

Proposal 4	RAN2 needs to perform the study based on at least the following principles for the AI model transfer:

	Model transfer/delivery: traffic should be transferred at a different priority, e.g., lower than user traffic.

	Differentiability: model transfer/delivery traffic should be differentiated from other user traffic.

	Security: there should be a guarantee that models are transferred securely, in a NW-aware manner, such that untrusted models cannot be downloaded.

	Addressability: Models need to be addressable such that the UE can request the transfer/delivery of a specific one

# References

- RP-243245 - New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2

- R2-2504635 – RAN2#130 – Operators views for the AI_ML model delivery options

- R2-2506169 – RAN2#131 – Operators views for the AI_ML model delivery options

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO, 3GPP TSG RAN#108, Prague, Czech Republic, June 9-13, 2025