3GPP TSG-WG2 Meeting #131-bis	R2-2507603

Prague, Czech Republic, October 13 – 17, 2025

Agenda Item:	8.20.1

Source:	Ericsson

Title:	Backwards compatibility for low NR band carrier aggregation switching

Document for:	Discussion, Decision

# 1	Introduction

An LS from RAN4 (R4-2511863) has provided input on low NR band CA switching (FDD+SDL band where the bands cannot be used simultaneously) and its impact to band combinations. This contribution discusses this input and proposes how to handle the backwards compatibility aspect.

# 2	Discussion

In RAN2#130 backwards compatibility was discussed for low band carrier switching - if this feature would be defined for a band combination deployed in the field, a network that does not understand this feature would consider that the UE supports regular CA for such band combination (i.e. simultaneous transmission on FDD and SDL) while the UE would only be able to support low band carrier switching for such band combination (i.e. transmission on either FDD or SDL).

For similar issues in the past a separate band combination list was introduced (as for the case of UL tx switching). However, this approach is costly since it adds complexity to handle multiple band combination lists, the fallback relation is not clear between the multiple band combinations and the overhead is larger. It is also not scalable to add parallel band combination lists for every feature that may have a risk of being non-backwards compatible. Lastly, it is not a single time effort since those lists need also maintenance to always refer to IEs defined by the usual band combination list. Hence, a simpler approach must be adopted.

Introducing a separate band combination list to ensure backwards compatibility has multiples issues and a simpler approach must be adopted.

In our view, it would be simpler to adopt a similar approach as done for NR-DC where an alternative FeatureSetCombinationId is defined (featureSetCombinationDC). With another FeatureSetCombinationId, a UE supporting a regular CA can indicate in the usual FeatureSetCombinationId support of both FDD and SDL bands simultaneously, while a UE supporting only low band carrier switching could indicate in the usual FeatureSetCombinationId support for e.g. FDD band while including FeatureSetDownlinkId set to 0 for the SDL band. This UE supporting an alternative FeatureSetCombinationId could then include support for both FDD and SDL carriers in this FeatureSetCombination.

Introduce an alternative FeatureSetCombinationId for a UE supporting low band carrier switching.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Introducing a separate band combination list to ensure backwards compatibility has multiples issues and a simpler approach must be adopted.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Introduce an alternative FeatureSetCombinationId for a UE supporting low band carrier switching.