3GPP TSG-RAN WG2 Meeting #131bis		R2-2507728

Prague, Czech Republic, 13th – 17th Oct., 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2123 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections on MAC for Mob Ph4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo (Rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-10-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TBD |  |  |  |  |  |  |  |  |
| Summary of change: |  | - TBD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TBD |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1xxxx |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

## 5.2b	Maintenance of UL Synchronization for CLTM candidate cell

The MAC entity shall for each CLTM candidate cell:

1>	when an LTM Candidate Timing Advance Command MAC CE described in clause 6.1.3.4b is received:

2>	if two TAGs are configured for the CLTM candidate cell:

3>	store the TA value in the LTM Candidate Timing Advance Command MAC CE for the indicated CLTM candidate cell for the indicated TAG as specified in clause 6.1.3.4b;

3>	start or restart the ltm-Candidate-TimeAlignmentTimer or ltm-Candidate-TimeAlignmentTimerTAG2 associated with the indicated LTM candidate cell for the indicated TAG as specified in clause 6.1.3.4b;

2>	else:

3>	store the TA value in the LTM Candidate Timing Advance Command MAC CE for the indicated CLTM candidate cell as specified in clause 6.1.3.4b;

3>	start or restart the ltm-Candidate-TimeAlignmentTimer associated with the indicated LTM candidate cell as specified in clause 6.1.3.4b.

NOTE:	If the UE receives more TA values than it can store, it is up to the UE implementation which TA values to store or discard.

1>	when the CLTM candidate configuration(s) or all execution condition(s) for a CLTM candidate configuration is released as specified in TS 38.331 [5]:

2>	stop the running ltm-Candidate-TimeAlignmentTimer and ltm-Candidate-TimeAlignmentTimerTAG2 associated with the corresponding CLTM candidate cell(s), if any;

2>	release the stored TA value for the corresponding CLTM candidate cell(s), if any.

Next change

### 5.4.4	Scheduling Request

The Scheduling Request (SR) is used for requesting UL-SCH resources for new transmission.

The MAC entity may be configured with zero, one, or more SR configurations. An SR configuration consists of a set of PUCCH resources for SR across different BWPs and cells. For a logical channel or for SCell beam failure recovery (see clause 5.17) and for consistent LBT failure recovery (see clause 5.21), at most one PUCCH resource for SR is configured per BWP. For a logical channel serving a radio bearer configured with SDT, PUCCH resource for SR is not configured for SDT. For beam failure recovery of BFD-RS set(s) of Serving Cell, up to two PUCCH resources for SR are configured per BWP. For positioning measurement gap activation/deactivation request, a dedicated SR configuration is configured. For event triggered L1 measurement report, a dedicated SR configuration may be configured.

Each SR configuration corresponds to one or more logical channels and/or to SCell beam failure recovery and/or to consistent LBT failure recovery and/or to beam failure recovery of a BFD-RS set and/or to positioning measurement gap activation/deactivation request and/or to event triggered L1 measurement report. Each logical channel, SCell beam failure recovery, beam failure recovery of a BFD-RS set and consistent LBT failure recovery, and event triggered L1 measurement report, may be mapped to zero or one SR configuration, which is configured by RRC. The SR configuration of the logical channel that triggered a BSR (clause 5.4.5) or a DSR (clause 5.4.9) or the SCell beam failure recovery or the beam failure recovery of a BFD-RS set or the consistent LBT failure recovery (clause 5.21) (if such a configuration exists) or positioning measurement gap activation/deactivation request (clause 5.25) or event triggered L1 measurement report (clause 5.35) is considered as corresponding SR configuration for the triggered SR. Any SR configuration may be used for an SR triggered by Pre-emptive BSR (clause 5.4.7) or Timing Advance reporting (clause 5.4.8).

RRC configures the following parameters for the scheduling request procedure:

-	sr-ProhibitTimer (per SR configuration);

-	sr-TransMax (per SR configuration).

The following UE variables are used for the scheduling request procedure:

-	SR_COUNTER (per SR configuration).

If an SR is triggered and there are no other SRs pending corresponding to the same SR configuration, the MAC entity shall set the SR_COUNTER of the corresponding SR configuration to 0.

When an SR is triggered, it shall be considered as pending until it is cancelled.

All pending SR(s) for BSR triggered according to the BSR procedure (clause 5.4.5) prior to the MAC PDU assembly shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the MAC PDU is transmitted and this PDU includes a Long, Refined Long or Short BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR (see clause 5.4.5) prior to the MAC PDU assembly. All pending SR(s) for BSR triggered according to the BSR procedure (clause 5.4.5) shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the UL grant(s) can accommodate all pending data available for transmission.

The MAC entity shall for each pending SR not triggered according to the BSR procedure (clause 5.4.5) for a Serving Cell:

1>	if this SR was triggered by Pre-emptive BSR procedure (see clause 5.4.7) prior to the MAC PDU assembly and a MAC PDU containing the relevant Pre-emptive BSR MAC CE is transmitted; or

1>	if this SR was triggered by beam failure recovery (see clause 5.17) of an SCell and a MAC PDU is transmitted and this PDU includes a MAC CE for BFR which contains beam failure recovery information for this SCell; or

1>	if this SR was triggered by beam failure recovery (see clause 5.17) for a BFD-RS set of a Serving Cell and a MAC PDU is transmitted and this PDU includes an Enhanced BFR MAC CE or a Truncated Enhanced BFR MAC CE which contains beam failure recovery information for this BFD-RS set of the Serving Cell; or

1>	if this SR was triggered by beam failure recovery (see clause 5.17) of an SCell and this SCell is deactivated (see clause 5.9); or

1>	if this SR was triggered by beam failure recovery (see clause 5.17) for a BFD-RS set of an SCell and this SCell is deactivated (see clause 5.9); or

1>	if the SR is triggered by positioning measurement gap activation/deactivation request (see clause 5.25) and the Positioning Measurement Gap Activation/Deactivation Request MAC CE that triggers the SR has already been cancelled; or

1>	if this SR was triggered by consistent LBT failure recovery (see clause 5.21) of an SCell and a MAC PDU is transmitted and the MAC PDU includes an LBT failure MAC CE that indicates consistent LBT failure for this SCell; or

1>	if this SR was triggered by consistent LBT failure recovery (see clause 5.21) of an SCell and all the triggered consistent LBT failure(s) for this SCell are cancelled; or

1>	if this SR was triggered by Timing Advance reporting (see clause 5.4.8) and all the triggered Timing Advance reports are cancelled; or

1>	if this SR was triggered by DSR procedure (see clause 5.4.9) and the DSR that triggered the SR has been cancelled; or

1>	if this SR was triggered by event triggered L1 measurement report procedure (see clause 5.35) and the event triggered L1 measurement report that triggered the SR has been cancelled:

2>	cancel the pending SR and stop the corresponding sr-ProhibitTimer, if running.

Only PUCCH resources on a BWP which is active at the time of SR transmission occasion are considered valid.

As long as at least one SR is pending, the MAC entity shall for each pending SR:

1>	if the MAC entity has no valid PUCCH resource configured for the pending SR; and

1>	if there is no ongoing RACH-less LTM cell switch; and

1>	if rach-LessHO is not configured:

2>	initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel the pending SR.

1>	else, for the SR configuration corresponding to the pending SR:

2>	when the MAC entity has an SR transmission occasion on the valid PUCCH resource for SR configured; and

2>	if sr-ProhibitTimer is not running at the time of the SR transmission occasion; and

2>	if the PUCCH resource for the SR transmission occasion does not overlap with a measurement gap:

3>	if the PUCCH resource for the SR transmission occasion does not overlap with any of a UL-SCH resource whose simultaneous transmission with the SR is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup, an SL-SCH resource, or an SL-PRS resource; or

3>	if the MAC entity is able to perform this SR transmission simultaneously with the transmission of the SL-SCH resource; or

3>	if the MAC entity is configured with lch-basedPrioritization, and the PUCCH resource for the SR transmission occasion does not overlap with the PUSCH duration of an uplink grant received in a Random Access Response or with the PUSCH duration of an uplink grant addressed to Temporary C-RNTI or with the PUSCH duration of a MSGA payload, and the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.4.5 overlaps with any other UL-SCH resource(s), and the physical layer can signal the SR on one valid PUCCH resource for SR, and the priority of the logical channel that triggered SR is higher than the priority of the uplink grant(s) for any UL-SCH resource(s) where the uplink grant was not already de-prioritized and its simultaneous transmission with the SR is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCHgroups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup, and the priority of the uplink grant is determined as specified in clause 5.4.1; or

3>	if both sl-PrioritizationThres and ul-PrioritizationThres are configured and the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.22.1.5 overlaps with any UL-SCH resource(s) carrying a MAC PDU, and the value of the priority of the triggered SR determined as specified in clause 5.22.1.5 is lower than sl-PrioritizationThres and the value of the highest priority of the logical channel(s) in the MAC PDU is higher than or equal to ul-PrioritizationThres and any MAC CE prioritized as described in clause 5.4.3.1.3 is not included in the MAC PDU and the MAC PDU is not prioritized by upper layer according to TS 23.287 [19]; or

3>	if an SL-SCH resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.4.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-SCH resource, and either transmission on the SL-SCH resource is not prioritized as described in clause 5.22.1.3.1a or the priority value of the logical channel that triggered SR is lower than ul-PrioritizationThres, if configured; or

3>	if an SL-SCH resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.22.1.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-SCH resource, and the priority of the triggered SR determined as specified in clause 5.22.1.5 is higher than the priority of the MAC PDU determined as specified in clause 5.22.1.3.1a for the SL-SCH resource; or

3>	if an SL-PRS resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.4.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-PRS resource, and either transmission on the SL-PRS resource is not prioritized as described in clause 5.22.1.3.1a or in the clause 5.22.1.3.5, or the priority value of the logical channel that triggered SR is lower than ul-PrioritizationThres, if configured; or

3>	if an SL-PRS resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.22.1.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-PRS resource, and the priority of the triggered SR determined as specified in clause 5.22.1.5 is higher than the priority of the MAC PDU and SL-PRS, if available, determined as specified in clause 5.22.1.3.1a or the SL-PRS resource in clause 5.22.1.3.5:

4>	consider the SR transmission as a prioritized SR transmission.

4>	consider the other overlapping uplink grant(s), if any, as a de-prioritized uplink grant(s), except for the overlapping uplink grant(s) whose simultaneous transmission is allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup;

4>	if the de-prioritized uplink grant(s) is a configured uplink grant configured with autonomousTx whose PUSCH has already started:

5>	stop the configuredGrantTimer for the corresponding HARQ process of the de-prioritized uplink grant(s);

5>	stop the cg-RetransmissionTimer for the corresponding HARQ process of the de-prioritized uplink grant(s).

4>	if SR_COUNTER < sr-TransMax:

5>	instruct the physical layer to signal the SR on one valid PUCCH resource for SR;

5>	if LBT failure indication is not received from lower layers:

6>	increment SR_COUNTER by 1;

6>	start the sr-ProhibitTimer.

5>	else if lbt-FailureRecoveryConfig is not configured:

6>	increment SR_COUNTER by 1.

4>	else:

5>	notify RRC to release PUCCH for all Serving Cells;

5>	notify RRC to release SRS for all Serving Cells;

5>	clear any configured downlink assignments and uplink grants;

5>	clear any PUSCH resources for semi-persistent CSI reporting;

5>	if rach-LessHO is not configured and if there is no ongoing RACH-less LTM cell switch:

6>	initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel all pending SRs.

3>	else:

4>	consider the SR transmission as a de-prioritized SR transmission.

NOTE 1:	Except for the cases specified in NOTE 3 below, the selection of which valid PUCCH resource for SR to signal SR on when the MAC entity has more than one overlapping valid PUCCH resource for the SR transmission occasion is left to UE implementation.

NOTE 2:	If more than one individual SR triggers an instruction from the MAC entity to the PHY layer to signal the SR on the same valid PUCCH resource, the SR_COUNTER for the relevant SR configuration is incremented only once.

NOTE 3:	When the MAC entity has pending SR for SCell beam failure recovery and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery of a BFD-RS set or for event triggered L1 measurement report) overlapping with PUCCH resource for SCell beam failure recovery for the SR transmission occasion, the MAC entity considers only the PUCCH resource for SCell beam failure recovery as valid. When the MAC entity has pending SR for beam failure recovery of a BFD-RS set of Serving Cell and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery or for event triggered L1 measurement report) overlapping with PUCCH resource for beam failure recovery of that BFD-RS set for the SR transmission occasion, the MAC entity considers only the PUCCH resource for beam failure recovery of that BFD-RS set as valid. When the MAC entity has pending SR for event triggered L1 measurement report and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery or for beam failure recovery of a BFD-RS set) overlapping with PUCCH resource for event triggered L1 measurement report for the SR transmission occasion, the MAC entity considers only the PUCCH resource for event triggered L1 measurement report as valid.

NOTE 4:	For a UE operating in a semi-static channel access mode as described in TS 37.213 [18], PUCCH resources overlapping with the set of consecutive symbols where the UE does not transmit before the start of a next channel occupancy time are not considered valid.

NOTE 5:	If the MAC entity is configured with lch-basedPrioritization, the MAC entity does not take UCI multiplexing according to the procedure specified in TS 38.213 [6] into account when determining whether the valid PUCCH resource for the SR transmission can be signalled by the physical layer and the SR transmission occasion overlaps with the PUSCH duration of an uplink grant of a MSGA payload.

NOTE 6:	When the MAC entity has overlapping PUCCH resource for pending SR for SCell beam failure recovery,  PUCCH resource for pending SR for beam failure recovery of a BFD-RS set, and PUCCH resource for pending SR for event triggered L1 measurement report for the SR transmission occasion, it is up to UE implementation to select PUCCH resource for SCell beam failure recovery or PUCCH resource for beam failure recovery of a BFD-RS set or PUCCH resource for event triggered L1 measurement report.

NOTE 7:	If an SL-SCH resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered by Uu MAC CEs except BSR/SL-BSR MAC CE, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-SCH resource, it is left to UE implementation to determine whether this SR transmission is prioritized over the SL transmission.

The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for BSR, which was initiated by the MAC entity prior to the MAC PDU assembly and which has no valid PUCCH resources configured, if:

-	a MAC PDU is transmitted using a UL grant other than a UL grant provided by Random Access Response or a UL grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload, and this PDU includes a BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR (see clause 5.4.5) prior to the MAC PDU assembly; or

-	the UL grant(s) can accommodate all pending data available for transmission.

The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for SL-BSR, which has no valid PUCCH resources configured, if:

-	a MAC PDU is transmitted using a UL grant other than a UL grant provided by Random Access Response or a UL grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload, and the ongoing Random Access procedure was initiated by the MAC entity prior to the MAC PDU assembly, and this PDU includes an SL-BSR MAC CE which contains buffer status up to (and including) the last event that triggered an SL-BSR (see clause 5.22.1.6) prior to the MAC PDU assembly; or

-	the SL grant(s) can accommodate all pending data available for transmission, and the ongoing Random Access procedure was initiated by the MAC entity prior to the sidelink MAC PDU assembly.

The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for SL-CSI reporting, which has no valid PUCCH resources configured, if:

-	the SL grant can accommodate SL-CSI reporting MAC CE for transmission.

The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for SL-DRX command indication, which has no valid PUCCH resources configured, if:

-	the SL grant can accommodate SL-DRX command indication for transmission.

The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for BFR of an SCell, which has no valid PUCCH resources configured, if:

-	a MAC PDU is transmitted using a UL grant other than a UL grant provided by Random Access Response or a UL grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload, and this PDU contains a MAC CE for BFR which includes beam failure recovery information of that SCell; or

-	the SCell is deactivated (as specified in clause 5.9) and all triggered BFRs for SCells are cancelled.

The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for BFR of a BFD-RS set of a Serving Cell, which has no valid PUCCH resources configured, if:

-	a MAC PDU is transmitted using a UL grant other than a UL grant provided by Random Access Response or a UL grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload, and this PDU contains an Enhanced BFR MAC CE or a Truncated Enhanced BFR MAC CE which includes beam failure recovery information of that BFD-RS set of the Serving Cell.

The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for consistent LBT failure recovery, which has no valid PUCCH resources configured, if:

-	a MAC PDU is transmitted using a UL grant other than a UL grant provided by Random Access Response or a UL grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload, and this PDU includes an LBT failure MAC CE that indicates consistent LBT failure for all the SCells that triggered consistent LBT failure; or

-	all the SCells that triggered consistent LBT failure recovery are deactivated (see clause 5.9).

The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for Sidelink consistent LBT failure recovery, which has no valid PUCCH resources configured, if one of the following conditions is met:

-	a MAC PDU is transmitted using a UL grant other than a UL grant provided by Random Access Response or a UL grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload, and this PDU includes an SL LBT failure MAC CE that indicates Sidelink consistent LBT failure; or

-	all the triggered Sidelink consistent LBT failure recovery are cancelled (see clause 5.31.2).

The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for positioning measurement gap activation/deactivation request, which has no valid PUCCH resources configured, if:

-	the Positioning Measurement Gap Activation/Deactivation Request MAC CE that triggers the SR corresponding to the Random Access procedure has already been cancelled.

The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for Timing Advance report, which has no valid PUCCH resources configured, if:

-	a MAC PDU is transmitted using a UL grant other than a UL grant provided by Random Access Response or a UL grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload, and this PDU includes a Timing Advance Report MAC CE (see clause 5.4.8).

The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for DSR, which has no valid PUCCH resources configured, if:

-	a MAC PDU is transmitted using a UL grant other than a UL grant provided by Random Access Response or a UL grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload, and this PDU includes either a DSR MAC CE or all the PDCP SDUs associated with the DSR (see clause 5.4.9); or

-	all the PDCP SDUs associated with the DSR have been discarded (see clause 5.4.9).

The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for SL-PRS Resource Request, which has no valid PUCCH resources configured, if:

-	a MAC PDU is transmitted using a UL grant other than a UL grant provided by Random Access Response or a UL grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload, and this PDU includes a SL-PRS Resource Request MAC CE (see clause 5.22.1.12).

The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for Event Triggered L1 Measurement Report, which has no valid PUCCH resources configured, if:

-	a MAC PDU is transmitted using a UL grant other than a UL grant provided by Random Access Response or a UL grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload, and this PDU includes a (Truncated) Event Triggered L1 Measurement Report MAC CE (see clause 5.35).

Next change

## 5.12	MAC Reset

If a reset of the MAC entity is requested by upper layers upon receiving RRCResume or RRCSetup, the MAC entity shall:

1>	stop the MBS multicast DRX timers;

1>	flush the soft buffers for all DL HARQ processes used for MBS multicast;

1>	for each DL HARQ process used for MBS multicast, consider the next received transmission for a TB as the very first transmission.

Otherwise, if a reset of the MAC entity is requested by upper layers or the reset of the MAC entity is triggered due to SCG deactivation as defined in clause 5.29, the MAC entity shall:

1>	if the MAC reset is not due to SCG deactivation:

2>	initialize Bj for each logical channel to zero;

1>	initialize SBj for each logical channel to zero if Sidelink resource allocation mode 1 is configured by RRC;

1>	if upper layers indicate SCG deactivation and bfd-and-RLM with value true is configured for the deactivated SCG:

2>	stop (if running) all timers except beamFailureDetectionTimer associated with PSCell and timeAlignmentTimers.

1>	else if upper layers indicate the reset is triggered by reconfiguration with sync:

2>	stop (if running) all timers, except MBS broadcast DRX timers, ltm-Candidate-TimeAlignmentTimer, and ltm-Candidate-TimeAlignmentTimerTAG2, if configured;

2>	consider all timeAlignmentTimers as expired and perform the corresponding actions in clause 5.2;

1>	else:

2>	stop (if running) all timers, except MBS broadcast DRX timers;

2>	consider all timeAlignmentTimers, inactivePosSRS-TimeAlignmentTimer, and cg-SDT-TimeAlignmentTimer, if configured, as expired and perform the corresponding actions in clause 5.2;

1>	set the NDIs for all uplink HARQ processes to the value 0;

1>	sets the NDIs for all HARQ process IDs to the value 0 for monitoring PDCCH in Sidelink resource allocation mode 1;

1>	stop, if any, ongoing Random Access procedure;

1>	discard explicitly signalled contention-free Random Access Resources for 4-step RA type and 2-step RA type, if any, except the contention-free Random Access Resources signalled in the LTM Cell Switch Command;

1>	flush Msg3 buffer;

1>	flush MSGA buffer;

1>	cancel, if any, triggered Scheduling Request procedure;

1>	cancel, if any, triggered Buffer Status Reporting procedure;

1>	cancel, if any, triggered Delay Status Reporting procedure;

1>	cancel, if any, triggered Power Headroom Reporting procedure;

1>	cancel, if any, triggered consistent LBT failure;

1>	cancel, if any, triggered Sidelink consistent LBT failure;

1>	cancel, if any, triggered BFR;

1>	cancel, if any, triggered Sidelink Buffer Status Reporting procedure;

1>	cancel, if any, triggered Pre-emptive Buffer Status Reporting procedure;

1>	cancel, if any, triggered Timing Advance Reporting procedure;

1>	cancel, if any, triggered Recommended bit rate query procedure;

1>	cancel, if any, triggered UL Rate Control procedure;

1>	cancel, if any, triggered Configured uplink grant confirmation;

1>	cancel, if any, triggered configured sidelink grant confirmation;

1>	clear, if any, configured sidelink grants;

1>	cancel, if any, triggered Desired Guard Symbol query;

1>	cancel, if any, triggered Positioning Measurement Gap Activation/Deactivation Request procedure;

1>	cancel, if any, triggered SDT procedure;

1>	cancel, if any, triggered IAB-MT Recommended Beam Indication query;

1>	cancel, if any, triggered Desired DL TX Power Adjustment query;

1>	cancel, if any, triggered Desired IAB-MT PSD range query;

1>	cancel, if any, triggered Case-6 Timing Request query;

1>	cancel, if any, triggered SL-PRS resource request;

1>	cancel, if any, triggered Event Triggered L1 Measurement Report;

1>	flush the soft buffers for all DL HARQ processes, except for the DL HARQ process being used for MBS broadcast;

1>	for each DL HARQ process, except for the DL HARQ process being used for MBS broadcast, consider the next received transmission for a TB as the very first transmission;

1>	release, if any, Temporary C-RNTI;

1>	clear, if any, Differential Koffset;

1>	if upper layers indicate SCG deactivation and bfd-and-RLM with value true is not configured; or

1>	if the MAC reset is not due to SCG deactivation:

2>	reset all BFI_COUNTERs;

1>	reset all LBT_COUNTERs;

1>	reset TTT for event triggered L1 measurement report triggering condition evaluation;

1>	reset all MR_SENT_COUNTER;

1>	clear all BEAM_ENTERING_LIST;

1>	clear all BEAM_LEAVING_LIST;

1>	clear all BEAM_REPORTED_LIST;

1>	clear all MR_LIST.

If a Sidelink specific reset of the MAC entity is requested for a PC5-RRC connection by upper layers, the MAC entity shall:

1>	flush the soft buffers for all Sidelink processes for all TB(s) associated to the PC5-RRC connection;

1>	consider all Sidelink processes for all TB(s) associated to the PC5-RRC connection as unoccupied;

1>	cancel, if any, triggered Scheduling Request procedure only associated to the PC5-RRC connection;

1>	cancel, if any, triggered Sidelink Buffer Status Reporting procedure only associated to the PC5-RRC connection;

1>	cancel, if any, triggered SL-PRS resource request only associated to the PC5-RRC connection;

1>	cancel, if any, triggered Sidelink CSI Reporting procedure associated to the PC5-RRC connection;

1>	cancel, if any, triggered Sidelink DRX Command Indication procedure associated to the PC5-RRC connection;

1>	cancel, if any, triggered Sidelink IUC-Request transmission procedure associated to the PC5-RRC connection;

1>	cancel, if any, triggered Sidelink IUC-Information Reporting procedure associated to the PC5-RRC connection;

1>	stop (if running) all timers associated to the PC5-RRC connection;

1>	reset the numConsecutiveDTX associated to the PC5-RRC connection;

1>	initialize SBj for each logical channel associated to the PC5-RRC connection to zero.

Next change

### 5.35.1	Introduction

The network may configure an RRC_CONNECTED UE to perform L1 beam level measurements for LTM candidate cell(s) and/or serving cell, and report the corresponding measurement results in accordance with the event triggered L1 measurement configuration. The measurement report is used for indicating to serving gNB of the L1 measurement results from the serving cell and/or candidate cell(s). The measurement configuration is provided by means of RRC dedicated signalling.

The network may configure the UE to report the following measurement information based on SS/PBCH block(s):

-	Measurement results per SS/PBCH block;

-	SS/PBCH block(s) resource indicator (SSBRI).

The network may configure the UE to report the following measurement information based on CSI-RS resources:

-	Measurement results per CSI-RS resource;

-	CSI-RS resource indicator (CRI).

RRC configures the following parameters in the LTM-CSI-ReportConfig for L1 measurement and event triggered L1 measurement reporting:

-	ltm-ResourcesForChannelMeasurement for the LTM resource configuration containing the RS(s) of LTM candidate cell(s) that may be measured for the event;

-	eventTriggered for the event-triggered measurement report;

-	eventLTM2, eventLTM3, eventLTM4, eventLTM5: events for the event-triggered measurement report;

-	timeToTrigger (TTT): time during which an entering/leaving condition needs to be consistently satisfied for reporting event triggered L1 measurement report or for cell switch execution to be met;

-	ltm-CandidateReportConfigList: List of report configurations for LTM candidate IDs;

-	ltm-EventTriggeredPeriodicReport: whether the event triggered L1 measurement report is sent periodically if an LTM event is triggered;

-	reportOnLeave: whether the event triggered L1 measurement report shall be triggered when the leaving condition for an event is satisfied;

-	ltm-EventTriggeredReportReportContent: the content of the event triggered L1 measurement report;

-	candidateSpecificOffset: offset for event condition that is applicable for all the reference signals belonging to the candidate cell with the candidate cell ID ltm-CandidateReportConfigId;

-	servingSpecificOffset: offset for event condition that is applicable for all the reference signals belonging to the serving cell with the candidate cell ID ltm-CandidateReportConfigId.

For L1 measurement and event triggered L1 measurement reporting, if an ltm-CSI-ReportConfig or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig is removed or modified from the current UE configuration, as specified in TS 38.331 [5], the MAC entity shall:

1>	remove the measurement reporting entry for the corresponding ltm-CSI-ReportConfigId from the MR_LIST, if included:

1>	stop the periodical reporting timer, if running, and reset the associated information (e.g. TTT and variables defined in 5.35.3) for the corresponding ltm-CSI-ReportConfigId.

Next change

### 5.35.3	Measurement report triggering

#### 5.35.3.1	General

The UE maintains the following UE variables for event triggered L1 measurement and report procedure:

-	MR_LIST: includes the list of ltm-CSI-ReportConfigId for which the L1 measurement report entering or leaving condition has been met for TTT for at least one applicable RS (of an LTM candidate cell) and, for eachltm-CSI-ReportConfigId, the RS resource index, and L1 measurement result of each measured applicable RS;

-	MR_SENT_COUNTER: the number of event triggered L1 measurement report(s) sent by UE if a triggering condition for the corresponding event is met for TTT for each ltm-CSI-ReportConfigId;

-	BEAM_ENTERING_LIST: list of RS resource index(es) of applicable RS(s) for each ltm-CSI-ReportConfigId, for which the L1 measurement report entering condition has been met for TTT;

-	BEAM_LEAVING_LIST: list of RS resource index(es) of applicable RS(s) for each ltm-CSI-ReportConfigId, which have been reported in the (Truncated) L1 measurement report MAC CE, and for which the L1 measurement report leaving condition has been met for TTT;

-	BEAM_REPORTED_LIST: list of RS resource index(es) of applicable RS(s) for each ltm-CSI-ReportConfigId, which have been reported in the (Truncated) L1 measurement report MAC CE, and the L1 measurement report leaving condition has not been met for TTT.

Unless explicitly specified otherwise, it is up to UE implementation how to store these variables.

The MAC entity shall:

1>	for each ltm-CSI-ReportConfigId included in the SpCell for L1 measurement and event triggered measurement report:

2>	if the corresponding ltm-ReportConfigType is set to eventTriggered and there is ltm-EventTriggeredReportContent configuration:

3>	if the eventLTM2 is configured in the corresponding ltm-CSI-ReportConfigId:

4>	if the associated LTM-CSI-ResourceConfig includes ltm-NZP-CSI-RS-ResourceSet:

5>	consider the current beam of serving cell, i.e. the RS configured in the indicated TCI State of the SpCell, as defined in clause 5.1.5 in TS 38.214 [7], to be applicable.

4>	else:

5>	consider the current beam of serving cell, i.e. the SSB QCLed with the RS configured in the indicated TCI State of the SpCell, as defined in clause 5.1.5 in TS 38.214 [7], to be applicable.

3>	if the eventLTM3, eventLTM4, or eventLTM5 is configured in the corresponding ltm-CSI-ReportConfigId:

4>	if ltm-CandidateReportConfigList is configured:

5>	consider any RS configured in the associated LTM-CSI-ResourceConfig of an LTM candidate cell other than the SpCell listed in ltm-CandidateReportConfigList, to be applicable.

4>	else:

5>	consider any RS configured in the associated LTM-CSI-ResourceConfig of an LTM candidate cell other than the SpCell listed, to be applicable.

2>	if the entry condition for the event associated with ltm-CSI-ReportConfigId is fulfilled for the measurement from lower layer during TTT for one or more applicable RSs, which is not in the BEAM_ENTERING_LIST and not in the BEAM_REPORTED_LIST:

3>	if the MR_LIST does not include a measurement reporting entry for the ltm-CSI-ReportConfigId (a first RS triggers the event):

4>	include a measurement reporting entry in the MR_LIST for this ltm-CSI-ReportConfigId;

3> for each applicable RS for which the entry condition is fulfilled during TTT and that is not in BEAM_LEAVING_LIST:

4>	include the SSBRI or CRI of the concerned RS in the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId;

3>	for each applicable RS for which the entry condition is fulfilled during TTT and that is in BEAM_LEAVING_LIST:

4>	remove the concerned RS in the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId;

4> if the SSBRI or CRI of the concerned RS has been removed from BEAM_REPORTED_LIST:

5>	include the SSBRI or CRI of the concerned RS in the BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId.

3>	consider L1 measurement reporting to be triggered for the ltm-CSI-ReportConfigId, as specified in 5.35.4.

2>	else if the leaving condition for the event associated with ltm-CSI-ReportConfigId is fulfilled for the measurement from lower layer during TTT for one or more applicable RSs included in the BEAM_ENTERING_LIST or BEAM_REPORTED_LIST:

3>	for each applicable RS for which the leaving condition is fulfilled during TTT and that is in BEAM_ENTERING_LIST:

4>	remove the concerned RS in the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId.

3>	for each applicable RS for which the leaving condition is fulfilled during TTT and that is in BEAM_REPORTED_LIST:

4>	remove the concerned RS in the BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId;

4>	include the SSBRI or CRI of the concerned RS in the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId.

3>	if reportOnLeave is set to true for this ltm-CSI-ReportConfigId:

4>	consider L1 measurement reporting to be triggered for the ltm-CSI-ReportConfigId, as specified in 5.35.4.

2>	upon expiry of the periodical reporting timer for this ltm-CSI-ReportConfigId:

3>	consider L1 measurement reporting to be triggered for the ltm-CSI-ReportConfigId, as specified in 5.35.4.

NOTE 1:	TTT is not restarted if the current beam of serving cell changes and the entry condition is still met with the new current beam.

NOTE 2:	To evaluate the L1 measurement reporting triggering event, the UE uses the latest L1-RSRP measurement from lower layer.

Next change

#### 5.35.3.3	Event LTM3 (Beam of candidate cell becomes offset better than beam of serving cell)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition LTM3-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition LTM3-2, as specified below, is fulfilled.

Inequality LTM3-1 (Entering condition)

Mn + Obn – Hys > Ms + Obs + Off

Inequality LTM3-2 (Leaving condition)

Mn + Obn + Hys < Ms + Obs + Off

The variables in the formula are defined as follows:

Mn is the beam measurement quantity of the LTM candidate cell based on SS/PBCH block or CSI-RS, not taking into account any offsets.

Obn is the offset of the LTM candidate cell (i.e. candidateSpecificOffset as defined in LTM-CSI-ReportConfig for this event). One offset is applied to all beam(s) associated with the LTM candidate cell.

Ms is the beam measurement quantity of the serving cell based on SS/PBCH block or CSI-RS, not taking into account any offsets. The beam associated with this event is the current beam, i.e corresponding to the RS configured in the indicated TCI state or the RS QCLed with the RS configured in the indicated TCI State in the serving cell as defined in clause 5.1.5 in TS 38.214 [7] in the serving cell.

Obs is the offset of the beam of the serving cell (i.e. servingSpecificOffset as defined in LTM-CSI-ReportConfig for this event).

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within LTM-CSI-ReportConfig for this event).

Off is the offset parameter for this event (i.e. ltm3-Offset as defined within LTM-CSI-ReportConfig for this event).

Mn, Ms are expressed in dBm in case of RSRP.

Obn, Obs, Hys, Off are expressed in dB.

Next change

### 5.35.4	Measurement report

The purpose of this procedure is to transfer L1 measurement results from the UE to the network.

RRC controls the event triggered L1 beam level measurement reporting by configuring the following parameter:

-	reportInterval: the periodicity of the event-triggered periodic measurement report;

-	reportAmount: number of measurement reports needs to be transmitted after the event is triggered;

-	maxNumberOfReportedBeams: number of beams whose measurements can be reported in the event triggered L1 measurement report by MAC CE regardless whether or not the report includes the current beam;

-	allowReportAnyBeam: whether the UE can report the measurement results for the beams not satisfying the conditions of the events;

-	reportCurrentBeam: whether the UE is required to report the measurement result of the current beam;

-	ltm-CandidateReportConfigId: LTM candidate cell ID for which the UE is required to measure reference signal and perform LTM event evaluation;

-	reportQuantity: the report quantity for the CSI report.

For each ltm-CSI-ReportConfigId included in the SpCell cell for L1 measurement and event triggered measurement report, the MAC entity shall:

1>	if at least one L1 measurement report has been triggered for the ltm-CSI-ReportConfigId as specified in 5.35.3 and not cancelled:

2>	if UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization:

3>	instruct the Multiplexing and Assembly procedure to generate the L1 measurement report MAC CE associated with the ltm-CSI-ReportConfigId as befined in clause 6.1.3.84 according to the measurement report information in the MR_LIST;

3>	if reportAmount is configured in the LTM-EventTriggeredPeriodicReport by RRC:

4>	if at least one L1 measurement report associated with the ltm-CSI-ReportConfigId has been triggered due to the reason other than the expiry of the periodical reporting timer and the L1 measurement report is not cancelled:

5>	set the MR_SENT_COUNTER to 0 for this ltm-CSI-ReportConfigId;

4>	increment the MR_SENT_COUNTER associated with the ltm-CSI-ReportConfigId by 1;

4>	stop the periodical reporting timer, if running;

4>	if reportAmount is configured in RRC, and the MR_SENT_COUNTER as defined for this ltm-CSI-ReportConfigId is less than reportAmount:

5>	restart the periodical reporting timer with the value of reportInterval for this ltm-CSI-ReportConfigId as defined within the corresponding LTM-CSI-reportConfig;

3>	include the SSBRI or CRI in the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId, if any, into the BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId;

3>	clear the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId;

3>	clear the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId;

3>	if the BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId is empty:

4>	remove the measurement reporting entry within the MR_LIST for this ltm-CSI-ReportConfigId;

4>	stop the periodical reporting timer for this ltm-CSI-ReportConfigId, if running.

3>	cancel the triggered L1 measurement report.

2>	else if the UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the Truncated L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization:

3>	instruct the Multiplexing and Assembly procedure to generate the Truncated L1 measurement report MAC CE associated with the ltm-CSI-ReportConfigId as defined in clause 6.1.3.84 according to the measurement report information in the MR_LIST by selecting the RS(s) based on a decreasing order of the priority for the type of beam: RS(s) in BEAM_ENTERING_LIST, RS(s) in BEAM_LEAVING_LIST, RS(s) in BEAM_REPORTED_LIST, and other RS(s) not in these three lists.

2>	else:

3>	trigger the SR for L1 measurement report;

3>	stop the periodical reporting timer for this ltm-CSI-ReportConfigId, if running.

NOTE 1:	After sending a Truncated L1 measurement report MAC CE, if the subsequent UL grant is still not big enough to fit all the remaining beams, it is up to UE implementation to select the beam(s) which were not included in the previous MAC CE.

NOTE 2: If more than one triggering events for L1 measurement report are pending and the UL grant is not sufficient for all regular L1 measurement report MAC CEs, it is up to UE implementation how to handle/include the L1 measurement report MAC CEs and/or Truncated L1 measurement report MAC CE.

NOTE 3: When a measurement report is triggered by entry condition for one or more RS(s), and included in the BEAM_ENTERING_LIST, another measurement report is triggered by leaving condition for the same RS(s), the UE considers the corresponding measurement report(s) as cancelled. When a measurement report is triggered by leaving condition for one or more RS(s), and included in the BEAM_LEAVING_LIST, another measurement report is triggered by entry condition for the same RS(s), the UE considers the corresponding measurement report(s) as cancelled.

Next change

### 5.36.1	Introduction

The UE may perform conditional LTM by using the L1 or L3 measurement for LTM cell switch conditions evaluation in accordance with the reconfiguration for conditional LTM. The RRC configures the following parameters for L1 trigger condition for CLTM procedure:

-	ltm-ExecutionCondition and ltm-ServingCellExecutionCondition for conditional LTM cell switch execution condition.

For conditional LTM cell switch, if an ltm-CSI-ReportConfig or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig is removed or modified from the current UE configuration, the MAC entity shall reset TTT for the corresponding ltm-CSI-ReportConfigId.

Next change

### 5.36.3	Conditional LTM execution

The conditional LTM cell switch procedure is triggered when:

-	the MAC entity determines that the event for conditional LTM is satisfied based on L1 measurements as specified in clause 5.36.2; or

-	the event(s) for conditional LTM is satisfied based on L3 measurements indicated by upper layers.

The MAC entity shall:

1>	if the event for conditional LTM is satisfied based on L1 measurements as specified in clause 5.36.2:

2>	select the SSB(s) or CSI-RS(s) with the L1 measurement that satisfied the event corresponding to the SSB-Index or NZP-CSI-RS-ResourceID in the LTM-CSI-ResourceConfig associated with the LTM-CSI-ReportConfig in which the satisfied event is included;

2>	indicate to upper layers that the event for the LTM cell switch procedure is triggered, and the Target Configuration ID, corresponding to ltm-CandidateId minus 1, for which the associated L1 measurement based event is satisfied.

1>	if the MAC reset operation as specified in clause 5.12 is performed, as requested by upper layers:

2>	acquire the Target Configuration ID from upper layers, if any, for the satisfied L3 measurement based event(s);

2>	if the event for conditional LTM is satisfied based on L1 measurement:

3>	if the cg-LTM-Configuration is configured for the CLTM target cell (i.e. the SpCell corresponding to the target configuration indicated by Target Configuration ID), if two TAGs are not configured for the CLTM target cell, and if the ltm-Candidate-TimeAlignmentTimer associated with the CLTM target cell is running in the first available CG occasion corresponding to one of the selected SSB/CSI-RS for initial uplink transmission according to clause 5.8.2:

4>	process the stored Timing Advance Command associated with the running ltm-Candidate-TimeAlignmentTimer (see clause 5.2);

4>	consider the RACH-less CLTM cell switch to be ongoing;

3>	if the cg-LTM-Configuration is configured for the CLTM target cell (i.e. the SpCell corresponding to the target configuration indicated by Target Configuration ID), if two TAGs are configured for the CLTM target cell, and if the ltm-Candidate-TimeAlignmentTimer or ltm-Candidate-TimeAlignmentTimerTAG2 associated with the Target Configuration ID for the TAG associated with the TCI state associated with one of the selected SSB/CSI-RS is running, in the first available CG occasion corresponding to the same selected SSB/CSI-RS for initial uplink transmission according to clause 5.8.2:

4>	process the stored Timing Advance Command associated with the running ltm-Candidate-TimeAlignmentTimer or ltm-Candidate-TimeAlignmentTimerTAG2 (see clause 5.2);

4>	consider the RACH-less CLTM cell switch to be ongoing.

3>	else if the UE is configured with UE-based Timing Advance measurement as specified in TS 38.331 [5] and the UE has successfully measured the Timing Advance for the CLTM target cell (i.e. the SpCell corresponding to the target configuration indicated by Target Configuration ID), and the measured Timing Advance is valid in the first available CG occasion corresponding to one of the selected SSB or CSI-RS for initial uplink transmission according to clause 5.8.2:

4>	process the measured Timing Advance (see clause 5.2);

4>	consider the RACH-less CLTM cell switch to be ongoing.

3>	else:

4>	initiate a Random Access procedure (see clause 5.1) on the SpCell;

4>	consider the RACH-based CLTM cell switch to be ongoing.

NOTE 1:	For L1 measurement based RACH-less CLTM, when multiple SSBs/CSI-RSs satisfy the event for conditional LTM, it is up to UE implementation to select a SSB/CSI-RS that satisfies the event and perform CLTM.

2>	if the event(s) for conditional LTM is satisfied based on L3 measurement triggered by upper layer:

3>	if cg-LTM-Configuration is configured for the CLTM target cell (i.e. the SpCell corresponding to the target configuration indicated by Target Configuration ID), if two TAGs are not configured for the CLTM target cell, and if at least one of the SSB(s) of the CLTM target cell with SS-RSRP above cg-RRC-RSRP-ThresholdSSB and amongst the SSBs associated with the CG is available:

4>	select the SSB(s) with SS-RSRP above the cg-RRC-RSRP-ThresholdSSB amongst the SSB(s) associated with the configured uplink grant;

4>	if the ltm-Candidate-TimeAlignmentTimer associated with the CLTM target cell is running in the first available CG occasion corresponding to one of the selected SSB for initial uplink transmission according to clause 5.8.2:

5>	process the stored Timing Advance Command associated with the running ltm-Candidate-TimeAlignmentTimer (see clause 5.2);

5>	consider the RACH-less CLTM cell switch to be ongoing.

3>	if cg-LTM-Configuration is configured for the CLTM target cell (i.e. the SpCell corresponding to the target configuration indicated by Target Configuration ID), if two TAGs are configured for the CLTM target cell, and if at least one of the SSB(s) of the CLTM target cell with SS-RSRP above cg-RRC-RSRP-ThresholdSSB and amongst the SSBs associated with the CG is available:

4>	select the SSB(s) with SS-RSRP above the cg-RRC-RSRP-ThresholdSSB amongst the SSB(s) associated with the configured uplink grant;

4>	if the ltm-Candidate-TimeAlignmentTimer or ltm-Candidate-TimeAlignmentTimerTAG2 associated with the CLTM target cell for the TAG associated with the TCI state associated with one of the selected SSB is running in the first available CG occasion corresponding to the same selected SSB for initial uplink transmission according to clause 5.8.2:

5>	process the stored Timing Advance Command associated with the running ltm-Candidate-TimeAlignmentTimer or ltm-Candidate-TimeAlignmentTimerTAG2 (see clause 5.2);

5>	consider the RACH-less CLTM cell switch to be ongoing.

3>	else if the UE is configured with UE-based Timing Advance measurement as specified in TS 38.331 [5] and the UE has successfully measured the Timing Advance for the CLTM target cell (i.e. the SpCell corresponding to the target configuration indicated by Target Configuration ID):

4>	select the SSB(s) with SS-RSRP above the cg-RRC-RSRP-ThresholdSSB amongst the SSB(s) associated with the configured uplink grant;

4>	if the measured Timing Advance is valid in the first available CG occasion corresponding to one of the selected SSB(s) for initial uplink transmission according to clause 5.8.2:

5>	process the measured Timing Advance (see clause 5.2);

5>	consider the RACH-less CLTM cell switch to be ongoing.

3>	else:

4>	initiate a Random Access procedure (see clause 5.1) on the SpCell;

4>	consider the RACH-based CLTM cell switch to be ongoing.

NOTE 2:	For L3 measurement based RACH-less CLTM, if there are multiple selected RSs corresponding to the CG with the measurement above the cg-RRC-RSRP-ThresholdSSB, it is up to UE implementation to select one of them to perform CLTM.

2>	if the RACH-less CLTM cell switch is considered to be ongoing:

3>	attempt to select a configured uplink grant for uplink transmission according to clause 5.8.2;

3>	if a valid configured uplink grant is selected:

4>	perform uplink transmission in the available CG occasion for RACH-less CLTM cell switch according to clause 5.8.2;

4>	monitor the PDCCH as specified in clause 5.7 and TS 38.213 [6].

2>	if the TimeAlignmentTimer associated with PTAG expires while the RACH-less CLTM cell switch is ongoing:

3>	initiate a Random Access procedure (see clause 5.1) on the SpCell;

3>	consider the RACH-based CLTM cell switch to be ongoing.

NOTE 3:	For RACH-based CLTM, if there are multiple selected RSs, it is up to UE implementation to select one of them to perform CLTM.

Next change

#### 6.1.3.84	Event Triggered L1 Measurement Report MAC CE

Event triggered L1 measurement report MAC CE consists of either:

-	event triggered L1 measurement report format (variable size); or

-	truncated event triggered L1 measurement report format (variable size).

The event triggered L1 measurement report formats are identified by MAC subheaders with an eLCIDs as specified in Table 6.2.1-2b.

For a truncated event triggered L1 measurement report MAC CE, at least the following fields should be included: report ID field, at least one triggered RS with corresponding measured quantity, and the current RS of serving cell as described in TS 38.215 [24] with corresponding measured quantity, if UE is configured to report the measurement result of current RS of the serving cell by reportCurrentBeam.

The fields in the (truncated) event triggered L1 measurement report MAC CE are defined as follows:

-	Report ID: This field indicates corresponding measurement report ID for this ltm-CSI-ReportConfigId associated with this event triggered measurement report. The length of the Report ID field is 6 bits;

-	Typei: This field indicates the type of the RS i of LTM candidate cell included in the event triggered L1 measurement report. The field is set to 00 to indicate the RS(s) that have satisfied the entry condition of the event associated with the report ID for TTT, and trigger this measurement report MAC CE, which are included in the BEAM_ENTERING_LIST associated with the report ID when the MAC CE is generated as specified in clause 5.35.3; it is set to 01 to indicate the RS(s) that have satisfied the leaving condition of the event associated with the report ID for TTT, and trigger this measurement report MAC CE, which are included in the BEAM_LEAVING_LIST associated with the report ID when the MAC CE is generated as specified in clause 5.35.3; it is set to 10 to indicate the RS(s) have been previously reported in the (Truncated) L1 measurement report MAC CE, which are  included in the BEAM_REPORTED_LIST associated with the report ID when the MAC CE is generated as specified in clause 5.35.3; it is set to 11 to indicate the RS(s) none of the BEAM_ENTERING_LIST / BEAM_REPORTED_LIST / BEAM_LEAVING_LIST, if allowReportAnyBeam specified in TS 38.331 [5] is configured. The RS(s) not satisfying the event for TTT are selected by decreasing value of measurement results. The RS(s) included in the truncated event triggered L1 measurement report MAC CE are selected based on the Type of RS(s), with the following priorities in decreasing order: 00, 01, 10, 11. If the (truncated) event triggered L1 measurement report MAC CE cannot accommodate all RS(s) with the same priority, the RS(s) are selected by decreasing value of measurement results. The length of the field is 2 bits;

NOTE:	For the measurement report triggered by LTM2, the RS with Type 00 or 01 is the current beam, which is always included in the last octet.

-	RSRIi: This field indicates the reference signaling resource index of the beam i of LTM candidate cell for the event triggered L1 measurement report (i.e. SS/PBCH Block Resource indicator (SSBRI) or CSI-RS resource indicator (CRI)). The maximum number of non-serving RS reported, i.e. M value, is configured by maxNumberOfReportedBeams if the measurement of current RS of serving cell is not included, or is maxNumberOfReportedBeams-1 if the measurement of current RS of serving cell is included. The first RS is the RS with the highest measured quality for non-serving RS in this measurement report MAC CE. The length of the RSRI index field is 9 bits;

-	RSRP1: This field indicates the measured quantity of the first beam based on SS/PBCH block or CSI-RS (i.e. the L1-RSRP) as described in TS 38.215 [24]. The length of the RSRP1 field is 7 bits;

-	DiffRSRPi: This field indicates the derived differential measured quantity for the beam i of LTM candidate cell based on SS/PBCH block or CSI-RS (i.e. the L1-RSRP) as described in TS 38.215 [24], with the reference of measured quality of the first beam. The length of the DiffRSRPi field is 4 bits;

-	RSRPserving: This field indicates the measured quantity based on SS/PBCH block or CSI-RS (i.e. the L1-RSRP) for current RS of serving cell as described in TS 38.215 [24] used for LTM event evaluation in clause 5.35.2, if UE is configured to report the measurement result of current RS of the serving cell by reportCurrentBeam. The length of the RSRPserving field is 7 bits;

-	R: Reserved bit, set to 0.

Figure 6.1.3.84-1: event triggered L1 measurement report and truncated event triggered L1 measurement report MAC CE

End of chang