3GPP TSG-RAN WG2 Meeting #132	R2-2508052

Dallas, USA, Nov. 17th– 21st, 2025

Source:	vivo

Title:	Consideration on Common Signal/Channel Design for 6GR

Agenda Item:	10.3.2.3

Document for:	Discussion and Decision

- Introduction

In RAN2#131bis meeting, initial agreements on the design of common signal/channel(BCCH, PCH, RACH) have been reached. In this paper, we will further discuss the detailed issues to be considered for the design of system information, paging and spectrum aggregation.

- Discussion

  - System information

In the RAN2#131bis meeting, the following agreements on 6G system information design[1] have been reached.

Agreements on System Information

- -	System information design should consider energy efficiency and low complexity for both network and UE.

-	System information design should ensure that the public warning requirements are met.

-	Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs)

-	Study how to improve flexibility and extensibility of scheduling SI design

-	Study mechanisms to improve SI update mechanism

-	Study aspects related to SIB1 size.  Understand the issues and desired content/size from RAN2 point of view.

-	Study areas specific SIB design.  Understand 5G pain points and what improvements can be considered.

NOTE: this is a starting list

In this section, we will discuss the issues related to  SIB1/SSB transmission, desired SIB1 content/size and OSI scheduling.

    - Broadcasting and On-demand Transmission

In Rel-15, NR introduced On-Demand Other System Information to reduce network energy consumption by avoiding always-on OSI broadcasting. In Rel-19, NR further introduced OD-SIB1 to achieve even greater network energy savings by minimizing the transmission of SIB1.

For 6GR NES technique related to system information, we think the OD-OSI/SIB1 mechanism specified in NR should be inherited. And the OD-SIB1 with UE request mechanism in NR Rel-19 can be further improved and extended to other scenarios from RAN2 perspective. The disadvantage for Rel-19 OD-SIB1 is that the deployment of OD-SIB1 cell mandates the network to ensure there is at least a cell-A with overlapping coverage with the OD-SIB1 cell. Otherwise, the UEs that have just switched to power on cannot obtain the OD-SIB1 request configuration from an OD-SIB1 cell itself if the SIB1 is not being broadcast and thus cannot trigger OD-SIB1 request procedure. Therefore, only capacity cells may apply Rel-19 OD-SIB1 mechanism for NES purposes. In 6GR, we think the OD-SIB1 mechanism should be extended to the scenario where the UEs can directly obtain the OD-SIB1 request configuration from the SSB of the OD-SIB1 cell, so that the UEs do not need to rely on an underlay cell (cell A) to obtain the OD-SIB1 request configuration of the OD-SIB1 cell, as Figure 1 shows.

Figure 1. Example of standalone OD-SIB1

Another shortcoming of Rel-19 OD-SIB1 request operation is that the OD-SIB1 cell still transmits always-on SSB, and thus, when the SSB periodicity is short, the NES gain from OD-SIB1 operation may be limited. Therefore, we think further NES gain may be achieved by the NES cell transmitting SSB in an on-demand manner too. In this case, the NES cell does not transmit always-on SSB. The UE relies on cell A’s SSB for time/frequency tracking and then requests OD-SSB & OD-SIB1 in the NES cell when necessary. We propose RAN2 to study cell A-assisted OD-SSB&OD-SIB1 in 6GR, as Figure 2 shows.

Figure 2. Example of OD-SSB & OD-SIB1

In sum, we propose RAN2 to design 6GR system information transmission to include an on-demand mechanism alongside the traditional always-on broadcasting.  Furthermore,  the existing NR OD-SIB1 mechanism should be extended to more scenarios for further NES gain.

- Both always-on broadcasting and OD-OSI/SIB1 mechanism specified in NR should be inherited in 6GR. And extend NR OD-SIB1 with UE request mechanism to cover: 1) standalone OD-SIB1 (e.g. the UE can obtain OD-SIB1 request configuration from SSB of the OD-SIB1 cell); 2) cell A-assisted OD-SSB&OD-SIB1 (i.e. cell A provides the OD-SSB&OD-SIB1 request configuration of the NES cell).

    - SIB1 Size/Content

As mentioned in the section 2.1, there is an agreement achieved on studying aspects related to SIB1 size at RAN2#131bis meeting, which includes understanding the issues and the desired content/size from RAN2 point of view. Although SIB1 size is also related to RAN1, we would like to provide our view on SIB1 size/content from RAN2 perspective based on the observations on the current commercial NR network and potential designs in 6GR from the following aspects.

- SIB 1 size limitation

To guarantee the coverage for UEs at the cell edge, the TB size for system information does not exceed 2976 bits in NR. Given that the expected frequency for 6GR deployment will be higher than 5G, and there will be more pathloss at the same distance, so it can be foreseen that the TB size limitation will not be relaxed. However, the SI TB limitation may lead to that complete SIB1 cannot be conveyed in one TB.

In 5G, SIB1 is extended in later releases with the introduction of new features, e.g. (e)RedCap, MT/MO SDT, positioning, etc., which causes the growth of SIB1 size. There is a risk that some features cannot be deployed due to there is not enough room to convey its related configurations in SIB1.

Furthermore, the harmonized design for IoT device and eMBB device in 6GR is expected, and more information will be carried in SIB1 to support IoT service, leading that the SIB1 size may be larger than NR(eMBB only), which also go against to the limited SIB1 size.

- SIB1 content

In NR, SIB1 mainly contains the information for cell selection, network discovery (i.e., cellAccessRelatedInfo), connection control (i.e., connEstFailureControl), SI scheduling, initial access related information (i.e., servingCellConfigCommonSIB), network congestion control related information (i.e., UAC), and UE timers/constants. In the current commercial networks, based on the test results of multiple intra-frequency cells under the same operators/vendor within an area, we observe that most of the parameters are configured with the same value, and most of the bits in SIB1 are same (i.e. nearly 80% as shown in the figure below). The different bits are mostly related to Cell ID, TAC, PRACH root sequence, zeroCorrelationZoneConfig within RACHconfig, etc.

Figure 3 Illustration of SIB1 configurations between different intra-frequency cells

In summary, we have the following observations on SIB1 size and content.

- 6GR SIB1 will convey more information, leading that SIB1 size exceeds the physical layer SI TB size limitation.

- Most of the parameters in NR SIB1 are configured with same value for the intra-frequency cells belongs to same operator/vender within an area.

Therefore, RAN2 should study some enhancements on SIB1 design, e.g. SIB1 splitting, and the following principles on SIB1 splitting can be considered.

- Principle 1, based on common configurations for all device types

As per the SID, 6GR shall support eMBB and IoT services with a single RAT. To provide corresponding configurations for eMBB and IoT service, one way is using independent SIB1 transmission as in LTE and NB-IoT, however, this method is inefficient since some common parameters are separately configured (e.g. cell selection and cell access related info). Another way is that both parameters for eMBB and IoT are transmitted via a same SIB1. On one hand, this will lead to larger SIB1 size which will exceeds the physical layer SIB1 size limitation, on the other hand, given that the coverage target for IoT service and eMBB service is different, i.e. there is approximately 10 dB extension for IoT service as per RAN1 discussion, repetition based SIB1 transmission is needed, this will lead to unnecessary repetition transmission for eMBB specific parameters. And from device perspective, this will also lead to unnecessary ASN.1 processing on eMBB specific parameters, which also add complexity to cost-limited IoT devices.

RAN2 needs to study the efficient SIB1 transmission to enable SI repetition and to avoid redundant transmission. For example, the reduction of redundant transmission can be realized by avoiding unnecessary repetition and independent transmission for the common parameters/configurations. In other words, SIB1 can be split into two parts which can be transmitted independently, i.e., the first part named basic SIB1 which includes the common configurations shared by all devices and is transmitted with repetitions; while the second part named additional SIB1 which includes specific configurations for eMBB device only and is transmitted without repetition or less repetitions. From UE perspective, the IoT device can read basic SIB1 only while the eMBB device needs to read both basic SIB1 and additional SIB1 and to combine them. One example is illustrated in the following figure.

Figure 4 Illustration of SIB1 splitting to support diverse device types

- Principle 2, based on common configurations within an area

Based on the observations on SIB1 configurations in real network, RAN2 needs to study energy-efficient SIB1 transmission with decreased SIB1 size, i.e. reduce the transmission of the parameters that share the same value within an area. For example, given that a majority of the parameters in SIB1 are consistently set to the same values within a specific region, to enhance efficiency and adaptability, we can restructure SIB1 into two segments. The first segment (area-specific) consists of parameters to maintain uniform values within an area, allowing them to be acquired as needed rather than being broadcast in each cell continually. The second segment (cell-specific) would encompass parameters with varying values crucial for cell camping, ensuring that essential information is periodically broadcast to support cell (re)selection with a minimum overhead. This approach is beneficial to reduce the always on SIB1 signalling overhead, which is helpful to reduce network power consumption as well.

- RAN2 to study enhancements on SIB1 transmission (e.g. SIB1 splitting) by taking the following aspects into account:

- Avoid redundant transmission, e.g. common configurations in SIB1 which are shared by different device types shall not be separately transmitted.

- Decrease common parameter transmission, e.g. configurations in SIB1 which are configured with same value within an area are not broadcast in each cell within this area.

    - Scheduling of Other SI

In NR, the SI scheduling scheme has been enhanced multiple times in different releases:

- In Rel-15, the SI messages carrying the SIBs are configured in the schedulingInfoList. The SI window for a SI message are determined according to the order of entry in the list of SI messages configured by schedulingInfoList. However, there may be some idle windows (red square box in Figure 5) that cannot be utilized, which limits the SI message capacity.

Figure 5: An example of Rel-15 SI scheduling

- In Rel-16, positioning feature is supported. The SI messages carrying the posSIBs are configured in the posSchedulingInfoList to support the transmission of posSIBs.

  - If the offsetToSI-Used is not configured, the SI window for a SI message are determined according to the order of entry in the concatenated list of SI messages by appending the posSchedulingInfoList to schedulingInfoList.

Figure 6: An example of Rel-16 SI scheduling without offsetToSI-Used

  - If the offsetToSI-Used is configured, the SI window for a SI message has a 80ms offset and are determined according to the order of entry in the concatenated list of SI messages by appending the posSchedulingInfoList to the list of SI messages with an associated si-Periodicity of 8 radio frames configured by schedulingInfoList. With the enhancement, the idle window between SFN 8 and SFN 16 can be utilized by the posSI(s), but the idle window between SFN 24 and SFN 32 may be not able to be utilized.

Figure 7: An example of Rel-16 SI scheduling with offsetToSI-Used

- In Rel-17, the SI messages carrying the SIBs or posSIBs introduced from Rel-17 are configured in the schedulingInfoList2. The SI window for a SI message is directly indicated by the gNB. With the enhancement, any idle window can be utilized.

The reason for the multiple enhancements in different releases is to better utilize idle SI windows to support the transmission of more SI messages and also the transmission of posSIBs. The consequence is that current SI scheduling scheme is quite complex. Now it seems unnecessary to distinguish the transmission of posSIB from that of SIB. The lesson learned from NR is that RAN2 should consider to design a unified SI scheduling scheme from the very beginning of 6G. The unified SI scheduling scheme should be cable to make full use of the idle SI windows and be applicable to all the type of SIBs.

- Study unified SI scheduling scheme with the following requirements:

- unified SI scheduling scheme is cable to make full use of the idle SI windows.

- unified SI scheduling scheme should be applicable to all the types of SIBs, e.g., SIB, posSIB, etc (if there are still different types of SIBs in 6G).

  - Paging

During the RAN2#131bis meeting, RAN2 agreed that paging design should take energy efficiency into consideration in 6GR [1]:

Agreements => Paging design should consider energy efficiency and simplicity for both UE and Network.

In this section, we will concentrate on the paging design principles regarding NW energy saving and UE power saving.

    - NW energy saving

In NR, achieving NES gain by paging adaptation was studied. It was identified that the gNB may have longer off-period to sleep if the POs are configured in clusters when the network load is low, such as the Figure 8 shows:

Figure 8. NES gain achieved by clustering POs to have longer off-duration

- Clustered POs in time domain is beneficial for NW energy saving when the network load is low.

In Rel-19, paging adaptation was specified, RAN2 agreed on a solution to extend N value with ‘T/32’ and Ns value with ‘eight’ while reusing the legacy PF/PO determination formula to support the NW semi-persistently configuring clustered POs (i.e. support only one paging frame to be configured per 32rf paging cycle), so that the time gap between PFs can be enlarged for the NW to have a longer off-duration to sleep.

However, the Rel-19 PF/PO determination formula has a significant drawback: it results in an excessively large configuration in SIB1, as we doubled the maximum number of POs (from 4 to 8) within a PF. Take one typical configuration in current network for instance (SCS=30kHz, T=320ms), if Ns = 8 and N = T/32, configuring firstPDCCH-MonitoringOccasionOfPO for each PO needs 13 bit (maximum symbol offset in a 32rf cycle is 8960 symbols). Consequently, the PCCH configuration in SIB1 consumes more than 13 * 8 = 112 bits. Furthermore, the signalling overhead will increase linearly if we allow for the configuration of more POs within a single PF in the 6G to achieve more NES gain. In other words, the Rel-19 PF/PO determination formula exhibits poor scalability.

- Paging adaptation in NR Rel-19 NES is supported by configuration of clustered POs with only one PF in a paging cycle, at the cost of increasing paging configuration signalling overhead in SIB1.

During last RAN2 meeting, issues related to SIB were discussed. RAN2 agreed to further study the aspects related to SIB1 size. As discussed in the previous section, the excessive size of the NR SIB1 is a pain point for NR, and we must address it in 6G, we should not under-estimate the signalling overhead from supporting clustered PO with long duty-cycle for NES. Considering the necessity to support NES in 6GR day-1 framework, if there is a need to further extend N/Ns value, the limitation from reusing Rel-19 paging adaptation method can be foreseen, e.g. paging configuration signalling overhead in SIB1 may further require hundreds of bits more and go beyond the acceptable level. Therefore, we propose RAN2 to study how paging is designed to support NES, while aiming to keep the paging configuration signalling overhead in SIB1 in an acceptable level.

- Study paging design to support both clustered PF/PO (e.g. for NES scenario) and distributed PF/PO configuration in 6GR Day-1, while aiming to keep the paging configuration signalling overhead in SIB1 in an acceptable level.

    - UE power Saving

Paging reception is always one of the dominant contributors to UE power consumption in RRC_IDLE state. Therefore, efficient paging power saving mechanisms are essential from Day-1 in 6G.

In 5G NR, multiple features were introduced to address paging-related power consumption. Paging Early Indication (PEI) is specified in Rel-17, while Low Power-Wake Up Signal (LP-WUS) was further studied and specified in Rel-18/19. Both were intended to reduce UE power consumption for paging monitoring, but in practice, the introduction of multiple duplicated functionalities created additional complexity. From an implementation and deployment perspective, having several features targeting the same problem has increased the burden on UEs and gNBs, without delivering fully consistent gains across all scenarios.

Besides, between these two functionalities, LP-WUS was shown to provide significant benefits in terms of paging monitoring efficiency, with a dedicated low-power wake-up receiver (WUR). However, LP-WUS was introduced relatively late in 5G (Rel-18/19), which limited its early adoption and integration into the ecosystem. In contrast, PEI provided only limited improvements and became largely overshadowed by LP-WUS. This indicates that a strong, unified, and effective solution is preferable from the beginning of a new generation, rather than incremental and overlapping mechanisms.

- In 5G, multiple duplicated features for paging power saving (e.g., PEI and LP-WUS) increased complexity while providing limited incremental benefit. A single unified solution for efficient paging power saving should be prioritized in 6G Day-1, avoiding the delays seen in 5G.

In 5G NR, an RRC_IDLE UE monitors the Paging Occasion for two distinct purposes:

- To receive a Paging Message in the PDSCH for Mobile-Terminated service establishment.

- To check for a short message in the PDCCH that contains specific system indications, such as Public Warning System notifications or System Information change notifications.

Since these two purposes may have different latency requirements for information delivery, the network configures two corresponding DRX cycles:

- The CN configures a UE-specific DRX cycle to satisfy the latency needs of MT service paging.

- The gNB configures a Cell-specific DRX cycle to meet the latency requirements for PWS notifications and other short indications.

Consequently, the UE selects the shorter of the two configured cycles as its effective DRX cycle for paging monitoring. This selection ensures that the latency requirements for both purposes are met. During each selected DRX cycle, the UE wakes up once and monitors both PDCCH and PDSCH. Most likely, 6GR will reuse the same scheme for the DRX cycle configuration and determination for paging PDCCH and PDSCH monitoring.

However, this mechanism can result in extra UE power consumption if the UE-specific DRX cycle is longer than the Cell-specific DRX cycle. For instance, as illustrated in the following figure, the UE is forced to PDSCH monitoring more frequently than needed for its MT service, increasing power consumption.

Figure 9. Extra Paging PDSCH monitoring in NR

In the provided figure above, the UE is configured with a 1280ms UE-specific DRX cycle and a 640ms cell-specific DRX cycle. According to the standard NR paging mechanism, the UE is required to monitor both the PDCCH and PDSCH once every 640ms. This results in the UE performing extra, potentially unnecessary, PDSCH monitoring once per 1280ms, as illustrated by the purple blocks. This NR approach provides better-than-expected latency performance for MT service but wastes power. During the Rel-17 "NR UE power saving enhancements" work item, it was confirmed that monitoring the Paging PDCCH&PDSCH typically necessitates more SSB measurements compared to merely monitoring the Paging PDCCH. Consequently, these extra PDSCH monitoring lead to additional UE power consumption for extra SSB measurements.

- In 5G, the short message and the Mobile-Terminated (MT) notification are carried in the PDCCH and PDSCH respectively, each having a distinct latency requirement for delivery. However, they are currently designed for joint monitoring to meet the stricter latency requirement, which unfortunately leads to extra UE power consumption.

Take the above two observations into account, we propose:

- The 6GR should study paging mechanism enable efficient UE power saving, including:

- Aim to support wake up mechanism for paging reception from Day-1. Similar design for LP-WUS/WUR in 5G NR is reused as the baseline.

- Investigate decoupling the monitoring of short message and MT notification to prevent the additional UE power consumption associated with the increased MT notification monitoring rate caused by joint design from Day-1.

  - Spectrum Aggregation for IDLE/INACTIVE mode operation（建辉）

During last meeting, RAN2 initiated some discussion on spectrum aggregation[1]:

Agreements on Spectrum aggregation 1 Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed. 2 Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc). Understand the deployment scenarios and problems to address

Since spectrum aggregation for CONNECTED mode operation has been specified in LTE/NR in CA mechanism and CA has been agreed to be supported in 6GR, we assume CA based spectrum aggregation should be the baseline for CONNECTED mode operation.  Hence, in this section, we focus on  spectrum aggregation for IDLE/INACTIVE mode operation. One contribution [2] regarding cell model of single cell with multiple carriers was discussed. We think the following online comments are valuable for further study of spectrum aggregation:

- What’s the benefit from supporting certain cell model so that further study is worthwhile? If the benefits are shared by the majority view, then what are the issues to be addressed;

- We need to understand the issues to be discussed are for IDLE/INACTIVE mode operation or for CONNECTED mode operation;

- We should first study the functional framework. How the cell model is defined, regardless it is by having a single cell with multi-carriers, multiple cells with anchor cell, etc., depends on the details within the framework.

In this section, we will focus on the functional framework for IDLE/INACTIVE mode operation in multi-carrier deployment scenario and analyze the potential benefits.

Functional framework for IDLE/INACTIVE mode operation in multi-carrier deployment scenario

Figure 10 is an example for illustration of the functional framework.

Figure 10. Illustration of a potential functional framework in 6GR multi-carrier deployment scenario

- SSB/SIB:

In 6GR, support of NES has been widely considered as an overall design principle. Currently, when multiple carriers are deployed as independent cells, SSB and SIB are transmitted in each cell. To achieve NES gain, the system information of some carriers/cells (non-anchor carrier/cell) can be saved on these cells and transmitted on an anchor carrier/cell, as Figure 10 shows. Considering that the SIBs of the multiple cells may have part of the duplicated information (e.g. they share the same area-specific OSI), the anchor carrier/cell does not need to actually transmit the whole system information set for the non-anchor carriers/cells.

As for SSB, whether the multiple carriers/cells can share a same SSB on anchor carrier/cell for synchronization depends on whether the anchor carrier/cell can be the reference cell of the other non-anchor carrier/cell. These part of study is up to RAN4 for further study. If the SSB can be shared, it means the SSB transmission on non-anchor carrier/cell can be saved, so that NES gain can be achieved.

According to the NR Rel-18 NES SI report [3], SSB/SIB-less operation in multi-carrier scenario can achieve noteworthy NES gain depending on different assumptions:

With both SSB-less and SIB1-less from one of two carriers for non-CA operation, with assumption that UE is able to acquire sync from a carrier from another band, - compared to baseline of 20ms SSB/SIB1 periodicity on both carriers, one source shows BS energy savings by 55.1%~44.4% with RO periodicity of 20ms~160ms at different loads, and one source shows 9.1%~14.8% energy savings at empty load if an anchor carrier carries additional SIB1 for another carrier; - at different loads, compared to baseline of 20ms SSB/SIB1 periodicity, one source shows BS energy savings by 36.0%~13.5% when combined with simplified SSB (i.e. PSS and SSS only); - one source shows that with baseline of SSB+SIB periodicity of {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell, BS energy savings can be 9.4%~98.4% if an anchor carrier carries the SSB and SIB1 for another carrier depending on the traffic load. Meanwhile, the SIB1 carried on another carrier increase the energy of that carrier by 2.3%~17.8%, resulting a total saving across two carries by 7.1%~80.6%.

- Paging

Currently, when multiple carriers are deployed as independent cells, paging message is independently transmitted in each cell within a tracking area for a certain UE. By network implementation, e.g. taking the UE capability into consideration, the network can either page the UE one by one at cell-level or frequency-level, or simultaneously on multiple cells/frequencies since the network does not on which cell/frequency the UE is camping.

In multi-carrier deployment scenario, if the carriers are aggregated, the network may only need to transmit the paging message for a certain UE on one of the carriers. Take the LTE NB-IOT multi-carrier cell for example. The UE monitors paging only on one carrier within the multi-carrier cell. The multi-carrier cell can configure paging weight for each carrier, and the UE can determine which carrier is for paging monitoring of its own according to its UE_ID and the paging weight of each carrier. From NES point of view, supporting POs distributed on multiple carriers may provide potential NES gain. The base station may have more off-duration for sleeping, as Figure 11 shows. It is quite similar to the NES gain logic in NR Rel-19 NES.

Figure 11. Potential NES gain from PO clustering in time-domain on multiple carriers

Therefore, the foremost issue for paging in multi-carrier deployment scenario, is whether the UE monitors paging only on anchor carrier/cell, or on one of multiple carriers/cells depending on the network configuration/indication. Figure 12 illustrates such two options.

Figure 12. Two options for paging operation in the multi-carrier deployment scenario

For option-1, the UE monitors paging only on anchor carrier/cell. As such, all UEs of various types within the cell monitor on the sole carrier/cell. This option may be a simple way-forward. However, it is hard to adapt to the demands of UEs of different types/capabilities. For instance, IOT-like UE may expect the network to configure PDCCH repetitions for better coverage, while the eMBB UE may not share this need. As a result, if the network configures PDCCH repetition on anchor carrier/cell for IOT-like UE, it can be a waste of resources as the majority of eMBB UEs do not share this need of PDCCH repetition. For another instance, UEs of different capabilities may not all support a bandwidth of the anchor carrier/cell for paging. Hence, in a compromised design, the anchor carrier/cell should be set to a bandwidth that the majority of the UEs of different types/capabilities can support, which may pose unnecessary restriction to real world deployment.

For option-2, POs are distributed on multiple carriers/cells. In this option, the network transmits paging for a certain UE only on one of the multiple carriers/cells if the NW and the UE can be aligned with the paging carrier/cell UE monitoring. Yet, there is also a coverage issue for option-2. As the Figure 13 shows, the carriers/cells aggregated may have a same/similar or an apparently different DL coverage. When the carriers/cells have a same/similar DL coverage, the paging carrier determination method for LTE NB-IOT may be reused. In comparison, when the carriers/cells have a different DL coverage, there is a chance that the network pages UE on a carrier/cell which does not cover UE’s location. In this case, paging missing will occur. Hence, some paging fall-back mechanism may be studied for this case.

Figure 13. Carriers with similar or diverse coverage due to the band location and signal power control

In conclusion, we think the UE monitors paging only on one carrier/cell within the multi-carrier deployment scenario. Yet, we need to further study whether this paging carrier/cell is only the anchor carrier/cell or can be one of the non-anchor carrier/cell.

- RACH

In NR Rel-15, SUL is defined, where two non-continuous carriers can be configured as NUL (Normal Uplink) and SUL (Supplementary Uplink) in one serving cell. The UE can perform RACH on either SUL or NUL at one time, depending on whether there is network indication, or by RSRP measurement for RACH carrier selection as Figure 14 shows.

Figure 14. RACH carrier selection between NUL/SUL in NR

In LTE Rel-14, NB-IOT UE can perform RACH on one of the UL carriers on which ROs are configured. When the UE needs to perform RACH, the UE first determine the CEL (Coverage-Enhancement Level) for each carrier, and then determines the carrier for RACH depending on the RACH selection probability of each carrier on which RO is configured for the determined CEL. The reason for this design is that the coverage of each UL carrier is assumed to be same and thus RSRP is not the solely factor, and there is the need for RACH load balancing among the carriers. Figure 15 shows an example. When the network configures ROs for CEL0 on four UL carriers, and nprach-ProbabilityAnchor-r14 for CEL0 is ‘oneFourth’, the probability for the UE to select RO on anchor carrier is 1/4=25%, and the probability for the UE to select RO on each non-anchor carrier is (1-1/4)/3=25%.

Figure 15. RACH carrier selection among anchor/non-anchor carriers in LTE NB-IOT

For multi-carrier deployment scenario in 6GR, we think the UE can also perform RACH on one of the multiple UL carriers if ROs are configured on those carriers. Further study is needed for how the UE selects the RACH carrier, and how the corresponding RACH procedure is designed.

Summary

In sum, we propose RAN2 to initiate the discussion on how to characterize the functional framework for IDLE/INACTIVE mode operation in multi-carrier deployment scenario, and the potential benefits of the framework.

- Spectrum aggregation for IDLE/INACTIVE mode enhances radio resource efficiency and provides significant network energy saving gains, include: 1. SSB/SIB on non-anchor carrier/cell is not transmitted; 2. Paging for a certain UE is transmitted only on one carrier/cell; 3. POs/ROs is configured in a time-domain-clustering manner on multiple carriers/cells to enable longer network DTX/DRX off-duration.

- RAN2 to discuss the functional framework for IDLE/INACTIVE mode operation in multi-carrier deployment scenario:

- SSB/SIB: Define anchor/non-anchor carrier/cell as:

- Anchor carrier/cell: a carrier/cell where the UE assumes SSB/SIB are transmitted.

- Non-anchor carrier/cell: a carrier/cell where the UE does not assume SIB is transmitted. FFS whether the UE assumes the SSB is transmitted or not.

- Paging: The UE monitors paging only on one carrier/cell within the multi-carrier deployment scenario, FFS this carrier/cell is the anchor carrier/cell or can be one of the non-anchor carrier/cell.

- RACH: The UE can perform RACH on one of the multiple carriers/cells on which RO is configured. FFS how the UE selects the RACH carrier, and how the corresponding RACH procedure is designed.

- Conclusion

In this paper, we discuss the detailed issues to be considered for the design of system information, paging and spectrum aggregation. And we have the following observations and proposals:

System Information

Proposal 1: Both always-on broadcasting and OD-OSI/SIB1 mechanism specified in NR should be inherited in 6GR. And extend NR OD-SIB1 with UE request mechanism to cover: 1) standalone OD-SIB1 (e.g. the UE can obtain OD-SIB1 request configuration from SSB of the OD-SIB1 cell); 2) cell A-assisted OD-SSB&OD-SIB1 (i.e. cell A provides the OD-SSB&OD-SIB1 request configuration of the NES cell).

Observation 1: 6GR SIB1 will convey more information, leading that SIB1 size exceeds the physical layer SI TB size limitation.

Observation 2:  Most of the parameters in NR SIB1 are configured with same value for the intra-frequency cells belongs to same operator/vender within an area.

Proposal 2: RAN2 to study enhancements on SIB1 transmission (e.g. SIB1 splitting) by taking the following aspects into account:

- Avoid redundant transmission, e.g. common configurations in SIB1 which are shared by different device types shall not be separately transmitted.

- Decrease common parameter transmission, e.g. configurations in SIB1 which are configured with same value within an area are not broadcast in each cell within this area.

Proposal 3: Study unified SI scheduling scheme with the following requirements:

- unified SI scheduling scheme is cable to make full use of the idle SI windows.

- unified SI scheduling scheme should be applicable to all the types of SIBs, e.g., SIB, posSIB, etc (if there are still different types of SIBs in 6G).

Paging

Observation 3: Clustered POs in time domain is beneficial for NW energy saving when the network load is low.

Observation 4:Paging adaptation in NR Rel-19 NES is supported by configuration of clustered POs with only one PF in a paging cycle, at the cost of increasing paging configuration signalling overhead in SIB1.

Proposal 4: Study paging design to support both clustered PF/PO (e.g. for NES scenario) and distributed PF/PO configuration in 6GR Day-1, while aiming to keep the paging configuration signalling overhead in SIB1 in an acceptable level.

Observation 5: In 5G, multiple duplicated features for paging power saving (e.g., PEI and LP-WUS) increased complexity while providing limited incremental benefit. A single unified solution for efficient paging power saving should be prioritized in 6G Day-1, avoiding the delays seen in 5G.

Observation 6:In 5G, the short message and the Mobile-Terminated (MT) notification are carried in the PDCCH and PDSCH respectively, each having a distinct latency requirement for delivery. However, they are currently designed for joint monitoring to meet the stricter latency requirement, which unfortunately leads to extra UE power consumption.

Proposal 5:The 6GR should study paging mechanism enable efficient UE power saving, including:

- Aim to support wake up mechanism for paging reception from Day-1. Similar design for LP-WUS/WUR in 5G NR is reused as the baseline.

- Investigate decoupling the monitoring of short message and MT notification to prevent the additional UE power consumption associated with the increased MT notification monitoring rate caused by joint design from Day-1.

Spectrum Aggregation for IDLE/INACTIVE mode operation

Observation 7:Spectrum aggregation for IDLE/INACTIVE mode enhances radio resource efficiency and provides significant network energy saving gains, include: 1. SSB/SIB on non-anchor carrier/cell is not transmitted; 2. Paging for a certain UE is transmitted only on one carrier/cell; 3. POs/ROs is configured in a time-domain-clustering manner on multiple carriers/cells to enable longer network DTX/DRX off-duration.

Proposal 6: RAN2 to discuss the functional framework for IDLE/INACTIVE mode operation in multi-carrier deployment scenario:

- SSB/SIB: Define anchor/non-anchor carrier/cell as:

- Anchor carrier/cell: a carrier/cell where the UE assumes SSB/SIB are transmitted.

- Non-anchor carrier/cell: a carrier/cell where the UE does not assume SIB is transmitted. FFS whether the UE assumes the SSB is transmitted or not.

- Paging: The UE monitors paging only on one carrier/cell within the multi-carrier deployment scenario, FFS this carrier/cell is the anchor carrier/cell or can be one of the non-anchor carrier/cell.

- RACH: The UE can perform RACH on one of the multiple carriers/cells on which RO is configured. FFS how the UE selects the RACH carrier, and how the corresponding RACH procedure is designed.

- Reference

- RAN2#131bis meeting minutes

- R2-2506799 Considerations on 6GR Control Plane vivo

- TR 38.864 Study on network energy savings for NR